IEEE P802.11
Wireless LANs

| Text Proposal for MDA Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-April 13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Juan Carlos Zuniga | InterDigital Communications | 1000, rue Sherebrook, Ouest, 10e étage, Montreal, QC, H3A 3G4 | +1514-904-6251 | j.c.zuniga@ieee.org |
| Guido R. Hiertz | Philips | ComNets, RWTH Aachen University, Kopernikusstr. 16, 52074 Aachen, Germany | +49-241-802-5829 | hiertz@ieee.org |
| Dee Denteneer | Philips | Philips Research, HTC 27 (WL 1.132), 5656 AE Eindhoven, The Netherlands | +31-402-746-937 | dee.denteneer@philips.com |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |
| Jarkko Kneckt | Nokia | Itämerenkatu 11-13 00180 Helsinki, Finland | +358504821550 | Jarkko.Kneckt@nokia.com |
| Donald Eastlake 3rd | Motorola | 111 Lock Drive Marlboro, MA 01752 USA | +1-508-786-7554 | donald.eastlake@motorola.com |

Abstract

This document proposes resolutions to the MDA Setup request comments: CID 15, 572, 1076, 1522, 5648, 4653, 527

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Problem Statement

This text proposal solves several comments received in the first WG Letter Ballot on MDA mechanism frames and definitions.

Proposed Solution

The following text provides a counter-proposal to the following comments: CID 15, 572, 1076, 1522, 5648, 4653, 527 in accordance with the agreements achieved during the 802.11s ad-hoc meeting of April, 2007 in Eindhoven NL.

In page 37, after line 20, replace section 7.3.2.65 by the following:

7.3.2.65 MDAOP Setup Request element

The MDAOP Setup Request information element is used by an MP to request the setup of a set of MDAOPs, identified by a single MDAOP Set ID, between itself (transmitter) and a receiver. This information element is transmitted in individually addressed MDA action frames. The format of the information element is as shown in Figure s36.

| Octets: 1 | 1 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- |
| Element ID | Length | MDAOP Duration | MDAOP Periodicity | MDAOP Offset |

The Element ID is set to the value given in Table 26 for this information element. The length is set to 10 octets.

The MDAOP Duration field specifies the duration of the MDAOP in multiples of 32 μs.

The MDAOP Periodicity field specifies the number of subinterval periods by which the Mesh DTIM interval is divided.

The MDAOP Offset field specifies the position of an MDAOP beginning from the beginning of the Mesh DTIM interval and subsequent subinterval periods within the Mesh DTIM interval. The value is specified in multiples of 32 μs.

An example of periodicity, duration, and offset values for a periodic MDAOP Info field is shown in

Figure s39. In this particular example, the periodicity equals four, so that there are four subintervals within the mesh DTIM interval. As further illustrated in the figure, the offset value indicates the start of the MDAOP relative to the start of the subintervals.

Mesh DTIM Interval

Periodicity = 4 (equal size subintervals)

Offset

Duration

= MDAOPs

Subinterval

In page 40, after line 28, replace the text paragraph and figure s43 by the following:

The format of the TX-RX Times and the Interfering Times Report field is shown in Figure s43. The fields involved are similar to the fields involved in the MDAOP Setup Request element. While the fields are the same as in an MDAOP setup request element, the TX-RX times and Interfering times reports can more efficiently report information compared to MDAOP setup request IEs. This is possible because different MDAOPs may be combined in an efficient way and reported. MDAOP Set IDs are not reported in the advertisements.

| Octets: 1 | 4 |  | 4 |
| --- | --- | --- | --- |
| Number of MDAOPs | MDAOP 1 | … | MDAOP n |