IEEE P802.11
Wireless LANs

| PANN Element with Generalized Metric |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Bahr | Siemens AG, CT | Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |

Abstract

This document provides changes to the current PANN element definition (Portal Announcement) that allow to use any arbitrary path selection metric in the PANN element. This extension is necessary in order to work with the extensibility framework of IEEE 802.11s.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Note to the editor: Change entry for Portal Announcement in Table 26 in Clause 7.3.2 as below:

- Information elements

| - Element IDs |  |  |
| --- | --- | --- |
| Information element | Element ID | Length (in octets) |
| Portal Announcement (PANN) | <ABSB> (see note below) | 16 to 255 |

Note to the editor: Change clause 7.3.2.71 according to the following text:

- PANN information element

The Portal Announcement (PANN) element is used for announcing in the mesh the presence of an MP configured as a Portal MP (that has a live connection to an external network). MPs may use this information to increase the efficiency of communication with stations outside the mesh.

The format of the PANN element is shown in PANN  element.

| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | L |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Flags | Hopcount | Time to Live | Originator Address | Sequence Number | Metric |  |  |
| - PANN element |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Element IDs for this information element. The length is set to 13+7.

The Flags field is reserved for future use.

The Hop Count field indicates the number of hops from the originator to the MP transmitting the request.

The Time to Live field indicates the maximum number of hops allowed for this element.

The Originator Address is set to the MAC address of the MP that is collocated with the portal.

The Sequence Number field is set to a sequence number specific for the originator.

The Metric field indicates a cumulative metric from the originator to the MP transmitting the announcement. The length L of the Metric field is taken from dot11spathselectionmetriclength which is set by the active path selection metric. The Metric field is 4 octets long if the default path selection metric is the active path selection metric (Airtime Link Metric, Clause 11A.5).

Detailed usage of the PANN element is described in 11A.4.

Change clause 11A.4.2 according to the following text:

- MPP announcement protocol

This subclause describes the function, generation and processing of the Portal Announcement (PANN).

Enabling the announcement protocol in a given MPP is beyond the scope of this standard.

- Function

The Portal Announcement (PANN) element, described in 7.3.2.71, is used to announce the presence of an MP configured as an MPP in the mesh. Portal Announcements allow MPs to select the appropriate MPP. An MP which receives a Portal Announcement message shall propagate the Portal Announcement as described below but may ignore its content.

The MPP sequence number of the Portal Announcement is incremented by the MPP before each transmission.

- Conditions for generating and sending a PANN

An MP shall transmit a PANN in the following cases:

Case A: Original transmission

All of the following applies:

- The MP is configured as an MPP

- At every PORTAL_ANNOUNCEMENT_INTERVAL

The content of a PANN element in Case A shall be as shown in Content of a PANN element in Case A.

| - Content of a PANN element in Case A |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 26 for the PANN element |
| Length | As required |
| Flags | Not used |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for the Portal Announcement |
| Address | MPP MAC address |
| Sequence Number | A sequence number specific for the MPP |
| Metric | initial value of active path selection metric (0 for default path selection metric, the airtime path metric) |

Case B:	Forwarding

All of the following applies:

- The MP has received a Portal Announcement

- PORTAL_PROPAGATION_DELAY has expired

- The decremented TTL of the Portal Announcement is equal to or greater than 1

The content of a PANN element in Case B shall be as shown in Content of a PANN element in Case B.

| - Content of a PANN element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Table 26 for the PANN element |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Originator Address | As received |
| Sequence Number | As received |
| Metric | As received  own metric towards the transmitting MP |

- PANN processing

A received Portal Announcement is subject to certain acceptance criteria. Processing depends on the contents of the Portal Announcement and the information available at the receiving MP.

- Acceptance criteria

The Portal Announcement will be discarded if

- the Sequence Number of the Portal Announcement is lower than the Sequence Number of the previously received Portal Announcement from this MPP, or

- the Sequence Number of the Portal Announcement is the same as the Sequence Number of the previously received Portal Announcement from this MPP, and the metric is worse that the metric of the previously received Portal Announcement from this MPP

- Effect of receipt

The following applies only to Portal Announcements that were not discarded during the check described in Acceptance criteria (Acceptance criteria) above.

- The receiving MP shall initiate a PANN_PROPAGATION_DELAY

- The receiving MP shall transmit a Portal Announcement as defined in Conditions for generating and sending a PANN (Conditions for generating and sending a PANN), Case B