IEEE P802.11
Wireless LANs

| TGn LB97 Submission for Miscellaneous PHY CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

2642, 618, 1293, 2662, 2669, 1903, 624, 1244, 2908, 733, 2675, 625, 2680, 182, 180, 1916, 1615, 1617, 1616, 1618, 1619, 1622, 1621, 1624, 1623, 1625, 2752, 2753, 2754, 2953, 3199, 1807, 2755, 2756, 2954, 749, 750, 507, 751, 752, 2759, 2760, 2761, 2762, 2763, 2766, 2767, 2916, 2917, 2769, 2770, 2771, 756, 757, 2776

The changes marked in this document are based on TGn Draft version P802 11n D2.0.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

| CID | Page | Line | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 2642 | 214.13 | 13 | 20.3.1 | "During transmission, the PSDU shall be appended to the PLCP preamble to create the PPDU" There's quite a lot of processing. I'm not sure "appended" captures this Also the normative "shall" is unnecessary here. | Reword: "During transmission, the PSDU is processed (i.e., scrambled and coded) and appended to the PLCP preamble to create the PPDU" | Accept: Refer to 07/0554 |
| 618 | 214.27 | 27 | 20.3.1 | It is not clear what is meant by "supports the SERVICE field" here? | clarification requested. | Counter: Refer to 07/0554 |
| 1293 | 230.59 | 59 | 20.3.2 | Missing information | Add HT-GF-STF and HT-LTF1 to the list of elements in HT packets | Counter: Refer to 07/0554 |
| 2662 | 232.12 | 12 | 20.3.3 | "scrambles the data to prevent long sequences of zeros or ones" Actually, it doesn't prevent them, because the scrambler is a 1:1 mapping of an input to an output, there will exist an input that results in all zeroes or all ones. | Reword: "scrambles the data to reduce the probability of long sequences of zeros or ones" | Accept: Refer to 07/0554 |
| 2669 | 232.63 | 63 | 20.3.3 | "Guard interval (GI) insertion: inserts the guard interval." Hmmm. Really helpful. | Give a little hint about what this entails or what the benefits are, or remove the useless "inserts the guard interval". | Counter: Refer to 07/0554 |
| 1903 | 233.01 | 1 | 20.3.3 | The non-HT portion and the HT signal field are not scrambled | remove scrambler from figure n63 | Accept: Refer to 07/0554 |
| 624 | 234.00 |  | 20.3.3 | The CSD operation is described after the IDFT in page 232, but in the Figure n64, it is shown before spatial mapping. | The Figure or the text in page 232 need to be modified. | Counter: Refer to 07/0554 |
| 1244 | 234.44 | 44 | 20.3.3 | EVM is undefined | spell it out. Provide a cross reference if it is defined somewhere in this standard | Counter: Refer to 07/0554 |
| 2908 | 234.56 | 56 | 20.3.4 | In a) PLCP Preamble also depends on CH_BANDWIDTH | Add CH_BANDWIDTH | Counter: Refer to 07/0554 |
| 733 | 235.30 | 30 | 20.3.4 | In step e) of the PPDU encoding process the zero padding (not tail bits) ignores the problem of even number of symbols in STBC) | Change " if necessary, the bit string is furhter extended with zero bits so that the resulting length is a multiple of N_DBPS," to "the number of symbols, N_sym, is calculated accroding to formula 20-32 and if necessary the bit string is extended with zero bits so that the length of the resulting string is N_sym X N_DBPS," | Accept: Refer to 07/0554 |
| 2675 | 235.38 | 38 | 20.3.4 | We have a structural problem with this description that e) talks about how to do LDPC encoding, and it occurs before f) which talks about scrambling. However this is derriere in front of arriere. | In e) describe how to calculate the length, and only that. Or describe the coding before point e), then e) can refer to the resultant length of the coding step. | Reject: Refer to 07/0554 |
| 625 | 236.00 |  | 20.3.4 | The CSD operation is not mentioned in step (r ). The CSD can be done in time samples after the IDFT. | Mention the CSD operation at suitable step. | Counter: Refer to 07/0554 |
| 2680 | 238.20 | 20 | 20.3.5 | "support of 400 ns guard interval," This is not an MCS parameter. Not all the other optional parameters (e.g. LDPC coding) are called out here, which is inconsistent. | Either remove any mention of guard interval here, or list all the PHY options and call them out as options. | Reject: Refer to 07/0554 |
| 182 | 263.00 |  | 20.3.10.2 | Scrambler is the weakest part of the system since it is transmitted at same rate as payload. We should use something more robust. | Replace with a self-synchronizing scrambler | Reject: Refer to 07/0554 |
| 180 | 292.50 | 50 | 20.3.14.2 | Allow only 20 MHz distant channels in 5GHz | Indicate valid operating channel numbers by reference to 17.3.8.3.3 or an adequate statement | Counter: Refer to 07/0554 |
| 1916 | 292.50 | 50 | 20.3.14.2 | Allow only 20 MHz distant channels in the 5GHz band | Indicate valid operating channel numbers by reference to 17.3.8.3.3 or an appropriate statement | Counter: Refer to 07/0554 |
| 1615 | 298.20 | 20 | 20.3.21.1 | There is a description of "The packet error rate (PER) shall be less than 10% for a PSDU length of 4096 octets with the rate-dependent input level listed in Table n75 or less." But it is strange expression. | Replace quoted sentence with; "The packet error rate (PER) shall be less than 10% for a PSDU length of 4096 octets with the rate-dependent input level listed in Table n75" [remove "or less"] or, "Increasing the input power, and the input power shall be less than or equal to the rate-dependent input level listed in Table n75 when the packet error rate (PER) becomes 10% for a PSDU length of 4096 octets." | Reject: Refer to 07/0554 |
| 1617 | 298.23 | 23 | 20.3.21.1 | There is a description of "The number of spatial streams under test shall be equal to the number of utilized transmitting STA antenna (output) port and also equal to the number of utilized Device Under Test input port." This is not possible for the STA that has 3 Rx antennas but support up to 2 spatial streams. | Specify how to treat such cases. I'm fine with using 2 Rx antennas, and the rest is disabled. | Reject: Refer to 07/0554 |
| 1616 | 298.23 | 23 | 20.3.21.1 | There is a description of "The number of spatial streams under test shall be equal to the number of utilized transmitting STA antenna (output) port and also equal to the number of utilized Device Under Test input port." This is not possible for STBC, for example, one Rx antenna supporting one spatial stream, but 2 Tx for 2x1 STBC. However, I don't think we should test STBC performance, especially for AWGN channel. | So, add explanations that this minimum input sensitivity test shall be applied for non-STBC case. | Accept: Refer to 07/0554 |
| 1618 | 298.31 | 31 | 20.3.21.1 | According to the Table n75, we cannot measure sensitivity for unequal modulation (MCS>32). On the other hand, I believe that MCSs using unequal modulation are not necessary to measure sensitivity. | Add statement that this sensitivity test shall be done only for MCSs using equal modulation. | Accept: Refer to 07/0554 |
| 1619 | 298.31 | 31 | 20.3.21.1 | There is no description in the case of using LDPC. | Add explanation that this test only uses BCC. Or, add sensitivity level table for PSDUs using LDPC. | Accept: Refer to 07/0554 |
| 1622 | 298.59 | 59 | 20.3.21.2 | Similar to the sensitivity test, this would work only for equal modulation, non-STBC, N_RX=N_STS, and/or maybe BCC. | Explicitly state that equal modulation, non-STBC, and BCC shall be used for this test, and state how to treat N_RX>N_STS case. (N_RX>N_STS means that the STA cannot support the number of spatial streams up to the number of receiving antennas.) | Counter: Refer to 07/0554 |
| 1621 | 298.59 | 59 | 20.3.21.2 | In this section, specifications are given by 17.3.10.2 in the 5GHz band or 19.5.2 in the 2.4GHz band for all transmissions in 20MHz channel width, or Table n75 for 40MHz. But this doesn't work, because we don't have BPSK r=3/4, and we have new 64QAM r=5/6. Why can't we use table n75 in any cases ? | Replace "The adjacent channel rejection shall follow 17.3.10.2 in the 5GHz band or 19.5.2 in the 2.4GHz band for all transmissions in 20MHz channel width with exception that 10% PER is required for 4096 octets packets rather than 10% PER for 1000 octets packets." with "For all trasmission in a 20MHz transmission channel width, the adjacent channel rejection shall be measured by setting the desired signal's streangth 3dB above the rate dependent sensitivity specified in Table n75 and raising the power of the interfering signal until 10% PER is caused for a PSDU length of 4096 octets. The power difference between the interfering and the desired channel is the corresponding adjacent channel rejection. The adjacent channel center frequencies shall be separated by 20MHz." And then, add line break between "....separated by 40MHz" and "The interfering signal in the..." on line-2 on page-299 to be read that the sentence after "The interfering signal in the..." shall be applied for the both of 20Mhz and 40Mhz. | Counter: Refer to 07/0554 |
| 1624 | 299.13 | 13 | 20.3.21.3 | Similar to the sensitivity test, this would work only for equal modulation, non-STBC, N_RX=N_STS, and/or maybe BCC. | Explicitly state that equal modulation, non-STBC, and BCC shall be used for this test, and state how to treat N_RX>N_STS case. (N_RX>N_STS means that the STA cannot support the number of spatial streams up to the number of receiving antennas.) | Counter: Refer to 07/0554 |
| 1623 | 299.13 | 13 | 20.3.21.3 | In this section, specifications are given by 17.3.10.3 in the 5GHz band for all transmissions in 20MHz channel width, or Table n75 for 40MHz. But this doesn't work, because we don't have BPSK r=3/4, and we have new 64QAM r=5/6. Why can't we use table n75 in any cases ? | Replace "The non-adjacent channel rejection shall follow 17.3.10.3 in the 5GHz band for all transmissions in 20MHz channel width with exception that 10% PER is required for 4096 octets packets rather than 10% PER for 1000 octets packets." with "For all trasmission in a 20MHz transmission channel width in the 5GHz band, the non-adjacent channel rejection shall be measured by setting the desired signal's streangth 3dB above the rate dependent sensitivity specified in Table n75 and raising the power of the interfering signal until 10% PER occurs for a PSDU length of 4096 octets. The power difference between the interfering and the desired channel is the corresponding non-adjacent channel rejection. The non-adjacent channel center frequencies shall be separated by 40MHz or more." And then, add line break between "....separated by 80MHz or more" and "The interfering signal in the..." on line-21 on page-299 to be read that the sentence after "The interfering signal in the..." shall be applied for the both of 20Mhz and 40Mhz. | Accept: Refer to 07/0554 |
| 1625 | 299.15 | 15 | 20.3.21.3 | For 40MHz channel bandwidth transmission, there is no explicit description in the 2.4GHz and/or in the 5GHz band, though the 20MHz is only for 5GHz. | Please clarify. | Counter: Refer to 07/0554 |
| 2752 | 301.14 | 14 | 20.3.22 | "Further, the PHY shall be set to operate at the appropriate frequency through station management via the PLME." Call out the mechanisms. "appropriate" makes it sound like the author of this sentence didn't know, or couldn't be bothered to check what these mechanisms were. | Add detail. | Counter: Refer to 07/0554 |
| 2753 | 301.20 | 20 | 20.3.22 | "A clear channel shall be indicated by PHY-CCA.indication(IDLE). The MAC considers this indication before issuing the PHY-TXSTART.request." This is unnecessary and wrong. The MAC considers this indication before some transmissions but not others. | Remove the quoted text. | Counter: Refer to 07/0554 |
| 2754 | 301.30 | 30 | 20.3.22 | "PMD_TX_PARAMETERS PMD_EXPANSIONS_MAT" Why call out expansions mat at this point? | Make expansions mat part of TX_Parameters. | Accept: Refer to 07/0554 |
| 2953 | 301.33 | 33 | 20.3.22 | Not sure if this is wrong notation or not. PMD_EXPANSIONS_MAT_ON is not defined any where in the standard. | Change this to PMD_EXPANSIONS_MAT_TYPE | Counter: Refer to 07/0554 |
| 3199 | 301.33 | 33 | 20.3.22 | PMD_EXPANSION_MAT_ON seems not be correct. To match the Fig. n81 and n82 PMD_DATA.request should be used. | Replace PMD_EXPANSION_MAT_ON with PMD_DATA.request | Counter: Refer to 07/0554 |
| 1807 | 301.33 | 33 | 20.3.22 | I think with the new definition of EXPANSIONS_MAT in D2.0 vs D1.0, we do not need PMD_EXPANSIONS_MAT_ON | remove PMD_EXPANSIONS_MAT_ON | Accept: Refer to 07/0554 |
| 2755 | 301.52 | 52 | 20.3.22 | "PHYTXSTART shall be disabled by the issuance of the PHY-TXEND.request." What the heck does this mean? It's a normative requirement. Does it mean that after any TXEND request, the PHY never transmits another packet? (i.e. disposable radios :0). | Reword to something meaningfull or delete. | Reject: Refer to 07/0554 |
| 2756 | 301.57 | 57 | 20.3.22 | "The packet transmission shall be completed and the PHY entity shall enter the receive state (i.e., PHYTXSTART shall be disabled)." Incomplete, doesn't say under what conditions this occcurs. | Indicate when this occurs. | Reject: Refer to 07/0554 |
| 2954 | 302.00 |  | 20.3.22 | This figure does not show PMD_EXPANSIONS_MAT_ON. | Show either PMD_EXPANSIONS_MAT_ON in the figure. If this is incorrect then show PMD_EXPANSIONS_MAT_TYPE. | Counter: Refer to 07/0554 |
| 749 | 302.29 | 29 | 20.3.22 | QBSPK is mentioned in the figure (n81) but it is never defined | Replace by either rotated BPSK or BPSK | Accept: Refer to 07/0554 |
| 750 | 303.27 | 27 | 20.3.22 | QBSPK is mentioned in the figure (n82) but it is never defined | Replace by either rotated BPSK or BPSK | Accept: Refer to 07/0554 |
| 507 | 304.01 | 1 | 20.3.22 | Figure n83 second column top box outputs a PHY-data.confirm, before a PHY-DATA.request is even made. That is a PHY-DATA.request can only be sent after receiving the PHY-TXSTART.confirm and a PHY-DATA.confirm cannot be recevied without first sending a PHY | Add missing first PHY-DATA.request after the last box in the first column and before the first box on in the second column. | Accept: Refer to 07/0554 |
| 751 | 304.20 | 20 | 20.3.22 | QBSPK is mentioned in the figure (n82) but it is never defined | Replace by either rotated BPSK or BPSK | Accept: Refer to 07/0554 |
| 752 | 304.25 | 25 | 20.3.22 | QBSPK is mentioned in the figure (n82) but it is never defined | Replace by either rotated BPSK or BPSK | Accept: Refer to 07/0554 |
| 2759 | 305.03 | 3 | 20.3.23 | "In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state." This is wrong. Firstly, there's no way the PHY can know in advance when a receive will occur. Secondly, there's no interface to allow the primitive to be disabled. Thirdly, you actually want to abort ongoing Rx if the MAC decides to start transmission at a particular time. An example is PCO, at the start of the uplink, although it may also be seen for regular data/ack, because we guarantee that TGn radios can acquire quicker than SIFS (i.e. RIFS). | Remove the quoted sentence. | Accept: Refer to 07/0554 |
| 2760 | 305.04 | 4 | 20.3.23 | "Further, through station management (via the PLME) the PHY is set to the appropriate frequency." This is not very informative. | Give details or remove. | Counter: Refer to 07/0554 |
| 2761 | 305.09 | 9 | 20.3.23 | "Upon receiving the transmitted PLCP preamble, PMD_RSSI.indication shall report a busy channel to the PLCP." "upon" is ambiguous. Does it mean the start or the end? How long is the preamble (i.e. which subfilelds are included). What is the timing constraint. | Indicate where, relative to the start of the premable the signal must be indicated. | Reject: Refer to 07/0554 |
| 2762 | 305.12 | 12 | 20.3.23 | "PHY-CCA.indication(BUSY) shall be issued for reception of a signal prior to correct reception of the PLCP frame." Duh? This is meaningless. | Turn into something meaningfull or delete. | Counter: Refer to 07/0554 |
| 2763 | 305.13 | 13 | 20.3.23 | "The PMD primitive PMD_RSSI is issued to update the RSSI and parameter reported to the MAC." Under what condition, and when is this issued. | Add details or delete. | Reject: Refer to 07/0554 |
| 2766 | 305.30 | 30 | 20.3.23 | "for all supported and unsupported modes except Reserved HT-SIG Indication." Ambiguous. Please relate to values of individual fields of the signal fields have specified values. | Add detail as described or refere to the later paragraph that defines this. | Counter: Refer to 07/0554 |
| 2767 | 305.34 | 34 | 20.3.23 | "except Reserved HT-SIG Indication," - ambiguous | Refer to the definition in the later paragraph. | Counter: Refer to 07/0554 |
| 2916 | 305.42 | 42 | 20.3.23 | When using the MM preamble, there is a better forward compatibility approach for Reserved HT-SIG Indication, namely, using falling back to the length in the L-SIG as an indication for the length to defer. | Introduce the suggested approach | Reject: Refer to 07/0554 |
| 2917 | 305.52 | 52 | 20.3.23 | ", and the PHY shall issue the error condition PHY-RXEND.indication(FormatViolation)", remove as this is in vialation with what has been said above by "Upon reception of a GF preamble by an HT STA which does not support GF, PHY-CCA.indication( BUSY) shall be maintained until either the predicted duration of the packet from the contents of the HT-SIG field, as per TXTIME in 21.4.3," | as suggested | Reject: Refer to 07/0554 |
| 2769 | 305.54 | 54 | 20.3.23 | "If the HT-SIG is not completely recognizable and supported the PHY shall issue the error condition PHY-RXEND.indication(UnsupportedRate)." What does "completely recognizable" mean | Reword to avoid using this term. | Counter: Refer to 07/0554 |
| 2770 | 305.57 | 57 | 20.3.23 | "Following training and signal fields, the PLCP SERVICE field and PSDU shall be received." This is misleading because it is the coded entity that is received. | reword: "Following training and signal fields, the coded PSDU (C-PSDU) (which comprises the coded PLCP SERVICE field and scrambled and coded PSDU) shall be received." | Accept: Refer to 07/0554 |
| 2771 | 306.02 | 2 | 20.3.23 | "After the reception of the final bit of the last PSDU octet the receiver shall be returned to the RX IDLE state," Is it possible to get a final symbol containing only the tail bits (plus padding)? If so this statement is in error. | Include effect of tail bits in the above statement, if necessary. | Accept: Refer to 07/0554 |
| 2773 | 306.06 | 6 | 20.3.23 | I have a general problem with the description of the PMD that understands the formatting of the PLCP C-PSDU. IMHO, if we define an abstraction (i.e. C-PSDU), we should define how to construct a C-PSDU in the PLCP, and how to transmit one in the PMD. | Restructure so that the procedures describe how to transmit and receive a C-PSDU. and add the mapping of length in the LENGTH parameter to the length of the C-PSDU. |  |
| 2775 | 306.31 | 31 | 20.3.23 | The receive procedures do not match the architecture. The PLCP defines the encoding of the signal fields. The PLCP needs to decode these fields in order to determine the demodulation of subsequent data symbols. However, there is no primitive in this diagram, nor in the PMD_SAP that allows it to do this. Further, detection of HT_MF or NON-ht depends on decoding the rotation of the HT-SIG. There is no signal that provides this information. | Add a primitive whereby the PLCP can set up the PMD to receive symbols after the signal field. Add a primitive that allows the PLCP to sense significant acquisition events that determine the packet format. Redraw the rx procedures to show these primitives in use. |  |
| 756 | 306.34 | 34 | 20.3.23 | QBSPK is mentioned in the figure (n84) but it is never defined | Replace by either rotated BPSK or BPSK | Accept: Refer to 07/0554 |
| 757 | 307.27 | 27 | 20.3.23 | QBSPK is mentioned in the figure (n85) but it is never defined | Replace by either rotated BPSK or BPSK | Accept: Refer to 07/0554 |
| 2776 | 308.20 | 20 | 20.3.23 | "Not HT-SIG: Refer to clause 17 or 19" I'm not sure that we can have a "refer to" jump. It's the equivalent of "go somewhere else", which, although a staple of my programming days occasionally let to suboptimal results. | Indicate where in clause 17 or 19. This might be done by indicating which box in those clauses. We could even edit the figures in clause 17 or 19 to provide a connector to reference here. Or provide text (easier) after the figure that defines the precise meaning of the connection. | Counter: Refer to 07/0554 |

# Comment Group: Scrambler

CID 182

Proposed Resolution: Reject

Reason for rejection: The simulated performance results in the submission 06/1368, which compared the scrambler in the draft with a genie-aided scrambler, show no significant degradation in performance due to scrambler currently in draft.

# Comment Group: Channelization

CID 180, 1916

Discussion:  In subclause 20.3.14.2, the comments request that the channels are limited to 20 MHz spacing.  They suggest adding a reference to 17.3.8.3.3 or an adequate statement.  This subclause only addresses channel numbering for 40 MHz.  At the beginning of 20.3.14 we already reference 17.3.8.3 for 20 MHz channels.  A reference to Annex J in 20.3.14 will be added for extra clarity.

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 292, line 15 add second paragraph as follows

The set of valid operating channel numbers by regulatory domain is defined in Annex J.

# Comment Group: PLCP

CID 2642

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 214, line 13, modify text as follows

During transmission, the PSDU shall be is processed (i.e., scrambled and coded) and appended to the PLCP preamble to create the PPDU.

CID 618

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 214, line 26, modify text as follows

The HT portion of the HT mixed format preamble also consists of the HT-SIG field that supports HT operation., and tThe SERVICE field is prepended to the PSDU.

CID 1293

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 230, line 59, modify text as follows

The HT-SIG, HT-STF, HT-GF-STF, HT-LTF1, and HT-LTFs exist only in HT packets.

CID 2662

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 232, line 13, modify text as follows

Scrambler: scrambles the data to prevent reduce the probability of long sequences of zeros or ones, see 20.3.10.2 (Scrambler).

CID 2669

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 232, line 63, modify text as follows

Guard interval (GI) insertion: inserts the guard interval prepend to the symbol a circular extension of itself.

CID 1903

Proposed Resolution: Accept

TGn Editor: In D2.0, Figure n63, Remove the scrambler block

CID 624

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 232, line 47, modify text as follows

Cyclic shift (CSD) insertion: insertion of the cyclic shifts prevents unintentional beamforming.  CSD insertion may occur before or after the IDFT.

CID 1244

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 234, line 43, modify text as follows

Different implementations are possible as long as they are equivalent within meet the EVM requirements (see 20.3.20.7.4).

CID 2908

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 234, line 57, modify text as follows

based on the FORMAT, NUM_EXTEN_SS, CH_BANDWIDTH, and MCS fields of the TXVECTOR.

CID 733

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 235, line 30, modify text as follows

The number of symbols, NSYM, is calculated according to Equation (20-32) and iIf necessary, the bit string is further extended with zero bits so that the resulting length is a multiple of NSYM x NDBPS, as described in 20.3.10.

CID 2675

Proposed Resolution: Reject

Reason for rejection: Step (e) discusses putting Service, PSDU, and tail bits together.  How many tail bits and the type of tails bits (zeros or repeated coded bits) is dependent on type of coding (BCC or LDPC) and STBC.  Describing coding before this step would be out of sequence.

CID 625

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 237, line 25, add following sentence at the end of step (r)

When beamforming is not used it is sometimes possible to implement the cyclic shifts in the time domain.

CID 2680

Proposed Resolution: Reject

Reason for rejection: 20.3.5 goes beyond MCS and describes the rate dependent parameters, which includes the guard interval (and 40 MHz as well).

CID 2752

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 301, line 14, modify text as follows

Further, the PHY shall be set to operate at the appropriate frequency through station management via the PLME, refer to 20.4.

CID 2753

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 301, line 20, modify text as follows

A clear channel shall be indicated by PHY-CCA.indication(IDLE). Note - under some circumstances, Tthe MAC uses the latest value of this indication considers this indication before issuing the PHY-TXSTART.request.

CID 2754, 2953, 3199, 1807, 2954

Accept: 2754, 1807

Counter: 2953, 3199, 2954

TGn Editor: In D2.0, pg 301, lines 31-33, delete as follows

— PMD_EXPANSIONS_MAT

— PMD_EXPANSIONS_MAT_ON

TGn Editor: In D2.0, pg 302, Figure n81, delete PMD_ EXPANSIONS_MAT.request

TGn Editor: In D2.0, pg 303, Figure n82, delete PMD_ EXPANSIONS_MAT.request

TGn Editor: In D2.0, pg 319, Table n79, delete rows for PMD_EXPANSIONS_MAT  and PMD_EXPANSIONS_MAT_ON

TGn Editor: In D2.0, pg 320, Table n80, modify the text as follows

| EXPANSION_MAT | PMD_EXPANSIONS_MAT.request PMD_TX_PARAMETERS.request | (NSD +NSP) complex matrices of size NTX ×N STS |
| --- | --- | --- |

TGn Editor: In D2.0, pg 320, Table n80, add row to table as follows

| EXPANSION_MAT_TYPE | PMD_TX_PARAMETERS.request | COMPRESSED_SV: EXPANSION_MAT contains a set of compressed beamforming matrices NON_COMPRESSED_SV: EXPANSION_MAT contains a set of non-compressed beamforming matrices CSI_MATRICES: EXPANSION_MAT contains a set of CSI matrices |
| --- | --- | --- |

TGn Editor: In D2.0, pg 325, line 9, modify the text as follows

PMD_TX_PARAMETERS.request (MCS, CH_BANDWIDTH, CH_OFFSET, STBC, SHORT_GI, ANTENNA_SET, PMD_EXPANSIONS_MAT, PMD_EXPANSIONS_MAT_TYPE)

TGn Editor: In D2.0, pg 325, delete subclauses 20.5.5.10, 20.5.5.10.1, 20.5.5.10.2, 20.5.5.10.3, and 20.5.5.10.4

CID 2755

Proposed Resolution: Reject

Reason for rejection: There is precedence for this sentence in 17.3.11.  It means that the PHY-TXSTART command that was issued can be terminated by a PHY-TXEND.request, as per previous sentence.

CID 2756

Proposed Resolution: Reject

Reason for rejection: There is precedence for this sentence in 17.3.11.

CID 749, 750, 751, 752, 756, 757

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 302-307, in Figures n81, n82, n83 (occurs twice in n83), n84, and n85  replace “QBPSK” with “rotated BPSK”

CID 507

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 304, modify Figures n83 as given by the yellow highlights below

CID 2759

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 305, line 3, delete text as follows

In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state.

CID 2760

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 304, line 4, modify text as follows

Further, through station management (via the PLME) the PHY is set to the appropriate frequency, refer to 20.4.

CID 2761

Proposed Resolution: Reject

Reason for rejection: There is precedence for this sentence in 17.3.12.  The time constraint is given in 20.3.21.5.

CID 2762

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 305, line 12, modify text as follows

PHY-CCA.indication(BUSY) shall also be issued as in initial indication of for reception of a signal prior to correct reception of the PLCP frame.

CID 2763

Proposed Resolution: Reject

Reason for rejection: There is precedence for this sentence in 17.3.12.  PMD_RSSI is issued under the conditions stated in the preceding sentences of the same paragraph.

CID 2766, 2767

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 305, lines 28-41, modify text as follows

— Upon reception of an HT mixed format preamble, the HT PHY shall maintain PHY-CCA.indication(BUSY) for the predicted duration of the transmitted frame, as per TXTIME in 21.4.3, for all supported and unsupported modes except Reserved HT-SIG Indication.  Reserved HT-SIG Indication is defined in the fourth bullet below.

— Upon reception of a GF preamble by an HT STA which does not support GF, PHY-CCA.indication(BUSY) shall be maintained until either the predicted duration of the packet from the contents ofthe HT-SIG field, as per TXTIME in 21.4.3, except Reserved HT-SIG Indication, or until thereceived level drops below the receiver minimum sensitivity level of BPSK, R=1/2 in Table n75 (Receiver minimum input level sensitivity) + 10 dB (-72 dBm for 20 MHz, -69 dBm for 40 MHz). Reserved HT-SIG Indication is defined in the fourth bullet below.

— Upon reception of a GF preamble by an HT STA which supports GF, the HT PHY shall maintain PHY-CCA.indication(BUSY) for the predicted duration of the transmitted frame, as per TXTIME in 21.4.3, for all supported and unsupported modes except Reserved HT-SIG Indication.  Reserved HT-SIG Indication is defined in the fourth bullet below.

CID 2916

Proposed Resolution: Reject

Reason for rejection: There are two issues with the proposal: (1) We have equated a Reserved HT-SIG Indication as an error in HT-SIG for both GF and MM for consistency.  An error in HT-SIG would almost guarantee an error in L-SIG.  (2) L-SIG TXOP results in the L-SIG length not necessarily equaling the length of the packet.

CID 2917

Proposed Resolution: Reject

Reason for rejection: The PHY-SAP services PHY-CCA.indication and PHY-RXEND.indication are separate primitives.  PHY-RXEND is an indication by the PHY to the local MAC entity that the MPDU currently being received is complete, either with NoError, FormatViolation, CarrierLost, or UnsupportedRate.  Only if the indication is NoError, does the MAC use PHY-RXEND.indication for channel access timing.  PHY-CCA is the indication by which the PHY conveys to the MAC the current state of the medium.

CID 2769

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 305, lines 53, modify text as follows

If the HT-SIG indicates an unsupported mode or Reserved HT-SIG Indication is not completely recognizable and supported the PHY shall issue the error condition PHY-RXEND.indication(UnsupportedRate).

CID 2770

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 305, line 57, modify text as follows

Following training and signal fields, the PLCP SERVICE field and PSDU  coded PSDU (C-PSDU) (which comprises the coded PLCP SERVICE field and scrambled and coded PSDU) shall be received.

CID 2771

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 306, line 2, modify text as follows

After the reception of the final bit of the last PSDU octet, and possible tail and padding bits, the receiver shall be returned to the RX IDLE state, as shown in Figure n86 (PLCP receive state machine).

CID 2776

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 308, line 20, modify Figure n84 as follows

Not HT-SIG:

Refer to clause

17.3.12 or 19.3.6

# Comment Group: Receiver Spec

CID 1615

Proposed Resolution: Reject

Reason for rejection: There is precedence for this sentence structure in 17.3.10.1.  The intent of the sentence is to replace “the rate-dependent input level listed in Table n75” with the appropriate value.  For example for BPSK, R=1/2: The packet error rate (PER) shall be less than 10% for a PSDU length of 4096 octets with -82 dBm or less.

CID 1617

Proposed Resolution: Reject

Reason for rejection: The paragraph describes the DUT in terms of input ports.  Therefore with two spatial streams, two input ports will be utilized regardless of how many more receive antennas.

CID 1616, 1618, 1619

Proposed Resolution: Accept

TGn Editor: In D2.0, pg 298, line 26, add the following text to end of paragraph as follows

Minimum sensitivity level specified in Table n75 and test only applies to non-STBC modes, MCSs 0-31, and BCC.

CID 1622

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 299, line 6, add the following text as a new paragraph to end of subclause 20.3.31.2 as follows

Adjacent channel rejection level specified in Table n75 and test only applies to non-STBC modes, MCSs 0-31, and BCC.

CID 1621

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 298, line 59 – pg 299 line 6, modify the text as follows (and add line break to create new paragraph with the last three sentences)

The adjacent channel rejection shall follow 17.3.10.2 in the 5 GHz band or 19.5.2 in the 2.4 GHz band for all transmissions in a 20 MHz channel width with the exception that 10% PER is required for 4096 octet packets rather than 10% PER for 1000 octet packets. For all transmissions in a 20 MHz channel width, the adjacent channel rejection shall be measured by setting the desired signal's strength 3 dB above the rate dependent sensitivity specified in Table n75 (Receiver minimum input level sensitivity) and raising the power of the interfering signal until 10% PER is caused for a PSDU length of 4096 octets. The power difference between the interfering and the desired channel is the corresponding adjacent channel rejection. The adjacent channel center frequencies shall be separated by 20 MHz when operating in the 5 GHz band and the adjacent channel center frequencies shall be separated by 25 MHz when operating in the 2.4 GHz band.  For all transmissions in a 40 MHz channel width, the adjacent channel rejection shall be measured by setting the desired signal's strength 3 dB above the rate dependent sensitivity specified in Table n75 (Receiver minimum input level sensitivity) and raising the power of the interfering signal until 10% PER is caused for a PSDU length of 4096 octets. The power difference between the interfering and the desired channel is the corresponding adjacent channel rejection. The adjacent channel center frequencies shall be separated by 40 MHz.

The interfering signal in the adjacent channel shall be a conformant OFDM signal, unsynchronized with the signal in the channel under test. For a conformed OFDM PHY, the corresponding rejection shall be no less than specified in Table n75 (Receiver minimum input level sensitivity). The interference signal shall have a minimum duty cycle of 50%.

CID 1624

Proposed Resolution: Counter

TGn Editor: In D2.0, pg 299, line 27, add the following text as a new paragraph to end of subclause 20.3.31.3 as follows

Non-adjacent channel rejection level specified in Table n75 and test only applies to non-STBC modes, MCSs 0-31, and BCC.

CID 1623, 1625

Proposed Resolution: Accept 1623, Counter 1625

TGn Editor: In D2.0, pg 299 line 13-26, modify the text as follows (and add line break to create new paragraph with the last four sentences)

The non-adjacent channel rejection shall follow 17.3.10.3 in the 5 GHz band for all transmissions in a 20 MHz

channel width with the exception that 10% PER is required for 4096 octet packets rather than 10% PER for

1000 octet packet.  For all transmissions in a 20 MHz channel width in the 5 GHz band, the non-adjacent channel rejection shall be measured by setting the desired signal's strength 3 dB above the rate-dependent sensitivity specified in Table n75 (Receiver minimum input level sensitivity), and raising the power of the interfering signal until a 10% PER occurs for a PSDU length of 4096 octets. The power difference between the interfering and the desired channel is the corresponding non-adjacent channel rejection. The non-adjacent channel center frequencies

shall be separated by 40 MHz or more. For all transmissions in a 40 MHz channel width in the 5 GHz band, the non-adjacent channel rejection shall be measured by setting the desired signal's strength 3 dB above the rate-dependent sensitivity specified in Table n75 (Receiver minimum input level sensitivity), and raising the power of the interfering signal until a 10% PER occurs for a PSDU length of 4096 octets. The power difference between the interfering and the desired channel is the corresponding non-adjacent channel rejection. The non-adjacent channel center frequencies shall be separated by 80 MHz or more.

The interfering signal in the non-adjacent channel shall be a conformant OFDM signal, unsynchronized with the signal in the channel under test. For a conformed OFDM PHY, the corresponding rejection shall be no less than specified in Table n75 (Receiver minimum input level sensitivity).  The interference signal shall have a minimum duty cycle of 50%. The non-adjacent channel rejection for transmissions in a 20 MHz or 40 MHz channel width is applicable only to 5 GHz band.