IEEE P802.11
Wireless LANs

| TGn LB97 Submission for PHY GF CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-06-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

328, 1636, 1672, 2970, 1563, 3402, 3361, 3089, 609, 339, 2915, 347, 839, 1678

The changes marked in this document are based on TGn Draft version P802 11n D2.0.pdf.

R1: the only difference between R0 and R1 is the inclusion of CID 839.

R2: the only difference between R2 and R1 is the inclusion of CID 1678.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

| CID | Page | Line | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 328 | 212.00 |  | 20.1.3 | Support for greenfield is intended to be optional, the mandatory behavior specified here for detecting greenfield packets (eg. decode HT-SIG, determine CRC pass/fail) is requiring too much and contradicts meaning of being optional. | Remove requirements to decode HT-SIG and determining CRC pass/fail. | Reject: refer to 07/0555 |
| 1636 | 212.58 | 58 | 20.1.3 | "Support for HT greenfield format is optional". Greenfield preamble is more efficient than mixed mode preamble due to its shorter length. Greenfield and/or greentime deployments of 802.11n will not be able to leverage the benefits of a shorter preamble if it is not made mandatory. | Make greenfield preamble mandatory. | Reject: refer to 07/0555 |
| 1672 | 212.59 | 59 | 20.1.3 | The text that begins "An HT STA that does not support the reception of the HT" and ends "..and determine if the HT-SIG redundancy check (CRC) passes" doesn't make any sense. The primary difference between GF and MM is the preamble. A station that doesn't support GF will not support demodulating the preamble, so this statement could be simplified to mean "A station that cannot demodulate the GF preamble shall demodulate the preamble". | End the section with "Support for HT greenfield format is optional" | Reject: refer to 07/0555 |
| 2970 | 212.59 | 59 | 20.1.3 | Green Field preamble: Make reception of frames with GF preamble mandatory for all 802.11n devices. In this way BSSs without legacy devices will not require protection. Transmission of GF preamble frames to remain optional. | Make reception of Green Field frames mandatory. | Reject: refer to 07/0555 |
| 1563 | 212.62 | 62 | 20.1.3 | There is a sentence of "In this case the receiver shall decode the HT-SIG and determine if the HT-SIG cyclic redundancy check (CRC) passes." But, it would no be true, because we decided to allow implementation without decoding HT-SIG as described in 20.3. | Add statement here such as ", or shall maintain CCA.indication(Busy) for HT GF packet input larger than -72dBm for 20MHz and -69dBm for 40MHz." | Reject: refer to 07/0555 |
| 3402 | 213.06 | 6 | 20.1.3 | Support of Greenfield Preambles should be mandatory for all HT devices. | Make Greenfield Preambles Mandatory. | Reject: refer to 07/0555 |
| 3361 | 213.06 | 6 | 20.1.3 | Greenfield Preambles should be mandatory for all HT devices for a more robust protection mechanim | Greenfield Preambles to be made Mandatory. | Reject: refer to 07/0555 |
| 3089 | 213.06 | 6 | 20.1.3 | In order to avoid interoperability problems, Greenfield Preambles should be mandatory for all 11n HT devices | Make Greenfield Preambles Mandatory | Reject: refer to 07/0555 |
| 609 | 259.00 |  | 20.3.9.5 | Mandatory GF is preferable for HT STA and should became mandatory in the specs. It can improve the overall throughput | Green Field preamble should became mandatory | Reject: refer to 07/0555 |
| 339 | 259.20 | 20 | 20.3.9.5 | While the greenfield PLCP frame will become shorter, it is unclear whether it will be more efficient. | Remove "and more efficient". | Reject: refer to 07/0555 |
| 2915 | 305.36 | 36 | 20.3.23 | According to Section 20.1.3 it is already required that an HT-STA in case of GF shall decode the HT-SIG and determine if the HT-SIG cyclic redundancy check (CRC) passes. Therefore, the length can be easily extracted from this information and a deferral until the received level drops below the receiver minimum sensitivity level of BPSK, R=1/2 in Table n75 is straight forward | Remove "+ 10 dB" and change "(-72 dBm for 20 MHz, -69 dBm for 40 MHz)" to "(-82 dBm for 20 MHz, -79 dBm for 40 MHz)" | Reject: refer to 07/0555 |
| 347 | 307.01 | 1 | 20.3.23 | Support for greenfield is intended to be optional, the mandatory behavior specified here for detecting greenfield packets (eg. decode HT-SIG, determine CRC pass/fail) is requiring too much and contradicts purpose of being optional. | Remove requirements to decode HT-SIG and determining CRC pass/fail. | Reject: refer to 07/0555 |
| 839 | 63.21 | 21 | 7.3.2.49.2 | The current support if the greenfield in PHY section actually mandates of detecting the greenfield preamble and Signal field so relatively small implementation effort is needed to get the data portion. The greenfield format can be very useful for small data packets like voice | Make the greenfield format mandatory. Remove the greenfield subfield from the HT Capabilities Info field | Reject: refer to 07/0555 |
| 1678 | 299.54 | 54 | 20.3.21.5.1 | It will be difficult for all 802.11 a/g/n receivers to tell whether GF preambles are a valid transmission. They will set their CCA threshold to -62 dBm when they detect Greenfield transmissions, and -82 dBm when they detect non-GF transmissions. This will cause an extremely high rate of collisions for GF transmissions | Either "Take GF transmissions out of the spec" or "lower the CCA threshold for unknown received signal types" | Reject: refer to 07/0555 |

# Comment Group: Greenfield

CIDs 328, 1636, 1672, 2970, 1563, 3402, 3361, 3089, 609, 339, 2915, 347, 839, 1678

Proposed Resolution: Reject

Reason for rejection:

- The motion set that included LB84 CID 130 that proposed that an HT STA that does not support greenfield format shall still decode the HT-SIG passes unanimously on 7/20/2006.

- Straw poll held on July 18, 2006 (06/968):  Shall the ability to process GF preambles be mandatory? Yes = 48, No = 78, Abstain = 9

- Given that a non-GF HT device must check the validity of the GF HT-SIG CRC, it enables two options for the non-GF HT device, added from the adoption of 06/1571

    - Evaluate the contents of the HT-SIG and defer based on TXTIME

    - OR, keep CCA busy until the received level drops below the CCA sensitivity level as adopted in 06-1571 (minimum modulation and coding rate sensitivity + 10 dB)

- Compromise:

    - CRC check can be performed without evaluating the contents of the HT-SIG, so non-GF HT devices are not required to evaluate the contents of the GF HT-SIG

    - GF packets will be robustly protected from non-GF devices based on the CCA procedure adopted in 06/1571

    - Given that the receive procedure in 06/1571 enhances the CCA threshold post a valid 8-bit CRC, the non-GF HT device is very well protected against false alarms

- The motion set that included LB84 CIDs covered by 06/1571 passed unanimously on 11/16/2006.

- Submission 06/1731 proposed that all LB84 comments related to GF be countered with 06/1571

  - The motion set that included LB84 CIDs covered by 06/1731 passed unanimously on 1/15/2007