IEEE P802.11
Wireless LANs

| LB97 Coex Comment Resolutions: DFS Channel Selection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Square, Suite 2A | +1-781-276-0912 | bbjerke@qualcomm.com |

Abstract

This submission suggests resolutions of LB97 Coex comments related to the sub-topic DFS Channel Selection. The following CIDs are addressed: 43, 255, 256, 257, 258, 259, 260, 261, 262, 583, 584, 589, 651, 852, 853, 1768, 1769, 1770, 1870, 2572, 2575, 2577, 2579, 2580, 2581, 2582, 2583, 2584, 2585, 2587, 2892, 2893, 2894, 3003, 3080, 3081, 3082, 3083, 3084, 3085, 3259, 3279, 3280, 3283, 3287, 3337.

The changes marked in this document are based on TGn Draft 2.0

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The following comments are concerned with clauses 11.9.7.1 and 11.7.9.2, which have been removed from the 802.11n Draft since 802.11n now has adopted 802.11y as part of the baseline.  The resolution of some of these comments should be left to TGy. However, below are recommended resolutions that will be forwarded to TGy for consideration.

| 583 | 198.47 | 11.9.7.1 | 11y/D1.x changes the paragraph following the paragraph 11n/D2.0 inserts, and the drafts have duplicate ECSA logic, but are out of phase. The operation on 20 MHz channel bandwidths should be the same. | Remove changes to 11.9.7.1 and 11.9.7.2 associated with 20 MHz channel bandwidth channel switch announcements. | Accept. The text in question has been removed from the 11n draft, leaving 11y as the single source. |
| --- | --- | --- | --- | --- | --- |
| 584 | 199.02 | 11.9.7.2 | 11y/D1.x changes the paragraph 11n/D2.0 inserts, and the drafts have duplicate ECSA logic, but 11n/D2.0 lacks the requirement for the DFS owner to maintain the associations. The operation on 20 MHz channel bandwidths should be the same. | Remove changes to 11.9.7.1 and 11.9.7.2 associated with 20 MHz channel bandwidth channel switch announcements. | Accept. The text in question has been removed from the 11n draft, leaving 11y as the single source. |
| 651 | 198.19 | 11.9.7.1 | The scanning prior to starting a new BSS requirement should be included here as well for 20 and 40 MHz BSS, otherwise new 20 and 40 MHz BSS's may step on existing 20/40 BSS's and causing the latter to adjust channel, a costly operation. | Add text specifiying that new 20 and 40 MHz BSS's should scan prior to starting the new BSS. If the AP chooses to overlap with existing BSS's, at least align the frequency. | Counter. Text specifying scanning requirements for 20/40 MHz HT operation is found in 11.9.8.3 where it belongs. |
| 1768 | 198.31 | 11.9.7.1 | there are different regulatory classes with the same channel, for example channel 149 is both ISM (reg class 5) and UNII upper band (reg class 3). If an AP were to switch to channel 149 with reg class 5, would a STA that only supports channel 149 reg class 3 be allowed to switch? Or would an AP need to announce that it is switching to ch 149 for both reg class 3 and 5? If so, how would the AP do this? | please clarify | My interpretation is that only STAs that specifically support the new regulatory class would be allowed to switch. Legacy STAs may be moved using the legacy CSA. In general, it’s up to the AP to decide how it wants to move the BSS. If a switch means “losing” some STAs, that’s the AP’s decision. Comment forwarded to TGy LB104. |
| 1769 | 198.55 | 11.9.7.2 | there are different regulatory classes with the same channel, for example channel 149 is both ISM (reg class 5) and UNII upper band (reg class 3). If the DFS owner were to switch to channel 149 with reg class 5, would a STA that only supports channel 149 reg class 3 be allowed to switch? Or would the DFS owner need to announce that it is switching to ch 149 for both reg class 3 and 5? If so, how would the DFS only do this? | please clarify | My interpretation is that only STAs that specifically support the new regulatory class would be allowed to switch. Legacy STAs may be moved using the legacy CSA. In general, it’s up to the DFS owner to decide how it wants to move the IBSS. If a switch means “losing” some STAs, that’s the DFS owner’s decision. Comment forwarded to TGy LB104. |
| 2572 | 198.40 | 11.9.7.1 | "If an AP is switching to a channel within the same regulatory class, and dot11ExtendedChannelSwitchImplemented is true, then the AP shall send the Extended Channel Switch Announcement element and Extended Channel Switch Announcement frame, else the AP shall send the Channel Switch Announcement element and Channel Switch Announcement frame. Optionally the Extended Channel Switch Announcement element and Extended Channel Switch Announcement element frame may be used." This option is always true for HT APs. So it says when we switch within the same regulatory class, we must use the extended element/frame. This would ensure that legacy .11h STA's get left behind. | Should allow the use of the normal channel switch signalling as well as the extended signalling. | Accept. Let the AP decide if it needs to use either or both of the legacy and extended channel switch announcements. |
| 2575 | 198.61 | 11.9.7.2 | "If a DFS owner is switching to a channel within the same regulatory class, and dot11ExtendedChannelSwitchImplemented is true, then the DFS owner shall send the Extended Channel Switch Announcement element and Extended Channel Switch Announcement frame," This option is always true for HT APs. So it says when we switch within the same regulatory class, we must use the extended element/frame. This would ensure that legacy .11h STA's get left behind. | Should allow the use of the normal channel switch signalling as well as the extended signalling. | Accept. Let the DFS owner decide if it needs to use either or both of the legacy and extended channel switch announcements. |
| 2892 | 198.41 | 11.9.7.1 | dot11ExtendedChannelSwitchImplemented is not included in MIB | Include dot11ExtendedChannelSwitchImplemented definition in the MIB | Accept Comment forwarded to TGy LB104. |
| 2893 | 198.41 | 11.9.7.1 | The dot11ExtendedChannelSwitchImplemented is subject of AP and STA as well. How knows the DFS owner that the dot11ExtendedChannelSwitchImplemented is true in all recipients of the Extended Channel Switch Announcement? | Add Extended Channel Switch Implemented subfield in the Extended Capabilities element. Add a following rules to this subclause "A STA shall set the Extended Channel Switch Implemented subfield in the Extended Capabilities element to 1 if the STA’s dot11ExtendedChannelSwitchImplemented dot11MultiDomainCapabilityEnabled are true; otherwise, it shall be set to 0. A STA shall set dot11ExtendedChannelSwitchImplemented to TRUE before associating with a BSS or IBSS in which the Extended Channel Switch Implemented subfield is set to 1 in the Extended Capabilities element in Beacon frames and Probe Response frames received from the BSS or IBSS." | Reject. Reasons for rejection: 1) dot11ExtendedChannelSwitchImplemented is true for all HT STAs. 2) In the 5 GHz band almost all STAs are .11h capable, and therefore understand the legacy channel switch announcement. 3) The AP can decide if it needs to use either or both of the legacy and extended channel switch announcements |
| 3003 | 198.19 | 11.9.7.1 | The scanning prior to starting a new BSS requirement should be included here as well for 20 and 40 MHz BSS, otherwise new 20 and 40 MHz BSS's may step on existing 20/40 BSS's and causing the latter to adjust channel, a costly operation. | Add text specifying that new 20 and 40 MHz BSS's should scan prior to starting the new BSS. If the AP chooses to overlap with existing BSS's, at least align the frequency. | Duplicate of 651 (Resolution of 651: Counter. Text specifying scanning requirements for 20/40 MHz HT operation is found in 11.9.8.3 where it belongs.) |

The following comments are concerned with clause 11.9.8.1 and 11.9.8.2

| 2894 | 199.14 | 11.9.8.1 | The definition mandates that HT STA sets to true all the following attributes: dot11RegulatoryClassesImplemented, dot11RegulatoryClassesRequired, dot11ExtendedChannelSwitchImplemented. It means that all HT STA implements the Extended Channel Switching. Not in all environments there is need for it. For example this feature is not so important for the 20MHz capable Home BSS that sits in 2.4GHz band. So this feature should be configurable | Add Extended Channel Switch Implemented subfield in the Extended Capabilities element. Add a following rules to this subclause "A STA shall set the Extended Channel Switch Implemented subfield in the Extended Capabilities element to 1 if the STA’s dot11ExtendedChannelSwitchImplemented and dot11MultiDomainCapabilityEnabled are true; otherwise, it shall be set to 0. A STA shall set dot11ExtendedChannelSwitchImplemented to TRUE before associating with a BSS or IBSS in which the Extended Channel Switch Implemented subfield is set to 1 in the Extended Capabilities element in Beacon frames and Probe Response frames received from the BSS or IBSS." | Reject. The E-CSA is a useful feature regardless of band, and it is a less complicated solution to simply mandate it for all HT STAs. Adding a subfield to the Extended Capabilities element contributes to beacon bloat. |
| --- | --- | --- | --- | --- | --- |
| 3283 | 199.23 | 11.9.8.1 | IS the requirement to set dot11extendedchannelswitchimplemented to true really only necessary for a STA that wants to support 40 mhz channel widths? | Conditionalize the setting of dot11extendedchannelswitchimplemented to true only for STA that want to support 40 mhz channel widths? | Reject. The E-CSA is equally useful for 20 MHz-only STAs as it is for 20/40 MHz STAs. |
| 2577 | 199.23 | 11.9.8.2 | "On detecting an overlapping BSS operating on the secondary channel," This is informal and ambiguous. | Reword: "An AP, on detecting an overlapping BSS whose primary channel is the AP's secondary channel..." | Counter (accept in principle). Reword as follows: “An AP operating a 20/40 MHz BSS, on detecting an overlapping BSS whose primary channel is the AP’s secondary channel, may choose to move to a different pair of channels or switch to 20 MHz BSS operation.” |

TGn Editor: change the text as shown below:

11.9.8.2 Introduction

An AP operating a 20/40 MHz BSS, Oon detecting an overlapping BSS operating on the secondary channel, the AP operating a 20/40 MHz BSS whose primary channel is the AP’s secondary channel, may choose to move to a different pair of channels or switch to 20 MHz BSS operation.

The following comments are concerned with clause 11.9.8.3

| 43 | 199.37 |  |  | 11.9.8.3 |  | "The AP may also use passive or active scan after the BSS has been started, with the same PPDU format restrictions as given above." There is no PPDU format restrictions in the former sentences. | Delete ", with the same PPDU format restrictions as given above" from the cited sentence. | Accept. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 852 |  | 199.32 | 11.9.8.3 |  | The text shown from lines 30 to 39 is more applicable for an AP looking to set up a 20/40 MHz BSS in 5 GHz band. For 40 MHz operation in 2.4 GHz, the scanning AP has to align its primary and secondary channels to coincide with the primary and secondary channels of the existing 20/40 MHz BSS if found. |  | Add the following text after line 39 "If the scanning AP chooses to start a 20/40 MHz BSS in 2.4 GHz, then the scanning AP shall ensure that the primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSS and thatthe secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of the existing 20/40 MHz BSS." | Accept. |
| 853 |  | 199.32 | 11.9.8.3 |  | The text shown from lines 30 to 39 is more applicable for an AP looking to set up a 20/40 MHz BSS in 5 GHz band. For 40 MHz operation in 2.4 GHz, the scanning AP has to align its primary and secondary channels to coincide with the primary and secondary channels of the existing 20/40 MHz BSS if found. |  | Change the following sentence " If the scanning AP chooses to start a 20/40 MHz BSS that occupies the same two channels as an existing 20/40 MHz BSS," to " If the scanning AP chooses to start a 20/40 MHz BSS in 5 GHz that occupies the same two channels as an existing 20/40 MHz BSS," | Accept. |
| 2579 |  | 199.42 | 11.9.8.3 |  | "The AP shall scan the channels (either itself or through the STAs) before making a transition from a 20 MHz BSS to a 20/40 MHz BSS." Which channels? |  | Reword: "The AP shall scan the candidate primary and secondary channels (either itself or through the STAs) before making a transition from a 20 MHz BSS to a 20/40 MHz BSS." | Accept. |
| 3080 |  | 199.00 | 11.9.8.3 |  | terms "scanning AP" and "existing BSS" in the context of the definition provided are ambigues |  | Clarify the denitions of "scanning AP" and "existing BSS" in a given context | Reject. Reasons for rejection: 1) It is clear from the previous sentences that the “scanning AP” is the AP that is about to start a 20/40 MHz BSS. 2) It is also clear from the context that “existing BSS” refers to a BSS that was started prior to the AP attempting to start a new 20/40 MHz BSS. |
| 3081 |  | 199.00 | 11.9.8.3 |  | The definition is ambiguous. |  | re-define the paragraph | Reject. Unclear which definition the commenter is referring to. |
| 3259 |  | 199.00 | 11.9.8.3 |  | What about 20/40 case of secondary's of two HT-BSS-40 exactly overlapping, but primary channels are in opposite directions? |  | Disallow a 40 mhz bss from being set up in this manner. | Counter. The current text in 11.9.8.3 and the Regulatory classes defined in Appendix J already disallow a BSS to be set up in this manner. |
| 3279 |  | 199.00 | 11.9.8.3 |  | Disallow 40 mhz BSS operation if the 40mhz BSS candidate cannot select a primary and secondary channel for operation that meet the following condition: Given the following definitions: P1 = the primary channel of the intended 40 MHz BSS S1 = the secondary channel of the intended 40 MHz BSS Pi = the set of primary operating channels for all established 40 MHz BSS operating within the channel range [P1-2, P1-1, P1, P1+1, P1+2, S1-2, S1-1, S1, S1+1, S1+2] in the target BSA Si = the set of secondary operating channels for all established 40 MHz BSS operating within the channel range [P1-2, P1-1, P1, P1+1, P1+2, S1-2, S1-1, S1, S1+1, S1+2] in the target BSA Ti = the set of sole operating channels for all established 20 MHz BSS operating within the channel range [P1-2, P1-1, P1, P1+1, P1+2, S1-2, S1-1, S1, S1+1, S1+2] in the target BSA An AP shall not establish a 40 MHz BSS unless it can meet the following conditions: 1) P1 = Pi for all values of I 2) P1 = Ti for all values of I 3) S1 = Si for all values of I When the right hand side of any of the above equalities is the null set, then the equality test result is defined to be true. If, after initial establishment of the 40 mhz BSS, the outcome of any of the equality tests above changes to FALSE, then the AP must immediately revert to 20mhz BSS operation. |  | Add the text shown in the comment to the subclause. | Transfer to topic group “40-20 in 2.4 GHz”, as a resolution has already been submitted that addresses this comment. |
| 3337 |  | 199.41 | 11.9.8.3 |  | refine the requirement for a scan - such as, change "scan" to "at least one overlapping BSS scan" |  | make the change | Counter. The resolution of CID 2579 accomplishes the desired refinement. |

TGn Editor: change the text as shown below:

11.9.8.3 Scanning prior to starting a BSS

Before an AP starts a 20/40 MHz BSS it shall scan the environment for existing BSSs. It may use passive

scan for at least MinChannelTime or active scan. If the scanning AP chooses to start a 20/40 MHz BSS in 5 GHz that occupies the same two channels as an existing 20/40 MHz BSS, then the scanning AP shall ensure that the

primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSS and that

the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of the existing 20/40

MHz BSS.  If the scanning AP chooses to start a 20/40 MHz BSS in 2.4 GHz, then the scanning AP shall ensure that the primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of an existing 20/40 MHz BSS.  The AP may also use passive or active scan after the BSS has been started, with the same PPDU format restrictions as given above.

The AP shall scan the candidate primary and secondary channels (either itself or through the STAs) before making a transition from a 20 MHz BSS to a 20/40 MHz BSS.

The following comments are concerned with clause 11.9.8.4

| 255 | 200.12 | 11.9.8.4 | Supported Channel Witdh Set (from HT Capability) is irrelevant. Specify any changes to STA Channel Width (in HT Information element) instead | Change the last part of the sentence, starting with "…the Supported Channel Width Set" to "…the STA Channel Width of the AP's HT Information element is set to 0". | Counter. Accept in principle, add “field” after “Width”. |
| --- | --- | --- | --- | --- | --- |
| 256 | 200.20 | 11.9.8.4 | Supported Channel Witdh Set (from HT Capability) is irrelevant. | Change "Supported Channel Width Set" to "STA Channel Width" | Accept. |
| 257 | 200.43 | 11.9.8.4 | Supported Channel Witdh Set (from HT Capability) is irrelevant. | In Table n52, row named "20 MHz channel", column named "Same 20 MHz channel": change "Supported Channel Width Set" to "STA Channel Width" | Counter. Implement the change in the column named “Different 20 MHz channel” instead of “Same 20 MHz channel” |
| 258 | 200.43 | 11.9.8.4 | Supported Channel Witdh Set (from HT Capability) is irrelevant. | In Table n52, row named "20 MHz channel", column named "Pair of 20 MHz channels wwith same…": change "Supported Channel Width Set" to "STA Channel Width" | Accept. |
| 259 | 200.43 | 11.9.8.4 | Supported Channel Witdh Set (from HT Capability) is irrelevant. | In Table n52, row named "20 MHz channel", column named "Pair of 20 MHz channels with different...": change "Supported Channel Width Set" to "STA Channel Width" | Accept. |
| 260 | 200.54 | 11.9.8.4 | Supported Channel Witdh Set (from HT Capability) is irrelevant. | In Table n52, row named "Pair of 20 MHz channels", column named "Same 20 MHz channel": change "Supported Channel Width Set: 1 (unchanged)" to "STA Channel Width: 0" | Accept. |
| 261 | 200.54 | 11.9.8.4 | Supported Channel Witdh Set (from HT Capability) is irrelevant. | In Table n52, row named "Pair of 20 MHz channels", column named "Different 20 MHz channel": change "Supported Channel Width Set" to "STA Channel Width" | Accept. |
| 262 | 200.54 | 11.9.8.4 | Supported Channel Witdh Set (from HT Capability) is irrelevant. | In Table n52, row named "Pair of 20 MHz channels", column named "Pair of 20 MHz channels with different...": change "Supported Channel Width Set" to "STA Channel Width" | Accept. |
| 589 | 199.58 | 11.9.8.4 | "While performing 20/40 MHz BSS to 20MHz BSS switch, the AP indicates it to the station in the HT capabilities IE. The draft says that the station should decide whether it needs to disassociate based on the new capabilities. However, the station may not get the correct idea of whether the AP can still support the BW for it's flows. The station may end up reassociating or not finding any other AP to associate with unnecessarily." [ST] | This can be taken care of by having the AP recalculate the bandwidth budget for all it's flows and delete some of the flows either based on priority or some other criteria. A new reason code can be added to indicate the tearing down of flows due to bandwidth switch. Only the affected stations can then decide to roam. | Reject. Unclear what specific change the commenter is suggesting. |
| 1770 | 201.01 | 11.9.8.4 | 11.9.8.1 states "For an HT STA, the following MIB attributes shall be set to true: dot11RegulatoryClassesImplemented, dot11RegulatoryClassesRequired, and dot11ExtendedChannelSwitchImplemented." so why is this if statement necessary? | remove paragraph | Accept. |
| 1870 | 200.00 | 11.9.8.4 | It is not clear if Extended Channel Switch Announcement provides capability to also change BSS from 20/40MHz operation to 20MHz operation. If not, please reword the statement in lines 11 to 13 to reflect the intention. | Please clarify. If however the text needs to stay then the word "same" maybe needed before "primary channel". | Counter. Accept in principle, add “using the same” just before “primary channel”. |
| 2580 | 199.49 | 11.9.8.4 | "adjust its channel width" - this needs to be related to existing description | Change to "switch the BSS to 20 MHz operation" | Accept. |
| 2581 | 199.57 | 11.9.8.4 | "An AP may switch the BSS capability between 20/40 MHz and 20 MHz by changing the value of the Supported Channel Width Set field of the HT capabilities element in the Beacon." There's no need to give the AP permission to use these fields, they are already defined for that purpose. | Reword: "An AP switches the BSS channel width between 20/40 MHz and 20 MHz by changing the value of the Supported Channel Width Set field of the HT Capabilities element in the Beacon." | Counter. Accept in principle, add “capability” after “channel width”. |
| 2582 | 199.59 | 11.9.8.4 | "The STAs may disassociate and associate or reassociate to accommodate the new capability." What is this saying? Re-associate with the same AP (which is unnecessary) or go elsewhere? What stops a STA from doing is already? - nothing | Remove the sentence or rewrite as a note saying they can go to another AP if they want to. | Counter. Accept in principle, rewrite as note and split current paragraph into two. |
| 2583 | 200.01 | 11.9.8.4 | What I don't understand is the role of regulatory classes when an AP switches between 20/40 and 20. When it swiches between these, must it also change regulatory class to a 40 MHz or 20MHz class? Or can it stay in the 40MHz class with the Secondary Channel Offset indicating 20MHz operation? | Please add text that describes how secondary channel offset and regulatory class are modified when switching between 20/40 and 40. | Counter. For clarification, move the very last sentence in 11.9.8.4 so that it appears immediately after the paragraph in question. |
| 2584 | 200.07 | 11.9.8.4 | "When changing from a single channel to a new channel, or from a pair of channels to a new pair of channels, the Supported Channel Width Set field of the HT capabilities element in the Beacon remains unchanged. When the Extended Channel Switch Announcement is used to switch the BSS from 20/40 MHz operation to 20 MHz operation on the primary channel, the Supported Channel Width Set field of the HT Capabilities element in the Beacon remains unchanged." I'm not sure what is the value of these statements. Also they are not complete, as they don't cover the switch from 20 MHz to 20/40. | I think there are several possible solutions: 1. The smallest one is to reword the second para "switch the BSS between 20/40 and 20 MHz operation" 2. However, we may be overconstraining. We could alternatively say: "An AP shall only set the Secondary Channel Offset to a non-zero value if its Supported Channel Width set is set to 1." This requires the AP to support 40MHz while operating a 20/40 MHz, BSS but doesn't prevent it switching its receive capability to 20MHz in a 20MHz BSS. Solution 2 is my preference. | Counter. The first paragraph simply says that the channel width capability of the AP doesn’t change in the two cases mentioned. The second paragraph is modified so that STA Channel Width is now used to signal a change in channel width, without changing the capability. Conclusion: same resolution as CID 255 |
| 2585 | 200.23 | 11.9.8.4 | Table n52 mistakenly believes that the Supported Channel Width set determines the 20/40 vs 20 mode of operation of the BSS. It does not. Secondary channel offset does. | Replace "Supported Channel Width Set: 0" with "Secondary Channel Offset set to 0" "Supported Channel Width Set: 1" with "Secondary Channel Offset set to a non-zero value" | Counter. STA Channel Width (of the HT Information element) is used to signal changes in channel width. Table n52 is only concerned with the use of the E-CSA, and not the legacy CSA/Secondary Channel Offset combo. Conclusion: same resolutions as CIDs 257, 258, 259, 260, 262, 262. |
| 2587 | 200.64 | 11.9.8.4 | "The AP may send the Extended Channel Switch Announcement element in a Beacon without performing a backoff, after determining the WM is idle for one PIFS period." This is unnecessary. The AP is also an HC. It can always access the medium after PIFS for any purpose whatsoever, including this. | Delete the quoted sentence or reword as a note. | Accept. (although the sentence appears in the base standard for legacy CSA and in 11y for the E-CSA) |
| 3082 | 199.49 | 11.9.8.4 | The definition "move BSS" is amboguous | clarify the definition | Reject. It is clear from the context and the subsequent sentence what “move BSS” means |
| 3083 | 199.00 | 11.9.8.4 | The definition is ambiguous. | re-define the paragraph | Reject. Unclear which definition the commenter is referring to. |
| 3084 | 200.00 | 11.9.8.4 | The definition "New Regulatoyry Class field specifies the position of the secondary channel relative to the new primary channel" is not correct | Remove this sentence. | Reject. The New Regulatory Class field does indeed specify the position of the secondary channel relative to the primary channel for 40 MHz channels (see Annex J) |
| 3085 | 200.65 | 11.9.8.4 | WM term is not defined | Change to wireless medium | Counter. The sentence containing “WM” has been deleted as per the resolution of CID 2587 |

TGn Editor: change the text as shown below:

11.9.8.4 Channel management at the AP

An AP operating a 20/40 MHz BSS may decide to move the BSS and/or switch the BSS to 20 MHz operation adjust its channel width if, for example, another BSS starts to operate in either or both of the primary or secondary channels, or radar is detected in either or both of the primary or secondary channels. Specifically, the AP may move its BSS to a different pair of channels, or change from a 20/40 MHz BSS to a 20 MHz BSS using either the primary channel of the previous channel pair or any other available 20 MHz channel. The AP may also change from a 20 MHz BSS to a 20/40 MHz BSS.

An AP may switches the BSS channel width capability between 20/40 MHz and 20 MHz by changing the value of the Supported Channel Width Set field of the HT capabilities element in the Beacon.

NOTE--The STAs may disassociate and associate or reassociate to accommodate the new capability of the AP.

In order to maintain existing associations while making a channel width change or while performing a channel pair relocation, an AP may inform associated HT STAs that it is making the change by including an Extended Channel Switch Announcement element in Beacon, Probe Response, and Extended Channel Switch Announcement frame transmissions until the intended channel switch time. The New Channel Number of the Extended Channel Switch Announcement element represents the new channel in the case that the BSS after relocation/width change will be a 20 MHz BSS, or the primary channel of the new pair of channels in the case that the BSS after relocation/width change will be a 20/40 MHz BSS. When changing to a new pair of channels, the New Regulatory Class field also specifies the position of the secondary channel relative to the new primary channel, i.e., either above or below. For 20 MHz operation when the New Regulatory Class signifies 40 MHz channel spacing, the 20 MHz channel

is the primary channel of the 40 MHz channel.

When changing from a single channel to a new channel, or from a pair of channels to a new pair of channels,

the Supported Channel Width Set field of the HT capabilities element in the Beacon remains unchanged.

When the Extended Channel Switch Announcement is used to switch the BSS from 20/40 MHz operation to

20 MHz operation using the same on the primary channel, the Supported STA Channel Width Set field of the AP’s HT Information element is set to 0. HT Capabilities element in the Beacon remains unchanged.

When an AP uses the Extended Channel Switch Announcement element to announce a channel switch or

channel width adjustment, the values for New Channel Number and New Regulatory Class shall comply with

the values given in Table n52 (Channel switching and channel width adjustment combinations). The values

for Supported STA Channel Width Set are also given in the table.

Table n52-Channel switching and channel width adjustment combinations

| Original channel | Destination channel |  |  |  |
| --- | --- | --- | --- | --- |
|  | Same 20 MHz channel | Different 20 MHz channel | Pair of 20 MHz channels with same primary channel as original | Pair of 20 MHz channels with different primary channel than original |
| 20 MHz channel | N/A | New Channel Number: any number different from the current number, satisfying channel allocation rules specified in 20.3.14.3 (40 MHz channelization) and Annexes I and J. STA Supported Channel Width Set: 0 (unchanged). | New Channel Number: same as the current number, satisfying channel allocation rules specified in definitions in 20.3.14.3 (40 MHz channelization) and Annexes I and J. STA Supported Channel Width Set: 1. | New Channel Number: any number different from the current number, satisfying channel allocation rules specified in 21.3.14.3 (40 MHz channelization) and Annexes I and J. STA Supported Channel Width Set: 1. |
| Pair of 20 MHz channels | New Channel Number: same as the current primary channel, satisfying channel allocation rules specified in 20.3.14.3 (40 MHz channelization) and Annexes I and J. STA Supported Channel Width: 0 Set: 1 (unchanged). | New Channel Number: any number different from the current primary channel, satisfying channel allocation rules specified in 20.3.14.3 (40 MHz channelization) and Annexes I and J. STA Supported Channel Width Set: 0. | N/A | New Channel Number: any number different from the current number, satisfying channel allocation rules specified in 20.3.14.3 (40 MHz channelization) and Annexes I and J. STA Supported Channel Width Set: 1 (unchanged). |

Movements of a 20/40 MHz BSS from one channel pair to a different channel pair and changes between 20

MHz and 20/40 MHz operation should be scheduled so that all STAs in the BSS, including STAs in power

save mode, have the opportunity to receive at least one Extended Channel Switch Announcement element

before the switch. The AP may send the Extended Channel Switch Announcement element in a Beacon without

performing a backoff, after determining the WM is idle for one PIFS period.

A STA shall support the Extended Channel Switch Announcement if dot11ExtendedChannelSwitchImplemented is true.

When the Extended Channel Switch Announcement element and Extended Channel Switch Announcement

frame are transmitted in bands where dot11SpectrumManagementRequired is true, the Channel Switch Announcement element and Channel Switch Announcement frame may also be transmitted. If both the Extended

Channel Switch Announcement element and the Channel Switch Announcement element are transmitted,

the New Channel Number of both elements shall be identical. An HT STA associated to an HT AP shall ignore

the Channel Switch Announcement element.

For 20 MHz operation when the New Regulatory Class signifies 40 MHz channel spacing, the 20 MHz channel

is the primary channel of the 40 MHz channel.

The following comments are concerned with Annex J

| 3280 | 459.00 | J | Annex J, table J1, reserved values are overlapping with some defined values. | Correct the J1 table values for the reserved values so that there is no overlap of reserved values with the non-reserved values. I.e. change reserved range from 31-255 to 34-255. | Accept. Make the change in Table J.1 as proposed. |
| --- | --- | --- | --- | --- | --- |
| 3287 | 459.00 | J | What is the relationship between channel spacing and channel set? I would think that the channels specified are separated by the channel spacing that is given, but this does not make any sense. The following two statements within J make me think that this table is incorrect: "The channel starting frequency variable is a frequency, used together with a channel number, to calculate a channel center frequency. Channel spacing is the frequency difference between nonoverlapping adjacent channel center frequencies." These statements together sort of imply that you multiple a channel number by the channel spacing to get the center frequency for that channel. The channel numbers, in for example the channel set for regulatory class 32 appear to be numbers that are representative of channels that are actually spaced at 20 mhz separation. | This table needs to be fixed. I do not currently have a suggestion. | Reject. Channel spacing is the bandwidth of the signal. Channel set are the valid channel numbers. Channel set is independent of channel spacing. Channel start frequency and channel number give you the channel center frequency (for the primary channel). The channel numbers for regulatory class 32 are spaced at 5 MHz. |