IEEE P802.11
Wireless LANs

| Network Type Assisted Selection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trpz.com |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503-264-8014 | necati.canpolat@intel.com |
| Vivek Gupta | Intel Corp | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 712 1754 | vivek.g.gupta@intel.com |
| Dave Stephenson | Cisco Systems | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Hong Cheng | Panasonic Singapore Laboratories | BLK1022 TaiSeng Ave #06-3530 TaiSeng Ind. Est. Singapore 534415 | +65 65505477 | hong.cheng@sg.panasonic.com |
| Amy Zhang | Huawei Technologies | Bantian, Longgang District Shenzhen 518129,P.R.China | +86 755 896 50361 | amyzhang@huawei.com |
| Colin Blanchard | BT Group | MLB1 PP8, BT Adastral Park Ipswich, Suffolk UK IP5 3RE | +44 1473 605353 | Colin.Blanchard@bt.com |
| Lars Falk | TeliaSonera | SE-20521 Malmö, Sweden | +46-70-5945555 | Lars.p.falk@teliasonera.com |
| Christian Kuhtz | EarthLink | 1375 Peachtree St, Level A, Atlanta, GA 30309 | 404 840 7684 | kuhtzch@corp.earthlink.net |
| Srinivas Sreemanthula | Nokia | 6000 Connection Dr, Irving, TX | +1 972 894 5000 | srinivas.sreemanthula@nokia.com |
| Angelo Centonza | Roke Manor Research | Old Salisbury Lane Romsey, Hampshire SO51 0ZN, UK | +44 1794 833185 | angelo.centonza@roke.co.uk |
| George Bumiller | Research in Motion | 295 Phillip Street Waterloo, Ontario Canada N2L 3W8 | +1 201 327 1136 | g.bumiller@ieee.org |
| Manoj Deshpande | Qualcomm | 5775 Morehouse Drive San Diego, CA 92121 | +1 858 651-3717 | manojd@qualcomm.com |
| Yongho Seok | LG Electronics |  |  | yhseok@lge.com |

Abstract

This document is the normative text proposal for network type selection, as first proposed in 11-07/0446.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

This proposal extends current mechanisms of communicating the security configuration of a wireless LAN to non-AP STAs.

Use Cases

- Many settings (e.g., downtown urban areas) have high numbers of available networks, many of which are private.  A device searching for a public attachment point to the Internet may need to sort through over 100 available networks to find the few that are public.  The Network Type bits attached to the HESSID enable this sort to be made easier.

- After the first-pass screen on the Network Type bits, GAS queries can be used to further find networks that meet the client criteria.

- Interactions with wireless LANs that do not support access to 802.21 services.  If a captive portal system such as UAM is in use, there is no way in current 802.11 implementations to tell before associating and being trapped by the captive portal system.

Benefits

- This proposal enables clients to find appropriate networks faster, thus improving the user experience as well as battery life by decreasing futile connection attempts.

- By communicating the type of authentication required, a piece of client software could use this information to automatically launch a web browser or other client (e.g. VPN) to automatically connect to the DN.

This is a normative text proposal that allows a non-AP STA to learn about how a network is configured, and what types of authentication it supports.  The reason for this is that such a STA may wish to ignore all the private networks in an area and search for networks that use a compatible type of authentication, or offer free access.

The specific points in the proposal are:

- Change the older term "ESSID" to "HESSID" where appropriate

- Add on bits to the HESSID to describe the type of network (private versus public, whether free Internet access is allowed, and whether further steps are required after 802.11 association)

- A new Generic Advertisement Service (GAS) native query that allows a non-AP STA to learn about the types of next steps that are required.

# Editing Instructions

      - Probe Response frame format

Change row 25 in table 15 to be HESSID instead of ESSID:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 25 | HESSID | HESSID shall be present if dot11InterworkingEnabled is present |

Make the indicated changes to the following clause, renumbering figures where appropriate:

      - HESSID Information element

The Homogenous Extended Service Set ID (ESSID) Information element contains description of HESSID. The format of the HESSID element is shown in Figure u1.

|  | Element ID | Length | HESSID | Network Type |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |

Figure u1—HESSID Information element format

The Element ID field is equal to the HESSID value in Table 26.

The value of the Length field is 6.

The HESSID field specifies the definition of HESSID.

In the infrastructure mode, the HESSID definition includes the BSSID value of one of the group of APs. The HESSID and the SSID together to provide is a unique value that is advertised in beacons and probe responses so that the non-AP STA is aware of continued applicability of previously discovered interworking and advertising services when moving from one AP to another within the scope of the HESSID.

The Network Type field is defined in Figure u13a.  This field is used to advertise the type of network for every SSID included in the HESSID set.

|  | b0 | b2 | b3 | b4 | b5 | b6 | b7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Network Type Code |  | NASR | Intranet/ Internet | Advertisement Policy |  | Reserved |
| Bits: | 3 |  | 1 | 1 | 2 |  | 1 |

Figure u13a—HESSID Network Type Field

Bits 0-2 are the Network Type Code bits.  These bits are set with the value from Table u1a.

Table u1a—Network Type Code bit assignments

| Network Type Code Bits (b0-b1-b2) | Meaning | Description |
| --- | --- | --- |
| 000 | Private network | Network requires user accounts for access |
| 001 | Private network with guest access | Network requires user accounts for access, but guest accounts area available |
| 010 | Chargeable | Access to the network requires payment. Further information on types of charges may be available through other methods (802.21, UAM, etc). |
| 011 | Free | No charges apply for network use. Bits 4-6 are also used in this case. |
| 100 | Emergency network | The network supports emergency services. Further information is available from a GAS native query (see clause 7.3.2.53) |
| 101-111 | Reserved | Not used. |

Bit 3 is the Next Authentication Step Required (NASR) bit.  This bit shall be set to one to indicate that the network requires a further authentication step, as defined in clause 7.3.2.54.

Bit 4 is the Internet/Intranet bit.  This bit is only used when the Network Type Code in Table u1a is set to 011.  It is set to 1 to indicate that the network provides Internet access, and 0 to indicate the network provides access only to local networks.  If the Network Type Code is not 011, these bits shall be set to 0.

Bits 5 and 6 are the Advertisement Policy field, which has values set according to Table u1b.  These bits are only used when the Network Type Code in Table u1a is set to 011  If the Network Type Code is not 011, these bits shall be set to 0.

Table u1b—Advertisement Policy bit assignments

| Network Type Code Bits (b0-b1-b2) | Meaning | Description |
| --- | --- | --- |
| 00 | No Advertisements | The network does not require the end user to view commercial advertisements |
| 01 | Advertisements present | The network requires end users to view advertisements to support the service |
| 10 | Mixed advertisement policy | The network may require end users to view advertisements, but only for certain services. |
| 11 | Reserved | Not used. |

Bit 7 is reserved and shall be set to zero.

Insert a new Native Query Type in Table u2 in the following clause:

      - SSID Container Information element

Table u2—Native Query Info ID definitions

| Info Name | Info ID | Native Info Element (clause) |
| --- | --- | --- |
| Capability List | 0 | 7.3.2.51 |
| mSSID List | 1 | 7.3.2.52 |
| Emergency Networks List | 2 | 7.3.2.53 |
| Network Authentication Type | 3 | 7.3.2.54 |
| Reserved | 3 – 255 |  |

Insert the following new subclause, renumbering figures and tables as appropriate:

      - Native Info Network Authentication Type Information element

The Native Info Network Authentication Type Information Element provides a list of authentication types that are used on the indicated SSID.

|  | Info ID (3) | Length | Status Code | Network Authentication Type Unit #1 | Network Authentication Type Unit #2 | … | Network Authentication Type Unit #N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | variable | variable | … | Variable |

Figure u27a—Native Info Network Authentication Type element format

The Info ID field is equal to 3, which corresponds to the Network Authentication Type element per Table u2.

The Length field is the length of the Network Authentication Type element and is the determined by the number and size of the Network Authentication Type Units, as described in this subclause.

The Status Code is a 2-octet field with values from Table 23.

Each Network Authentication Type Unit has the structure shown in Figure u27b.

|  | Network Authentication Type Indicator Value | Network Authentication Type Unit Length | Network Authentication Type Indicator data |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure u27b—Network Authentication Type Unit

The Network Authentication Type indicator value has one of the values show in Table u2a.

The Network Authentication Type Unit Length field is set to the number of octets in the Network Authentication Type Unit.

Each indicator value defines additional information that may be communicated.

Table u2a—Network Authentication Type indicator value

| Value | Meaning |
| --- | --- |
| 0 | Acceptance of legal terms and conditions |
| 1 | On-line enrollment supported |
| 2 | UAM |

If the Network Authentication Type indicator value is 0, the network requires the user to accept terms and conditions.  No Network Authentication Type data is present.

If the Network Authentication Type indicator value is 1, the network supports on-line enrolment.  Therefore, higher-layer software on the non-AP STA may indicate to the user that accounts may be created.  No Network Authentication Type Data is present.

If the Network Authentication Type indicator value is 2, the network supports the browser-based Universal Access Method.  In this case, the Network Authentication Type data is used to indicate the version of UAM in use, as shown in Figure u27c.

|  | Network Authentication Type Indicator Value (2 - UAM) | Network Authentication Type Unit Length (2) | UAM Version |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure u27c—Network Authentication Type Unit for UAM

The UAM Version is a one-octet field indicating the version of UAM in use.  When the version number is equal to 1, there is no further data required.

References:

[1] IEEE P802.11u/D0.04, baseline document.