IEEE P802.11
Wireless LANs

| TGn LB97 Submission for PHY FEC Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Road, San Jose CA, 95131 | +1.408.451.8109 | George.Vlantis@st.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following twenty-eight (28) LB97 comments:

919, 655, 725, 1597, 1782, 1808, 1810, 1811, 1812, 1900, 1902, 1906, 2652, 2663, 2702, 2707, 2708, 2713, 2715, 2772, 2784, 2797, 2798, 2977, 3231, 3232, 3389, 3390.

Counter proposals are given for 22 of the 28 CIDs.  The 6 exceptions are:  2652 (Transfer to MAC [or Defer and Assign to Jim Petranovich]); 2772 (Defer and Assign to Eldad Perahia); 2784 (Defer and Assign to Eldad Perahia); 2797 (Accept); 2977 (Deferred for Discussion); and 3390 (Accept).

The changes marked in this document are based on TGn Draft version P802 11n D2.0.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolutions

Click on icon below for modified “FEC” sheet of PHY Comment Resolution spreadsheet Doc #11-07-0341r2.  Changes are in red text.

# (1) CID 919 (G.10.1: page 442; line 4)

Proposed Resolution:  Counter

Discussion:  There are two errors here describing the length of the message and the PSDU.  The commenter reported the second of the two errors.  (Both are artifacts of using LDPC Example 1’s text to create LDPC Example 2’s text.)  However, the data in the associated tables are correct and so are their lengths.

TGn Editor: In D2.0, page 439, line 57, modify the text as follows:

The length of the message was increased by 40 octets from 72 characters to 144112 characters, in order to illustrate padding (rather than puncturing) and encoding of multiple codewords.

TGn Editor: In D2.0, page 442, line 4, modify the text as follows:

The resulting 100140 octets PSDU is shown in Table G.39.

# (2) CID 655 (20.3.9.4.3: page 250; line 47)

Proposed Resolution:  Counter

Discussion:  The HT SIGNAL field is always encoded with a single BCC encoder, so 6 tail bits of zeroes are enough for a single BCC codec to output its data.  There are two reasons why a single BCC encoder is used: (1) if a single BCC encoder were not used, then the receiver would have to decode the HT SIGNAL field with both a single decoder and a dual decoder, because the receiver would not know the data rate and hence the number of encoders for the payload a priori; and (2) the modulation of the HT_SIGNAL field is BPSK, and the code rate R=1/2 which produces a data rate comparable to the 6Mbp/s of 802.11a/g.  (The description of the encoding of the HT SIGNAL field on page 251 lines 46-47 parallels the description of the encoding of the legacy SIGNAL field in subclause 17.3.4.)   The unencoded and encoded data rates for the preamble are well within the capabilities of a single BCC decoder and do not warrant dual encoders.

However, the confusion raised by this comment stems from the definition of NES. There are three occurrences of the definition of NES in draft 2.0 and to be more precise the current definition of NES, i.e. “Number of FEC encoders” should be changed to “Number of BCC encoders for the Data field”.  See CIDs #1808 and 1906 for the accepted resolutions.  CID #1782 is also related.

# (3) CID 725 (20.3.2: page 217; line 35)

Proposed Resolution:  Counter

Discussion:  The commenter notes that the 6 Tail bits specified in Figure n62 are incorrect for the LDPC case (which is noted in the figure) and that 6 Tail bits are inadequate in the dual BCC encoder case.  Rather than further qualifying the diagram, as the commenter suggests, by changing “(non-LDPC case only)” to “(non-LDPC single encoder only)”, the string “Tail 6 bits” can simply be replaced with “Tail 6•NES bits”.

A related CID, CID #1900 suggests deleting the number “6” as a remedy.  Since this Figure is qualified for BCC encoding only, and NES is defined in the BCC encoding case, the solution provided offers the reader more information.

Related CID are CIDs #1900 and #2702.  The first change below resolves CID #2702.

TGn Editor: In D2.0, page 217, line 35, modify Figure n62 as follows:

Tail 66•NES bits

# (4) CID 1597 (20.3.10.3: page 263; line 25)

Proposed Resolution:  Counter

Discussion:  The commenter raises several points in this comment.  For the LDPC points, the commenter’s “Proposed Change” would introduce much redundancy into the draft, except that the concatenation of the SERVICE field with the PSDU should be mentioned clearly in subclause 20.3.10 “The Data Field”, and the BCC cases should be disambiguated. With regard to the point that BCC decoding capability is mandatory, but is never stated appears to be valid.

The optionality of the LDPC codec is clearly stated in Clause 20.3.10.6.1, page 264, Lines 49-52 and Line 56-57. Likewise, the implications of "LDPC coding" capability are described clearly in Figure n30 (page 62) and in the first row of Table n26 (page 63), both of Subclause 7.3.2.49.2.

However, the commenter seems correct that a statement that BCC encoding capability of the Data Field is mandatory is missing from subclauses 20.3.10.3, 20.3.10.4, and 20.3.10.5 and elsewhere in the text, and that 17.3.5.5 mandates usage of the BCC encoder.   In the commenter's "Proposed Change", he hints that "The Data field" definition should be used and fixed for both BCC and LDPC.

TGn Editor: In D2.0, page 262, line 17, modify the text as follows:

The When BCC encoding, the Data field consists of the 16-bit SERVICE field, the PSDU, either six or twelve tail bits, depending whether thare are one or two encoding streams, and pad bits.  When LDPC encoding, the Data field consists of the 16-bit SERVICE field and the PSDU,  processed by the procedure in subclause 20.3.10.6.5.

TGn Editor: In D2.0, page 263, line 15, modify the text as follows:

The data areThe Data field is encoded using either the binary convolutional code (BCC) defined in 17.3.5.5, or the low density parity check (LDPC) code defined in 20.3.10.6 (Low density parity check (LDPC) codes).

TGn Editor: In D2.0, page 263, line 25, append the following text to the existing paragraph:

The capability to receive BCC frames is mandatory.

# (5) CID 1782 (20.3.10.6: page 264; line 49)

Proposed Resolution:  Counter

Discussion:  The commenter has two points.  The commenter’s first point is that the commenter could not find a subclause where the number of LDPC encoder is stated to be one.  As several drafts have read and draft 2.0 currently reads, the specification of the number of BCC and LDPC encoders, the number of encoders that are employed, and their optionality are contained in subclause 20.3.10.3 “Coding”.  The commenter will find that in subclause 20.3.10, page 264, line 19, the following sentence: “A single FEC encoder is always used when LDPC coding is used”.

The commenter’s second point is that the MCS Tables n82-n87 and n90-n94 specify NES=1 and MCS Tables n88–n89 and n95-n96 include a column of NES values for each MCS entry.   When LDPC encoding is used, the value of NES in the MCS tables is of subclause 20.6 should be ignored and a single LDPC encoder is always employed.  The commenter’s “Proposed Change” to modify the definition of NES in Table n81 is accepted in principle.

However, the same commenter submitted another comment CID #1808, which immediately follows this comment resolution in the PHY spreadsheet and this document.  CID #1808 makes the same point and has the same “Proposed Change”.  To simplify the editor’s task, this CID #1782 is countered with the resolution of CID #1808, which is accepted.

# (6) CID 1808 (20.6: page 328; line 33)

Proposed Resolution:  Counter

Discussion:  The comment is accepted in principle.  See CID #1782 for discussion of related comment on the same Table n81 by the same commenter.  See CID #1906 for discussion of similar comment but on a different Table n59.  See CID #655 for the reasoning behind adding the qualifying text “for the Data field”.

TGn Editor: In D2.0, page 328, line 33, modify the definition of NES in Table n81 as follows:

Number of FECBCC encoders for the Data field.

# (7) CID 1810 (G.9: page 430; line 12)

Proposed Resolution:  Counter

Discussion:  The commenter’s sentiment is accepted in principle, and the actionable text is provided in this counter-proposal.  Appendix subclause G.9 is G.2 in Draft 2.1.  The TXVECTOR parameter names are changed below to the Draft 2.0 definitions below.  In response to editorial comments, the editor has changed the formatting of these lines Draft 2.1.

TGn Editor: In D2.0, page 430, lines 12-21, modify the text as follows (maintaining the formatting of D2.1):

— LDPC_CODING = LDPC_CODING = 1                             (LDPC encoder; not BCC)

— TxVectorCW CH_BANDWIDTH = HT_CBW20 = 0        (TXVECTOR ChannelWidth CH_BANDWIDTH = 0 => 20 MHz)

— TxVectorMCS = 4                                                                (MCS = 4; QAM16; Code Rate = 3/4)

— Code Rate R = 3/4

— TxVectorLength LENGTH = 100 octets                              (with 16-bit SERVICE field becomes 102 octets = 816

bits to scramble and encode)

— m_STBC = 1 STBC = 0                                                       (STBC = 0 => OFF; m_STBC = 1)

# (8) CID 1811 (G.10: page 439; line 61) and (9) CID 1812 (G.10; page 441 line 44)

Proposed Resolution:  Counter

Discussion:  The commenter’s sentiment is accepted in principle, and the actionable text is provided in this counter-proposal.  Appendix subclause G.10 is G.3 in Draft 2.1.  The TXVECTOR parameter names are changed below to the Draft 2.0 definitions below.  In response to editorial comments, the editor has changed the formatting of these lines Draft 2.1.  Furthermore, the lines referred to in CIDs #1811 and #1812 have been coalesced into a single block in Draft 2.1, in response to another editorial CID.

TGn Editor: In D2.0, page 439, lines 61-65 and in D2.0, page 441, lines 44-49, modify the text as follows (maintaining the formatting of D2.1):

— LDPC_CODING = LDPC_CODING = 1                             (LDPC encoder; not BCC)

— TxVectorCW CH_BANDWIDTH = HT_CBW40 = 1        (TXVECTOR ChannelWidth CH_BANDWIDTH = 1 => 40 MHz)

— TxVectorMCS = 1                                                                (MCS = 1; QPSK; Code Rate = 1/2)

— Code Rate R = 1/2

— TxVectorLength LENGTH = 140 octets                              (with 16-bit SERVICE field becomes 142 octets =

1136 bits to scramble and encode)

— m_STBC = 2 STBC = 1                                                       (STBC = 1 => ON; m_STBC = 2)

# (10) CID 1900 (20.3.2: page 217; line 35)

Proposed Resolution:  Counter

Discussion:  This CID points out the same error as CID #725, but with a different “Proposed Change”.  See CID #725 for the Resolution.  Another related comment is CID #2702.

# (11) CID 1902 (20.3.3: page 232; line 14)

Proposed Resolution:  Counter

Discussion:  The commenter’s sentiment is accepted in principle, and the commenter’s “Suggested Change” would be agreeable, except that the actionable text below adds consistency with the language that is used in step e) Interleaver on lines #22-23 on the same page.  When LDPC encoding, both the b) Encoder parser and the e) Interleaver steps are bypassed.   The inclusion of the “for the Data field” phrase in the text is to be consistent with the other two definitions of NES in the parameter tables.  See related CIDs #655, #1782, #1808, and #1906.

TGn Editor: In D2.0, page 430, lines 14-15, modify the text as follows:

b) Encoder parser: if BCC encoding is to be used, de-multiplexes the scrambled bits among NES (number of FECBCC encoders for the Data field) FECBCC encoders, in a round robin manner.

# (12) CID 1906 (20.3.5: page 240; line 14)

Proposed Resolution:  Counter

Discussion:  The comment is accepted in principle.  See CID #1808 for similar comment but on a different Table n81.  CIDs #655 and #1782 also relate.

TGn Editor: In D2.0, page 328, line 33, modify the definition of NES in Table n59 as follows:

Number of FECBCC encoders for the Data field..

# (13) CID 2652 (20.3.2: page 225; line 31)

Proposed Resolution:  Transfer to MAC ad-hoc (Suggested Alternative:  Defer and Assign CID 2652 to Jim Petranovich who has a similar Comment from the commenter regarding Short GI.)

Reason for Transfer:  The commenter is requesting rules for the MAC to set the LDPC_CODING parameter.   After briefly discussing this comment with the commenter in Orlando, it was evident that this comment is a place-holder for the MAC team to do some work on such language, and it is clear that such guidance does not belong in clause 20, but rather in clause 9 or some such.  Such rules certainly don’t belong in a table, such as Table n56, which this CID cites.

After further discussion of how to implement such rules, based on a variety of inputs, it was clear that the rules being requested would be similar to rules for selecting when to use STBC, 1/2 GI, 40MHz, beam-forming, or just simply changing the MCS, etc.  While some rules could be provided, over-specifying what amounts to be an implementation-specific choice (comparable to rate adaptation for MCS) would not be desirable.  Also, there did not seem to be much enthusiasm for specifying rules for using all the other TXVECTOR parameters by the MAC, such as the ones listed above.

My suggestion is to transfer this CID #2652 to the MAC ad-hoc and allow them to reject it or allow the commenter to withdraw it.  However, the commenter suggested as an alternative of assigning CID #2652 to Jim Petranovich who is working on a similar comment regarding the MAC usage of Short GI.

# (14) CID 2663 (20.3.3: page 232; line 14)

Proposed Resolution:  Counter

Discussion:  The commenter is requesting that the parenthesized definition of NES in the line below be deleted because it is repetitive.

b) Encoder parser: de-multiplexes the scrambled bits among NES (number of FEC encoders) FEC encoders, in a round robin manner.

Note that this line is changed by the resolution to CID #1902 to read:

b) Encoder parser: if BCC encoding is to be used, de-multiplexes the scrambled bits among NES (number of BCC encoders for the Data field) BCC encoders, in a round robin manner.

Unfortunately, this line is the first occurrence of the parameter NES in the draft, and NES should be defined at or before its first usage.  If the parenthetical definition were to be deleted, then there would be no definition of the parameter NES in this line.  It would be a simple task to rewrite the sentence without introducing NES here, but the parameter NES is then used nine more times on pages 237 and 238 before its definition is given in tabular form in Table n59 on page 240.  (Incidentally, the parameter is redundantly redefined one more time in Table n81 on page 328 and used repeatedly elsewhere throughout the draft.)  The introduction of the parameter NES and its parenthetical definition were included due to a comment requesting clarification of the number of encoders in an earlier draft, so its not advisable to risk going backwards.  See related CIDs #655, #1782, #1808, #1902, and #1906.

# (15) CID 2702 (20.3.9.5.3: page 261; line 17)

Proposed Resolution:  Counter

Discussion:  This CID points out the same error as CID #725 and CID #1902 with a comparable “Suggested Change”, but is devoid of actionable text.  See CID #725 for the Resolution.  The first instruction to the editor resolves this CID.

# (16) CID 2707 (20.3.10.4: page 263; line 39)

Proposed Resolution:  Counter

Discussion:  The commenter’s “Comment” and “Proposed Change” may or may not be technically correct, but the commenter’s sentiment that more than one implementation of the algorithm is possible.  However, the commenter is neglecting to take into account that the padding bits (which fill out the OFDM symbols) follow the tail bits.  (I once made the same mistake when correcting the Annex G.1 BCC Example, for the single BCC encoder case in draft 1.0.  It wasn’t until I reread 17.3.5.2, and discovered the additional text with the example in subclause G.5.2 “Scrambling” of “IEEE Std. 802.11TM-1999 (R2003)”, that I discovered that the commenter’s shortcut can cause difficulties.)

Whether one or two encoders are used (NES = 1 or NES = 2), either the six or twelve tail bits are inputted to the scrambler or the scrambler shift register must be clocked 6•NES times where the tail bits appear in the stream, because not doing one or the other would effect the scrambler’s output for the padding bits, and hence effect the input values for BCC encoder.  In 17.3.5.2, the SERVICE field, the PSDU, the tail bits and the padding bits are treated as a single Data field.  The tail bits are specified to be set again to zero after scrambling the Data field and before encoding, but the scrambled padding bits remain untouched.  Not clearing the tail bits before the BCC encoder would effect both the encoded tail bits and the encoded padding bits.  While the padding bits are typically ignored by the receiver, the values of the transmitted padding bits will differ if the scrambler is not clocked 6•NES times when the tail bits should appear in the stream.  The 17.3.5.2 algorithm is the clearest algorithmic explanation.  Hardware implementations would tend to pipeline the stream.

That being said, I do want to thank the commenter for bringing subclause 20.3.10.4 to my attention.  There are two minor clarifications that need to be made to this subclause to be consistent with the previous subclause:

TGn Editor: In D2.0, page 263, line 30, modify the first sentence of the paragraph as follows:

If two BCC encoders are used, the scrambled data bits are divided between the encoders by sending alternating bit to different encoders.

TGn Editor: In D2.0, page 263, line 38, modify the text as follows:

Following the parsing operation, 6 scrambled “zero” bits following the end of the message bits in each FECBCC input sequence are replaced by unscrambled “zero” bits, as described in 17.3.5.2.

# (17) CID 2708 (20.3.10.6.1: page 264; line 54)

Proposed Resolution:  Counter

Discussion:  The commenter points out that the standard cannot have a normative requirement to “apply”, only to behave in a certain way when certain events occur.  There is an additional editorial flaw in this sentence regarding the subscripting of NES and NSS.  Finally, the value of NES does not apply to LDPC encoding.

TGn Editor: In D2.0, page 263, lines 52 to 54, modify the text as follows:

The rate-dependent parameters in Table n82 (Rate dependent parameters for mandatory 20 MHz, NSSNSS =1 MCSs, NESNES = 1) to Table n96 (Rate-dependent parameters for optional 40 MHz, NSSNSS = 4 MCSs) shall still apply to both LDPC and BCC encoding, with the exception that the NES parameter applies only to BCC encoding.

# (18) CID 2713 (20.3.10.6.6: page 269; line 6)

Proposed Resolution:  Counter

Discussion:  The commenter requests a clarification of where the code words are “sent”.  Timbuktoo is not the correct answer.  The intent of the first sentence of the paragraph is to specify that the succession of codewords is converted back into a bitstream.  The second sentence describes the ordering.  The third sentence identifies the destination processing for the bitstream.  The following text clarifies this intent.  The third sentence remains specifying the destination of the output.

See Discussion of CID #3232 which relates to the replacement of “shortened and punctured codewords that result from the encoding process” with “codewords that results from the encoding process of 20.3.10.6.5 (LDPC PPDU encoding process)”.   The formerlanguage could be mistakenly interpreted to mean that all codewords are shortened and punctured.

TGn Editor: In D2.0, page 269, lines 6 to 10, modify the text as follows:

The succession of LDPC shortened and punctured codewords that result from the encoding process of 20.3.10.6.5 (LDPC PPDU encoding process) shall be sent converted into a bitstream in sequential fashion.  Within each codeword, bit i0 is sent ordered first.  The parsing of this encoded data stream into spatial streams shall follow exactly the parsing rules defined for the BCC encoder, as defined in 20.3.10.7.1.

# (19) CID 2715 (20.3.10.7.3: page 270; line 36)

Proposed Resolution:  Counter

Discussion:  The commenter points out that “skip” is an instruction to the reader, and not a description of an operation performed by a STA.  The proposed resolution employs the commenter’s first alternative “Proposed Change”.

TGn Editor: In D2.0, page 270, line 36, modify the text as follows:

If LDPC encoding was used, no frequency interleaving is performed, hence skip to the parsed streams are immediately mapped to symbols as defined in 20.3.10.8 (QAM mapping) in order to map the parsed streams.

# (20) CID 2772 (20.3.23: page 306; line 6)

Proposed Resolution:  Defer and Assign to Eldad Perahia

Discussion:  Eldad Perahia has been assigned to similar Comments (CID #2773 and #2275) on the PLCP and PMD transmit and receive procedures.  He has volunteered to resolve this comment.

# (21) CID 2784 (20.5.5.3.2: page 321; line 24)

Proposed Resolution:  Defer and Assign to Eldad Perahia

Discussion:  Eldad Perahia has been assigned to similar Comments (CID #2773 and #2275) on the PLCP and PMD transmit and receive procedures.  He has volunteered to resolve this comment.

# (22) CID 2797 (G.9.1: page 430; line 43)

Proposed Resolution:  Accept

TGn Editor: In D2.0, page 430, line 43 and 44, modify the text as follows:

NOTE 1—the message for LDPC example 1 is identical to the corrected message for the BCC example, meaning that the FCS field (octets 97-100) has the correctsame CRC 32 value.

# (23) CID 2798 (G.9.2: page 430; line 57)

Proposed Resolution:  Counter

Discussion:  Accept in principle the commenter’s “Proposed Change” to delete the text referring to the correction, but combine the preceding and succeeding sentences to read like the text preceding other Tables.

TGn Editor: In D2.0, page 430, lines 55 to 57, modify the text as follows:

The resulting 816 bits are shown in Table G.34, with the bit-

NOTE—the CRC32 value is correct in bits 784-815.  Bit-ordering of octets is being most significant bit first.

# (24) CID 2977 (A.4.17.2: page 364; line 58)

Proposed Resolution:  Defer

Discussion:  The commenter states, “LDPC decoder mandatory at AP:  For the single antenna client, 802.11n only provides LDPC encoding to increase range on the uplink.  The capability to receive LDPC frames should be mandatory at the AP.”  There is a similar comment by the commenter regarding increased range on the downlink for single antenna clients by making  STBC transmission capability mandatory at the AP.  Discussion needed.

# (25) CID 3231 (20.3.10.6.5: page 267; line 46)

Proposed Resolution:  Counter

Discussion:  The commenter points out that the word “encode” appears both in step c) and in step f), which might suggest to the reader that the data is encoded twice.  It is accepted in principle that the redundancy should be removed, except that an earlier commenter noted that the encoding step was not mentioned in step c), where it belongs between the shortening bits being added and then the shortening bits being removed.  Therefore, the proper place to remove the redundant use of the word “encode” is in step f).  The clarification of paragraph f) is to state the data is processed codeword-by-codeword using steps c) through step e).

TGn Editor: In D2.0, page 268, lines 58 to 62, modify the text as follows:

f) For each of the NCW codewords, process Encode the data per 20.3.10.6.3 (LDPC encoder), 20.3.10.6 (Low density parity check (LDPC) codes), and 20.3.10.6.4 (Parity check matrices) using the number of shortening bits per codeword as computed in step c) for encoding, and puncture or repeat bits per codeword as computed per step d) and e), as illustrated in Figure n74 (LDPC PPDU encoding padding and puncturing of a single codeword).

# (26) CID 3232 (20.3.10.6.6: page 269; line 6)

Proposed Resolution:  Counter

Discussion:  The commenter correctly points out that the sentence may mistakenly imply that all codewords are shortened and punctured.  The commenter also correctly points out that repetition of codewords for padding purposes is not mentioned.  Actually, there are conditions where each of the shortening, puncturing and repetition of codewords steps are bypassed (e.g. the length of the encoded data is exactly a multiple of the longest codeword size).  Only the encoding step is dictated in all cases.

Because CID #2713 changes this paragraph, see CID #2713 for the actionable text.  The phrase “shortened and punctured codewords that result from the encoding process” is replaced with “codewords that results from the encoding process of 20.3.10.6.5 (LDPC PPDU encoding process)”.

# (27) CID 3389 (20.3.3: page 233; line 13)

Proposed Resolution:  Counter

Discussion:  The commenter asks to qualify the usage of the Interleaver to BCC encoding, as the Interleaver does not apply to LDPC encoding.  However, Figure n63 only applies to the non-HT portion and the HT signal field of the HT mixed format packet.  In these cases, BCC encoding is always used and LDPC encoding is never used.  Because the Interleaver is always associated with BCC encoding (and never with LDPC encoding), the Interleaver is always used in Figure n63.

LDPC encoding and bypassing the Interleaver (in the LDPC case only) applies to the HT portion of the data field.  The co mmenters submitted CID #3390 to address Figure n64.

Rather than rejecting the comment, the fact that the comment is confused by Figure n63 seems to indicate some clarity is required in the figure.  The modification that comes to mind is to remove the ambiguity of “FEC Encoder” and specify the “BCC Encoder”..

TGn Editor: In D2.0, page 233, line 13, replace the text below in the second box from the left of Figure n63 as follows:

FECBCC Encoder

# (28) CID 3390 (20.3.3: page 234; line 31)

Proposed Resolution:  Counter

Discussion:  The commenter correctly points out that the the “Interleaver” box in Figure n64 applies only to the BCC encoder and not to the LDPC encoder.

TGn Editor: In D2.0, page 234, lines 31, insert the text below in Figure n64 as follows:

—the stream parser may have 1,2, 3 or 4 outputs

—when LDPC encoding, the interleavers are not used

—when STBC is used, the STBC block has more outputs than inputs