IEEE P802.11
Wireless LANs

| LB97 HT WEP TKIP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |
| Henry Ptasinski | Broadcom | 190 Mathilda Place Sunnyvale, CA 94040 | +1 408 543 3316 | henryp@broadcom.com |

Abstract

This document contains proposed resolutions for three CIDs from the MAC group comments of LB97 that relate to the use of WEP and TKIP by HT STA.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

General discussion:

These comments:

| CID | Commentor | Page | Clause | Proposed Resn Status | Comment | Proposed Change | Proposed Resolution |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 300 | Cam-Winget, Nancy | 90.19 | 8.2 |  | This statement should be more proactive to the use of CCMP rather than a passive "should not use WEP or TKIP". | Either delete the sentence or replace with "HT STA shall use CCMP when communicating with other STA." | Counter – delete the text as per doc 11-07-0590r1. That doc proposes some changes to other parts of clause 8 that create the same effect as that desired by the commentor but in a manner that is more consistent with other parts of clause 8. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 830 | Lefkowitz, Martin | 90.19 | 8.2 |  | "HT STA should not use WEP or TKIP when communicating with other STA that support stronger ciphers. HT STA shall not use pre-RSNA security methods to protect unicast frames if the RA or address1 of the frame corresponds to an HT STA." The two sentances above mean the same thing. | Delete the second sentence. | Counter – the two sentences are different – the second refers exlusively to HT-STA-HT-STA links and the first is more broad. However, CID 1113 suggested another change that resulted in a deletion of the referenced text as per 11-07-0590r1. |
| 1113 | Marshall, Bill | 90.38 | 8.3.1 |  | pairwise ciphers are negotiated between the STAs. This statement says that when the negotiated results is TKIP, the STAs shall not communicate | Delete this sentence. Insert at end of first paragraph of 8.4.2 "A STA that has dot11HighThroughputOptionImplemented set to true shall include CCMP as an available pairwise cipher suite." Insert at end of third paragraph of 8.4.3 "A STA that has dot11HighThroughputOptionImplemented set to true shall not select TKIP as the pairwise cipher suite if CCMP is advertised by the AP." | Counter – the conclusion drawn by the commentor is correct – HT STA shall not communicate using TKIP. Group agrees to add more language and delete some language in order to indicate how pairwise cipher is selected, not exactly as commentor suggests, but as per 11-07-0590r1. |

TGn Editor: Delete all of the changes shown for subclause “8.2 Pre-RSNA Security methods” on page 90 at approximately lines 3 through 24 of TGn draft D2.0.

TGn Editor: Delete the editor instruction that begins with “Inser the following new paragraph” and continuing through and including the entirety of the sentence that begins with “An HT STA shall not use TKIP as the pairwise cipher suite” from the changes to sublcause “8.3.1 Overview” on page 90 near line 35 of TGn draft D2.0.

TGn Editor: Insert the following text and editing instructions immediately preceding the header for subclause “8.6.3 Mapping PTK to CCMP keys” on page 91 near line 58 of TGn draft D2.0:

8.4.3 RSNA policy selection in an ESS

Change the third paragraph of 8.4.3 as follows:

The STA’s SME initiating an association shall insert an RSN information element into its (Re)Association Request; via the MLME-ASSOCIATE.request primitive, when the targeted AP indicates RSNA support. The initiating STA’s RSN information element shall include one authentication and pairwise cipher suite from among those advertised by the targeted AP in its Beacon and Probe Response frames. It shall also specify the group cipher suite specified by the targeted AP. If at least one RSN information element field from the AP’s RSN information element fails to overlap with any value the STA supports, the STA shall decline to associate with that AP. A STA with a value of true for the MIB attribute dot11HighThroughputOptionImplemented shall eliminate TKIP as a choice for the pairwise cipher suite if CCMP is advertised by the AP or if the AP included an HT Capabilities element in its Beacons and Probe Responses. The elimination of TKIP as a choice for the pairwise cipher suite may result in a lack of overlap of the remaining pairwise cipher suite choices, in which case, the STA shall decline to associate with that AP.

8.4.4 RSNA policy selection in an IBSS

Change the first paragraph of 8.4.4 as follows:

In an IBSS, all STAs must use a single group cipher suite, and all STAs must support a common subset of pairwise cipher suites. However, the SMEs of any pair of non-HT-STAs may negotiate to use any common pairwise cipher suite they both support. Each STA shall include the group cipher suite and its list of pairwise cipher suites in its Beacon and Probe Response messages. Two STAs shall not establish a PMKSA unless they have advertised the same group cipher suite. Similarly, the two STAs shall not establish a PMKSA if the STAs have advertised disjoint sets of pairwise cipher suites. A STA with a value of true for the MIB attribute dot11HighThroughputOptionImplemented shall eliminate TKIP as a choice for the pairwise cipher suite if CCMP is advertised by the other STA or if the other STA included an HT Capabilities element in its Beacons and Probe Responses. The elimination of TKIP as a choice for the pairwise cipher suite may result in a lack of overlap of the remaining pairwise cipher suites choices, in which case, the STAs shall not exchange encrypted frames.

References: