IEEE P802.11
Wireless LANs

| LB97 operating mode bits |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document contains a proposed resolution for CID 1832 of LB97 from the COEX group of comments. CID 1832 asks for clarification of an inconsistency in the descriptions of the operating mode bit settings. This document attempts to provide a solution for that problem. The following additional CIDs relate to the same issue, or to other issues with respect to the operating mode bits of the HT Information element, and therefore, these comments could also be addressed with the adoption of this document: CID 16, 18, 317, 318, 457, 460, 550, 1830, 1831, 1937, 1938, 2830.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

General discussion:

CID 1832 proposes a clarification of the use of the operating mode bits of the HT Information element. Specifically, some of the bit combination usage descriptions seem to overlap. The following proposal is an attempt to create a consistent usage for the bits that does not impact the protection mechanism descriptions of Clause 9.13 because that clause was written with various assumptions about the meaning of the operating mode bit values that are reflected in this proposal.

| CID | Commentor | Page | Clause | Proposed Resn Status | Comment | Proposed Change | Proposed Resolution |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 1832 | Qian, Luke | 73.30 | 7.3.2.50 | A | The rules to set operating mode to 0 is incosistent with the rules to set it to none zero. For example, when all STAs in the BSS are 20/40 MHz STAs, it should be set to 0. However, in the mean time there may be some non-HT STAs in other BSS in the primary channel therefore the operating bit should be set to 1 accordingly. | Fix the inconsistency. Recommend descirbing the use of the two bits individually. | Accept – changed to the description as found in document 11-07-xxxxry and in the subsequent draft(s). |
| --- | --- | --- | --- | --- | --- | --- | --- |

TGn Editor: Change the contents of the “Encoding” column box for the row that contains “Operating Mode” for the value found in the “Field” column of table n30 “HT Information element” on page 73 at about line 30 in TGn draft D2.0 as follows:

Set to 0 if:

- There are no non-HT STAs in the primary or the secondary channel or associated with this BSS

- And either,

- all STAs associated with this BSS are 20/40 MHz HT

- in a 20/40 MHz BSS,

- or

- this BSS is a 20 MHz BSS

Set to 1 (HT non-member protection mode) if:

- there is at least one non-HT STA in either the primary or the secondary channel, but not associated with this BSS

- And either,

- all STAs associated with this BSS are 20/40 MHz HT in a 20/40 MHz BSS,

- or

- this BSS is a 20 MHz BSS

Set to 2 if:

- There are no non-HT STAs in the primary or the secondary channel or associated with this BSS

- And

- This BSS is a 20/40 MHz BSS

- And

- There is at least one 20 MHz HT STA associated with this BSS

Set to 3 (HT mixed mode) if:

- there is at least one non-HT STA associated with this BSS

- And

- This BSS is a 20/40 MHz BSS

- Or

- This BSS is a 20 MHz BSS

| 16 | Adachi, Tomoko | 7.3.2.50 | 73 | 31 | "- all STAs in the BSS are 20/40 MHz HT, or - in a 20/40 MHz BSS, or" Correct this. | Change the cited part to "- all STAs in the BSS are 20/40 MHz HT STAs in a 20/40 MHz BSS, or". | Counter - see CID 1832 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | Adachi, Tomoko | 7.3.2.50 | 73 | 39 | For 1, do non-HT STAs need to be in both the primary channel and the secondary channel even if their presence is may? | Change "if there may be non-HT STAs in both the primary and the secondary channel" to "if there may be non-HT STAs in the primary or the secondary channel". | Counter - see CID 1832 |
| 317 | Chan2, Douglas | 7.3.2.50 | 73 | 32 | "-in a 20/40 MHz BSS", unclear | clarify it. | Counter - see CID 1832 |
| 318 | Chan2, Douglas | 7.3.2.50 | 73 | 38 | "in both the primary and the secondary channel", incorrect | should be "in either the primary or the secondary channel" | Counter - see CID 1832 |
| 457 | Chan, Douglas | 7.3.2.50 | 73 | 32 | "-in a 20/40 MHz BSS", unclear | clarify it. | Counter - see CID 1832 |
| 460 | Chan, Douglas | 7.3.2.50 | 73 | 38 | "in both the primary and the secondary channel", incorrect | should be "in either the primary or the secondary channel" | Counter - see CID 1832 |
| 550 | Dorsey, John | 7.3.2.50 | 73 | 38 | The directions for the value 1 speak of "non-HT STAs in both the primary and the secondary channel." This says that if there are non-HT STAs in one channel, but not the other, then this protection signaling does not occur. | Change "both the primary and the secondary" to "either the primary or the secondary". | Counter - see CID 1832 |
| 1830 | Qian, Luke | 7.3.2.50 | 73 | 32 | "-in a 20/40 MHz BSS", unclear | clarify it. | Counter - see CID 1832 |
| 1831 | Qian, Luke | 7.3.2.50 | 73 | 38 | "in both the primary and the secondary channel", incorrect | should be "in either the primary or the secondary channel" | Counter - see CID 1832 |
| 1937 | Smith, Matt | 7.3.2.50 | 73 | 30 | Conditions for setting Operating Mode to "0" are confusing. | Clarify conditions for setting mode to "0". | Counter - see CID 1832 |
| 1938 | Smith, Matt | 7.3.2.50 | 73 | 38 | Intention is unclear for mode "1" | Change "in both the primary and secondary channel" to "in either the primary or secondary channels" | Counter - see CID 1832 |
| 2830 | Trainin, Solomon | 7.3.2.50 | 73 | 33 | The definition of the operating mode =0 is not clear and looks like typo | Replace the "— all STAs in the BSS are 20/40 MHz HT, or — in a 20/40 MHz BSS, or" by " — all STAs in the 20/40 BSS are 20/40 MHz HT, or" | Counter - see CID 1832 |

References: