IEEE P802.11
Wireless LANs

| LB97 operating mode bits |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document contains a proposed resolution for CID 1832 of LB97 from the COEX group of comments. CID 1832 asks for clarification of an inconsistency in the descriptions of the operating mode bit settings. This document attempts to provide a solution for that problem. The following additional CIDs relate to the same issue, or to other issues with respect to the operating mode bits of the HT Information element, and therefore, these comments could also be addressed with the adoption of this document: CID 16, 18, 317, 318, 457, 460, 550, 1830, 1831, 1937, 1938, 2830.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

General discussion:

CID 1832 proposes a clarification of the use of the operating mode bits of the HT Information element. Specifically, some of the bit combination usage descriptions seem to overlap. The following proposal is an attempt to create a consistent usage for the bits that does not impact the protection mechanism descriptions of Clause 9.13 because that clause was written with various assumptions about the meaning of the operating mode bit values that are reflected in this proposal.

| CID | Commentor | Page | Clause | Proposed Resn Status | Comment | Proposed Change | Proposed Resolution |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 1832 | Qian, Luke | 73.30 | 7.3.2.50 | A | The rules to set operating mode to 0 is incosistent with the rules to set it to none zero. For example, when all STAs in the BSS are 20/40 MHz STAs, it should be set to 0. However, in the mean time there may be some non-HT STAs in other BSS in the primary channel therefore the operating bit should be set to 1 accordingly. | Fix the inconsistency. Recommend descirbing the use of the two bits individually. | Accept – changed to the description as found in document 11-07-0591r2 and in the subsequent draft(s). |
| --- | --- | --- | --- | --- | --- | --- | --- |

TGn Editor: Change the contents of the “Encoding” column box for the row that contains “Operating Mode” for the value found in the “Field” column of table n30 “HT Information element” on page 73 at about line 30 in TGn draft D2.0 as follows:

Set to 0 if:

- There are no non-HT STAs detected in the primary or the secondary channel or associated with this BSS

- And either,

- all STAs associated with this BSS are 20/40 MHz HT

- in a 20/40 MHz BSS,

- or

- this BSS is a 20 MHz BSS

Set to 1 (HT non-member protection mode) if:

- there is at least one non-HT STA detected in either the primary or the secondary channel or both the primary and secondary channels, but not associated with this BSS

- And either,

- all STAs associated with this BSS are 20/40 MHz HT in a 20/40 MHz BSS,

- or

- this BSS is a 20 MHz BSS

Set to 2 if:

- There are no non-HT STAs detected in the primary or the secondary channel

- And

- This BSS is a 20/40 MHz BSS

- And

- There is at least one 20 MHz HT STA associated with this BSS

Set to 3 (HT mixed mode) otherwise.

| 16 | Adachi, Tomoko | 73.31 | 7.3.2.50 | C | "- all STAs in the BSS are 20/40 MHz HT, or - in a 20/40 MHz BSS, or" Correct this. | Change the cited part to "- all STAs in the BSS are 20/40 MHz HT STAs in a 20/40 MHz BSS, or". | Counter - see CID 1832 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | Adachi, Tomoko | 73.39 | 7.3.2.50 | C | For 1, do non-HT STAs need to be in both the primary channel and the secondary channel even if their presence is may? | Change "if there may be non-HT STAs in both the primary and the secondary channel" to "if there may be non-HT STAs in the primary or the secondary channel". | Counter - see CID 1832 |
| 317 | Chan2, Douglas | 73.32 | 7.3.2.50 | C | "-in a 20/40 MHz BSS", unclear | clarify it. | Counter - see CID 1832 |
| 318 | Chan2, Douglas | 73.38 | 7.3.2.50 | C | "in both the primary and the secondary channel", incorrect | should be "in either the primary or the secondary channel" | Counter - see CID 1832 |
| 457 | Chan, Douglas | 73.32 | 7.3.2.50 | C | "-in a 20/40 MHz BSS", unclear | clarify it. | Counter - see CID 1832 |
| 460 | Chan, Douglas | 73.38 | 7.3.2.50 | C | "in both the primary and the secondary channel", incorrect | should be "in either the primary or the secondary channel" | Counter - see CID 1832 |
| 550 | Dorsey, John | 73.38 | 7.3.2.50 | C | The directions for the value 1 speak of "non-HT STAs in both the primary and the secondary channel." This says that if there are non-HT STAs in one channel, but not the other, then this protection signaling does not occur. | Change "both the primary and the secondary" to "either the primary or the secondary". | Counter - see CID 1832 |
| 1830 | Qian, Luke | 73.32 | 7.3.2.50 | C | "-in a 20/40 MHz BSS", unclear | clarify it. | Counter - see CID 1832 |
| 1831 | Qian, Luke | 73.38 | 7.3.2.50 | C | "in both the primary and the secondary channel", incorrect | should be "in either the primary or the secondary channel" | Counter - see CID 1832 |
| 1937 | Smith, Matt | 73.30 | 7.3.2.50 | C | Conditions for setting Operating Mode to "0" are confusing. | Clarify conditions for setting mode to "0". | Counter - see CID 1832 |
| 1938 | Smith, Matt | 73.38 | 7.3.2.50 | C | Intention is unclear for mode "1" | Change "in both the primary and secondary channel" to "in either the primary or secondary channels" | Counter - see CID 1832 |
| 2830 | Trainin, Solomon | 73.33 | 7.3.2.50 | C | The definition of the operating mode =0 is not clear and looks like typo | Replace the "— all STAs in the BSS are 20/40 MHz HT, or — in a 20/40 MHz BSS, or" by " — all STAs in the 20/40 BSS are 20/40 MHz HT, or" | Counter - see CID 1832 |

Treatment of CID 2134, 2509

Several choices:

Accept the resolutions shown in the following table, or use the text that follows the table, or ??:

Straw Poll:

Use the proposed resolutions in the table:

Y:

N:

Use the text below the table:

Y:

N:

Other:

| 2134 | Stephens, Adrian | 85.05 | 7.4.9.10 | C | “The STA Channel Width bit indicates the current operating width of the transmitting STA. An HT-AP that indicated a value of 0 in its most recently transmitted STA Channel Width field shall not transmit a 40 MHz mask PPDU.” This creates a problem. The semantics of the STA Channel Width field in the HT information element is somewhat different. The intention of that field was that a STA can monitor activity on the secondary channel and recommend its peer to send 20MHz frames if 40MHz frames are likely to be 11nterfered with in the locality. It is specifically not a switch of receiver capability – i.e. a 40MHz frame may or may not get through. The point is that a STA updating this field has no effect on the Operating Mode field, which is required to distinguish support of 20MHz-only STA. However, “current operating width” is the “receiver’s current capability” | “Remove operating mode 2 (because there is no behaviour associated with it). Explicitly allow a STA that sends a Notify Channel Width to change its mask. Require that a peer STA honour the Notify Channel Width request with a “”should”” statement.” | Counter – The first part of the second requested change by the commentor is not needed, since the converse is stated on page 167 at line 12 of D2.0. The second part of the second requested change by the commentor is added per changes in doc 11-07-0614r6. The operating mode 2 definition is changed as per 11-07-0591r3, rather than removing that mode. It is noted that mode 2 provides information to receivers that can be used by those receivers to modify behaviour, but that modification is not mandatory. So, while there is no mandatory behaviour associated with mode 2, there is optional behaviour. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2509 | Stephens, Adrian | 165.47 | 9.20.4 |  | "Recommending a transmission width of 20 MHz has no effect on the BSS requirements regarding protection of 40 MHz transmissions or on the STA Channel Width advertised by the associated AP. A STA should reassociate with its Supported Channel Width Set field set to 0 (indicating only 20 MHz operation) to obtain this protection." I believe that: 1. Notify Channel Width should allow a change in receiver operating channel width 2. Operating mode 2 is useless. Operating mode of 0 is usually paired with 2. The only place in the spec where it doesn't is in 11.16 (PCO). However, it makes no sense, IMHO for an AP to be operating PCO unless there are active 20MHz STA, so no reason to explicitly signal their presence. If operating mode 2 is removed, there are no BSS requirements regarding protection of 40 MHz transmissions dependent on the presence of 20MHz HT STA and the quoted paragraph can be removed. | Reserve operating mode 2 in n30. Remove all references to operating mode 2. Remove the quoted paragraph. | Counter – see CID 2134 |

CID 2134, 2509

TGn Editor: Insert the following text as a new sentence to appear at the end of the subclause “9.13.3.1 General” at about page 127 line 58 in TGn draft D2.0 as follows:

When the Operating Mode field is set to 0 and the Secondary Channel Offset field is set to a non-zero value, then a 40 MHz HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to HT_CBW40) may initiate a TXOP. When the Operating Mode field is set to a non-zero value or the Secondary Channel Offset field is set to zero, then a 40 MHz HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to HT_CBW40) shall not initiate a TXOP.

References: