IEEE P802.11
Wireless LANs

| LB97 PHY Comment Resolutions: Smoothing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |
| Tomoya Yamaura | Sony Corp. | 5-1-2 Kitashinagawa Shinagawa-ku, Tokio 1410001 Japan | +81 3 5448 4018 | yamauratomoya@yahoo.co.jp |

Abstract

This submission suggests resolutions of LB97 PHY comments related to the sub-topic Smoothing. The following CIDs are addressed: 1673, 2651, and 2961.

The changes marked in this document are based on TGn Draft 2.0

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CID        Sec.            Pg.          Ln.             Comment                         Proposal                        Proposed Resolution

| 1673 | 20.3.2 | 224 | 19 | Indicates whether frequency-domain smoothing is recommended as part of channel estimation. | How is this recommendation made? | Counter, accept in principle. Add a note in the table that refers to the following paragraph in Sec. 20.3.10.10.1 : “If 95 percent of the sum of the energy from all impulse responses of the time domain channels between all space time streams and all transmit chain inputs, induced by the CSD added according to Table n62 (Cyclic shift values of HT portion of the packet) and the frequency-dependence in the matrix , is contained within 800 ns, the smoothing bit should be set to 1, otherwise it shall be set to 0.” |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 224, line 22, Table n56, 3nd column, add the following note:

“NOTE – Setting of the smoothing bit is defined in Sec. 20.3.10.10.1 (Spatial mapping).”

| 2651 | 20.3.2 | 224 | 19 | The MAC needs to know how to set each TXVECTOR parameter. So what are the rules under which SMOOTHING is set described? | Add to the MAC a description of how to set the SMOOTHING parameter, or remove it from the VECTORs. | Counter. See CID 1673 proposed resolution. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter, added a note in Table n56 - see CID 1673 proposed resolution.

| 2961 | 20.3.9.4.3 | 250 | 12 | Both the HT-Signal Fields in Table n63 of subclause 20.3.9.4.3 page 250, lines 12-15 and the TXVECTOR Fields in Table n56 of subclause 20.3.2, page 224, lines 10-48, are unclear about the usage of all 8 possible combinations of the "Not Sounding" bit, the "Smoothing" bit, and the "HT-LENGTH" field being zero or non-zero. Please consider the table submitted in Doc #11-07/0317r0 as context for the clarification and possible incorporation into the draft. | Clarify the usage of all 8 combinations of the "Not Sounding" bit, the "Smoothing" bit, and the "HT-LENGTH" field being zero or non-zero. Please consider the table submitted in Doc #11-07/0317r0 as context for the clarification and possible incorporation into the draft. | Counter. Table may not be necessary. Include a note that when Length = 0 then Not Sounding shall be set to 0. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter

TGn Editor: on page 250, line 21, Table n63 (Not Sounding row), add the following sentence:

“When Length = 0 then Not Sounding shall be set to 0.”