IEEE P802.11
Wireless LANs

| LB97 COEX Comment Resolutions: 20/40 MHz Coexistence |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |
| Matthew Fischer | Broadcom |  |  | mfischer@broadcom.com |

Abstract

This submission suggests resolutions of LB97 COEX comments related to the sub-topic 20/40 MHz. The following CIDs are addressed: 186, 16, 18, 317, 318, 457, 460, 550, 715, 716, 717, 1740, 1830, 1831, 1832, 1867, 1937, 1938, 2465, 2466, 2467, 2468, 2830, 2964, 3257, 3271, 3272, and 3277.

CID 1832 and CIDs referred to it are covered in submission 11-07-0591r1.

CIDs 1740, 1867, 3271, and 2468 are covered in submission 11-07-0614r0.

The changes marked in this document are based on TGn Draft 2.0

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CID    Sec.     Pg.    Ln.             Comment                                  Proposal                        Proposed Resolution

| 186 | 9.20 | 163 | 23 | 20/40 Functional description: When a 20 MHz station is transmitting or receiving on the primary channel, none of the other channels can be used for other transmissions in the BSS. It is possible, however, to increase the BSS throughput even beyond what is achievable by a 40 MHz channel if the MAC included the option for multichannel use, while respecting use of the same channels by OBSSs. | Provide a way for other channel(s) to be used simultaneously with the primary channel in a BSS, while observing virtual carrier sense on all channels. The CCC MAC makes this possible. It can be used in the 5 GHz band alone and in both the 5 GHz and the 2.4 GHz bands together. | Withdrawn by commenter 3/14/07 |
| --- | --- | --- | --- | --- | --- | --- |

| 16 | 7.3.2.50 | 73 | 31 | "- all STAs in the BSS are 20/40 MHz HT, or - in a 20/40 MHz BSS, or" Correct this. | Change the cited part to "- all STAs in the BSS are 20/40 MHz HT STAs in a 20/40 MHz BSS, or". | Counter - see CID 1832, document 11-07-0591r1. |
| --- | --- | --- | --- | --- | --- | --- |
| 18 | 7.3.2.50 | 73 | 39 | For 1, do non-HT STAs need to be in both the primary channel and the secondary channel even if their presence is may? | Change "if there may be non-HT STAs in both the primary and the secondary channel" to "if there may be non-HT STAs in the primary or the secondary channel". | Counter - see CID 1832, document 11-07-0591r1. |
| 317 | 7.3.2.50 | 73 | 32 | "-in a 20/40 MHz BSS", unclear | clarify it. | Counter - see CID 1832, document 11-07-0591r1. |
| 318 | 7.3.2.50 | 73 | 38 | "in both the primary and the secondary channel", incorrect | should be "in either the primary or the secondary channel" | Counter - see CID 1832, document 11-07-0591r1. |
| 457 | 7.3.2.50 | 73 | 32 | "-in a 20/40 MHz BSS", unclear | clarify it. | Counter - see CID 1832, document 11-07-0591r1. |
| 460 | 7.3.2.50 | 73 | 38 | "in both the primary and the secondary channel", incorrect | should be "in either the primary or the secondary channel" | Counter - see CID 1832, document 11-07-0591r1. |
| 550 | 7.3.2.50 | 73 | 38 | The directions for the value 1 speak of "non-HT STAs in both the primary and the secondary channel." This says that if there are non-HT STAs in one channel, but not the other, then this protection signaling does not occur. | Change "both the primary and the secondary" to "either the primary or the secondary". | Counter - see CID 1832, document 11-07-0591r1. |

| 715 | 9.20.1 | 163 | 30 | “operating in 20MHz mode” – what is meant by “20 MHz mode” | Define 20MHz mode using PLME parameters or MASK | Counter – delete the sentence. See also CID 2465: “Rules in this spec use the terms 20MHz capable and refer to the supported channel width set as synonyms. Now we're explicitly introducing the possibility that a STA can "lie" about its capabilities.” |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter.

TGn Editor: on page 163, line 30, delete the following sentence:

“A 20/40 capable STA operating in 20 MHz mode follows the rules for a 20 MHz capable STA.”

| 716 | 9.20.1 | 163 | 32 | "operates as defined in 11.16" does it means that the rules in the rest of this subclause (9.20) do not apply | Please clarify | Counter. Refer to CID 2468, see submission 11-07-0614r0. |
| --- | --- | --- | --- | --- | --- | --- |

| 717 | 9.20.1 | 163 | 35 | “operates as defined in 9.13.5” – does it mean thet the rules in the rest of this subclause (9.20) do not apply | Please clarify | Counter. Refer to CID 2468, see submission 11-07-0614r0. |
| --- | --- | --- | --- | --- | --- | --- |

| 1740 | 9.20 | 163 | 23 | add a statement that 9.20 and all subclauses only applies to 20/40 capable STAs | As in comment | Counter - text is rewritten with newly defined terms to make it explicit in each instance as to which STA the rules of behavior apply - see document 11-07-0614r0. |
| --- | --- | --- | --- | --- | --- | --- |

| 1830 | 7.3.2.50 | 73 | 32 | “-in a 20/40 MHz BSS”, unclear | clarify it. | Counter - see CID 1832, document 11-07-0591r1. |
| --- | --- | --- | --- | --- | --- | --- |
| 1831 | 7.3.2.50 | 73 | 38 | “in both the primary and the secondary channel”, incorrect | should be “in either the primary or the secondary channel” | Counter - see CID 1832, document 11-07-0591r1. |

| 1832 | 7.3.2.50 | 73 | 30 | The rules to set operating mode to 0 is incosistent with the rules to set it to none zero. For example, when all STAs in the BSS are 20/40 MHz STAs, it should be set to 0. However, in the mean time there may be some non-HT STAs in other BSS in the primary channel therefore the operating bit should be set to 1 accordingly. | Fix the inconsistency. Recommend descirbing the use of the two bits individually. | Accept – changed to the description as found in document 11-07-0591r1 and in the subsequent draft(s). |
| --- | --- | --- | --- | --- | --- | --- |

| 1867 | 9.20.1 | 163 | 38 | The text “Otherwise, a 20/40 capable STA shall operate as defined in 9.20.2” implies that STAs operating under PCO and L-SIG TXOP protection (as mentioned in the previous two paragraphs in the same section) DO NOT need to comply with rules of section 9.20.2. | It is NOT always possible to fully protect against OBSS activities on the secondary channel and as such the HT STA must always check for secondary CCA at the beginning of the TXOP before it commences transmission. PCO and L-SIG TXOP protection mechanism cannot avoid this condition/requirement. Edit the text to reflect the requirement. | Counter, see submission 11-07-0614r0. |
| --- | --- | --- | --- | --- | --- | --- |

| 1937 | 7.3.2.50 | 73 | 30 | Conditions for setting Operating Mode to "0" are confusing. | Clarify conditions for setting mode to "0". | Counter - see CID 1832, document 11-07-0591r1. |
| --- | --- | --- | --- | --- | --- | --- |
| 1938 | 7.3.2.50 | 73 | 38 | Intention is unclear for mode "1" | Change "in both the primary and secondary channel" to "in either the primary or secondary channels" | Counter - see CID 1832, document 11-07-0591r1. |

| 2465 | 9.20.1 | 163 | 30 | "follows the rules for a 20 MHz capable STA." This is awkward. Rules in this spec use the terms 20MHz capable and refer to the supported channel width set as synonyms. Now we're explicitly introducing the possibility that a STA can "lie" about its capabilities. It's also inconsistent with the definition in table n25. The result is that roughly half the statements in the spec now need to be rewritten to say, "A 20 MHz capable STA, or a 20/40 MHz Capable STA that sets it Supported Channel Width set to 0 ..." | I think the easiest solution is to remove the quoted line. | Accept. Implemented in CID 715. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept. Implemented in CID 715.

| 2466 | 9.20.1 | 163 | 30 | “A 20/40 capable STA” There is no definition of this term | Add a definition of this term, or relate to existing 5ignaling. | Counter. Delete the sentence. Implemented in CID 715. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter. Implemented in CID 715.

| 2467 | 9.20.1 | 163 | 30 | "operating in 20 MHz mode" - relate to existing signalling | Replace with "that sets its Supported Channel Width Set field in the HT Capabilities element to 0" | Counter. Delete the sentence. Implemented in CID 715. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter. Implemented in CID 715.

| 2468 | 9.20.1 | 163 | 35 | “A 20/40 capable STA operating under L-SIG TXOP protection operates as defined in 9.13.5 (L-SIG TXOP protection).Otherwise, a 20/40 capable STA shall operate as defined in 9.20.2 (STA CCA sensing 20/40 MHz BSS) to 9.20.4 (Switching between 40 MHz and 20 MHz).” This appears to excuse a STA using L-SIG TXOP protection from anything in 9.20.2-9.20.4, including, for example, the need to operate the “intolerant” bit. Same comment for the previous line on PCO operation. | I think we need to establish clearly which of the rules in 9.20.2 are only operated if you are not PCO and are not L-SIG and then find a way to represent this exclusivity here. Needs a submission. | Counter, see submission 11-07-0614r0. |
| --- | --- | --- | --- | --- | --- | --- |

| 2830 | 7.3.2.50 | 73 | 33 | The definition of the operating mode =0 is not clear and looks like typo | Replace the "— all STAs in the BSS are 20/40 MHz HT, or — in a 20/40 MHz BSS, or" by " — all STAs in the 20/40 BSS are 20/40 MHz HT, or" | Counter - see CID 1832, document 11-07-0591r1. |
| --- | --- | --- | --- | --- | --- | --- |

| 2964 | 9.13.1 | 127 | 46 | 20-40MHz Coexistence: non-HT duplicate and HT duplicate (MCS 32) modes do not work properly in 2.4GHz due to channel spacing that is not centered on 20MHz (normally 25MHz, i.e. channels 1,6,11.) | Maintain definition of 40MHz operation in the 5GHz band but preclude it from the 2.4GHz Band | Reject. Reason for rejection: coexistence mechanism are defined in Sections 7, 9, and 11 that allow for proper sharing of spectrum between 20 MHz and 40 Mhz devices in 2.4 GHz. |
| --- | --- | --- | --- | --- | --- | --- |

| 3257 | 9.20.4 | 164 | 32 | Need to add allowance to transmit control types in secondary channel | Make the change indicated in the comment. | Accept. Implemented in CID 3272. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept. Implemented in CID 3272.

| 3271 | 9.20 |  |  | make a reference to 11.9.8.3 Scanning prior to… -- there are additional rules about 20/40 MHz BSS operation in that subclause | Make the change indicated in the comment. | Counter - rewrite of 9.20 per submission 11-07-0614r0 removes the need for this reference. |
| --- | --- | --- | --- | --- | --- | --- |

| 3272 | 9.20.4 | 164 | 32 | Allow alternative 20/40 coexistence mechanisms. | Change "A non-AP STA that is a member of a 40 MHz BSS shall not transmit 20 MHz frames in the secondary channel." to "A non-AP STA that is a member of a 40 MHz BSS shall not transmit non-Control type 20 MHz frames in the secondary channel." | Accept. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 164, line 32-33, modify the sentence as follows:

“A non-AP STA that is a member of a 40 MHz BSS shall not transmit non-Control type 20 MHz frames in the secondary channel.”

| 3277 | 9.20 | 163 | 38 | wrong preposition - "to" means up to the beginning of that item, "through" means up to the end of that item | change "to" to "through" | Accept |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

TGn Editor: on page 163, line 38, modify the sentence as follows:

“Otherwise, a 20/40 capable STA shall operate as defined in 9.20.2 (STA CCA sensing 20/40 MHz BSS) to through 9.20.4 (Switching between 40 MHz and 20 MHz).”