IEEE P802.11
Wireless LANs

| LB97 CID 75 20-40 COEX assignee MattF CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +972547885738 | solomon.trainin@intel.com |
| Eldad Perahia | Intel |  |  | eldad.perahia@intel.com |

Abstract

This document contains proposed resolutions for most of the COEX group comments from LB97 that were assigned to MattF (Matthew Fischer). Only a few of the over 200 comments from that assignment group are not covered by this document. Those comments are expected to be reviewed individually by the COEX group or by a larger group. The comments addressed in this document address issues of detail relating to the mechanism for coexistence between 20/40 MHz BSS and 20 MHz BSS. Most of these comments relate to subclauses 9.20.4 and 11.9.8.3 and to some clause 7 subclauses that relate to frames that are involved in the signalling surrounding 20/40 MHz BSS operation.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

General discussion:

The comments for 20/40 COEX fall into various categories, not all of which are listed here, nor is each CID listed here:

- Questioning the use of 40 mhz BSS in 2.4 GHz channels

  - Forbid 40 mhz BSS in 2.4 GHz – CID 170, 171, 804, 2816

  - Allow the use of 40 mhz BSS in 2.4 GHz, with modified restrictions – CID 75, 1686, 1720

- Scanning responsibility

  - 40-mhz capable HT STA – CID 721

  - All associated HT STA – CID 1741

  - 40-mhz capable HT STA that indicate use of 40 mhz – CID 119

  - HT AP only – CID 1665

  - Add HT AP requirement for scanning – CID 1667

- Scanning style

  - Allow active probing for quick response – CID 1666, 1669

- Text clarifications – CID 42, 1729, 719, 720, 798, 1102, 1229, 1501, 1502, 1668, 1724, 1725, 1728, 1746, 1747, 1749, 1750, 1751, 1752, 1753, 1754, 1755, 1756, 1757, 1758, 1760, 1761, 1762, 1763, 1764, 1871, 1872, 1982, 2107, 2128, 2130, 2132, 2133, 2134, 2480,

- Responsibility for monitoring forty_mhz_intolerant bit

  - All HT STA

  - Only 40 MHz capable HT STA – CID 1730

- Reception of forty_mhz_intolerant (HT Info Exch) from neighbour BSS

  - do not allow – CID 1230

- 11k requirements clarification

  - Make it more explicit, but do not expand – CID 253

- Corner cases

  - CID 351

- IBSS

  - Disallow IBSS 40 MHz at 2.4 GHz - CID 722

- misc

  - CID 626, 1103, 1105, 2488

- Thirty minutes is too long

  - CID 1635, 1664, 2481,

- Beacon detection instead of traffic detection – CID 1748, 2482,

- Eliminate exception for low-bandwidth device – CID 1759

- Secondary channel CCA testing – CID 1817

- Legacy DUP response problem – CID 1891

- STA behaviour upon trigger detection (channel width) – CID 2485

General nature of the changes:

- modified affected channels definition (specifically name clause 19)

- deleted sensitive channels definition

- plethora of AP and STA definitions to create a shorthand for “operating in 2.4”, “operating in 5 GHz”, “forty MHz capable”, “not forty MHz capable” and all of the combinations thereof

- remove the Beacon Request AP-based scanning mechanism (i.e. use of 11k frame for AP-initiated scanning) – replace with a requirement for STA-initiated scanning that includes a new element to convey scanning parameter values from AP to STA

  - parameter elements can appear in beacons and association response frames and in HT Info mgmt frames, allowing the AP to provide individualized scanning parameters

- remodel the HT Information Exchange frame by:

  - move it to the Spectrum Mgmt category, since these frames are all Class 1, and therefore, never protected, thereby allowing cross-BSS signalling of 40MHz-intolerance

  - change the name to 20/40 BSS Coexistence mgmt frame (note that this is a global instruction – the doc still uses HT information exchange throughout)

  - change to two octets for info bits

  - allow inclusion of zero or more instances of the AP Channel Report element to indicate the channels on which other BSSs exist

  - remove “40 mhz intolerant” bit from this frame – use newly-named “20 MHz BSS Width Request” bit for signalling this from STA to AP – 40 mhz intolerant bit now only in the HT Cap element

  - move the field descriptions to subclause 7.1.3.34+

- created MIB variable dot11FortyMHzIntolerant and use it to tell when to set the bit

- redefined OBSS Scan operation using a new set of MIB variables with specific defaults and ranges for those new variables, now 1 minute for scanning, 5 minutes for width-switching

- created MIB variables for all of the overlapping BSS scan parameters

- rearranged 9.20.4, to separate the various functions and delete some text that was redundant to some text in 11.15.1 and moved some text to a new 11.15.1a

- carefully chose names of STA and AP, e.g. FC-HT-STA vs FC-HT-STA-19, etc. as appropriate

- deleted various paragraphs of 9.20.4 which were deemed by some commentors as extraneous

- redefined the BSS width trigger events, with specific emphasis on using BEACONS for detection, not traffic

- redefinition of BSS width trigger events includes a method for identifying the difference between events for 2.4 and 5 GHz, such that all trigger events and channel switching and scanning requirements are now marked as being applicable only for 2.4 GHz operations

- some specific changes with respect to addressing of frames – look for multicast

- in 11.9.8.3 – very explicit restrictions on starting a 40 mhz bss – scan before you start the BSS

In the CID listings the following should be noted:

- A small number of CIDs do NOT have a proposed resolution – this is to allow easier inclusion of the proposed resolutions into a spreadsheet later.

- Those CIDs without a resolution are generally given a shaded appearance.

- The table has been split into three pieces because I think that word behaviour is improved as a result

| CID | Commentor | Page | Clause | Proposed Resn Status | Comment | Proposed Change | Proposed Resolution |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 75 | Bagby, David | 0.00 | General | A | In LB 84 I made the comment that "The interoperation of the 20 MHZ and 40 MHZ modes of operation are inadequate and are considered not to work effectively. There are serious concerns over the impact of 40 MHZ operation on legacy 20 MHz devices - this is evidenced by the TG's own ad-hoc group which is debating these issues. This reviewer requires that TGn not have significant negative impact on legacy devices. Yes, I realize that "significant" is a value call - I will consider technical solutions proposed and adopted by TGn to see if they satisfy this requirement." I have monitored and participated in the discussions over the last year that have resulted in an attempt to resolve this comment (and similar comments); Unfortunately, TGN draft 2.0 has not adequately addressed the technical problems of 20/40 coexistence, particularly in the 2.4 GHz band. The current mechanism requires scanning for 20 devices by 40 devices and is an improvement, however significant technical problems remain with the Draft 2.0 approach. The problems include: 1) Conflicts can occur between 20 and 40 devices for significant periods of time (depending on the periodicity of scanning) - as the period between scans can be long, this can result in unacceptable amounts of interference; 2) Interference can become repetitive - the scanning patterns can result in quiet periods making 40 devices think there are no 20 devices present (overnight for example); which will result in an almost guaranteed "scanning period" duration of interference (ex: the next morning); 3) It does not appear that the channels to be scanned algorithm is sufficient to remove significant potential for interference (a 40 device in 2.4 would have to essentially scan all of the avail channels to check the primary and secondary 40 MHz widths (due to 2.4 Ghz channel overlap). If the scan period were adjusted to resolve 1) and 2), and then the scans were to include all potential interfering channels for 3), the result appears to be a lot of work on the 40 station's part - only to discover that it can not operate in 40 mode. So the commuter is back to believing that prohibition of 40 operation in 2.4 is a better solution. |  |  | Further rework the 20/20 interoperability mechanisms to remove negative impact on legacy devices. A solution which is still acceptable to this reviewer is to restrict TGn to only 20 MHz operation in the 2.4 GHz band and to allow 40MHz operation only in the 5Ghz band(s). I will also consider additional clever proposals on their merits. |  | Accept – a general rework of the 20/40 coexistence mechanism has been performed, addressing, it is believed, the concerns expressed by the commentor. The changes resulting from that work are found in document 11-07-xxxxry and in the subsequent draft(s). |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 351 | Chan2, Douglas | 0.00 | General | A | The current 20/40 coexistence scheme is not sufficient to cover commonly arising 40/40 coexisting scenarios. Specifically, consider if there is another 40 MHz BSS that has its secondary channel overlapping with us, but their primary has a different offset. For example, let's say we're using channels 1 and 5 respectively for secondary and primary, but the other BSS has 5 for secondary and 9 for primary. Then these OBSSs cannot each other. |  |  | Analyze and provide changes to solve this problem. Since beacons are only transmitted on the primary, the solution is likely that we would need to extend the number of scanned channels to avoid this. (In the example scenario provided here, we could solve it by scaning channels from 1 to 12. ) |  | Accept – make changes to 11.9.8.3 as shown in 11-07-xxxxry. |  |
| 3007 | Worstell, Harry | 0.00 | General |  | A 40MHZ channel bandwidth should Not be used in the 2.4GHz Band. It completely destroys the band for other users. |  |  | Remove the 40MHz channelization in the 2400-2485MHz band in all instances of the draft. |  |  |  |
| 3250 | Erceg, Vinko | 1.00 | 3 | C | Narrow the definition of the affected devices by qualifying an HT-AP-19 with the condition that the AP is capable of operating a 20/40 MHz BSS |  |  | change "HT-AP-19: An HT AP that is operating on a 20 MHz channel or set of 20 MHz channels that belong to the channel set that is defined in clause 19." to "HT-AP-19: An HT AP that is operating on a 20 MHz channel or set of 20 MHz channels that belong to the channel set that is defined in clause 19 and that indicates its capability to operate a 20/40 MHz BSS through the transmission of HT Capability Information elements that have a non-zero value in the Supported Channel Width Set field." |  | Counter – add another definition that includes the qualifier – see changes in 11-07-xxxxry |  |
| 1724 | Perahia, Eldad | 1.29 | 3 | A | clarify that 40 MHz affected channel only applies to 2.4 GHz |  |  | as in comment |  | Accept – see change to definition of 40 MHz affected channel as per 11-07-xxxxry |  |
| 1871 | Roy, Richard | 1.29 | 3 | R | The definition of 40 MHz affected channel is confusing and targetted to a specific system bandwidth. |  |  | Rewrite to eliminate mention of "seven" and make it clear what is intended which is presumably any 20MHz channel that is affected by a 40MHz channel in proximity. |  | Reject – the 40 mhz affected channels definition is only to be used with respect to a specific band and the description provides the required accuracy. |  |
| 1725 | Perahia, Eldad | 1.46 | 3 | A | clarify that 40 MHz sensitive channel only applies to 2.4 GHz |  |  | as in comment |  | Accept – change the definition of 40 MHz sensitive channel as per 11-07-xxxxry |  |
| 1872 | Roy, Richard | 1.46 | 3 | A | Why define a 40 MHz sensitive channel as "any 20MHz channel that a STA can operate on"? |  | Remove the definition of the term and replace its (one and only) use in the document with "any allowed 20 MHz channel" or something similar. |  |  | Accept – definition deleted, and text modified as found in 11-07-xxxxry |  |
| 1982 | Stephens, Adrian | 1.46 | 3 | A | The definition for 40 MHz sensitive channel is particularly useless. It translates to "any permitted channel". A STA cannot observe events outside the 40 MHz channel set, because to do so would be to operate outside the channels permitted by the regulatory domain. |  | Remove the definition and any references to it. |  |  | Accept – see CID 1872 and change in 11-07-xxxxry |  |
| 1729 | Perahia, Eldad | 64.18 | 7.3.2.49.2 | A | 40 MHz is optional. Is this use of the Forty MHz Intolerance bit mandatory for 20 MHz only APs? |  | please clarify |  |  | Accept – add the following sentence to 9.20.1 as indicated in 11-07-xxxxry – “A 20 MHz capable STA may set the Forty MHz Intolerant field to 1 in transmitted HT Capabilities elements.” |  |
| 1730 | Perahia, Eldad | 64.18 | 7.3.2.49.2 | C | A STA that is only 20 MHz capable should be able to ignore the Forty MHz Intolerant bit. Clarify this in the definition and encoding |  | as in comment |  |  | Counter – Changes to the bit fields are not needed, but behavioral descriptions in other subclauses need clarification: Create new definition of forty MHz capable HT-AP-19 (FC-HT-AP-19) to specifically exclude 20 MHz-only capable AP as per 11-07-xxxry |  |
| 1501 | Morioka, Yuichi | 64.18 | 7.3.2.49.2 | A | According to 9.20.4, this subfield is only used for operation at 2.4GHz. |  | Explicitly state this definition and encoding is for HT Capabilities field when it is transmitted at 2.4GHz band, and add encoding rule for it when it is transmitted at 5GHz (or other bands) as reserved. Or, explicitly state how to set/use this field at non-AP HT STA operating at 5GHz and at AP that is not HT-AP-19. |  |  | Accept – definitions of STA and AP include the appropriate band restriction – bit descriptions in 7.3.2.49.2 have been changed to include such language as per 11-07-xxxxry |  |
| 1502 | Morioka, Yuichi | 64.18 | 7.3.2.49.2 | C | Forty Intolerant subfield transmitted by non-AP HT STAs would be a kind of capability. But, the information transmitted by AP seems a kind of current operation mode, rather than capability. This means it would be better to place in IE, not in capability element. |  | Remove definition and encoding for transmission by AP. Replacing one reserve subfield in HT information element with "Forty Intolerant Mode", and give definition and encoding as same as existing ones for Forty Intolerant field in HT Capabilities element transmitted by AP for IEs at 2.4GHz, and define it as reserve for IEs at other bands.. Then rewrite 9.20.4 accordingly. |  |  | Counter – The bit is an advertisement of “intolerance” – either a STA is intolerant, or a BSS is intolerant. It is never used as a command bit for the local BSS. The BSS allowed channel width is determined by examining the secondary channel offset field of the HT Information element. – to clarify: Add text to the definition of the forty_mhz_intolerant bit in the table in 7.3.2.49.2 as shown in 11-07-xxxry. |  |
| 1728 | Perahia, Eldad | 64.18 | 7.3.2.49.2 | A | Since the definition and encoding use "AP", this would apply to 5 GHz devices. I thought the intent was to restict "Forty MHz Intolerance" to 40 MHz devices in 2.4 GHz. |  | modify subfield to only apply to 2.4 GHz, and not to be used in 5 GHz |  |  | Accept – see changes as per 11-07-xxxxry |  |
| 2107 | Stephens, Adrian | 64.23 | 7.3.2.49.2 | C | The description: "When sent by a STA, indicates whether the AP associated with this STA is required to prohibit 40 MHz transmissions by all members of the BSS. (See 9.20.4 (Switching between 40 MHz and 20 MHz))" Is not the comlete picture. The STA sets this bit according to local matters, or when it detects any of the BSS width trigger events, which include whether its AP declares "intolerance", or whether an OBSS AP declares intolerance, or it locally detects data on a 40MHz affected channel. |  |  | Replace with: "When sent by a STA, indicates one of the following conditions: whether the AP associated with this STA is required to prohibit 40 MHz transmissions by all members of the BSS, or The STA receives a beacon from an OBSS that asserts the Forty MHz Intolerant field, or The STA observes OBSS data transmissions on a 40MHz sensitive channel, or according to local policy. (See 9.20.4 (Switching between 40 MHz and 20 MHz))" |  | Counter – accept spirit of changes, see modified proposed change as part of 11-07-xxxxry |  |
| 1686 | Nanda, Sanjiv | 64.24 | 7.3.2.29.2 | R | By permitting the STA to set the Intolerant bit to 1 to restrict use of 40 MHz by the BSS, and moreover by requiring the STA to set the bit to 1 if it observes legacy transmissions, the use of 40 MHz in 2.4 GHz is severely curtailed, as to make it worthless. However, the complexity of the feature is significantly increased, without commensurate benefit. |  |  | Either find a method to enable more scenarios where 40 MHz can be used in 2.4 GHz, or completely disallow the use of 40 MHz in 2.4 GHz. For example, could decrease the amount of time that 40 MHz is disallowed and allow higher fraction of time that coexistence is possible. Further restrictions must be placed on when a STA can set the Intolerant bit, for how long, under what conditions. Duplicate-DSSS mode proposed during earlier discussions can allow coexistence of 40 MHz transmissions with legacy STA. |  | Reject – In a good number of instances, finding more scenarios where 40 MHz can be used in 2.4 GHz is a matter of replacing exsiting equipment with newer equipment, a process which is always occurring. By including the provisions for 40 mhz operation in 2.4 GHz, this future migration is made possible. Disallowing the use of 40 mhz in 2.4 GHz does not improve the operation of existing devices and it does not improve the operation of future devices, but allowing 40 mhz in 2.4 ghz allows the possibility of future improvements. |  |
| 3105 | Zaks, Artur | 84.00 | 7.4.9.10 | R | HT Information Exchange frame format is redundant and can be removed. |  |  | STA and AP indicate channel width in HT Capabilities IE. Channel width 20 MHz is sufficient to assume that STA is 40MHz Intolerant ( definition of Forty MHz Intolerant field of HT Information Exchange frame). STA Channel Width=0 in HT Indication IE is sufficient to assume that AP should not use 40MHz operation ( definition of the STA Channel Width field of HT Information Exchange frame). |  | Reject - Without the HT Information Exchange frame, a STA would be required to dissociate and re-associate in order to communicate changes in the Forty MHz Intolerant field value. Such dissociation events are disasterous for some QoS applications. |  |
| 3106 | Zaks, Artur | 84.00 | 7.4.9.10 | C | HT Information Exchange mentionsSecondary Channel Offset field that is not defined in the document. |  |  | Remove HT Inforamtion Exchange frame definition. |  | Counter – Secondary Channel offset field is a reference to the HT Information element – the commentor is correct, in that the text does not fit in this subclause and should be moved to subclause 11.15.1 as per 11-07-xxxxry |  |
| 2128 | Stephens, Adrian | 84.17 | 7.4.9.10 | C | The ability to come up with a good descriptive name for something generally shows it is well defined and understood. So the name "HT Information Exchange" should sound a warning. Firstly, the "HT" part is misleading because this particular frame format may also be supported by a non-HT STA (which is why support for this feature is part of the Extended Capability mechanism, rather than the HT Capability element). So it should be called "information exchange". But surely all frames exchange information in some sense, or there is no point sending them! |  |  | Rename the frame HT Coexistence Management frame. |  | Counter – editor to rename “HT Information Exchange frame” to “20/40 BSS Coexistence Management frame” throughout the document, including the “HT Information Exchange support” bit of the Extended Capabilities element – as per 11-07-xxxxry |  |
| 1102 | Marshall, Bill | 84.41 | 7.4.9.10 | C | if the length of the HT Information field is variable, the length need to be stored somewhere in the frame |  |  | add a length field to the frame |  | Counter – frame is variable, but now uses fixed two-octet field for some info and existing element (from TGk) to convey other information – the element information includes length – see 11-07-xxxxry |  |
| 2130 | Stephens, Adrian | 84.41 | 7.4.9.10 | C | "The HT Information field contains at least one octet" The HT Information Field is a fixed field, not an element. As such it's length cannot vary in the future, because it is necessary to know the length in order to parse any vendor-specific elements that follow the Acton Body. |  |  | Either turn the HT Information Field into an element or change the quoted text to: "The HT Information Field contains one octet" |  | Counter – see CID 1102 |  |
| 1103 | Marshall, Bill | 84.56 | 7.4.9.10 | R | normative statements don't belong in clause 7 |  |  | move this paragraph to an appropriate place in the standard |  | Reject – clause 7 may include normative statements. As an example, the base standard already includes many normative statements in clause 7. |  |
| 2132 | Stephens, Adrian | 84.60 | 7.4.9.10 | C | "When the Information Request bit is set to 0, the recipient of the HT Information field may transmit an HT Information Exchange management action frame with the transmitting STA as the recipient, but should not." You cannot say "may, but should not". Either you allow it or you do not. |  | Replace with: "When the Information Request bit is set to 0, the recipient of the HT Information field is not required to respond with an HT Information Exchange frame." Or consider replacing "is not required" with "should not" or "shall not". Actually "shall not" may be the better semantics because otherwise we risk a never-ending ping-pong of these frames between two darwin-worthy STAs. |  | Counter – generally agreed, but different wording used, as per 11-07-xxxxry. |  |  |
| 3289 | Erceg, Vinko | 85.00 | 7.4.9.10 | C | Misleading statement. |  | change the sentence: "When the Forty MHz Intolerant bit is set to 1, it indicates that the STA that has transmitted the management action frame containing the HT Information field is intolerant of 40 MHz operation." to "When the Forty MHz Intolerant bit is set to 1, it indicates that the STA that has transmitted the management action frame containing the HT Information field is indicating that 40 MHz channel operation is not to be employed." |  | Counter – see CID 2133 and document 11-07-xxxxry |  |  |
| 3290 | Erceg, Vinko | 85.00 | 7.4.9.10 | R | Two instances of "40 MHz mask PPDU" occur - these should be replaced with more accurate terms, such as "MPDUs or A-MPDUs that have a TXVECTOR parameter BW equal to 40 mhz" or whatever is the appropriate description of a PPDU at the MAC layer that will be sent using a 40 mhz mask. |  | Replace the use of "40 MHz mask PPDU" with more accurate terms, such as "MPDUs or A-MPDUs that have a TXVECTOR parameter BW equal to 40 mhz" or whatever is the appropriate description of a PPDU at the MAC layer that will be sent using a 40 mhz mask, something like "that has any value other than NON_HT_CBW40 for the TXVECTOR parameter CH_BANDWIDTH" |  | Reject – already covered - see definitions in clause 3. |  |  |
| 2133 | Stephens, Adrian | 85.01 | 7.4.9.10 | A | “When the Forty MHz Intolerant bit is set to 1, it indicates that the STA that has transmitted the management action frame containing the HT Information field is intolerant of 40 MHz operation.” | As it stands merely saying a STA is intolerant of 40 MHz operation is not enough. We need to refer to the subclauses that define this behaviour. I guess the question is whether the Forty MHz Intolerant bit is intended to be set under all conditions in which the bit of the same name is set in the HT Capability element. (this is probably the intention, which is why the field has the same name). If that is the case, rather than duplicating that description (with the danger that it diverges and introduces loopholes in the spec), we should refer to the definition of the other bit. In that case we could say: “The Forty MHz Intolerant bit indicates whether the STA transmitting the frame is tolerant of 40 MHz operation. The definition of this field is the same as in the HT Capabilities element (see 7.3.2.49), and it is operated as described in 9.20.4.” |  |  |  | Accept – see changes in 11-07-xxxxry |  |
| 2134 | Stephens, Adrian | 85.05 | 7.4.9.10 | C | “The STA Channel Width bit indicates the current operating width of the transmitting STA. An HT-AP that indicated a value of 0 in its most recently transmitted STA Channel Width field shall not transmit a 40 MHz mask PPDU.” This creates a problem. The semantics of the STA Channel Width field in the HT information element is somewhat different. The intention of that field was that a STA can monitor activity on the secondary channel and recommend its peer to send 20MHz frames if 40MHz frames are likely to be 13nterfered with in the locality. It is specifically not a switch of receiver capability – i.e. a 40MHz frame may or may not get through. The point is that a STA updating this field has no effect on the Operating Mode field, which is required to distinguish support of 20MHz-only STA. However, “current operating width” is the “receiver’s current capability” | “Remove operating mode 2 (because there is no behaviour associated with it). Explicitly allow a STA that sends a Notify Channel Width to change its mask. Require that a peer STA honour the Notify Channel Width request with a “”should”” statement.” |  |  |  | Counter – text that differentiates operating mode 0 from operating mode 2 is provided as per changes from doc 11-07-xxxxry. The first part of the second requested change by the commentor is not needed, since the converse is stated on page 167 at line 12 of D2.0. The second part of the second requested change by the commentor is added per changes in doc 11-07-zzzzry. |  |
| 2831 | Trainin, Solomon | 85.05 | 7.4.9.10 | R | There is no clear need to communicate the STA channel width this way because the actual channel width depends on Secondary Channel Offset | Remove this field from the HT Information Exchange frame |  |  |  | Reject – the Secondary Channel Offset provides the allowable channelization for the BSS, but it does not indicate the channel width that is being used by the STA. |  |
| 1105 | Marshall, Bill | 85.10 | 7.4.9.10 | R | normative statements don’t belong in clause 7 | move this paragraph to an appropriate place in the standard |  |  |  | Reject – clause 7 may include normative statements. As an example, the base standard already includes many normative statements in clause 7. |  |
| 2832 | Trainin, Solomon | 85.10 | 7.4.9.10 | A | This subclause is about HT Information Exchange frame format but Secondary Channel Offset field is not a part of the HT Information Exchange frame format | Remove the entire paragraph that start with “An HT-AP that indicated a value of 0 in its most recently transmitted Secondary Channel Offset …” |  |  |  | Accept – see changes in 11-07-xxxxry |  |
| 171 | Batra, Anuj | 109.00 | 9.6.8 | R | RTS/CTS in 40MHz env. may become partially 20MHz if initiated as non-HT duplicate (when the AP is aware of existing non-HT devices). This is a coex issue if the sequence is initated by a STA which is "hidden" from the non-HT device using an extension channel even if such sees/is seen by the AP, as the AP's response shall be 20MHz legacy in the control channel |  |  | Disallow using of 40MHz in 2.4 band |  | Reject - The frequency of occurrence of the situation described is expected to be small and therefore, insignificant to the operation of either of the networks in question and therefore should not be used as a rationale to disallow 40 mhz operation in 2.4 GHz. |  |
| 1891 | Sherlock, Ian | 109.00 | 9.6.8 | R | RTS/CTS in 40MHz channel environment may become partially 20MHz if initiated as legacy duplicate (when the AP is aware of existing non-HT devices). This is a coexistence issue if the sequence is initated by a STA which is "hidden" from the non-HT device using an extension channel even if such sees/is seen by the AP, as the AP's response shall be 20MHz legacy in the control channel. |  |  | Do not allow use of 40MHz channels in the 2.4GHz band. |  | Reject – see CID 171 |  |
| 2986 | Waters, Deric | 109.00 | 9.6.8 | R | RTS/CTS in 40MHz env. may become partially 20MHz if initiated as legacy duplicate (when the AP is aware of existing non-HT devices). This is a coex issue if the sequence is initated by a STA which is "hidden" from the non-HT device using an extension channel even if such sees/is seen by the AP, as the AP's response shall be 20MHz legacy in the control channel |  |  | Disallow using of 40MHz in 2.4 band |  | Reject – see CID 171 |  |
| 1741 | Perahia, Eldad | 163.30 | 9.2 | A | Based on this sentence, in 2.4 GHz, a 20/40 MHz STA operating in 20 MHz mode does NOT need to perform scanning and do the "Forty MHz intolerance" stuff in 9.20.4. So if the BSS has switched from 40 to 20 MHz due to "Forty MHz intolerance", and now operates by rules for a 20 MHz capable STA, how does it do the scanning and other functions to allow it to resume use of 40 MHz in 2.4 GHz. We need to answer the fundamental question of what defines "operating in 20 MHz mode" is based on the STA Channel Width or Supported Channel Width Set or what? |  |  | please clarify |  | Accept – Delete the cited sentence as indicated in 11-07-xxxxry |  |
| 2876 | Trainin, Solomon | 164.00 | 9.20.4 | A | The Forty MHz Intolerant field is used for two different purposes: 1) to represent the non-AP STA or BSS intolerance to 40MHz transmission of any other STA; 2) to signal the trigger detection from STA to associated AP. Due to this duality the current definition of the switching between 40MHz and 20MHz does not prevent from propagation of 40MHz intolerance - the non-AP STA becomes 40MHz intolerant while seeing any other STA 40MHz intolerance and may transmit its HT Capabilities element with Forty MHz Intolerant field while re-associating. |  |  | Separate the signaling of trigger detection from the intolerance notification. Define mechanism that allows non-AP STA transfer the trigger of 40MHz intolerance to their AP w/o making this non-AP STA 40MHz intolerant thus preventing from intolerance propagation. |  | Accept – include a new bit in the HT Information field “20 MHz BSS Width Request” in 7.4.9.10 and accompanying behavioural description in 9.20.4 and delete the instruction to STA to reassociate with the Forty MHz Intolerant bit set to 1 when responding to a trigger event, as per 11-07-xxxxry |  |
| 2877 | Trainin, Solomon | 164.00 | 9.20.4 | C | The Forty MHz Intolerant field is used for two different purposes: 1) to represent the non-AP STA or BSS intolerance to 40MHz transmission of any other STA; 2) to signal the trigger detection from STA to associated AP. Due to this duality the current definition of the switching between 40MHz and 20MHz does not prevent from propagation of 40MHz intolerance |  |  | Add "Twenty MHz mask" field in the HT Information Exchange management action frame. Non-AP STA should use the "Twenty MHz mask" field set to 1 to signal the trigger events to AP. The "Forty MHz Intolerant" field should be set to 0 when the non-AP STA uses the HT Information Exchange to signal about trigger event. The "Twenty MHz mask" field should be set to 0 when the non-AP STA responses to Information Request of the "Forty MHz Intolerant" field state. |  | Counter – see CID 2876 |  |
| 2906 | van Zelst, Allert | 164.00 | 9.20.4 |  | The protection mechanism is inadequate for OBSSs, particularly during the scanning period started once an OBSS is detected. |  |  | Add duplicate DSSS to allow for the well-known MAC protection mechanisms in 20/40 MHz operation in 2.4 GHz. |  |  |  |
| 3251 | Erceg, Vinko | 164.00 | 9.20.4 | A | Create an exception for a STA receiving forty_mhz_intolerant information from its own BSS. If a STA receives forty_mhz_intolerant = 1 from a frame that is marked with its own BSSID, then it is not a trigger event |  |  | As per the comment. |  | Accept – see changes in 11-07-xxxxry |  |
| 3252 | Erceg, Vinko | 164.00 | 9.20.4 | A | OBSS Scan operation should cover all "sensitive" channels, not just "affected" channels. |  |  | Make the change indicated in the comment. |  | Accept – in order to allow more unlicensed users in the 2.4 ghz band, any intolerance in the band should prevent 40 mhz bss operation anywhere in the band. See doc 11-07-xxxxry |  |
| 3253 | Erceg, Vinko | 164.00 | 9.20.4 | A | AP Channel Report should cover all "sensitive" channels, not just "affected" channels |  |  | Make the change indicated in the comment. |  | Counter – see CID 3252 – the range of channels is extended, but the AP Channel Report is no longer used as per 11-07-xxxxry |  |
| 3254 | Erceg, Vinko | 164.00 | 9.20.4 | C | Need to define the scan operation parameters more thoroughly |  |  | Describe the minimum scanning in terms of the maximum amount of time that is allowed to perform the scan of all of the channels in the AP channel report, and the minimum time to remain on each channel, which could vary depending on whether a channel is scanned actively or passively. |  | Counter – scanning now described as STA-initiated with MIB Variables defining the parameters for the scanning operation as per 11-07-xxxxry |  |
| 3255 | Erceg, Vinko | 164.00 | 9.20.4 | A | Intolerant bit can also appear in a management action frame as part of a field which is not an element - this case needs to be covered by the same restriction. |  |  | Change "Before transmitting a frame containing an HT Capability element with the Forty MHz Intolerant bit set to 0," to "Before transmitting a frame containing an HT Capability element or an HT Information field with the Forty MHz Intolerant bit set to 0," |  | Accept – see changes to 9.20.4 as per doc 11-07-xxxxry |  |
| 3261 | Erceg, Vinko | 164.00 | 9.20.4 | C | Refine the 802.11k measurement request OBSS Scan requirements - elements of the behavior seem to be missing. |  |  | The scan request does not provide a time limit or indicate whether the STA may make the measurements by scanning a channel, then coming back to the home channel, then scanning another channel in the target set, etc. If the AP makes the scan requests, and can know the exact behavior of the STA depending on how it set the parameters in the scan request, then the AP can know when the STA will be on channel and available for communication. |  | Counter – scanning now described as STA-initiated with MIB Variables defining the parameters for the scanning operation as per 11-07-xxxxry |  |
| 3262 | Erceg, Vinko | 164.00 | 9.20.4 | C | Require that OBSS Scan requirement includes a requirement to scan for offset periods over the 30 minutes, such that the scanning offsets completely cover an assumed 100 msec periodicity of beacon interval. Allow a PS STA to clump the measurement. |  |  | Make the change indicated in the comment. |  | Counter – scanning is changed to STA-initiated scanning, in which case, the STA can make the scanning offsets match the desired behaviour – see 11-07-xxxxry |  |
| 3263 | Erceg, Vinko | 164.00 | 9.20.4 | C | Is the scanning really going to be directed by the AP through the beacon measurement request, or can the STA do it independently? And if STA do this independently, then is the requirement uniform for all types of STA? E.g. PS vs non-PS STA? AC_VO flow STA vs AC_BE STA? TSPEC with UP=5,6,7 STA vs STA without such accepted TSPECS? |  |  | Either define the requirements very clearly for the 11k scan request, or drop the 11k mechanism in favor of STA-initiated scanning, and clearly define the requirements for STA-initiated scanning based on various STA parameters including but not limited to the PS mode and QoS considerations. |  | Counter – scanning now described as STA-initiated with MIB Variables defining the parameters for the scanning operation as per 11-07-xxxxry |  |
| 3264 | Erceg, Vinko | 164.00 | 9.20.4 | A | Add option to allow STA detection or AP detection of legacy beacon to cause fallback from 40 to 20. |  |  | Make the change indicated in the comment - or eliminate the traffic requirement and use a beacon requirement instead (or use both). |  | Accept – make beacons a trigger condition, eliminating traffic as a condition, and make it clear that legacy beacons are also a trigger, as per 11-07-xxxry |  |
| 3265 | Erceg, Vinko | 164.00 | 9.20.4 | C | it would be best to scan with some memory of the phasing of previous scans relative to some fixed time. This works when there is some common multiple of beacon interval that can be assumed for all target BSSs. E.g. if all BSSs have beacon intervals from the set (10,20,25,50,100), then a STA that scans a specific channel for 10 msec every 190 msec (scan start times) will eventually overlap at least two TBTT for any given BSS with any of those beacon intervals. We might allow one beacon interval or less total scan time per channel if there is an accompanying requirement for active scanning. And we could allow scanning on any channel to end prematurely if an OBSS is found. |  |  | Make the change indicated in the comment. |  | Counter – scanning is changed to STA-initiated scanning, in which case, the STA can make the scanning offsets match the desired behaviour – see 11-07-xxxxry |  |
| 3275 | Erceg, Vinko | 164.00 | 9.20.4 | A | Make it clear that when a STA scans for intolerant bits, it needs to ignore received intolerant=1 indications if the BSS of the received frame matches its own BSS. |  |  | Make the change indicated in the comment. |  | Accept – see changes to 9.20.4 as per 11-07-xxxxry |  |
| 3276 | Erceg, Vinko | 164.00 | 9.20.4 | A | Make it clear that 20mhz-only capable STA are not required to scan, and therefore, may set the intolerant bit to 0 even if they do not scan. |  |  | Make the change indicated in the comment. |  | Accept – see changes to 9.20.4 as per 11-07-xxxxry |  |
| 3278 | Erceg, Vinko | 164.00 | 9.20.4 | A | Disallow any 40 mhz BSS to operate in the ISMii band unless it matches exactly the primary and secondary channels of any existing 40 mhz BSS in ISMii. |  |  | This might be better done in 11.9.8.3 - see the comments for that subclause. |  | Accept – see changes to 11.9.8.3 as per 11-07-xxxxry |  |
| 3336 | Erceg, Vinko | 164.00 | 9.20.4 | A | add a statement or two that makes it clear that either a STA or AP may transmit the value of 1 for the forty_mhz_intolerant bit for reasons other than those explicitly given in the text and that the address1 value of the transmitted frame that contains the value of 1 for the forty_mhz_intolerant bit may be mcast or bcast or ucast |  |  | make the change as suggested in the comment |  | Accept – see changes to 9.20.4 in 11-07-xxxxry |  |
| 1746 | Perahia, Eldad | 164.24 | 9.20.4 | A | The definition of HT-AP-19 does not seem to be restricted to 20/40 capable AP's. Clarify that this entire subclause may be ignored by a 20 MHz only capable AP in 2.4 GHz (and 5 GHz as well, for that matter) |  |  | as in comment |  | Accept – see new definition of FC-HT-AP-19 which includes 40 MHz operating capability and new definition STA-19 and changes that use these new terms as per 11-07-xxxxry |  |
| 1747 | Perahia, Eldad | 164.24 | 9.20.4 | C | Clarify that all the 40 MHz intolerance, tigger events, and similar stuff only apply to 2.4 GHz operation |  |  | put all the stuff in to a separate subclause identified specifically to 2.4GHz |  | Counter – introducing new terms to clarify which STA and AP in which bands need to follow these rules. See changes introduced by 11-07-xxxxry |  |
| 2480 | Stephens, Adrian | 164.24 | 9.20.4 | C | It is not clear whether an AP scans for intolerance when it starts up |  |  | Add a statement that a 20/40 MHz HT-AP-19 does an OBSS scan before selecting a channel position and width. |  | Counter – agree in principle, but place this requirement in 11.9.8.3 instead of 9.20.4 as per 11-07-xxxxry |  |
| 2481 | Stephens, Adrian | 164.24 | 9.20.4 | C | 30 Minutes to detect a legacy OBSS seems to long, particularly if both HT and legacy BSSs are severely impacted during this period. |  |  | Reduce scanning period to 1 minute. |  | Counter – generally agree, but want some hysteresis in the operation – one minute scan frequency and five minutes for reversion to 40 mhz operation - see changes to D2.0 in 11-07-xxxxry |  |
| 2482 | Stephens, Adrian | 164.24 | 9.20.4 | A | The requirement of seeing data on a legacy OBSS before determining a channel width event is too unreliable. The scan may just happen to sample a channel while it is idle. |  |  | Change the definition of a the BSS width trigger event from data to beacon. |  | Accept – see changes in 11-07-xxxxry |  |
| 2485 | Stephens, Adrian | 164.24 | 9.20.4 | A | A STA reports intolerance, but should not be required to change its operating channel width unilaterally. If, and only if, the AP decides to change BSS width, the STA then has to respond. The STA can (if it wants to) send a Notify Channel Width and set its Tx and Rx mask to 20Mhz, but that is a local decision. Also, there should be no requirement anywhere in the spec that a STA is required to modify its supported channel width set. This field should be purely static, like other STA capabilities. |  |  | Remove any requirement that a STA modify its Supported Width Set. Remove any requirement that the STA unilaterally modify its "operating width" on the basis of a BSS width set trigger. |  | Accept – see changes in 11-07-xxxxry |  |
| 2488 | Stephens, Adrian | 164.24 | 9.20.4 | C | The “intolerant” bit is a partial solution to the partial overlap situation in 2.4 GHz. It solves the situation where an overlapping BSS only overlaps the secondary channel, but does not overlap the primary channel. It does not solve the situation where an OBSS partly overlaps both the primary and secondary channels. While operating in 40MHz increases the probability of overlap, it is not fundamentally the cause. For example, consider a 40MHz HT BSS on channel 1 (primary) and 5 (secondary). A legacy OBSS exists on channel 2. This is detected and reported via the “intolerant” bit. The HT BSS switches to 20 MHz operation on channel 1. The result is that it spends more time interfering with channel 2, and channel 2 sees more power. Without information about the cause of “intolerance”, the HT cannot make an informed decision. |  |  | Add a new field to accompany the “intolerant” field. This contains the channel number of the interference in an OBSS case, or 0 when intolerance is due to some other cause (i.e. to use in the collocated BT case). Adrian Check in draft: whether 40 MHz capable STA scans in a 20MHz only BSS? (should it?). AP should be allowed not to send beacon measurement requests if it has no intention of switching to 40MHz. Also check that a 20MHz-only HT AP doesn’t have to do this. |  | Counter – add a field that contains bits per channel for 2.4 ghz and a separate bit for use in 5ghz to specifically indicate a secondary channel trigger event as per 11-07-xxxxry |  |
| 2489 | Stephens, Adrian | 164.24 | 9.20.4 | C | What are the rules for whether 40 MHz capable STA scans in a 20MHz only BSS?. IMHO, the 40MHz capable STA should not be required to do any scanning when its AP is only 20MHz capable, as the AP will never make use of the information. AP should be allowed not to send beacon measurement requests if it has no intention of switching to 40MHz. i.e. it should be allowed to operate in 20MHz with no scanning, but before it switches back to 40MHz, it must have had all its eligble STA perform a scan. Also check that a 20MHz-only HT AP doesn’t have to ask its STA to perform any scanning. |  |  |  |  | Counter – Rules of behaviour of forty-capable vs not-forty-capable STA and AP have been clarified by including new terms FC-HT-STA-19 and FC-HT-AP-19 as per 11-07-xxxxry – rules on scanning initiation change from AP-initiated to STA-initiated as per 11-07-xxxxry. AP that does not want to have associated FC-STA to scan has the choice of itself advertising as 20 mhz only capable, which is reasonable, since it already knows that it will not use 40 mhz. |  |
| 1664 | Myles, Andrew | 164.25 | 9.20.4 | C | There needs to be provision for probing, or rapid scanning when an AP transitions from 20 to 40 MHz mode so that it can quickly change mode. |  |  | Modify the section to include a provision for rapid transition from 20 MHz to 40 MHz mode. |  | Counter – the language has been changed to require a more frequent scanning operation that facilitates the quick mode transition requested by the commentor – see 11-07-xxxxry |  |
| 1665 | Myles, Andrew | 164.25 | 9.20.4 | R | We don't want to put the burden of scanning on clients. The Aps probing at their lowest data rates (1 Mbps and 6 Mbps) have sufficient range to detect an OBSS that will interfere with 40 MHz communications |  |  | Have the Aps probe the 40 MHz affected channels, not the clients. |  | Reject – the group feels that: 1) scanning is already typically performed by STA, and therefore, these new requirements re-use existing behaviour 2) the potential consequences to QOS and BE traffic from the AP leaving the primary channel to scan are more costly than the similar consequences for STA, especially when accounting for the already established procedures for STA scanning 3) the STA have generally better opportunity to hear OBSS in a greater range |  |
| 804 | Ketchum, John | 164.27 | 9.20.4 | R | Mechanisms to deal with legacy interop issues when operating in 20/40 mode at 2.4 GHz significantly complicate the standard, marginalize the use of 40 MHz frame formats at 2.4 GHz, and do not fully protect legacy devices operating at 2.4 GHz from 20/40 HT devices. |  |  | Eliminate the use of 20/40 operation in the 2.4 GHz band, and remove content on 40 MHz intolerant field |  | Reject – the mechanisms described are optional, needing only to be implemented by STA that desire to employ 40 mhz in 2.4 GHz – as such, the complexity is of choice by implementor and thusfar, seems to represent a tradeoff that a super-majority of the members accepts. |  |
| 1817 | Petranovich, James | 164.27 | 9.20.4 | C | The rules for switching from 40 Mhz to 20 MHz in 2.4 GHz take too long to force a swtich. For example, if a legacy OBSS appears in or overlapping the secondary channel, it will take a long time (as much as 30 minutes under ideal conditions) for the BSS to swtich to 20 MHz mode. |  |  | Force devices that regularly see CCA set in the secondary channel (more than 10% of the time in a 1 minute period) to either scan for overlapping OBSS in the secondary channel quickly (within 5 minutes) or switch to 20 MHz mode. |  | Counter – determining the appropriate threshold for such a requirement is a difficult task – instead, the scanning requirement is increased to once per minute, such that the discovery process should take place much more quickly as per changes to scanning found in 11-07-xxxxry |  |
| 2494 | Stephens, Adrian | 164.35 | 9.20.4 | C | "An HT-AP-19 that set the Supported Channel Width Set field of its most recently transmitted HT Capability element to 0 may set the Forty MHz Intolerant field to 1 in transmitted HT Capabilities elements. An HT-AP-19 that set the Supported Channel Width Set field of its most recently transmitted HT Capability element to a non-zero value may set the Forty MHz Intolerant field to 1 in transmitted HT Capabilities elements." This is tortuous and can be said more succinctly |  |  | Reword thus: "An HT-AP-19 may set the Forty MHz Intolerant field to 1 in transmitted HT Capabilities elements regardless of the value of the Supported Channel Width Set field." |  | Counter – accepted this change but also changed the sentence a bit for some other comments, without modifying the intent of this comment as per 11-07-xxxxry |  |
| 3109 | Zaks, Artur | 164.35 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 719 | Kasher, Assaf | 164.36 | 9.20.4 | C | The two paragraphs in lines 35 through 41 basicaly say that "IF X then Y" and the "IF not X then Y" where X is setting the supported channel width to zero and Y is "may set the Forty MHz Intolerant bit to zero". This means only that An HT-AP-19 may set the Forty MHz Intolerant bit. (which is next to saying nothing) |  |  | Replace both paragraphs with "An HT-AP-19 may set the Forty MHz intolerant bit" or with nothing. |  | Counter – text similar to that which was suggested is incorporated in 11-07-xxxxry |  |
| 3110 | Zaks, Artur | 164.39 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 3111 | Zaks, Artur | 164.40 | 9.20.4 | R | non-zero value of the field that is one-bit long is ambigues |  |  | Replace a "non-zero value" to "1" |  | Reject – there is no ambiguity. |  |
| 1720 | Nanda, Sanjiv | 164.43 | 9.20.4 | R | These criteria for switching BSS channel width to 20 MHz in the 2.4 GHz band, will result in making 40 MHz unusable in this band, due to the presence of legacy STA and since no constraints are placed on when a STA may set the Forty MHz Intolerant bit. Rather than implement these set of complex procedures, including significant measurement burden, signaling overhead, the use of 40 MHz in the 2.4 GHz band must be disallowed. |  |  | Either disallow the use of 40 MHz in the 2.4 GHz band, or relax the conditions under which it can be used. This can be done by (i) reducing the measurement burden, (ii) constraining when a STA may set the Forty MHz Intolerant bit, (iii) allowing protection of legacy transmissions on "sensitive" channels through a duplicate-DSSS transmission format. |  | Reject – many scenarios exist where, despite the switching criteria, 40 mhz operation will be possible - most STA implementations already perform scanning at a much higher rate than what is proposed (albeit the scanning rate is proposed to increase beyond the values found in D2.0, but still within reason compared to most implementations) – similar arguments can be made and have been made regarding higher speed modes of previous amendments, for example, (i.e. that they have limited useability) – the overhead of the proposed duplicate DSSS transmission format makes the use of 40 mhz much less interesting |  |
| 2962 | Vlantis, George | 164.44 | 9.20.4 | C | 20-40MHz Coexistence--Switching between 40MHz and 20MHz: Subsequent to the inclusion of the 40/20MHz secondary channel sensing scheme into the 1.10 draft, lab testing of the sensing scheme using the sampling procedure in subparagraphs a) to c) has proven to be unreliable in several usage scenarios in the 2.4GHz Band (c.f. Intel results). |  |  | Maintain definition of 40MHz operation in the 5GHz band but preclude it from the 2.4GHz Band |  | Counter – the indicated lab testing data have not been produced to support the argument – additionally, the mechanisms from D1.10 have been modified to address some of the “possible” issues suggested by the author alluded to in the comment, as per 11-07-xxxxry, with 40 mhz operation still allowed in the 2.4 GHz band |  |
| 3112 | Zaks, Artur | 164.44 | 9.20.4 | R | "BSS width trigger" is it identifier? This expression is not crearly defines the intention of the events |  |  | re-define the event group name |  | Reject – while it has been considered to change the name to something slightly more descriptive – e.g. BSS Width Change Trigger, that name is deemed to be too long and therefore unwieldy to be useful – the commentor is invited to propose alternatives |  |
| 1666 | Myles, Andrew | 164.45 | 9.20.4 | C | It is better to have the AP actively probe the 40 MHz affected channels at 2.4 GHz and the secondary channel at 5 GHz (if permitted in the regulatory domain). That allows for a quicker response. |  |  | Modify the section to include a provision for the AP actively probing the 40 MHz affected channels. |  | Counter – active probing is not prohibited by the subclause, neither is probing by the AP prohibited. |  |
| 1748 | Perahia, Eldad | 164.45 | 9.20.4 | A | the requirement of detecting a data frame may result in not detecting a neighboring AP during momentary quiet period. Then when the legacy OBSS resumes active flow of traffic, it will be interfered with by the 11n BSS which switched to 40 MHz under false pretenses. |  |  | consider only using the beacon as the trigger event for a) |  | Accept – see changes from 11-07-xxxxry |  |
| 3113 | Zaks, Artur | 164.45 | 9.20.4 | C | "40 MHz affected channel" expression is ambigues |  |  | change with " channel that complies to 40-MHz channel definition" |  | Counter – see definition in clause 3, where the exact meaning of the phrase “40 MHz affected channel” is provided |  |
| 3114 | Zaks, Artur | 164.45 | 9.20.4 | A | The definition is not understandable |  |  | re-define this event crearly |  | Accept – this trigger event has been redefined, albeit with different meaning from the original as per 11-07-xxxxry |  |
| 3115 | Zaks, Artur | 164.45 | 9.20.4 | C | The definition is not understandable |  |  | Detection of frame of any type other that Null Data or QOSNulldata belonging to an OBSS on the channel STA operates on, that complies to 40-MHz channel definition. |  | Counter – the definition is changed to detect beacons instead of traffic as per 11-07-xxxxry. |  |
| 3116 | Zaks, Artur | 164.45 | 9.20.4 | C | The time for the event detiction should not be 500ms hard-coded time, but configurable |  |  | Define the time as "dot11HT40ChannelEventAgilityTime", UINT32, range: 0-60000ms. Default: 3000ms. |  | Counter – reference to the time of 500 msec has been removed as has the provision that employed that value as per 11-07-xxxxry |  |
| 3335 | Erceg, Vinko | 164.45 | 9.20.4 | A | should we be looking for beacons instead of traffic? Consider the case of the peaceful coexistence of a 20 mhz BSS and a 40 mhz BSS until traffic starts on the 20 mhz BSS, in which case, the 40 mhz BSS has 30 minutes to detect the traffic - will it find the traffic more quickly than this? if not, then the traffic will not likely last 30 minutes - of course, if the scanning requirements are modified, then maybe the traffic will be detected more quickly. How quickly is fast enough? |  |  | change traffic detection to beacon detection in trigger condition a) |  | Accept – see 11-07-xxxxry |  |
| 1749 | Perahia, Eldad | 164.46 | 9.20.4 | C | to be explicitly clear that this only applies to 2.4 GHz, AP should be changed to HT-AP-19 |  |  | as in comment |  | Counter – see new terms FC-HT-AP-19 and FC-HT-STA-19 that are used in 11-07-xxxxry throughout the text that describes what happens when a STA encounters a trigger event – the cited text – the definition of the tigger event, does not need to discriminate, since there is no normative behavior described in that text |  |
| 3273 | Erceg, Vinko | 164.49 | 9.20.4 | C | The 500 msec time does not make much sense, since it presumes that scanning for beacons will occur at a minimum rate of once per 500 msec, when the requirements elsewhere in this subclause suggest a minimum scanning rate of once per thirty minutes. |  |  | Change 500 msec to thirty minutes to match other scanning times. |  | Counter – the time and the behaviour that employed that value have been removed as per 11-07-xxxxry |  |
| 2495 | Stephens, Adrian | 164.50 | 9.20.4 | C | bullet item b) "reception on any 40 MHz sensitive channel, of a frame that contains an HT Capabilities element with the value of 1 in the Forty MHz Intolerant field" would allow two STA that set up a DLS link at least once every 30 minutes to lock themselves into an "intolerant" state, regardless of external observations. |  |  | Call out the frame types where this may be observed, excluding the DLS setup frames, thus: "reception on any 40 MHz sensitive channel, of a Beacon, Measurement Pilot, Association Request, or Reassociation Request frame that contains an HT Capabilities element with the value of 1 in the Forty MHz Intolerant field" |  | Counter – adding a separate signaling bit for communicating trigger detection to the AP will alleviate the lock problem, but the idea of listing frames is still the correct thing to do, the list is shorter that the list suggested here – see 11-07-xxxxry |  |
| 2496 | Stephens, Adrian | 164.50 | 9.20.4 | A | "reception on any 40 MHz sensitive channel," "40 MHz sensitive channel: any 20 MHz channel of the set of 20 MHz channels that the STA may operate on in the current regulatory environment." The definition of 40MHz sensitive channel is useless. Assuming that STA operate within the required regulatory constraints, it has no effect. |  |  | Replace with "reception" and delete the definition of 40 MHz sensitive channel. |  | Accept – with a note that originally, it was suggested that the sensitive set might be smaller than all channels, or that affected set might be used instead…see changes in 11-07-xxxxry |  |
| 2873 | Trainin, Solomon | 164.50 | 9.20.4 | C | 40MHz capable STA has to scan to look for the Forty MHz intolerance=1 in the HT Capabilities element in any 20 MHz channel of the set of 20 MHz channels that the STA may operate on in the current regulatory environment. The HT Capabilities element is represented in beacon, association, and probe response frames. There is no clear reason of all 20MHz channels scanning |  |  | Reduce the number of channels to scan to the secondary channel that can be compromised as scanning of number of channels with 5MHz resolution |  | Counter – the set of frames for which the forty mhz intolerant bit is checked is reduced in the “all channels” trigger b) case to only include the Coex mgmt action frame and the probe request frame as per 11-07-xxxxry |  |
| 2874 | Trainin, Solomon | 164.50 | 9.20.4 | C | Reason of the Forty MHz intolerance assertion in the HT Capabilities element is detecting of trigger event or any other unspecified motive. Assuming that the reason to assert Forty MHz intolerance is stations BT activity what is the reason to scan all the 20MHz channels |  |  | Reduce the number of channels to 40 MHz affected channels |  | Counter – now that separate signalling bits exist for STA-AP communication of trigger detection and specific forty mhz intolerant signalling, the frames applicable to trigger condition b) are reduced in number and the meaning of those trigger events is more strictly to “not use 40 mhz in the band” as opposed to the meaning of the trigger a) events as per 11-07-xxxxry |  |
| 3117 | Zaks, Artur | 164.50 | 9.20.4 | C | The expression "40 MHz sensitive channel" is ambiues |  |  | Change "40 MHz sensitive channel" to " channel that complies to 40-MHz channel definition" |  | Counter – 40 mhz sensitive channel, where still used, has been changed to “the channel set defined in Clause 19” as per 11-07-xxxxry |  |
| 3118 | Zaks, Artur | 164.50 | 9.20.4 | A | The definition "of a frame that contains an HT Capabilities element …" is ambigues |  |  | Change the definition to include frame type, BSSID frame belongs to, Address1 (Broadcast or Unicast), Primary or Secondary channel. |  | Accept – see changes that specify frame type/subtype and addressing information where appropriate, as per 11-07-xxxxry |  |
| 3260 | Erceg, Vinko | 164.50 | 9.20.4 | R | BT AFH allows operation with a minimum of 20 channels - so it is not necessary for the item b) to specify the detection of the intolerant bit over the entire 2.4 ghz band, but just over the sensitive channel set. A 40 mhz BSS takes about 45 1 mhz channels, leaving at least 31 or more channels for BT to operate on. |  |  | Make the change indicated in the comment. |  | Reject – with one 40 mhz BSS and one 20 mhz BSS operating, no channels are left for BT – in such cases, a complete prohibition over the band is required to protect BT. |  |
| 3266 | Erceg, Vinko | 164.50 | 9.20.4 | C | For item b), change "reception" to "detection" to allow neighbor OBSS to inform the receiving STA in the adjacent BSS to respond to the forty_mhz_intolerant bit |  |  | Make the change indicated in the comment. |  | Counter – the term “reception” is already defined in the beginning of clause 7 to not include any address checks – the language cited has been modified a bit to ensure that it is clear that neighbour BSSs will respond to trigger type b) events – changes as per 11-07-xxxxry |  |
| 2816 | Surineni, Shravan | 164.56 | 9.20.4 | C | A method of operation/switching of 20/40 MHz channels is presented. However, this is very complicated. Further, in a BSS with a lot of traffic, chaging of a BSS from 40 MHz to 20 MHz will have a cascaded effect as same number of STAs are now contending for half of the available bandwidth. As the overlapping BSS scenario is very likely to happen in 2.4 GHz, atleast 40 MHz operation has to be banned in this band. |  |  | Restrict 40 MHz operation to 5GHz only. |  | Counter – the mechanisms described are optional, needing only to be implemented by STA that desire to employ 40 mhz in 2.4 GHz – as such, the complexity is of choice by implementor and thusfar, seems to represent a tradeoff that a super-majority of the members accepts – additionally, the switching between 20 and 20/40 BSS operation will be reduced as per the modifications to the mechanism of 9.20.4, such that impacts on QoS and other bandwidth-sensitive features will be minimized. |  |
| 3119 | Zaks, Artur | 164.56 | 9.20.4 | A | The definition is ambigues. |  |  | Re-define the requirement |  | Accept – task group is assuming that the commentor is referring to the trigger c) item – it has been rewritten to clarify the intent, as per 11-07-xxxxry |  |
| 3120 | Zaks, Artur | 164.56 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 3121 | Zaks, Artur | 164.58 | 9.20.4 | R | definition "in transmitted HT Information elements beginning at ht e next DTIM or next TBTT if no DTIMs are transmitted" is not clear |  |  | Change to "in transmitted Beacon at next TBTT" |  | Reject – the next DTIM language is needed to avoid the condition that the BSS changes to 20 mhz operation (no secondary channel) while some associated STA are in PS mode – they could wake up and attempt a 40 mhz transmission which will not be received by the AP. Therefore, the AP shall wait until the next DTIM before making the change. Changing the language to just “beacon” means that a loophole would exist, wherein the AP could transmit some other non-beacon frames that contain the element before the DTIM, implying that the AP has actually made the 40 to 20 mhz switch without first announcing this to the PS STA. |  |
| 3274 | Erceg, Vinko | 164.60 | 9.20.4 | C | The text does sort of imply that everything after page 164 line 64 is part of the operational restrictions. |  |  | Change "restrictions" to "two restrictions" near the end of the paragraph that begins with "An HT-AP-19 that detects any of the BSS width trigger events a), b) or c) shall" |  | Counter – changed it by making the requirements generic to any FC-HT-AP-19 as per 11-07-xxxxry |  |
| 2497 | Stephens, Adrian | 164.63 | 9.20.4 | C | It is not clear which of the following restrictions follow on from the colon. They need to be distinguished as a list. |  |  | The next two paragraphs should be formatted as a dashed list. |  | Counter – changed it by making the requirements generic to any FC-HT-AP-19 as per 11-07-xxxxry |  |
| 1635 | Mujtaba, Syed Aon | 164.65 | 9.20.4 | C | HT-AP-19 waiting 30 minutes during which there is no BSS width trigger event before setting STA Channel Width = 1 seems arbitrary and excessive. For example, a single 20 MHz STA would have a dispropotionate impact on the throughput and capacity of a 40MHz 2.4 GHz 802.11n BSS. |  |  | Use a random backoff timer, whose minimum backoff is 1 min, and maximum backoff is 30 minutes (preferably 15 minutes). This would alleviate the determinisitic 30 minute latency. |  | Counter – reduce detection time to 1 minute with 5 minute hysteresis for returning to 40 mhz operation as per 11-07-xxxry |  |
| 3025 | Zaks, Artur | 164.65 | 9.20.4 | R | frame that contains HT IE is not defines |  |  | Define frames that contain HT IE to be transmitted. |  | Reject – any frame that contains such an element is subject to the rules, since any such frame would potentially be advertising a change in BSS operating width. |  |
| 3122 | Zaks, Artur | 164.65 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 3026 | Zaks, Artur | 165.01 | 9.20.4 | C | 30 min fixed values is not correct |  |  | define as "dot11Channel Width Agility Time", UINT32, in seconds default: 60. |  | Counter – agree that MIB variable is correct solution and agree that 60 seconds is correct default, disagree on name and exact details of the MIB expression in the annex – see changes are per 11-07-xxxxry |  |
| 3027 | Zaks, Artur | 165.04 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 3028 | Zaks, Artur | 165.04 | 9.20.4 | C | The definition is ambigues. |  |  | re-define the paragraph |  | Counter – paragraph is reworded as per 11-07-xxxxry |  |
| 3029 | Zaks, Artur | 165.04 | 9.20.4 | R | The frames HT IE is sent are not defined |  |  | Define the frames HT IE are sent in |  | Reject – any frame that contains such an element is subject to the rules, since any such frame would potentially be advertising a change in BSS operating width. See clause 7 to determine which frames may contain the element. |  |
| 3030 | Zaks, Artur | 165.05 | 9.20.4 | C | 30 min fixed values is not correct |  |  | define as “dot11Channel Width Agility Time”, UINT32, in seconds default: 60. |  | Counter – see CID 3026 |  |
| 720 | Kasher, Assaf | 165.07 | 9.20.4 | A | “An HT-AP-19 that has an active association with at least one non-HT STA may set the Forty MHz Intolerant bit in transmitted HT Capability elements to 1, and may set the Forty MHz Intolerant bit to 1 at other times.” This only say again that an HP-AP-19 may set the Forty MHz Intolerant bit” to which we agreed before (see previous comment” |  |  | Remove paragraph |  | Accept. Delete the sentence as per 11-07-xxxxry |  |
| 2498 | Stephens, Adrian | 165.07 | 9.20.4 | C | “An HT-AP-19 that has an active association with at least one non-HT STA may set the Forty MHz Intolerant bit in transmitted HT Capability elements to 1, and may set the Forty MHz Intolerant bit to 1 at other times.” This is unnecessary language. The AP can set that bit to 1 whenever it wants, so it’s not necessary to specify the first may. However if the information is usefull, it should be retained as a note. |  |  | Reword thus: “An HT-AP-19 may set the Forty MHz Intolerant bit to 1 at any time according to implementation-defined criteria. NOTE-An HT-AP-19 that has an active association with at least one non-HT STA can set the Forty MHz Intolerant bit to 1.” |  | Counter – paragraph has been deleted as per 11-07-xxxxry. |  |
| 2878 | Trainin, Solomon | 165.07 | 9.20.4 | A | This paragraph defines a case already covered in the 3rd and 4th paragraphs after subclause header on page 164. |  |  | Remove the paragraph that starts “An HT-AP-19 that has an active association with at least one non-HT STA may …” |  | Accept – see CID 720 |  |
| 3031 | Zaks, Artur | 165.07 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 3032 | Zaks, Artur | 165.07 | 9.20.4 | C | The definition is ambigues. |  |  | Re-define the paragraph |  | Counter – paragraph is deleted as per 11-07-xxxxry |  |
| 3033 | Zaks, Artur | 165.07 | 9.20.4 | R | The frames HT IE is sent are not defined |  |  | Define the frames HT IE are sent in |  | Reject – any frame that contains such an element is subject to the rules, since any such frame would potentially be advertising a change in BSS operating width. See clause 7 to determine which frames may contain the element. |  |
| 3034 | Zaks, Artur | 165.07 | 9.20.4 | C | “at other times” definition is ambigues |  |  | refine at what occasions AP may set the bit. |  | Counter – paragraph has been deleted as per 11-07-xxxry |  |
| 2879 | Trainin, Solomon | 165.11 | 9.20.4 | A | This paragraph defines a case already covered in the paragraph before the last on page 164 |  |  | Remove the paragraph that starts “An HT-AP-19 that has an active association with at least one HT STA whose last successfully …” |  | Accept – paragraph deleted as per 11-07-xxxxry |  |
| 3035 | Zaks, Artur | 165.11 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 3037 | Zaks, Artur | 165.11 | 9.20.4 | C | The definition is ambigues. |  |  | Re-define the paragraph |  | Counter – paragraph has been modified per 11-07-xxxxry |  |
| 3036 | Zaks, Artur | 165.12 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 3038 | Zaks, Artur | 165.13 | 9.20.4 | R | definition "in transmitted HT Information elements beginning at ht e next DTIM or next TBTT if no DTIMs are transmitted" is not clear |  |  | Change to "in transmitted Beacon at next TBTT" |  | Reject – see CID 3121 |  |
| 3039 | Zaks, Artur | 165.16 | 9.20.4 | R | definition "in transmitted HT Information elements beginning at ht e next DTIM or next TBTT if no DTIMs are transmitted" is not clear |  |  | Change to "in transmitted Beacon at next TBTT" |  | Reject – see CID 3121 |  |
| 1750 | Perahia, Eldad | 165.19 | 9.20.4 | C | clarify that a 20 MHz-only capable STA may ignore the request |  |  | as in comment |  | Counter – the request acceptance is actually based on whether a STA has indicated support in the extended capabilities element. In any case, such a statement has been added to the paragraph and to 11.17 as per 11-07-xxxxry |  |
| 2499 | Stephens, Adrian | 165.19 | 9.20.4 | C | "An HT-AP-19 may transmit an HT Information Exchange management frame with a value of 1 in the RequestInformation bit to a STA including an Information element in order to request an update of the status of the Forty MHz Intolerant field of the HT Capabilities element." There's nothing special about the AP. However, this "may" is too broad because not all STA support the use of this frame. |  |  | Reword as a note: "NOTE-An HT-AP-19 can transmit an HT Information Exchange management frame with a value of 1 in the Request Information bit to a STA that supports the HT Information Exchange in order to request an update of the status of the Forty MHz Intolerant field of its HT Capabilities element." |  | Counter – reworded differently, but in particular, broadened the concept as per 11-07-xxxxry |  |
| 3040 | Zaks, Artur | 165.19 | 9.20.4 | R | HT-AP-19 identifier is not defined |  |  | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |  |
| 3041 | Zaks, Artur | 165.19 | 9.20.4 | C | The definition is ambigues. |  |  | re-define the paragraph |  | Counter – paragraph has been reworded as per 11-07-xxxxry |  |
| 2880 | Trainin, Solomon | 165.20 | 9.20.4 | A | There is no such a field as Information element defined in the HT Information Exchange |  |  | Remove "including an Information element" |  | Accept – reworded to correct the mistyped reference as per 11-07-xxxxry |  |
| 3042 | Zaks, Artur | 165.21 | 9.20.4 | A | "STA including an Information element" is ambigues |  |  | define what IE should be sent |  | Accept – reworded to correct the mistyped reference to the information request bit of the ht information exchange mgmt action frame as per 11-07-xxxxry |  |
| 1751 | Perahia, Eldad | 165.24 | 9.20.4 | C | "...an HT-AP-19 may complete an overlapping BSS scan…". Since this is a may and not a shall, why does it matter when the HT-AP-19 completes the scan. It "may" complete the scan whenever it feels like. |  |  | Change the language to say that if the HT-AP-19 chooses to scan, it is suggested that the scan is performed at time no earlier than one beacon interval preceeding the transmission. |  | Counter – paragraph has been deleted as per 11-07-xxxxry |  |

SPLIT

| 2500 | Stephens, Adrian | 165.24 | 9.20.4 | C | "Before transmitting a frame containing an HT Information element with the Secondary Channel Offset field set to non-zero value, an HT-AP-19 may complete an overlapping BSS scan (defined below) at a time no earlier than one beacon interval preceeding the transmission." This "may" is meaningless. There's no reason why it can't already do that. Turn it into a note. | Reword: "NOTE-Before transmitting a frame containing an HT Information element with the Secondary Channel Offset field set to non-zero value, an HT-AP-19 can complete an overlapping BSS scan (defined below), which is best performed at a time no earlier than one beacon interval preceeding the transmission." |  | Counter – paragraph has been deleted as per 11-07-xxxxry |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1667 | Myles, Andrew | 165.25 | 9.20.4 | C | Change "may" to "shall" | Change "may" to "shall" |  | Counter – paragraph has been deleted as per 11-07-xxxxry |
| 3043 | Zaks, Artur | 165.25 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 |  | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 1668 | Myles, Andrew | 165.26 | 9.20.4 | C | It is impossible to complete a passive OBSSS scan in 1 beacon interval. | There is no reason to have the 1 beacon interval constraint. 1 minute shoud be sufficient. |  | Counter – paragraph has been deleted as per 11-07-xxxxry |
| 3044 | Zaks, Artur | 165.27 | 9.20.4 | C | Fixed value of one TBTT is not acceptable | Define value as MIB IE |  | Counter – paragraph has been deleted as per 11-07-xxxxry |
| 1669 | Myles, Andrew | 165.29 | 9.20.4 | C | The method for scanning is too slow. At 2.4 GHz, we should have the AP actively probe the 7 40 MHz affected channels once per minute. At 5 GHz, we should have the AP probe the secondary channel once per minute where permitted, and have it passively scan once per minute where active probing is not permitted. | Modify the scanning scheme for relatively frequent active probing where possible, and frequent passive scanning when that is the only alternative. |  | Counter – accept the one minute change to scanning minimum, active probing is already allowed, create text for 5 ghz operation, including new definitions, as per 11-07-xxxxry |
| 1752 | Perahia, Eldad | 165.29 | 9.20.4 | C | Clarify that overlapping BSS scan operation only applies to 2.4 GHz | as in comment |  | Counter – the line cited by the commentor is simply the creation of a definition – an OBSS Scan could be applied in any band of spectrum, and that point is not material for this paragraph of the specification. Other statements in 9.20.4 clearly indicate that the defined OBSS Scan is only used in the 2.4 ghz band, some statements have been clarified to this point. |
| 2881 | Trainin, Solomon | 165.29 | 9.20.4 | C | The defined scan time should be related to passive scan and not to active one | Replace the text "An overlapping BSS scan operation is defined as a passive or active scan of all of the 40 MHz affected channel excluding the primary channels contained in the AP Channel Report set wherein, the per-channel scan duration is a minimum of 10 TU and where each channel in the set is scanned at two times per thirty minutes ..." by "An overlapping BSS scan operation is defined as a passive or active scan of all of the 40 MHz affected channel excluding the primary channels contained in the AP Channel Report set wherein. Each channel in the set is scanned at two times per thirty minutes. If the scanning is passive the per-channel scan duration is a minimum of 10 TU ..." |  | Counter – separated active and passive requirements – see 11-07-xxxry for exact changes. |
| 3045 | Zaks, Artur | 165.29 | 9.20.4 | C | The definition is ambigues. |  | Re-define the paragraph | Counter – paragraph is reworded per 11-07-xxxxry |
| 2502 | Stephens, Adrian | 165.30 | 9.20.4 | C | “the AP Channel Report set” – “set” is the wrong term |  | replace with “the AP Channel Report element” | Counter – cited text has been deleted as per 11-07-xxxry |
| 2503 | Stephens, Adrian | 165.30 | 9.20.4 | C | “An overlapping BSS scan operation is defined as a passive or active scan of all of the 40 MHz affected channel excluding the primary channels contained in the AP Channel Report set wherein,” The parsing of the sentence is ambiguous. Also, the definition of 40 MHz affected channel probably means that the exclusion cited above is unnecessary. It could mean: An overlapping BSS scan operation is defined as a passive or active scan of (all of the 40 MHz affected channel excluding the primary channels) contained in the AP Channel Report set wherein, or An overlapping BSS scan operation is defined as a passive or active scan of all of the 40 MHz affected channel excluding the (primary channels contained in the AP Channel Report set) wherein, Also, what is “wherein” doing here? |  | Reword so it’s not ambiguous. I recommend changing the languages so that it is based on actions, i.e. “To perform an OBSS scan, a STA shall scan (actively or passively) each channel in the AP Channel report set that is also a 40MHz affected channel…” | Counter – reworded with the goal of clarity as desired by the commentor – see 11-07-xxxxry for exact changes. |
| 42 | Adachi, Tomoko | 165.31 | 9.20.4 | A | “the per-channel scan duration is a minimum of 10 TU and where each channel in the set is scanned at two times per thirty minutes and the minimum total scan time per channel per thirty minutes is 200 msec.” How can you make the minimum total scan time per channel per thirty minutes 200 msec by minimum scan duration of 10 TU twice per thirty minutes? |  | Clarify how to achieve the requirement for the minimum total scan time. | Accept – Add the following sentence to 9.20.4 as per 11-07-xxxxry: “Note: The values provided in the previous paragraph indicate the minimum requirements. In some cases, not all minimums can be satisfied simultaneously.” |
| 3046 | Zaks, Artur | 165.32 | 9.20.4 | C | 10TU fixed per-channel time is not aceptable |  | Define value as MIB IE, in milliseconds, default=20 | Counter – MIB variables created with default values and ranges as per 11-07-xxxxry |
| 3047 | Zaks, Artur | 165.32 | 9.20.4 | C | fixed 2 times per 30 minutes definition is not aceptable |  | Define value as MIB IE | Counter – MIB variable for time interval added as per 11-07-xxxxry |
| 2882 | Trainin, Solomon | 165.33 | 9.20.4 | C | Two times of 10 TU is 20TU and not 200msec |  | Change “ … and the minimum total passive scan time of all channels per thirty minutes is 200 msec” | Counter – see the note that was added per 11-07-xxxxry – the times given are minimums – it is not possible or necessary to meet all minimums simultaneously. |
| 3048 | Zaks, Artur | 165.33 | 9.20.4 | A | fixed 200ms definition is not aceptable |  | Define value as MIB IE | Accept – see changes in 11-07-xxxxry |
| 2883 | Trainin, Solomon | 165.35 | 9.20.4 |  | The STA Channel Width field actually is related to the receiver only. The first STA should transmit 40MHz PPDUs to the second STA in respect to value of this field received from the second STA. Any value in the Channel Width field transmitted by AP does not prevent this AP from transmitting 40MHz PPDU as long the Supported Channel Width in its own HT Capability element is set to 1 and the Secondary Channel Offset is not zero. So this 39ignalling does not tell the STA that the AP will not transmit 40MHz thus preventing the STA from switching to 20 MHz mask. The rule of An HT-AP-19 that detects any of the BSS width trigger events on page 164 solves the problem saying that both STA Channel width and Secondary Channel Offset has to reflect the BSS width changes. Only the HT Information element contains both fields. So only the HT Information element issued by AP is a proper solution to change the BSS channel width. |  | Replace the paragraph “The AP may communicate its STA Channel Width either by transmitting the HT Information Element (in beacons and probe response frames) or by transmitting the Notify Channel Width management action frame.” By “The HT-AP-19 may communicate its BSS channel width by transmitting the beacons and probe response frames with related values of the STA Channel width and Secondary Channel Offset in HT Information Element.” Remove the last paragraph of this subclause. |  |
| 2884 | Trainin, Solomon | 165.35 | 9.20.4 | C | This text is not related to the purpose of this subclause. It is related to any 40/20 capable AP and not to HT-AP-19 only and should be defined separately |  | Move the text that starts with "The AP may communicate its STA Channel Width either by transmitting the HT Information …" and ends with " … request has been adopted after having received an HT frame of requested specified channel width." to new subclause "Switching between 40Mhz PPDUs and 20Mhz PPDUs under 40MHz mask". Remove the "STA Channel Width" field from the HT Information Exchange frame. | Counter – the text, and the two subsequent paragraphs of text are moved to the beginning of the subclause, the STA Channel Width field is removed from the HT information exchange frame – as per 11-07-xxxxry |
| 2509 | Stephens, Adrian | 165.47 | 9.20.4 |  | "Recommending a transmission width of 20 MHz has no effect on the BSS requirements regarding protection of 40 MHz transmissions or on the STA Channel Width advertised by the associated AP. A STA should reassociate with its Supported Channel Width Set field set to 0 (indicating only 20 MHz operation) to obtain this protection." I believe that: 1. Notify Channel Width should allow a change in receiver operating channel width 2. Operating mode 2 is useless. Operating mode of 0 is usually paired with 2. The only place in the spec where it doesn't is in 11.16 (PCO). However, it makes no sense, IMHO for an AP to be operating PCO unless there are active 20MHz STA, so no reason to explicitly signal their presence. If operating mode 2 is removed, there are no BSS requirements regarding protection of 40 MHz transmissions dependent on the presence of 20MHz HT STA and the quoted paragraph can be removed. |  | Reserve operating mode 2 in n30. Remove all references to operating mode 2. Remove the quoted paragraph. | Counter – see CID 2134 |
| 2885 | Trainin, Solomon | 165.47 | 9.20.4 | A | There is no more requirement to protect especial 40MHz transmission so this paragraph is not relevant any more |  | remove the paragraph starts with "Recommending a transmission width of 20 MHz has no effect on the BSS requirements …" | Accept – removed as shown in 11-07-xxxxry |
| 2886 | Trainin, Solomon | 165.47 | 9.20.4 | C | The Forty MHz Intolerant field is used for two different purposes: 1) to represent the non-AP STA or BSS intolerance to 40MHz transmission of any other STA; 2) to signal the trigger detection from STA to associated AP. Due to this duality the current definition of the switching between 40MHz and 20MHz does not prevent from propagation of 40MHz intolerance - the non-AP STA becomes 40MHz intolerant while seeing any other STA 40MHz intolerance and may transmit its HT Capabilities element with Forty MHz Intolerant field while re-associating. The solution is not asserting the Forty MHz Intolerant field to 1 in the HT Capabilities element when the trigger event happens. |  | Change the text that starts with "— disassociate from the HT-AP-19 and reassociate with the HT-AP-19" as follows: "— disassociate from the HT-AP-19 and reassociate with the HT-AP-19 with its Supported Channel Width Set field set to 0, indicating support only for 20 MHz operation and successfully transmit to its associated HT-AP-19 an HT Information Exchange management action frame with the Forty MHz Intolerant field set to 1". Remove the text "— The STA shall not reassociate with the HT-AP-19 with a Capability element containing a value of 0 for the Forty MHz Intolerant field before a period of thirty minutes has elapsed during which there are no BSS width trigger events a) or b)." Change the "Before transmitting a frame containing an HT Capability element with the Forty MHz Intolerant bit set to 0 ..." to "Before transmitting an HT Information Exchange management action frame with the Forty MHz Intolerant bit set to 0 ..." | Counter – alternate solution was suggested, which is to provide separate bits, as is done per 11-07-xxxxry |
| 3052 | Zaks, Artur | 165.47 | 9.20.4 | C | The definition is ambigues. |  | re-define the paragraph | Counter – paragraph has been deleted as per 11-07-xxxxry |
| 1753 | Perahia, Eldad | 165.53 | 9.20.4 | A | clarify that a 20 MHz-only capable STA may ignore this |  | as in comment | Accept – clarified with the use of the new term FC-HT-STA-19 as per 11-07-xxxxry |
| 1754 | Perahia, Eldad | 165.53 | 9.20.4 | A | a 20/40 STA that is operating as 20 MHz I assume can ignore the first sub-bullet. However, does it need to perform the operation in the second bullet? |  | please clarify | Accept – 20 mhz only STA does not need to perform either operation – all is rewritten to clarify as per 11-07-xxxxry |
| 1755 | Perahia, Eldad | 165.53 | 9.20.4 | A | is there a reason that in this line we use HT STA, and in other lines, like 65 we only use STA? |  | clarify terminology | A – yes there was a reason, because not all STA associated to an HT-AP-19 are subject to these rules – further clarified in rewrite of 11-07-xxxxry |
| 3053 | Zaks, Artur | 165.53 | 9.20.4 | R | HT-AP-19 identifier is not defined |  | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 2510 | Stephens, Adrian | 165.56 | 9.20.4 | C | “”disassociate from the HT-AP-19 and reassociate with the HT-AP-19 with its Supported Channel Width Set field set to 0,”” I’m concerned that we have two different mechanisms to communication information, and they have different side-effects. This adds complexity, and probably is the cause of a number of bugs in the draft standard – not all of which I may have found. I would prefer to simplify. A STA always has the option of re-assocation if it wants to change its capabilities. We don’t need to describe this in this section. |  | Make the HT Information Exchange mechanism mandatory for all HT STA (or all 40MHz HT STA if other changes made by other comments result in 20MHz STA no longer being required to communicate this information) Describe all normative signalling related to communication of this bit in terms of the HT Information Exchange mechanism. Remove any mention of re-association in this subclause. | Counter – general agreement with the comment, but the specifics of the changes are not quite exactly as per the commentor’s suggestions – see 11-07-xxxxry for exact changes |
| 3054 | Zaks, Artur | 165.56 | 9.20.4 | R | HT-AP-19 identifier is not defined |  | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 3055 | Zaks, Artur | 165.60 | 9.20.4 | R | HT-AP-19 identifier is not defined |  | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 1756 | Perahia, Eldad | 165.65 | 9.20.4 | A | clarify that a 20 MHz-only capable STA may ignore this |  | as in comment | Accept – does not apply to 20 MHz only STA - rewritten to clarify as per 11-07-xxxxry |
| 2511 | Stephens, Adrian | 165.65 | 9.20.4 | C | “conditions and performed at least one of the above operations” It only performs one of them because line 55 says “either”, not “either or both of”. |  | Reword: “conditions and performed one of the above operations” | Counter – language modified to remove the confusing construct as per 11-07-xxxxry |
| 3056 | Zaks, Artur | 165.65 | 9.20.4 | C | The definition “ the above operations” is ambigues. |  | Define explicitely which operations | Counter – language modified to remove the confusing construct as per 11-07-xxxxry |
| 3057 | Zaks, Artur | 166.06 | 9.20.4 | C | 30 min fixed value is not correct |  | define as “dot11Channel Width Agility Time”, UINT32, in seconds default: 60. | Counter – MIB variable has been defined and put in place, but with different name and default value as per 11-07-xxxxry |
| 3058 | Zaks, Artur | 166.08 | 9.20.4 | R | HT-AP-19 identifier is not defined |  | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 2875 | Trainin, Solomon | 166.09 | 9.20.4 | A | 20MHz capable station has reduced obligations; this STA has no obligation to respond to beacon request and support HT information exchange frames but in some circumstances shall scan. |  | Leave the 20MHz capable STA with no obligations at all | Accept – 20 MHz STA are not obliged to perform these actions, as per 11-07-xxxxry |
| 3059 | Zaks, Artur | 166.09 | 9.20.4 | C | 30 min fixed value is not correct |  | define as “dot11Channel Width Agility Time”, UINT32, in seconds default: 60. | Counter – MIB variable has been defined and put in place, but with different name and default value as per 11-07-xxxxry |
| 721 | Kasher, Assaf | 166.18 | 9.20.4 | C | "Before transmitting a frame containing an HT Capability element with the Forty MHz Intolerant bit set to 0, an HT STA that is associated with an HT-AP-19 shall complete an overlapping BSS scan at a time no earlier than one beacon interval preceeding the transmission." This shall apply only to station with that are operating in 40MHz. |  | Change to "Before transmitting a frame containing an HT Capability element with the Forty MHz Intolerant bit set to 0, an HT STA that has set the Supported Channel Width Set field to a non zero value and is associated with an HT-AP-19 shall complete an overlapping BSS scan at a time no earlier than one beacon interval preceeding the transmission." | Counter – same outcome, different wording – now applies only to 40 mhz capable STA as per 11-07-xxxxry |
| 1757 | Perahia, Eldad | 166.18 | 9.20.4 | A | clarify that a 20 MHz-only capable STA may ignore this |  | as in comment | Accept – see changes in 11-07-xxxxry |
| 1758 | Perahia, Eldad | 166.18 | 9.20.4 | A | If a 20/40 capable STA is operating in 20 MHz mode with no intent on operating in 40 MHz, my interpretation of 9.20.1 page 163, line 30 is that it skips this section. However, what should it do with the Forty MHz Intolerant bit in the HT Capability element , especially if it wants to transmit a frame containing an HT Capability Element for another reason. |  | please clarify | Accept – the sentence at p 163 line 30 has been deleted. STA operating in 20 mhz mode have no responsibility for identifying BSS width trigger events, as per changes found in 11-07-xxxxry |
| 2512 | Stephens, Adrian | 166.18 | 9.20.4 | C | General question on scanning for OBSS. The scanning requirements on a 20MHz STA are much reduced compared to a 40MHz STA. A 20MHz STA is only required to scan after detecting and reporting an intolerant state (i.e. rx a HT beacon from OBSS with the intolerant bit set on its primary channel), and then reporting this state cleared, and then only if it uses reassociation (i.e. not the HT Information element mechanism) to report it. Either it was the intention of the group to relieve a 20MHz STA of the responsibility of scanning, or it was not. This half-way house makes no logical sense. I recommend that 20MHz STA share exactly the same burden of scanning as 40Mhz STA. My rationale is as follows: 1. A 20MHz STA is capable of scanning these channels - i.e. the ability to tune to and receive on a 40 MHz affected channel is not specific to 40MHz STA. 2. A 20MHz STA may be the only STA of a HT BSS in range of a HT OBSS with the intolerant bit set. |  | Remove all language that limits scanning to 40Mhz STA. For example, remove phrases such as: "that indicates in the Supported Channel Width Set field of its most recently transmitted HT Capability element a value of 1" on line 23. | Counter – the commentor refers to the mix of required responsibilities of 20 mhz-only capable STA – the group has expressed its desire that the requirements for scanning only apply to 40 mhz-capable STA, with changes to reflect this decision reflected in 11-07-xxxxry |
| 2513 | Stephens, Adrian | 166.18 | 9.20.4 | C | "Before transmitting a frame containing an HT Capability element with the Forty MHz Intolerant bit set to 0, an HT STA that is associated with an HT-AP-19 shall complete an overlapping BSS scan at a time no earlier than one beacon interval preceeding the transmission." There should be the same requirement on the HT Information element. |  | Insert: "or an HT Information element with the Forty MHz Intolerant bit set to 0," | Counter – correct reference is to the HT Information field of the HT Information exchange mgmt action frame, not the HT information element – see changes as per 11-07-xxxxry |
| 2514 | Stephens, Adrian | 166.18 | 9.20.4 | C | It is not clear who is responsible for scanning. We have: 1. A STA must scan before telling it's AP the intolerant bit is 0 2. A STA must accept scan requests from its AP 3. The AP must send scan requests at least every 30 minutes It seems to me that there is some unnecessary specification. Surely if the STA must respond to the AP and the AP must ask it to scan every 30 minutes, then STA-initiated scanning is redundant. |  | Please document as an informative note in the draft why the different scanning responsibilities are distributed thus. | Counter – the original intent was to provide a requirement that STA perform some scanning, and if the AP-requested scans caused an automatic fulfilment of this requirement, then no redundant behaviour was expected – in any case, the group has spoken such that it prefers only STA-initiated scanning to occur and the draft language has been changed to reflect this desire as per the changes of 11-07-xxxxry |
| 3060 | Zaks, Artur | 166.19 | 9.20.4 | C | 30 min fixed value is not correct |  | define as "dot11Channel Width Agility Time", UINT32, in seconds default: 60. | Counter – MIB created, but with different name as per 11-07-xxxxry |
| 3061 | Zaks, Artur | 166.22 | 9.20.4 | C | The definition is ambigues. |  | re-define the paragraph | Counter – paragraph has been deleted as per 11-07-xxxxry |
| 119 | Adachi, Tomoko | 166.23 | 9.20.4 | R | "An HT STA that indicates in the Supported Channel Width Set field of its most recently transmitted HT Capability element a value of 1 shall accept all Beacon Measurement Requests from its associated HT-AP-19 that have a value of 254 for the reporting condition, except when the total number of octets in the transmitted MSDUs and received unicast MSDUs during the past 30 minutes did not exceed 10000 octets." The condition relating to the Supported Channel Width Set field is too restrictive. If the HT STA is only using the 20 MHz mask, it doesn't need to accept the request. The HT STA needs to disassociate and associate, or re-associate to change its capability. In 7.3.1.24 Channel Width field, it says "When the Channel Width field is set to 0, it indicates use of the 20 MHz primary channel. When set to 1, it indicates use of any channel width enabled under the Supported Channel Width Set." This field in the Notify Channel Width frame can be used instead. The point is that an HT STA doesn't need to transmit the Notify Channel Width frame. |  | Change "An HT STA that indicates in the Supported Channel Width Set field of its most recently transmitted HT Capability element a value of 1 shall accept all Beacon Measurement Requests from its associated HT-AP-19 that have a value of 254 for the reporting condition, except when the total number of octets in the transmitted MSDUs and received unicast MSDUs during the past 30 minutes did not exceed 10000 octets." to "If an HT STA indicates in the Supported Channel Width Set field of its most recently transmitted HT Capability element a value of 1, and whenever it transmits a Notify Channel Width frame if the most recently transmitted Channel Width field is also set to 1, it shall accept all Beacon Measurement Requests from its associated HT-AP-19 that have a value of 254 for the reporting condition, except when the total number of octets in the transmitted MSDUs and received unicast MSDUs during the past 30 minutes did not exceed 10000 octets." It is preferable that this sentence is divided into shorter sentences. | Reject – There are two aspects to consider – 1) the potential operating condition of the BSS 2) the potential operating condition of the associated STA – if the BSS has the potential to operate at 40 MHz, then scanning must be performed while the BSS is operating at 20 MHz in order to allow a possible future transition to 40 MHz. So for the BSS, the potential for 40 MHz should be the condition to test when determining if scanning is necessary. The argument is similar for the STA. |
| 626 | Gosteau, Jeremy | 166.23 | 9.20.4 | C | To gain on power efficiency and have interest to use 40MHz in power saving mode, scanning at 40MHz could be combined with the usual 20MHz scanning in a smart manner |  | Try to find a solution in a Coexistence 20/40MHz group | Counter – see CID 2887 |
| 2515 | Stephens, Adrian | 166.23 | 9.20.4 | C | What this section doesn't say is that a STA that receives a Beacon Measurement request with a condition of 254 shall perform the measurement, but is not required to return the result. However a side effect of the measurement may be the detection of a BSS width trigger event. The issue is what is meant by "accept" on line 24. |  | Reword: "An HT STA that indicates in the Supported Channel Width Set field of its most recently transmitted HT Capability element a value of 1 shall perform measurement as indicate by a Beacon Measurement Request from its associated HT-AP-19 that has a value of 254 for the reporting condition, except when the total number of octets in the transmitted MSDUs and received unicast MSDUs during the past 30 minutes did not exceed 10000 octets. The STA may choose to return a Beacon Measurement Report or not according to local policy. NOTE-The main purpose of this scanning is to allow BSS width trigger events to be detected." | Counter – scanning is now STA-initiated as per 11-07-xxxxry |
| 3100 | Kakani, Naveen | 166.23 | 9.20.4 | C | A STA can associate with an 20/40 AP as a 20/40 STA but it can switch to "Supported Channel Width = 0" in between. Can the AP exempt this STA from scanning. If yes then there is a potential problem. |  | Clarify and if the switch is possible then disallow the STAs to use 40 MHz for certain period of time (ex: 1 minute) | Counter – clarified here, but no change to the language of the draft – in that the STA that is 40 mhz capable as per the HT Capability Supported Channel Width field shall perform scanning, independently of the notify channel width indication – if it instead, switches its supported channel width to 20 mhz, it can only do this through dissociation and a new association, in which case, there is no problem. |

SPLIT

| 3101 | Kakani, Naveen | 166.23 | 9.20.4 | C | The intent of the text in lines 23-26 is to allow exemption for power save STAs. However, the text allows only for power save for STAs that are already associated with the AP and have signalled 20/40 capability and are not involved in any active session. Allow flexibility for power save STAs to use 40 MHz for their sessions but at the same time pay penalty for not scanning in the form of not using 40 MHz for certian period of time after they are out of power save mode (Power Managment bits in the Frame control field) | Submission 11-07-129r3 along with the following conditions : - If a STA is in Power Save Mode it shall not set it's channel width to 40 MHz atleast until 1 minute after it has come out of power save mode | Counter – the question is not one of making it difficult to use both PS and 40 mhz channel width, but rather of sharing in the burden of ensuring that other BSS are not harmed by 40 mhz operations. Any STA that wishes to use 40 mhz must share this burden. By defining a low level of activity, the STA can be said to not be sharing in the benefit of using 40 mhz, and in that case, may be exmpeted from sharing the cost of using 40 mhz. See changes introduced by CID 2887. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3062 | Zaks, Artur | 166.24 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 1759 | Perahia, Eldad | 166.25 | 9.20.4 | C | If the HT STA doesn't want to scan and is hardly sending any data, then it seems simpler to for it to switch to 20 MHz operation rather than add loop-holes to the spec | remove exception and and information saying the device may switch to 20 MHz operation to avoid scanning. | Counter – use a percentage airtime criteria, rather than octets – see CID 2887 |
| 2887 | Trainin, Solomon | 166.25 | 9.20.4 | A | "except when the total number of octets in the transmitted MSDUs and received unicast MSDUs during the past 30 minutes did not exceed 10000 octets." The limitation of 10000 octets is not related to the MAC parameters and cannot be justified. Bring another criteria to allow low power devices not to scan when the activity is very low. | Bring another criteria to allow low power devices not to scan when the activity is very low. | Accept – add the following language to the draft in place of the existing allowance of a 10000 octet exception “ An FC-HT-STA-19 that is associated with an FC-HT-AP-19 shall perform at least one OBSS scan every dot11BSSWidthTriggerScanInterval seconds, except when the total duration of the transmitted MSDUs and received unicast MSDUs during the past 30 minutes did not exceed 0.5% (i.e. did not exceed 9 seconds).” As per 11-07-xxxxry |
| 2888 | Trainin, Solomon | 166.25 | 9.20.4 | C | The limitation of 10000 octets is not related to the MAC parameters and cannot be justified. Bring another criteria to allow low power devices not to scan when the activity is very low. | Replace the text "except when the total number of octets in the transmitted MSDUs and received unicast MSDUs during the past 30 minutes did not exceed 10000 octets." by "except when the device is going to sleep and not receiving beacons for average of ten seconds during the thirty minutes interval." | Counter – see CID 2887 |
| 2516 | Stephens, Adrian | 166.26 | 9.20.4 | C | "minutes did not exceed 10000 octets" - grammar | "minutes does not exceed 10000 octets" | Counter – language changed as per CID 2887 – still uses a similar construct – not convinced that the event being measured was not in the past. |
| 722 | Kasher, Assaf | 166.27 | 9.20.4 | A | * | Disallow 40MHz @ 2.4 for IBSS | Accept – statement to this effect, “An HT-STA-19 that is a member of an IBSS shall not set the Secondary Channel Offset field of the HT Information element to a non-zero value in transmitted frames. “ added to 9.20.1 as per 11-07-xxxxry |
| 1760 | Perahia, Eldad | 166.29 | 9.20.4 | A | wouldn't this paragraph also apply to an AP and STA operating in 5 GHz? | please clarify | Accept – 5 GHz rules clarified as per changes indicated in 11-07-xxxxry |
| 2518 | Stephens, Adrian | 166.29 | 9.20.4 | C | "An HT-AP-19 that indicates in the Secondary Channel Offset field of its most recently transmitted HT Information element a non-zero value shall assume a value of 1 for the STA Channel Width of a STA that has associated with a value of 1 in the Supported Channel Width Set field of its HT Capability element until it receives an HT Information Exchange management action frame from that STA." This is both wrong and unnecessary. If a STA sends the AP a Notify Channel Width frame, this paragraph says "ignore it". There's no need to talk about "assuming" values. This case is no different than the 5GHz case just after association. | Remove the quoted paragraph. | Counter – this paragraph has been rewritten to correct the errors as per 11-07-xxxxry |
| 3063 | Zaks, Artur | 166.29 | 9.20.4 | A | The definition is ambigues. | re-define the paragraph | Accept – this paragraph has been rewritten as per 11-07-xxxxry |
| 3064 | Zaks, Artur | 166.29 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 1761 | Perahia, Eldad | 166.35 | 9.20.4 | A | wouldn't this paragraph also apply to an AP and STA operating in 5 GHz? | please clarify | Accept – 5 GHz rules clarified as per changes indicated in 11-07-xxxxry |
| 2519 | Stephens, Adrian | 166.35 | 9.20.4 | C | "An HT-AP-19 that indicates in the Secondary Channel Offset field of its most recently transmitted HT Information element a value of 0 shall assume a value of 0 for the STA Channel Width of a STA that has associated with a value of 1 in the Supported Channel Width Set field of its HT Capability element until it receives an HT Information Exchange management action frame from that STA." This is wrong and unnecessary. An AP that indicates a secondary channel offset of 0 is operating a 20MHz BSS. It and all its STA can only use 20MHz. It doesn't need to "assume" any other fields have any other values to generate this behaviour. It's wrong because it says "Ignore Notify Channel Width frames until you see an HT Information Exchange frame" - which is clearly daft logic. | Remove the quoted paragraph. | Counter – this paragraph has been rewritten to address these issues as per 11-07-xxxxry |
| 3065 | Zaks, Artur | 166.35 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 3066 | Zaks, Artur | 166.35 | 9.20.4 | A | The definition is ambigues. | re-define the paragraph | Accept – this paragraph has been rewritten as per 11-07-xxxxry |
| 1762 | Perahia, Eldad | 166.41 | 9.20.4 | C | If I understand this paragraph correctly, the STA assumes that the AP assumes that it is 40 MHz capable? Isn't it safer for the AP to assume that the STA is only 20 MHz capable until it hears otherwise? And whatever this paragraph means, I don't see anything specific to 2.4 GHz, so should it also apply to 5 GHz? | please clarify | Counter – 2.4 vs 5 GHz operation clarified as per 11-07-xxxxry – as for width capability, that is explicitly visible in the HT Cap element. What is assumed in this instance is not capability, but current operating width. |
| 2520 | Stephens, Adrian | 166.41 | 9.20.4 | A | "An HT STA shall assume that its associated HT-AP-19 has assumed a value of 1 for the STA Channel Width value for corresponding to its association until the HT STA successfully transmits an HT Information Exchange management action frame to the HT-AP-19." This is ambiguous specification, and completely unnecessary. The STA Channel Width field is unnecessary. There is no reason that an AP would want to allow a BSS to operate in 40MHz while itself it can only receive in 20MHz from any of its STA. | Remove the quoted text. | Accept – see 11-07-xxxxry |
| 3067 | Zaks, Artur | 166.41 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 3068 | Zaks, Artur | 166.43 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 1763 | Perahia, Eldad | 166.46 | 9.20.4 | C | This paragraphs makes it sound like in the first two minutes of association, the HT STA does not need to scan prior to operating in 40 MHz mode. If this is correct, then it should be changed that the HT STA must scan prior to first transmission after association. The HT STA should not be causing interference for two minutes. | as in comment | Counter – scanning requirement changed from AP-initiated to STA-initiated as per 11-07-xxxxry, effectively eliminating the cited paragraph |
| 3069 | Zaks, Artur | 166.46 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 3070 | Zaks, Artur | 166.49 | 9.20.4 | C | fixed 2 minutes definition is not aceptable | Define value as MIB IE | Counter – MIB variable defined as per 11-07-xxxxry, but the cited text has been deleted |
| 3071 | Zaks, Artur | 166.54 | 9.20.4 | C | fixed 10TU definition is not aceptable | Define value as MIB IE | Counter – MIB variable defined as per 11-07-xxxxry, but the cited text has been deleted |
| 2527 | Stephens, Adrian | 166.60 | 9.20.4 | C | "the reporting condition shall be 254 (report not necessary), and" - informal | Reword thus: "the Reporting Condition field shall be set to 254 (representing Report Not Necessary), and" | Counter – text has been deleted as per 11-07-xxxxry |
| 2889 | Trainin, Solomon | 166.63 | 9.20.4 | C | The RCPI is related to the beacon only measurement | Replace the word "frame" by "beacon" | Counter – text has been deleted as per 11-07-xxxxry |
| 2529 | Stephens, Adrian | 166.65 | 9.20.4 | C | "Received beacons that do not meet this minimum RCPI threshold do not need to be processed." This is behaviour for the STA, not the HT-AP-19. It is somewhat buried here, and easy to overlook. Also, it is not clear exactly which of the procedures do not need to be processed. For example, it may affect only reporting to the AP. Or it may be intended to filter and reject beacons containing the intolerant bit set according to RCPI. | Clarify the intent. One of the following statements may be a suitable additional paragraph: 1. "An HT STA that receives a Beacon Request Measurement frame containing a non-zero Threshold/Offset field value shall use the specified only to filter the beacons that comprise the Beacon Report frame. The Threshold/Offset field value has no other effect." 2. "An HT STA that receives a Beacon Request Measurement frame containing a non-zero Threshold/Offset field value shall use the specified value to filter beacons on the specified Channel Number. This filtering shall be applied before operating any of the procedures defined in this subclause." | Counter – text has been deleted as per 11-07-xxxxry |
| 1764 | Perahia, Eldad | 167.01 | 9.20.4 | C | clarify that a 20 MHz-only capable STA may ignore this | as in comment | Counter – text has been deleted as per 11-07-xxxxry |
| 2530 | Stephens, Adrian | 167.01 | 9.20.4 | C | "During each successive thirty minute period of continued association by an HT STA, the associated HT-AP-19 shall transmit additional Beacon Request Measurement requests to that HT STA such that all of the channels in the AP Channel Report shall have been scanned at least twice by the HT STA for a minimum duration of 10 TU per scanned channel during the thirty minutes." This is not dependent on STA width capability, where elsewhere it is. | I propose elswhere to move this dependency. But if that comment is rejected, this needs to be modified to say "an HT STA that set the Supported Channel Width set field to 1 in its last HT Capabilities element transmission" | Counter – text has been deleted as per 11-07-xxxxry |
| 3072 | Zaks, Artur | 167.01 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 3073 | Zaks, Artur | 167.01 | 9.20.4 | C | The definition is ambigues. | re-define the paragraph | Counter – paragraph has been deleted as per 11-07-xxxxry |
| 3074 | Zaks, Artur | 167.05 | 9.20.4 | C | fixed 10TU definition is not aceptable | Define value as MIB IE | Counter – text has been deleted as per 11-07-xxxxry |
| 2532 | Stephens, Adrian | 167.07 | 9.20.4 | C | "An HT-AP-19 that set the Supported Channel Width Set field of its most recently transmitted HT Capability element to a non-zero value shall transmit an AP Channel Report containing at least all of the 40 MHz affected channels excluding the primary channel." This doesn't say when it transmits the report. It is probably intended to relate to earlier text in this subclause. Also "excluding the primary channel" is unnecessary because the definition of 40 MHz affected channels achieves this. | Reword: "An HT-AP-19 that set the Supported Channel Width Set field of its most recently transmitted HT Capability element to a non-zero value shall ensure that any AP Channel Report it transmits contains at least all of the 40 MHz affected channels." | Counter – relevant text has been deleted in favour of STA-initiated scanning as per 11-07-xxxxry |
| 3075 | Zaks, Artur | 167.07 | 9.20.4 | R | HT-AP-19 identifier is not defined | Define HT-AP-19 | Reject – HT-AP-19 is defined in the definitions clause (clause 3) |
| 253 | Bjerke, Bjorn | 167.08 | 9.20.4 | C | As the text in 9.20.4 is currently written, it is implicit that .11k Beacon measurement request and AP Channel Report are mandatory for HT STAs operating at 2.4 GHz. | Make this requirement explicit, and preferably up front rather than keeping it buried towards the end of the clause | Counter – AP-initiated scanning has been deleted – all scanning is now STA-initiated as per 11-07-xxxxry |
| 3076 | Zaks, Artur | 167.08 | 9.20.4 | C | "a non-zero-value" definition is ambigues | change to "1" | Counter – cited text has been deleted as per 11-07-xxxxry |
| 3077 | Zaks, Artur | 167.08 | 9.20.4 | C | "at least all" deginition is ambigues | change to "all" | Counter – cited text has been deleted as per 11-07-xxxxry |
| 798 | Ketchum, John | 206.00 | 11.17 | A | second paragraph in 11.17 is a verbatim repetition of the first paragraph, with the exception that "STA" is replaced by "AP". This is redundant, since an AP is also a STA | remove the paragraph at lines 43-44 on page 206 | Accept – delete the second paragraph of 11.17 as indicated in 11-07-xxxxry. |
| 3088 | Zaks, Artur | 206.00 | 11.17 | C | This definition is not correct: when 802.11w is applied, MMPDUs are encrypted. Oher APs will not be able to decrypt. | Remove this paragraph | Counter – the HT Information exchange mgmt action frame has been moved to the Spectrum Management action group – all of those frames are defined as class 1 – see the changes found in 11-07-xxxxry |
| 2622 | Stephens, Adrian | 206.37 | 11.17 | C | The name "HT Information Exchange" is poor. The frame is used to manage coexistence. So, I prefer: HT Coexistence Mangement frame. | Rename: "HT Coexistence Mangement frame" here and elsewhere in the document. | Counter – use “20/40 BSS Coexistence Management” as per 11-07-xxxxry |
| 1229 | Marshall, Bill | 206.43 | 11.17 | A | an AP is a STA. An AP is a STA. An AP is a STA. | delete this sentence. It duplicates the one at line 39 | Accept – see CID 798. |
| 2624 | Stephens, Adrian | 206.43 | 11.17 | A | "An AP that supports the HT Information Exchange management action frame type shall set the HT Information Exchange Support bit to 1 in transmitted Extended Capabilities information elements." This is redundant given the previous paragraph as an AP is a STA. | Delete the quoted text. | Accept – see changes in 11-07-xxxxry |
| 2625 | Stephens, Adrian | 206.46 | 11.17 | C | "A STA that supports the transmission of and reception of the HT Information Exchange management action frame may transmit any information that is indicated in the HT Information Exchange management action frame format to any other STA that also supports the transmission of and reception of the HT Information Exchange management action frame, including to the AP with which it is associated." It is not clear if this assumes that the AP supports this. Note, an AP is a STA. So the last phrase is unnecessary unless it is implying that the AP must support it (which is not defined anywhere else). | Delete the quoted text. | Counter – deleted the last phrase of the quoted sentence because, as the commentor pointed out, it may contradict the AP’s advertised support – but left the rest of the sentence as per 11-07-xxxxry |
| 2626 | Stephens, Adrian | 206.53 | 11.17 | A | “Additionally, a STA that supports the transmission of and reception of the HT Information Exchange management action frame may transmit any information that is indicated in the HT Information Exchange management action frame format to any APs with which it is not associated using either an individual or group addressed frame.” This implies that all Aps, past and present, support this feature. | Reword: “Additionally, a STA that supports the transmission of and reception of the HT Information Exchange management action frame may transmit any information that is indicated in the HT Information Exchange management action frame format to an AP with which it is not associated that supports the supports the HT Information Exchange management action frame type using either an individual or group addressed frame.” | Accept – see changes in 11-07-xxxxry |
| 1230 | Marshall, Bill | 206.55 | 11.17 | A | sending an HT Information Exchange action frame while unassociated isn’t allowed by the rules in 11.3 | make this sentence consistent with 11.3. Either fix this sentence, or fix 11.3. | Accept – changed frame from HT Action category to Spectrum Management category, since those frames are always class 1, as per 11-07-xxxxry |
| 2896 | Trainin, Solomon | 206.55 | 11.17 | R | The basic .11ma spec does not allow STA associated in BSS transmission of multicast frames | Replace “group addressed” to broadcast | Reject – baseline subclauses 7.1.3.1.3 and 9.2.7 clearly allow MCAST transmissions by STA in a BSS. |
| 3334 | Erceg, Vinko | 206.61 | 11.17 | A | should broaden exception to include mcast | change “broadcast” to “broadcast or multicast” | Accept – see changes in 11-07-xxxxry |
| 2627 | Stephens, Adrian | 207.02 | 11.17 | A | “shall supersede any earlier received information” – this is meaningless normative specification | reword: “supersedes any earlier received information” | Accept – changes are specified in 11-07-xxxxry |
| 2628 | Stephens, Adrian | 207.07 | 11.17 | A | “The most recently successfully transmitted information from within an HT Information Exchange management action frame shall supersede any earlier transmitted information, regardless of the frame type and subtype that was used to communicate that information. For example, information may be provided within HT Information Exchange management action frames, within Management Action frames, and within other management frames.” I think this statement is unnecessary. It is only the effect on the receiver that is important. The transmitter knows why it transmitted this frame, presumably in response to some change in local state that has already taken place. | Remove the quoted paragraph. | Accept – changes described in 11-07-xxxxry |
| 2629 | Stephens, Adrian | 207.14 | 11.17 | A | “A STA may transmit an HT Information Exchange management action frame that contains a value of 1 for the Request Information bit to another STA that supports the transmission of and reception of HT Information Exchange management action frame.” We’re already done the “who can I send this frame to” logic. We don’t need to talk about support here. Also, we need to prevent endless loops, so there has to be a guard on when this bit is set to 1. | Reword thus: “A STA that transmits an HT Information Exchange frame may set the Request Information field to 1, except when this frame is a response to an HT Information Exchange frame in which the Request Information field is set to 1.” | Accept – changes are found in 11-07-xxxxry |
| 170 | Barr, John | 292.12 | 20.3.14 | C | Allowing operation with 40 MHz channels in 2.4 GHz spectrum will not coexist with over 1 billion Bluetooth (IEEE 802.15.1a) devices present around the world. In addition, operation of 40 MHz channels in 2.4 GHz spectrum will be subject to high levels of interference from Bluetooth devices. With only 80 MHz allocated in 2.4 GHz spectrum allocation of half of that spectrum to a single WLAN limits access by other radios sharing that spectrum. Coexistence analysis shows a significant degradation of 802.11n performance in the presense of Bluetooth devices, even with significant separation. Many devices include both Bluetooth and 802.11 making inteference even more significant. AFH defined in IEEE 802.15.2 was designed to allow IEEE 802.15.1 devices to reasonably avoid 20 MHz wide 802.11 devices. None of the billion Bluetooth devices deployed at this time have been designed to avoid 40 MHz 802.11n devices. | Change When using 40 MHz channels, it can operate in the channels defined in 20.3.14.1 (Channel allocation in the 2.4 GHz Band) and 20.3.14.2 (Channel allocation in the 5 GHz band)." to "When using 40 MHz channels, it can only operate in the channels defined in 20.3.14.2 (Channel allocation in the 5 GHz band)." Also change other places in the draft that imply operation with 40 MHz channels in 2.4 GHz spectrum. | Counter – a mechanism is provided which allows devices to “protect” BT devices by explicitly forbidding the use of 40 mhz in the band. The mechanism was implicitly available in the D2.0, and is modified by changes from document 11-07-xxxxry, such as to be more explicitly useful for this purpose. In particular, the use of the Forty_MHz_intolerant bit is per local STA MIB setting and can be transmitted via class 1 frames to non-associated BSSs to force them to operate only using 20 mhz of bandwidth. BT can be fully protected using this mechanism. In fact, providing such a mechanism is an improvement over the current situation, where many devices that claim conformance to IEEE 802.11 include proprietary 40 MHz operational modes that do not include any features for cooperative sharing of the band. |
| 2802 | Stephens, Adrian | 475.46 | T.2 |  | T.2 doesn't mention the "intolerant" bit. Seeing as this is perhaps the most complex part of BSS management, it certainly needs description here. I feel that the rules for "intolerant" are so complex, that without this informative material, there is a significant risk of mis-interpretation, with subsequent risk of non-interoperation. This is why I've marked this as part of my "no" vote, even though Annex T is informative. | Add to the table, or add a separate table. Ideally I'd like to see usage models described that show what happens and when when a legacy BSS starts operation or stops it in a way that requires a response by a 40MHz 2.4 GHz BSS. |  |

Group TWO

| 1771 | Perahia, Eldad |  |  | 201.44 |  | 11.15.1 |  | A |  | what's the difference between 11.15.1 and 9.20? Both subclauses describe 20/40 operation |  | please clarify |  | Accept – all of 9.20 now moved to 11.15 as per 11-07-xxxxry |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2871 | Trainin, Solomon |  |  | 164.00 |  | 9.20.4 |  | C |  | This subclause covers the specific case of switching the whole BSS between transmission of 40MHz mask PPDUs and 20MHz mask PPDUs. Heading of this subclause does not reflect context of it |  | Change the name of this subclause to "Switching BSS between transmission of 40MHz mask PPDUs and 20MHz mask PPDUs" |  | Counter – change the name of the 11.15 subclause to “20/40 MHz BSS operation” and move 9.20 to be a set of subclauses beneath 11.15 as per 11-07-xxxxry |
| 2483 | Stephens, Adrian |  |  | 164.24 |  | 9.20.4 |  |  |  | How does 20/40 switching work with ISSS? IBSS is an HT feature. We have not properly addressed how an IBSS operates channel width switching. |  | I propose that 40MHz in 2.4 GHz not be allowed in an IBSS (this is because the 20/40 coexistence rules require the presence of the AP to manage response to BSS width tirgger events). Review the language attached to the channel switching and use of extended channel switch announcement to ensure that it speaks of STAs, not APs. This will allow its use by the DFS owner of a 5GHz 40MHz IBSS. |  |  |
| 2490 | Stephens, Adrian |  |  | 164.27 |  | 9.20.4 |  | A |  | "Once associated, a non-AP STA shall support the same channel width capabilities it declared in its Association Request." This essentially says "don't lie in your association request". We don't need this kind of statement as the definition of the field in clause 7 suffices. |  | Remove the quoted sentence. |  | Accept |
| 2491 | Stephens, Adrian |  |  | 164.28 |  | 9.20.4 |  | A |  | "This capability is asserted in the Supported Channel Width Set in the HT capabilities element." What is "assertion"? I assert we don't need this statement. |  | Remove the quoted text. |  | Accept |
| 2493 | Stephens, Adrian |  |  | 164.32 |  | 9.20.4 |  | R |  | "A non-AP STA that is a member of a 40 MHz BSS shall not transmit 20 MHz frames in the secondary channel." This should also be a requirement on an AP |  | Remove "non-AP" from the above. |  | Reject – PCO requires AP transmissions of 20 MHz frames in the control channel. |
| 2589 | Stephens, Adrian |  |  | 201.40 |  | 11.15 |  | A |  | I don't understand why we've got bits of 20/40 switching in 9.20 and bits here. I would have thought that most of 9.20.4 actually lives in 11, as it does not relate to the transfer of data, but to management of the channel. |  | Move 9.20.4 into 11.15 and merge with the existing sections there. |  | Accept – move all of 9.20 to 11.15, placing the existing subclauses of 11.5 to appear before any subclause of 9.20 (either pre-existing or new) as per 11-07-xxxxry |
| 2590 | Stephens, Adrian |  |  | 201.57 |  | 11.15.1 |  | A |  | "An HT AP declares the BSS channel width in the BSS Channel Width subfield in the HT Information element." Not so, there is no such field. |  | Replace: "An HT AP declares the BSS channel width by setting the Secondary Channel Offset field of the HT information element to a zero or non-zero value." |  | Accept |
| 3079 | Zaks, Artur |  |  | 164.00 |  | 9.20.4 |  |  |  | Operation in the IBSS is not defined |  | Define the operation in the IBSS |  |  |
| 3279 |  | Erceg, Vinko | 199.00 |  | 11.9.8.3 |  | C |  | Disallow 40 mhz BSS operation if the 40mhz BSS candidate cannot select a primary and secondary channel for operation that meet the following condition: Given the following definitions: P1 = the primary channel of the intended 40 MHz BSS S1 = the secondary channel of the intended 40 MHz BSS Pi = the set of primary operating channels for all established 40 MHz BSS operating within the channel range [P1-2, P1-1, P1, P1+1, P1+2, S1-2, S1-1, S1, S1+1, S1+2] in the target BSA Si = the set of secondary operating channels for all established 40 MHz BSS operating within the channel range [P1-2, P1-1, P1, P1+1, P1+2, S1-2, S1-1, S1, S1+1, S1+2] in the target BSA Ti = the set of sole operating channels for all established 20 MHz BSS operating within the channel range [P1-2, P1-1, P1, P1+1, P1+2, S1-2, S1-1, S1, S1+1, S1+2] in the target BSA An AP shall not establish a 40 MHz BSS unless it can meet the following conditions: 1) P1 = Pi for all values of I 2) P1 = Ti for all values of I 3) S1 = Si for all values of I When the right hand side of any of the above equalities is the null set, then the equality test result is defined to be true. If, after initial establishment of the 40 mhz BSS, the outcome of any of the equality tests above changes to FALSE, then the AP must immediately revert to 20mhz BSS operation. |  | Add the text shown in the comment to the subclause. |  | Counter – changes similar to this are added as per 11-07-0614r1 |  |

TGn Editor: Change the definition 3.n3 “40 MHz affected channel” at about page 1 line 29 in TGn draft D2.0 as follows:

3.n3 40 MHz affected channel: any 20 MHz channel of the channel set that is defined in Clause 19 and that is one of the seven 20 MHz channels that are closest to the secondary channel and centered on the secondary channel.

TGn Editor: Delete the definition 3.n6 “40 MHz sensitive channel” at about page 1 line 46 in TGn draft D2.0.

TGn Editor: Insert the following definition “forty megahertz capable high throughput accesspoint 19 (FC-HT-AP-19)” at about page 2 line 13 in TGn draft D2.0 renumbering it and the existing definitions as appropriate:

3.n15a forty megahertz high throughput access point 19 (FC-HT-AP-19): an HT-AP-19 that included a non-zero value in the Supported Channel Width Set field (indicating its capability to operate on a 40 MHz channel) of its most recent transmission of a frame containing an HT Capabilities element.

TGn Editor: Insert the following definitions “station 19 (STA-19)” and “station 17 (STA-17)” at about page 4 line 47 in TGn draft D2.0 renumbering them and the existing definitions as appropriate:

3.n57a station 17 (STA-17): a STA that is operating on a channel that belongs to any regulatory class that has a value of “20” or “40” for the entry in the column labeled “Channel Spacing (MHz)” of any of the tables found in Annex J.

3.n57b station 19 (STA-19): a STA that is operating on a 20 MHz channel or set of 20 MHz channels that belong to the channel set that is defined in Clause 19.

TGn Editor: Insert the following definitions “high throughput station 17 (HT-STA-17)”, “high throughput station 19 (HT-STA-19)” at about page 4 line 47 in TGn draft D2.0 renumbering it and the existing definitions as appropriate:

3.n57c high throughput station 17 (HT-STA-17): an HT-STA that is also a STA-17.

3.n57d high throughput station 19 (HT-STA-19): an HT-STA that is also a STA-19.

TGn Editor: Insert the following six definitions at about page 4 line 47 in TGn draft D2.0 renumbering it and the existing definitions as appropriate:

3.n57e forty megahertz capable high throughput AP (FC-HT-AP): an HT-AP that included a non-zero value in the Supported Channel Width Set field (indicating its capability to operate on a 40 MHz channel) of its most recent transmission of a frame containing an HT Capabilities Information element.

3.n57f forty megahertz capable high throughput station (FC-HT-STA): an HT-STA that included a non-zero value in the Supported Channel Width Set field (indicating its capability to operate on a 40 MHz channel) of its most recent transmission of a frame containing an HT Capabilities Information element.

3.n57g forty megahertz capable high throughput AP 17 (FC-HT-AP-17): an HT-AP-17 that is also an FC-HT-AP.

3.n57h forty megahertz capable high throughput station 17 (FC-HT-STA-17): an HT-STA-17 that is also an FC-HT-STA.

3.n57i forty megahertz capable high throughput AP 19 (FC-HT-AP-19): an HT-AP-19 that is also an FC-HT-AP.

3.n57j forty megahertz capable high throughput station 19 (FC-HT-STA-19): an HT-STA-19 that is also an FC-HT-STA.

TGn Editor: Insert a new row in the second table that appears in subclause “7.2.3.1 Beacon frame format” on page 34 near line 42 of TGn draft D2.0, as shown:

|  | Overlapping BSS Scan Parameters | The Overlapping BSS Scan Parameters element may be present if the dot11FortyMHzOptionImplemented is true. |
| --- | --- | --- |

TGn Editor: Insert a new row in the second table that appears in subclause “7.2.3.5 Association Response frame format” on page 35 near line 17 of TGn draft D2.0, as shown:

|  | Overlapping BSS Scan Parameters | The Overlapping BSS Scan Parameters element may be present if the dot11FortyMHzOptionImplemented is true. |
| --- | --- | --- |

TGn Editor: Delete all of the changes to subclause “7.3.2.21.6 Beacon Request” which are modifications to Table 29b that are found on page 55 beginning at about line 27 of TGn draft D2.0.

TGn Editor: Insert the following text to appear immediately preceding the text “7.3.2.37 Neighbor Report element” on page 59 at about line 45 of TGn draft D2.0:

7.3.2.36 AP Channel Report element

Change the first sentence of the sublcause as follows:

The AP Channel Report element contains a list of channels where a STA is likely to find an AP when the element is not part of an HT Information Exchange frame. When the element is part of an HT Information Exchange frame, the AP Channel Report element contains a list of channels on which a STA has found conditions that disallow the use of a 20/40 MHz BSS.

TGn Editor: Insert the following text and editor instruction and figure to appear as a new subclause, immediately preceding the text “7.4 Action frame format details” on page 76 at about line 53 of TGn draft D2.0, appropriately numbering the new subclause and adjacent subclauses to account for the presence of the new subclause, and appropriately formatting the figure, including providing an appropriate number for the figure:

Insert the following new subclause:

7.3.2.52a Overlapping BSS Scan Parameters element

The Overlapping BSS Scan Parameters element is used by an AP in a BSS to indicate the values to be used by BSS members when performing overlapping BSS scan operations.

| Octets | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | OBSS Scan Passive Dwell | OBSS Scan Active Dwell | BSS Width Trigger Scan Interval | OBSS Scan Passive Total Per Channel | OBSS Scan Active Total Per Channel | BSS Wide Channel Transition Delay |

Figure nxxx-0

The element ID value is to be assigned by the IEEE 802.11 WG ANA.

The length field shall be set to 12.

The OBSS Scan Passive Dwell field contains a value in TU that receiving STA are to use to set their dot11OBSSScanPassiveDwell MIB variable as described in 11.15.1a Scanning and reporting requirements for 40 MHz capable STA.

The OBSS Scan Active Dwell field contains a value in TU that receiving STA are to use to set their dot11OBSSScanActiveDwell MIB variable as described in 11.15.1a Scanning and reporting requirements for 40 MHz capable STA.

The BSS Width Trigger Scan Interval field contains a value in seconds that receiving STA are to use to set their dot11OBSSWidthTriggerScanInterval MIB variable as described in 11.15.1a Scanning and reporting requirements for 40 MHz capable STA.

The OBSS Scan Passive Total Per Channel field contains a value in TU that receiving STA are to use to set their dot11OBSSScanPassiveTotalPerChannel MIB variable as described in 11.15.1a Scanning and reporting requirements for 40 MHz capable STA.

The OBSS Scan Active Total Per Channel field contains a value in TU that receiving STA are to use to set their dot11OBSSScanActiveTotalPerChannel MIB variable as described in 11.15.1a Scanning and reporting requirements for 40 MHz capable STA.

The BSS Wide Channel Transition Delay field contains a value in seconds that receiving STA are to use to set their dot11BSSWideChannelTransitionDelay MIB variable as described in 11.15.1a Scanning and reporting requirements for 40 MHz capable STA.

The use of each of these parameters is described in 11.15.1a Scanning and reporting requirements for 40 MHz capable STA.

TGn Editor: Change the text found in the “Encoding” column of the row that contains “Forty MHz Intolerant” in the “Subfield” column of table n25 found in subclause “7.3.2.49.2 HT Capabilities Info field” on page 64 at about line 17 of TGn draft D2.0 as follows:

Set to 0 by an HT-AP to indicate to neighboring BSSs that the transmitting HT-AP is not requiring a prohibition of the use of 40 MHz transmissions in the neighboring BSS.

Set to 1 by an HT-AP if the AP does not allow use of 40 MHz transmissions

in to indicate  neghboring BSSs that the transmitting HT-AP is requiring a prohibition of the use of 40 MHz transmissions in the neighboring BSS.

When set to 0 by a STA, indicates to a a receiving AP that the AP is not required to prohibit the use of 40 MHz transmissions within its BSS.

When set to 1 by a STA as determined through local policy, indicates to a receiving AP that the AP is required to prohibit the use of 40 MHz transmissions within its BSS.

TGn Editor: Delete the last sentence of subclause “7.4.9.10 HT Information Exchange frame format” of TGn draft D2.0, that begins with “An HT-AP that indicated a value of 0” and that is found on page 85 at about line 10.

TGn Editor: Change figure n40 in subclause “7.4.9.10 HT Information Exchange frame format” on page 84 at about line 41 of TGn draft D2.0 by changing the one-bit “STA Channel Width” field at bit position 2 of the existing depicted octet of the HT Information field to the name “20 MHz BSS Width Request”.

TGn Editor: Change the text and figure n40 in subclause “7.4.9.10 HT Information Exchange frame format” on page 84 at about line 41 of TGn draft D2.0 by adding an additional octet to the existing octet to appear to the right of the existing octet of the HT Information field and modifying the text to account for the existence of the new octet with all bits of the new octet indicated as reserved in the figure and in the text.

TGn Editor: Change the text and table n40 in subclause “7.4.9.10 HT Information Exchange frame format” on page 84 at about line 33 of TGn draft D2.0 by adding an item labeled “AP Channel Report(s)” as a new field appearing as order number 4 and noting in the added text that the AP Channel Report element may appear multiple times and providing in the text, a reference to the AP Channel Report element subclause.

TGn Editor: Change the text and table n40 in subclause “7.4.9.10 HT Information Exchange frame format” on page 84 at about line 33 of TGn draft D2.0 by adding an item labeled “Overlapping BSS Scan Parameters” as a new field appearing as order number 5 and noting in the added text that the inclusion of the Overlapping BSS Scan Parameters element is optionally present, and including a reference to the new subclause (7.3.2.52a) that describes the new Overlapping BSS Scan Parameters element.

TGn Editor: Change the name “HT Information Exchange” to “20/40 BSS Coexistence Management” in all places that it occurs in TGn draft D2.0, and within any other edits that occur to TGn draft D2.0.

TGn Editor: Change the name “HT Information field” to “20/40 BSS Coexistence field” in all places that it occurs in TGn draft D2.0, and within any other edits that occur to TGn draft D2.0.

TGn Editor: Change the text in subclause “7.4.9.10 HT Information Exchange frame format” on page 84 starting at about line 60 of TGn draft D2.0 with the words “When the Information Request bit is set to 0”, as follows:

The Forty MHz Intolerant bit, when set to 1, indicates a request to the receiving STA to operate the receiving STA’s BSS using a 20 MHz operating channel width. Otherwise it is set to 0. The definition of this field is the same as in the HT Capabilities element (see 7.3.2.49) and its operation is described in 9.20.4.

The 20 MHz BSS Width Request field, when set to 1, indicates a request to the receiving STA to operate the receiving STA’s BSS using a 20 MHz operating channel width.  Otherwise it is set to 0. The operation of this field is described in 9.20.4.

TGn Editor: Change the text on page 84 at about line 41 of TGn draft D2.0 by moving the description, including the figure, of the HT Information field from the subclause “7.4.9.10 HT Information Exchange frame format” to a new subclause to be named approximately as “7.3.1.34 HT Information field” with the heading numbering to be determine exactly by the editor, and with the editor to provide the appropriate insertion instruction to the WG editor,  to appear at about page 52, just before 7.3.2 Information Elements”, the portion to be moved beginning with “The HT Information field contains...” and ending with the end of the subclause.

TGn Editor: Change the location of the “HT Information Exchange frame” from an action of the category “HT Action” to an action of the category “Spectrum Management” by moving the entire subclause 7.4.9.10 to appear as a new subclause immediately following subclause 7.4.1.6 on about page 78, line 11 of TGn draft D2.0 and add a new entry for the Spectrum Management action to correspond to this new action within table 44 on page 77 of TGn draft D2.0, adjusting the reserved action values as is appropriate, and by deleting the entry for “HT Information Exchange” found in the table n31 on page 79 in subclause 7.4.9.1 and adjusting the reserved values in the table as is appropriate.

TGn Editor: Insert the following text as a new sentence to appear at the end of the subclause “9.13.3.1 General” at about page 127 line 58 in TGn draft D2.0 as follows:

When the Operating Mode field is set to 0 and the Secondary Channel Offset field is set to a non-zero value, then a 40 MHz HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to HT_CBW40) may initiate a TXOP. When the Operating Mode field is set to a non-zero value or the Secondary Channel Offset field is set to zero, then a 40 MHz HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to HT_CBW40) shall not initiate a TXOP.

TGn Editor: Change the location of all of the subclauses of “9.20 20/40 Functional description” to appear as subclauses of “11.15 20/40 MHz operation”, with all of the 9.20 subclauses appearing AFTER any existing and any new subclause of 11.15 that are added by the adoption of this document, renumbering subclauses in Clause 9 and in Clause 11 as appropriate, in TGn draft D2.0.

TGn Editor: Change the name of the subclause “11.15 20/40 MHz operation” to “11.15 20/40 MHz BSS operation”, in TGn draft D2.0.

TGn Editor: Change the text of subclause “9.20.1 Rules for operation in 20/40 MHz BSS” on page 163 at about line 29 of TGn draft D2.0, as follows:

A 20/40 MHz capable HT STA shall set the HT Information Exchange support field to 1 in transmitted Extended Capabilities elements.

An HT-STA-19 that is a member of an IBSS shall not set the Secondary Channel Offset field of the HT Information element to a non-zero value in transmitted frames.

All 20/40 capable STA shall operate as defined in 9.20.2 (STA CCA sensing 20/40 MHz BSS) through 9.20.4 (Switching between 40 MHz and 20 MHz).

Additionally, a 20/40 capable STA operating under PCO operates as defined in 11.16 (Phased Coexistence Operation).

Additionally, a 20/40 capable STA operating under L-SIG TXOP protection operates as defined in 9.13.5 (L-SIG TXOP protection).

TGn Editor: Insert a new subclause and accompanying editor instruction to appear between the subclauses “9.20.1 Rules for operation in 20/40 MHz BSS” and “9.20.2 STA CCA sensing 20/40 MHz BSS” at about page 163 line 40 in TGn draft D2.0, appropriately modifying the clause numbering of the new subclause heading and subsequent subclauses to account for the new subclause:

Insert the following new subclause:

9.20.1a STA transmission width restrictions for 20/40 MHz BSS

When an FC-HT-STA is associated with an FC-HT-AP whose last indicated operating mode is 40 MHz BSS and that declares a Supported Channel Width of 20/40 MHz, the FC-HT-STA may transmit frames in the 20 MHz wide primary channel as well as in the 40 MHz wide channel to receivers that support receptions of those frames. A non-AP STA that is a member of a 40 MHz BSS shall not transmit 20 MHz frames in the secondary channel.

TGn Editor: Change the last sentence of subclause “9.20.1 Rules for operation in 20/40 MHz BSS” at about page 163 line 38 in TGn draft D2.0, which references the subclauses of 9.20 to account for the changes to the subclauses of 9.20, by making the first reference refer to the proposed 9.20.1a and the last reference to remain unchanged.

TGn Editor: Insert the following text and accompanying editor instructions to appear between the subclauses “9.20.3 NAV assertion in 20/40 MHz BSS” and “9.20.4 Switching between 40 MHz and 20 MHz” at about page 164 line 21 in TGn draft D2.0, appropriately modifying the clause numbering of the new subclause headings and subsequent subclauses to account for the new subclauses:

Insert the following new subclause:

9.20.3a Signaling 40 MHz intolerance

An FC-HT-STA-19 shall set the Forty MHz Intolerant bit to 1 in transmitted HT Capabilities elements and HT Information fields in frames that use either a unicast or broadcast/multicast Address1 value if and only if the value of the MIB attribute dot11FortyMHzintolerant is true and independently of the value of the Supported Channel Width set field in the same element.

An FC-HT-STA-17 shall set the Forty MHz Intolerant bit to 0 in transmitted HT Capabilities elements and HT Information fields in frames that use either a unicast or broadcast/multicast Address1 value.

TGn Editor: Change the text of subclause “9.20.4 Switching between 40 MHz and 20 MHz” at about page 164 line 24 in TGn draft D2.0 as follows:

9.20.4 Switching between 40 MHz and 20 MHz

The following events are defined to be BSS width trigger events:

a) on any 40 MHz affected channel, reception of a Beacon frame that

does not contain an HT Capabilities element

b) on any of the channels of the channel set defined in Clause 19, reception of a 20/40 BSS Coexistence Management or Beacon or Probe Request frame that contains a value of 1 in a Forty MHz Intolerant field and that has the Address1 field set to a unicast, multicast or broadcast address value, with no further addressing qualifications

c) reception of an HT Information Exchange frame with the 20 MHz BSS Width Request

field set to 1 from a STA with which the receiver has an active association

An FC-HT-AP-19 that detects any of the BSS width trigger events a), b) or c) during an overlapping BSS scan (see 11.15.1a), or at any other time, shall set the Secondary Channel Offset field to 0 in transmitted HT Information elements beginning at the next DTIM or next TBTT if no DTIMs are transmitted to indicate that no secondary channel is present (i.e., that the BSS operating width is 20 MHz).

An FC-HT-AP-19 shall not set the STA Channel Width field to 1 in transmitted HT Information elements and shall not set the Secondary Channel Offset field to a non-zero value in transmitted HT Information elements unless the following two conditions have been met:

- A period of dot11BSSWideChannelTransitionDelay seconds have elapsed during which there are no BSS width trigger events, a), b) or c); and

- The most recently received 20 MHz BSS Width Request field from each associated FC-HT-STA-19 did not contain a value of 1.

To request an update of the status of the 20 MHz BSS Width Request field, an FC-HT-AP-19  can transmit an HT Information Exchange frame with a value of 1 in the Information Request bit to a STA that supports the HT Information Exchange, as described in 11.17.

An FC-HT–STA-19 that is associated with an FC-HT-AP-19 and that detects either of the BSS width trigger events a) or b) during an overlapping BSS scan, or at any other time, shall

transmit to its associated FC-HT-AP-19 an HT Information Exchange management action frame with the 20 MHz BSS Width Request field set to 1 and a set of one or more AP Channel Report elements if any of the following three conditions is true:

- if the last successfully transmitted indication of the 20 MHz BSS Width Request field to the FC-HT-AP-19 since association to the FC-HT-AP-19 was 0

- there has been no successfully transmitted HT Information Exchange management frame to the FC-HT-AP-19 since association with the FC-HT-AP-19

- the value for the AP Channel Report element(s) of the last successfully transmitted HT Information Exchange frame to the FC-HT-AP-19 since association is different from the new value of the AP Channel Report element(s), where the channels to be included in the AP Channel Report element(s) are those on which a trigger event a) or b) has occurred

An

FC-HT-STA-19 shall not transmit to its associated FC-HT-AP-19 an HT Information Exchange management action frame with the 20 MHz BSS Width Request field set to 0 until a period of dot11BSSWideChannelTransitionDelay seconds have elapsed during which there are no BSS width trigger events a) or b).

Before transmitting a frame containing an HT Information field with the 20 MHz BSS Width Request bit set to 0, an FC-HT–STA-19 that is associated with an FC-HT-AP-19 shall complete an overlapping BSS scan at a time no earlier than one beacon interval preceeding the transmission.

TGn Editor: Change the two paragraphs of subclause “11.9.8.3 Scanning prior to starting a BSS” at about page 199 line 28 in TGn draft D2.0 as follows:

11.9.8.3 Scanning prior to starting a BSS

Before an AP starts a 20/40 MHz BSS it shall scan the environment for existing BSSs. It may use passive scan for at least dot11OBSSPassiveScanTotalPerChannel TU or active scan for at least dot11OBSSActiveScanTotalPerChannel TU on the intended secondary channel. If the scanning AP chooses to start a 20/40 MHz BSS in 5 GHz that occupies the same two channels as an existing 20/40 MHz BSS, then the scanning AP shall ensure that the primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of the existing 20/40 MHz BSS. If the scanning AP chooses to start a 20/40 MHz BSS in 2.4 GHz, then the scanning AP shall ensure that the primary channel of the new BSS is identical to the primary channel of any existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of an existing 20/40 MHz BSS. The AP may also use passive or active scan after the BSS has been started.

An FC-HT-AP-19 may set the Secondary Channel Offset field to a non-zero value in transmitted HT Information elements beginning at BSS startup provided that two sequential overlapping BSS scan operations are performed immediately before BSS startup, and that no BSS width trigger events are detected during those scans.

After BSS startup, the AP shall perform at least one overlapping BSS scan of the candidate primary and secondary channels either itself or through its associated STAs, before making a transition from a 20 MHz BSS to a 20/40 MHz BSS.

Given the following definitions:

- P1 = the primary channel of the intended 20/40 MHz BSS
S1 = the secondary channel of the intended 20/40 MHz BSS

- C = the set of all channels whose center frequency falls within the closed interval [(P1+ S1)/2 – 20, (P1+ S1)/2 + 20] and that are allowable channels within the current operational regulatory domain

- OPi = the ith member of the set of primary operating channels of all 20/40 MHz BSS operating within the AP’s BSA that are members of the channel set C

- OSi = the ith member of the set of secondary operating channels of all 20/40 MHz BSS operating within the AP’s BSA that are members of the channel set C

- OTi = the ith member of the set of operating channels of all 20 MHz BSS operating within the AP’s BSA that are members of the channel set C

An FC-HT-AP-19 may transmit a frame containing an HT Information element with the Secondary Channel Offset field set to a non-zero value only if the following three conditions are true:

1) P1 = OPi for all values of i
2) P1 = OTi for all values of i
3) S1 = OSi for all values of i

If, after initial establishment of the 20/40 MHz BSS, the outcome of any of the equality tests above changes to FALSE, then the FC-HT-AP-19 shall immediately revert to 20 MHz BSS operation or the FC-HT-AP-19 shall immediately move the BSS channel(s) to a channel (or pair of channels) where the above equalities are true.

An FC-HT-AP-17 may start a 20/40 MHz BSS on a channel if a Beacon has been detected on the intended secondary channel during the scanning described in this subclause.

TGn Editor: Insert the following text at the end of the subclause “11.9.8.4 Channel management at the AP” at about page 201 line 15 in TGn draft D2.0 as follows:

An FC-HT-AP-19 shall maintain a record (known as the STA Channel Width record) of the STA Channel Width field value most recently received from each of its associated STA. When a STA associates with an FC-HT-AP-19, the initial value for the STA Channel Width record for that STA shall be set to the value of 1 if an HT Capability element with a value of 1 for the Supported Channel Width set is received as part of the association exchange and shall be set to the value 0, otherwise. An FC-HT-AP that receives any frame containing a STA Channel Width field from an associated STA shall set the value of the STA Channel Width record to the corresponding value from the received STA Channel Width field. An FC-HT-AP-19 shall not transmit a 40 MHz PPDU to a STA if the corresponding STA Channel Width record has the value of 0.

TGn Editor: Delete all of the changes to subclause “11.10 Radio Measurement Procedures” and its subclauses that are found on page 201 beginning at about line 17 of TGn draft D2.0.

TGn Editor: Insert the following text into subclause “11.15.1 Basic functionality in BSS 20/40 MHz mode” of TGn draft D2.0, before the sentence that begins with “A non-AP HT STA declares its chanel width capability” that is found on page 201 at about line 61.

An HT-AP that indicated a value of 0 in its most recently transmitted Secondary Channel Offset field shall not transmit a 40 MHz mask PPDU.

TGn Editor: Insert the following text into subclause “11.15.1 Basic functionality in BSS 20/40 MHz mode” of TGn draft D2.0, before the sentence that begins with “A 20 MHz capable STA shall” that is found on page 202 at about line 11.

An AP may communicate its STA Channel Width either by transmitting an HT Information Element (in beacons and probe response frames) or by transmitting the Notify Channel Width management action frame.

An HT STA whose most recently successfully acknowledged transmission of the STA Channel Width field in any frame was set to 0 shall not transmit a 40 MHz mask PPDU.

A STA that receives from another STA, a Notify Channel Width frame with a value of 0 in the STA Channel Width field should not transmit a 40 MHz mask PPDU to that STA. The reaction of an FC-HT-STA or FC-HT-AP to change the channel width of PPDUs addressed to the sender of the Notify Channel Width frame might not be immediate after receiving the frame. A STA that has transmitted a Notify Channel Width frame knows that the requested channel width has been adopted after receiving an HT frame of the requested channel width from the STA to which it addressed the Notify Channel Width frame.

TGn Editor: Insert the following text and editor instruction as a new subclause to appear before the subclause “11.15.2 Support of DSSS/CCK in 40 MHz” of TGn draft D2.0 on page 202 at about line 47, renumbering the new subclause and the existing subclauses as appropriate to account for the insertion of the new subclause:

Insert the following new subclause:

11.15.1a Scanning and reporting requirements for 40 MHz capable STA

An overlapping BSS scan operation is defined as a passive or active scan of all 40 MHz affected channels when the scanning STA is an FC-HT-STA-19. FC-HT-STA-17 are not required to perform overlapping BSS scan operations.

During an overlapping BSS scan operation, the per-channel scan duration is a minimum of dot11OBSSScanPassiveDwell TU when scanning passively and a minimum of dot11OBSSScanActiveDwell TU when scanning actively. During an overlapping BSS scan operation, each channel in the set is scanned at least two times per dot11BSSWidthTriggerScanInterval seconds and the minimum total scan time per channel per dot11BSSWidthTriggerScanInterval seconds is dot11OBSSScanActiveTotalPerChannel TU for an active scan and dot11OBSSScanPassiveTotalPerChannel TU for a passive scan.

NOTE: The values provided in the previous paragraph indicate the minimum requirements. In some cases, not all minimum values can be achieved simultaneously.

Upon receipt of a frame containing an Overlapping BSS Scan Parameters element from the AP with which an FC-HT-STA-19 has an active association, the receiving FC-HT-STA-19 MLME shall update the values of the MIB variables used during overlapping BSS scanning operations according to the received parameter values.

An FC-HT-AP-19 may transmit frames containing an Overlapping BSS Scan Parameters element to any or all associated STA in order to provide overlapping BSS scan parameter values that are different from the default values.

An FC-HT-STA-19 that is associated with an FC-HT-AP-19 shall perform at least one OBSS scan every dot11BSSWidthTriggerScanInterval seconds, except when the total duration of the transmitted MSDUs and received unicast MSDUs during the past 30 minutes did not exceed 3.0% (i.e.  did not exceed 54 seconds).

TGn Editor: Delete the second paragraph of subclause “11.17 STA-STA HT Information Exchange” at page 206 about line 43 in TGn draft D2.0, the text beginning with the words “An AP that supports the HT Information Exchange”:

TGn Editor: Change the third paragraph of subclause “11.17 STA-STA HT Information Exchange” at page 206 about line 46 in TGn draft D2.0, as follows:

A STA that supports the transmission of and reception of the HT Information Exchange management action

frame may transmit any information that is indicated in the HT Information Exchange management action

frame format to any other STA that also supports the transmission of and reception of the HT Information

Exchange management action frame.

TGn Editor: Change the fourth paragraph of subclause “11.17 STA-STA HT Information Exchange” at page 206 about line 52 in TGn draft D2.0, as follows:

Additionally, a STA that supports the transmission of and reception of the HT Information Exchange management

action frame may transmit any information that is indicated in the HT Information Exchange management

action frame format to an AP that supports the HT Information Exchange management action frame type, but with which it is not associated .

TGn Editor: Change the fifth paragraph of subclause “11.17 STA-STA HT Information Exchange” at page 206 about line 58 in TGn draft D2.0, as follows:

A STA shall not transmit to another STA, an HT Information Exchange management action frame with a unicast address in the address1 field if the most recently received Extended Capabilities element from the recipient STA contained a value of 0 in the HT Information Exchange support field. A STA may transmit to any STA, an HT Information frame with a broadcast/multicast address in the address1 field.

TGn Editor: Insert the following text as the new last paragraph of subclause “11.17 STA-STA HT Information Exchange” at page 207 about line 23 in TGn draft D2.0:

A STA shall not transmit an HT Information Exchange management frame to a STA if the most recently received Extended Capabilities element from that STA contained a value of 0 in the HT Information Exchange support field.

TGn Editor: Change the seventh paragraph of subclause “11.17 STA-STA HT Information Exchange” at page 207 about line 1 in TGn draft D2.0, as follows:

The most recently received information from within an HT Information Exchange management action frame

supersedes any earlier received information, regardless of the frame type and subtype that was used to

communicate that information. For example, information may be provided within HT Information Exchange

management action frames, within Management Action frames, and within other management frames.

TGn Editor: Delete the eighth paragraph of subclause “11.17 STA-STA HT Information Exchange” that begins with the words “The most recently successfully transmitted information” at page 207 about line 7 in TGn draft D2.0.

TGn Editor: Change the text of the ninth paragraph of subclause “11.17 STA-STA HT Information Exchange” at page 207 about line 14 in TGn draft D2.0:

A STA that transmits an HT Information Exchange frame may set the Request Information field to 1, except when this frame is a response to an HT Information Exchange frame in which the Request Information field is set to 1

. A STA that receives an HT Information Exchange management action frame that contains a value of 1 for the Request Information bit shall transmit an HT Information Exchange management action frame that has the value of its RA corresponding to the address of the STA from which it received the HT Information Exchange management action frame that contained a value of 1 for the Request Information bit.

TGn Editor: Insert the new MIB attribute “dot11FortyMHzOptionImplemented” in the appropriate location of annex D of TGn draft D2.0 with the default value for the attribute as “false”, as follows, with the appropriate value of dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11OperationEntry list found at about page 373, line 37.

dot11FortyMHzOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the STA is capable of transmitting and receiving on a 40 MHz channel using a 40 MHz Mask. The default value of this attribute is FALSE.”

::= { dot11OperationEntry xx }

TGn Editor: Insert the new MIB attribute “dot11FortyMHzIntolerant” in the appropriate location of annex D of TGn draft D2.0 at about page 377, line 29 with the default value for the attribute as “false”, as follows, with the appropriate value of dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11OperationEntry list found at about page 373, line 37.

dot11FortyMHzIntolerant OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the STA requests that 40 MHz Mask PPDUs are not transmitted within range of the STA. The default value of this attribute is FALSE.”

::= { dot11OperationEntry xx }

TGn Editor: Insert the new MIB attribute “dot11BSSWidthTriggerScanInterval” in the appropriate location of annex D of TGn draft D2.0 at about page 377, line 29 with the default value for the attribute as “60 seconds”, as follows, with the appropriate value of dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11OperationEntry list found at about page 373, line 37.

dot11BSSWidthTriggerScanInterval OBJECT-TYPE

SYNTAX INTEGER (10…180)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, indicates the maximum interval in seconds between scan operations to be performed to detect BSS width trigger events. The default value of this attribute is 60.”

::= { dot11OperationEntry xx }

TGn Editor: Insert the new MIB attribute “dot11BSSWideChannelTransitionDelay” in the appropriate location of annex D of TGn draft D2.0 at about page 377, line 29 with the default value for the attribute as “300 seconds”, as follows, with the appropriate value of dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11OperationEntry list found at about page 373, line 37.

dot11BSSWideChannelTransitionDelay OBJECT-TYPE

SYNTAX INTEGER (300…9000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, indicates the minimum interval in seconds to transition from BSS operation without a secondary channel present to BSS operation with a secondary channel present. The default value of this attribute is 300.”

::= { dot11OperationEntry xx }

TGn Editor: Insert the new MIB attribute “dot11OBSSScanPassiveDwell” in the appropriate location of annex D of TGn draft D2.0 at about page 377, line 29 with the default value for the attribute as “20 TU”, as follows, with the appropriate value of dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11OperationEntry list found at about page 373, line 37.

dot11OBSSScanPassiveDwell OBJECT-TYPE

SYNTAX INTEGER (20…500)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, indicates the minimum amount of time in TU that the STA continuously scans each channel when performing a passive OBSS scan operation. The default value of this attribute is 20.”

::= { dot11OperationEntry xx }

TGn Editor: Insert the new MIB attribute “dot11OBSSScanActiveDwell” in the appropriate location of annex D of TGn draft D2.0 at about page 377, line 29 with the default value for the attribute as “10 TU”, as follows, with the appropriate value of dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11OperationEntry list found at about page 373, line 37.

dot11OBSSScanActiveDwell OBJECT-TYPE

SYNTAX INTEGER (10…500)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, indicates the minimum amount of time in TU that the STA continuously scans each channel when performing an active OBSS scan operation. The default value of this attribute is 10.”

::= { dot11OperationEntry xx }

TGn Editor: Insert the new MIB attribute “dot11OBSSScanPassiveTotalPerChannel” in the appropriate location of annex D of TGn draft D2.0 at about page 377, line 29 with the default value for the attribute as “200 TU”, as follows, with the appropriate value of dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11OperationEntry list found at about page 373, line 37.

dot11OBSSScanPassiveTotalPerChannel OBJECT-TYPE

SYNTAX INTEGER (200…10000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, indicates the minimum total amount of time in TU that the STA scans each channel when performing a passive OBSS scan operation. The default value of this attribute is 200.”

::= { dot11OperationEntry xx }

TGn Editor: Insert the new MIB attribute “dot11OBSSScanActiveTotalPerChannel” in the appropriate location of annex D of TGn draft D2.0 at about page 377, line 29 with the default value for the attribute as “20 TU”, as follows, with the appropriate value of dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11OperationEntry list found at about page 373, line 37.

dot11OBSSScanActiveTotalPerChannel OBJECT-TYPE

SYNTAX INTEGER (20…1000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, indicates the minimum total amount of time in TU that the STA scans each channel when performing an active OBSS scan operation. The default value of this attribute is 20.”

::= { dot11OperationEntry xx }

References: