IEEE P802.11
Wireless LANs

| Link Metric Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32746 | +1 407 562 4050 | Guenael.strutt@motorola.com |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave, Sunnyvale, CA 94085 | +1-408-331-6834 | audeh@tropos.com |
| Jan Kruys | Cisco Systems | Haarlerbergpark, 1101CH Amsterdam, Netherlands | + 31 20357 2447 | jkruys@cisco.com |
| W Steven Conner | Intel Corporation | JF3-206, 2111 NE 25th Ave, Hillsboro, OR 97124 | +1-503-264-8036 | w.steven.conner@intel.com |

Abstract

Resolution of LB93 comments related to link quality reporting

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of changes:

- Local link state report is generalized to any metric of any length

- “Local Link State Announcement” is renamed “Link Metric Report”

- The link metric report mechanism can be used in a mesh probe request/response or in a standalone message

Rename 11A.3.4 “Frame addressing and forwarding in a mesh network” to 11A.3.5

Add 11A.3.4 with the following

11A.3.4 Link metric reporting

The purpose of the link metric reporting procedure is to determine the link metric associated with a particular link.

If bi-directional link metrics are required in the network, each MP may request a link metric report from a peer MP, or may voluntarily submit a link metric report to a peer MP.  Upon reception of a link metric report, an MP may update its local link metric information using the link metric information received.

To request a link metric report, an MP sends a link metric request to a peer MP.  An MP receiving a link metric request shall reply with a link metric report containing the measured metric for the link to the requesting MP.

To submit a link metric report, an MP sends a link metric report frame to a peer MP.

(7.3.2.56)

(replace 7.3.2.56 with the following)

7.3.2.56 Link metric report element

A link metric report element is transmitted by an MP to a peer MP to indicate the quality of the link between them. This information may be used to ensure that the link metric is symmetric for all mesh links if the path selection protocol so requires.

The contents of the element are shown in Error: Reference source not found.

| Octets: 1 | 1 | variable |  |
| --- | --- | --- | --- |
| ID | Length | metric M |  |
| - Link Metric Report element |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to the length of the metric field, as defined by the active path selection metric.

The metric M is the value of the link metric associated with the link from the peer MP sending the link metric report and the local MP.

The link metric report element may be used in the generation of a link metric such as the airtime metric defined in 11A.5.

(7.4.9.1)

Replace 7.4.9.1 with the following

7.4.9.2 Link Metric Report frame format

If the Link Metric Report is sent without other information elements, the frame format used is the Action frame body format. It is transmitted by an MP to a peer MP in a mesh to advertise metric information. This frame is transmitted in an individually addressed manner. The frame format is shown in Error: Reference source not found.

| Octets: 1 | 1 | Variable |
| --- | --- | --- |
| Category | Action | Link Metric Report Element |

Figure s1: Link Metric Report frame format

The Category field shall be set to 5 (representing mesh management).

The Action field shall be set to 0 (representing Link metric report)

Add 7.4.9.1 with the following

7.4.9.1 Link Metric Request frame format

The Link Metric Request frame format uses the Action frame body format and is transmitted by an MP to a peer MP in a mesh to request metric information. This frame is transmitted in an individually addressed manner. The frame format is shown in Error: Reference source not found.

| Octets: 1 | 1 |
| --- | --- |
| Category | Action |

Figure s2: Link Metric Request frame format

The Category field shall be set to 5 (representing mesh management).

The Action field shall be set to 1 (representing Link metric request)

(11A.1.6)

(delete clause 11A.1.6)

11A.1.6 Link quality measurement

Once a mesh link has been established with a peer MP, prior to using this mesh link as a candidate for establishing a path a measure of the quality of the link shall be established.  This information is required to allow the path selection algorithm/metric to function properly.

Before a link quality measurement has been established, the link is marked as being unavailable.  Path selection frames received from an MP whose link is unavailable shall be discarded.  Once a link quality measurement has been established, the link is marked as being available.  At this point, the MP is able to fully participate in the selection of paths (and by extension, it is able to fully participate in frame forwarding).

Since an MP may use any path selection metric, the MP shall measure a link quality value that is relevant to the path selection metric.  For example, 7.3.2.56 describes a Local Link State Report element comprised of a transmit bit rate and a frame error rate. 11A.3.4 Link  describes procedures for using link quality measurements.

References:

- doc.: IEEE 802.11-07/0023r13 Resolution of comments received during IEEE 802.11 Letter Ballot 93