IEEE P802.11
Wireless LANs

| Airtime metric |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32746 | +1 407 562 4050 | Guenael.strutt@motorola.com |
| Michael Bahr | Siemens AG, CT | Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |
| Jan Kruys | Cisco Systems | Haarlerbergpark, 1101CH Amsterdam, Netherlands | + 31 20357 2447 | jkruys@cisco.com |
| Malik Audeh | Tropos | 555 Del Rey Ave, Sunnyvale, CA 94085 | +1-408-331-6834 | audeh@tropos.com |

Abstract

Resolution of LB93 comments related to the airtime metric (section 11A.5 in Draft D1.03)

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of changes:

- Airtime metric is simplified and does not refer to PHY-specific values

Note: All instructions to the editor are based on IEEE 802.11s Draft D1.03.

Replace 11A.5 page 139-141 with the following (Remove 11A.5.1 and 11A.5.2.)

  - Airtime Link Metric

In order to compute the forwarding table for individually addressed frames, the MP shall first calculate the link metric for each pairwise link to its peer MPs in the Mesh.  This subclause defines a default link metric that may be used by a path selection protocol to identify an efficient radio-aware path. The extensibility framework allows this metric to be overridden by any path selection metric as specified in the active profile.

The default link metric is the airtime. Airtime reflects the amount of channel resources consumed by transmitting the frame over a particular link. This measure is approximate and designed for ease of implementation and interoperability.

The airtime for each link is calculated as:

ca=O+Btr11−ef

where O and Bt are constants listed in Error: Reference source not found, and the input parameters r and ef are the data rate in Mbit/s and the frame error rate for the test frame size Bt respectively. The rate r represents the data rate at which the MP would transmit a frame of standard size Bt based on current conditions and its estimation is dependent on local implementation of rate adaptation. The frame error rate ef is the probability that when a frame of standard size Bt is transmitted at the current transmission data rate r, the frame is corrupted due to transmission error,; its estimation is a local implementation choice. Frame drops due to exceeding TTL should not be included in this estimate as they are not correlated with link performance.

Table s21: Airtime Cost Constants

| Parameter | Recommended value | Description |
| --- | --- | --- |
| O | varies depending on PHY | Channel access overhead, which includes frame headers, training sequences, access protocol frames etc. |
| Bt | 8192 | Number of bits in test frame |

The airtime link metric shall be measured in increments of 10.24 microseconds, or one hundredth of a TU.

Table sXX: Parameters of Airtime Link Metric for Extensible Path Selection Framework

| Path Selection Metric ID | See table s4 in 7.3.2.53.2 |
| --- | --- |
| Data type | Unsigned Integer, 0  metric value  4,294,967,296 |
| Length of metric field | 4 octets |
| Operator for metric aggregation | addition ( + ) |
| Comparison operator | less than, equal to, greater than as used with integers metric a is better than metric b iff a < b metric a is equal to metric b iff a = b metric a is worse than metric b iff a > b |
| Initial value of path metric | 0 |

References:

- doc.: IEEE 802.11-07/0023r27 Resolution of comments received during IEEE 802.11 Letter Ballot 93