IEEE P802.11
Wireless LANs

| LB97 PHY Comment Resolutions: CSD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom Corp. | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission suggests resolutions of LB97 PHY comments related to the sub-topic CSD. The following CIDs are addressed: 175, 336, 413, 2668, 2690, 2695, 2690, 3202, and 3394.

The changes marked in this document are based on TGn Draft 2.0.

CID 1909 was re-assigned to Assaf Kasher.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CID        Sec.             Pg.         Ln.             Comment                         Proposal                       Proposed Resolution

| 175 | 20.3.10.10.1 | 277 | 39-44 | For robust channel estimation with the smoothing the CSD should be shorter than the cyclic prefix. Therefore change: "If 95 percent of the sum of the energy" … "induced by the CSD" … "is contained within 800 ns, the smoothing bit should be set to 1" | To: "If 90 percent of the sum of the energy" … "induced by the CSD" … "is contained within 80 precent of the cyclic prefix duration, the smoothing bit should be set to 1" | Reject. Reason for rejection: The paragraph “If 90 percent ..” defines setting of the smoothing bit. The setting of the smoothing bit should be independent of the cyclic prefix length. Requirement is that CSD does not create too much frequency selectivity so that the smoothing is efective which is the case for the CSDs defined in Table n62. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Reject,

| 336 | 20.3.9.3.2 | 245 | 27-34 | Shorter cyclic shifts for the non-HT portion would aid legacy devices when receiving these transmission. | Determine and use shorter cyclic shifts for the non-HT portion. | Reject. Reason for rejection: the cyclic shifts in Table n61 are already short. Shorter cyclic shifts increase probability of unintentional beamforming. |
| --- | --- | --- | --- | --- | --- | --- |

| 413 | 20.3.9.3.2 | 245 | 27-34 | Shorter cyclic shifts for the non-HT portion would aid legacy devices when receiving these transmission. | Determine and use short cyclic shifts for the non-HT portion. | Duplicate of 336. Reject. |
| --- | --- | --- | --- | --- | --- | --- |

| 2668 | 20.3.3 | 232 | 60 | "When beamforming is not used it is sometimes possible to implement the cyclic shifts in the time domain." Is this true of all 3 types? | If so, move quoted text into the main point j). | Couter. Delete the sentence. See also resolution to CIDs 624 and 625 where the same sentence was modified and moved in front of all 3 types. The new sentence is: “CSD insertion may occur before or after the IDFT.” |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter.

TGn Editor: on page 226, line 61-62, delete the following sentence:

“A cyclic shift MCSD(k) may be applied as a part of the spatial mapper, see 20.3.10.10.1 (Spatial mapping). When beamforming is not used it is sometimes possible to implement the cyclic shifts in the time domain.”

| 2690 | 20.3.9.3.2 | 245 | 35 | "NOTE: With more than four TX chains, each cyclic shift on the additional TX chains shall not be less than -200 ns nor greater than 0 ns." A note cannot contain normative language. | Remove "NOTE:" and possibly move the statement into the enclosing subclause text. | Accept |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept

TGn Editor: on page 245, line 35-37, remove the following sentence in the table n61:

“NOTE: With more than four TX chains, each cyclic shift on the additional TX chains shall not be less than 200 ns nor greater than 0 ns.”

TGn Editor: on page 245, line 39, add the following paragraph:

“With more than four TX chains, each cyclic shift on the additional TX chains shall not be less than -200 ns nor greater than 0 ns.”

| 2695 | 20.3.9.4.2 | 249 | 31 | "NOTE—These cyclic shift values do not apply to the HT-SIG in an HT mixed format PPDU." There appears to be a contradiction going on with the title. The note has no normative effect. | Change the title to "Cyclic shift definition for the HT-LTF of the HT mixed format preamble" and change wording appropriately. Find a home for the NOTE in 20.3.9.4.1 | Counter. Change title to: ".. preamble excluding HT-SIG". It is appropriate to keep the note in Sec. 20.3.9.4.2. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter

TGn Editor: on page 249, line 32, modify the title as follows:

“20.3.9.4.2 Cyclic shift definition for the HT portion of the HT mixed format preamble excluding the HT SIGNAL field”

TGn Editor: on page 244, line 58, modify the title as follows:

“20.3.9.3.2 Cyclic shift definition for the non-HT fields and the HT SIGNAL field”

| 2990 | 20.3.10.10.1 | 277 | 39-44 | For robust channel estimation with the smoothing the CSD should be shorter than the cyclic prefix. Therefore change: "If 95 percent of the sum of the energy" … "induced by the CSD" … "is contained within 800 ns, the smoothing bit should be set to 1" | To: "If 90 percent of the sum of the energy" … "induced by the CSD" … "is contained within 80 precent of the cyclic prefix duration, the smoothing bit should be set to 1" | Duplicate of CID 175. Reject. |
| --- | --- | --- | --- | --- | --- | --- |

| 3202 | 20.3.9.3.2 | 245 | 41-44 | When an HT device transmits a Clause 17 or Clause 19 packet (rates 3, 4.5, 6, 9, 12, 24 ,27, 36, 48, 54Mb/s) using more than one transmit chain, it shall apply the cyclic shifts defined in Table n61 (Cyclic shift for non-HT portion of the packet) to the transmission in each chain. | Too short CSDs create large gain control inaccuracies. Make CSD range up to -400 ns instead of -200 ns. | Withdrawn by commenter (Vinko Erceg). |
| --- | --- | --- | --- | --- | --- | --- |

| 3394 |  | 275 |  | When NTX>NSTS, cyclic shift between the transmission from multiple antennaes is not mandated for HT transmissions. | Mandate per transmit chain cyclic shift for all HT transmissions (for HT-DATA in MM, for the whole packet in GF). | Reject. Reason for rejection: cyclic shift as a part of spatial mapping can be used in this case (M_CSD). |
| --- | --- | --- | --- | --- | --- | --- |