IEEE P802.11
Wireless LANs

| TGn LB97 Submission for Category "SM Power Save" in Frame Format ad-hoc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tomoko Adachi | Toshiba Corporation | 1, Komukai Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582 Japan |  | tomo.adachi@toshiba.co.jp |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comments:

- 11 and 1499

The changes marked in this document are based on TGn Draft version 2.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

TGn Editor: Please delete all those indicating the related CID as "(Ed: CID XXX)" after appropriate time.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# 11 (C), 1499 (A)

# Proposed Resolution

| CID | Comment | Proposed change | Resolution |
| --- | --- | --- | --- |
| 11 | SM Power Save mode is described here in the HT Capabilities Info field and also in the SM Power Control field, 7.3.1.25 but the bit allocation is different. By applying the same allocation used in the SM Power Control field to the HT Capabilities Info field, the expression will be unified. | Change the allocation as follows and to be consistent with those in the SM Power Control field of the SM Power Save Management Action Frame: 00 SM Power Save Disabled (no restrictions on what may be sent to the STA) 01 Reserved 10 Static SM Power Save mode 11 Dynamic SM Power Save mode | Counter. See doc. 11-07/XXXX |
| 1499 | It is not clear what is the meaning of value=3, even if I read 11.2.3. I guess it would be corresponding to the description of the paragraph starting line-1 of page-195. But I don't know what is the "SM enable" and I don't know this is for dynamic SM PS mode or static SM PS mode. | Please specify more clearly. | Accept. |

## Current status and confirmation of the encoding

The SM Power Save subfield in HT Capabilities Info field is defined in clause 7.3.2.49.2 as follows:

0: Static SM Power Save mode

1: Dynamic SM Power Save mode

2: Reserved

3: SM enabled

The SM Power Control field is used in SM Power Save frame. The usage of SM Power Save Enabled bit and SM Mode bit in the SM Power Control field is described in clause 7.3.1.25 and the summary of those combinations is as follows:

| SM Power Save Enabled (B0) | SM Mode (B1) | Encoding |
| --- | --- | --- |
| 0 | 0 | SM Power Save disabled |
| 0 | 1 | SM Power Save disabled |
| 1 | 0 | Static SM Power Save mode |
| 1 | 1 | Dynamic SM Power Save mode |

"SM enabled" by the SM Power Save should mean that RX chains are always enabled, which is equivalent to the SM Power Save Enabled bit set to 0 in SM Power Control field.

When the SM Power Save Enabled set to 0, it doesn't make sense indicating what the mode of operation is. Thus, the combination of SM Power Save Enabled bit set to 0 and the SM Mode bit set to 1 should be equivalent to the reserved case of the SM Power Save subfield in the HT Capabilities Info field.

TGn Editor: Change row 4, column 3 in table n25 as follows:

| Subfield | Definition | Encoding |
| --- | --- | --- |
| LDPC coding capability | Indicates support for receiving LDPC coded packets | Set to 0 if not supported Set to 1 if supported |
| Supported channel width set | Indicates which channel widths the STA supports. See 11.15 (20/40 MHz Operation). | Set to 0 if only 20 MHz operation is supported Set to 1 if both 20 MHz and 40 MHz operation is supported |
| SM Power Save | Indicates the Spatial Multiplexing (SM) Power Save mode. See 11.2.3 (SM Power Save). | Set to 0 for SM Power Save disabled Set to 1 for Static SM Power Save mode Set to 3 for Dynamic SM Poiwer Save mode The value 2 is reserved (Ed: CID 11) (Ed: CID 1499) |

TGn Editor: Change the last paragraph in clause 7.3.1.25 as follows:

The SM Mode field indicates the mode of operation when the SM Power Save Enabled field is set to 1. A value of 1 indicates Dynamic SM Power Save mode, a value of 0 indicates Static SM Power Save mode. A value of 1 with the SM Power Save Enabled field set to 0 is reserved. The modes are described in 11.2.3 (SM Power Save).

(Ed: CID 11)

## References:

[1] Draft-P802.11n_D2.0.pdf