IEEE P802.11
Wireless LANs

| Proposal for comment resolution for Explicit Quant of Beamforming in LB97 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joonsuk Kim | Broadcom | 190 Mathilda Place, Sunnyvale, CA94086 | 408-543-3455 | joonsuk@broadcom.com |

Abstract

This document proposes resolutions to LB97 comments assigned to Beamforming adhoc.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# CID 220

| 220 | 69.47 | 47 | 7.3.2.49.6 |  |  | "Definition" part of "Explicit Compressed Beamforming Feedback Matrix" is vague. Is the intention to state if the STA is capable of returning compressed beamforming matrix? | Clarify definition |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Counter.

Change “Definition” column for Explicit Compressed Beamforming Feedback Matrix in line 48, page 69 to “Indicates whether or not this receiver can return compressed beamforming feedback matrix explicit feedback”

# CID 343,1606,1608,1797,1955, 2731

| 343 | 285.00 |  | 20.3.11.2.1 |  |  | The quantization scheme presented here does not cover the full range of two's complement encoding with N_b bits. Specifically, the smallest value with this scheme is -(2^{N_b-1}-1), but the smallest value in a N_b-bit two'complement is -2^{N_b-1}. As a result we would lose precision in the negative values. | Redine Eq. 20-77 and 20-78 to correct this problem. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1606 | 284.44 | 44 | 20.3.11.2.1 |  |  | Here is a description of "The CSI matrix, H_eff, shall be determined from the transmitter spatial mapper input to the receiver FFT output (the beamformee removes the CSD in table n67 from the measured channel matrix)." Yes, we all know that the CSD shall be removed by beamformee. but how about P_HTLTF specified in equation (20-27) ? I expect it shall be removed by the beamformee also, and it is worth to explicitly state this in the spec. | Add "and P_HTLTF defined at (20-27)", so this sentence would be; "The CSI matrix, H_eff, shall be determined from the transmitter spatial mapper input to the receiver FFT output (the beamformee removes the CSD in table n67 and P_HTLTF defined at (20-27) from the measured channel matrix)." |
| 1608 | 285.45 | 45 | 20.3.11.2.1 |  |  | In these equation, maximum functions for -N_SR<k<N_SR are used. When the grouping is not used, i.e., Ng has non-zero value, should we take into account the amplitude of unused subcarriers for these equation ? | Please clarify whether all subcarriers shall be used for calculating these maximum functions or only selected subcarriers for grouping shall be used. |
| 1797 | 285.44 | 44 | 20.3.11.2.1 |  |  | Eq. 20-76 doesn't make any sense to me. Line 26 says that Eq. 20-76 is the linear counterpart of 20-75. However, the linear counterpart of 20-75 is just the denominator of 20-76 | please clarify |
| 1955 | 284.00 |  | 20.3.11.2.1 |  |  | This clause already gives the rule how to decode received CSI matrices. The encoding (quanization) scheme is up to the implementation of vendors. The example of encoding processing is unncessarily complicated. The long and confusing encoding example does not serve the purpose of clarification. In fact, the old example provided in draft 1.0 is far better than the new one in draft 2.0. Comment resolution is supposed to bring clarification, not confusion. | Remove the example of the encoding process and replace it by a brief explanation of the meaning of the carrier matrix amplitude field (Mh(k)). |
| 2731 | 286.17 | 17 | 20.3.11.2.2 |  |  | "The beamformee shall encode the matrices so a beamformer applying the procedure below will optimally reconstruct the matrix." This is a topsy turvy way of specifying anything. I don't see any need for this obfuscation. | Please indicate how to encode the values, not how to decode them. |

Suggestion: Reassigned to Assaf

CID 1800

| 1800 | 286.23 | 23 | 20.3.11.2.2 |  |  | what are the indices m,l for the matrix V? | please clarify |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Counter

Insert following sentence at the end of the first sentence in in line 57, page 284, “, where m is from 1 to Nr and l is from 1 to Nc.”

Insert following sentence at the end of the sentence in in line 25, page 286, “, where m is from 1 to Nr and l is from 1 to Nc.”

CID 2734

| 2734 | 288.45 | 45 | 20.3.11.2.3 |  |  | "Even if N_ESS=0 in a sounding packet, the Not Sounding field shall be set to zero." This is kind of useless, because a sounding packet is defined as one in which the not sounding field is set to 0. | Remove the quoted sentence. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept (page 289)

CID 3206

| 3206 | 286.13 | 13 | 20.3.11.2.2 |  |  | The sentence is duplicate and does not provide any information in the subclause. | replace "These matrices are sent to the transmitter" with "These matrices V(k) are sent to the beamformer in the order of real and imaginary components per tone as specified in 7.3.1.31" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Counter

Replace “These matrices are sent to the transmitter. The transmitter can use ….” with "The beamforming matrices V(k) found by the beamformee are sent to the beamformer in the order of real and imaginary components per tone as specified in 7.3.1.31. The beamformer can use …"

Replace “receiving STA” with “beamformee” in line 10, page 286

Replace “transmitter” with “beamformer” in line 13, page 286

Replace “transmitting STA” and “receiving STA” with “beamformer” and “beamformee”, respectively, from line 40 to line 45 in page 284, under 20.3.11.2.1

CID 3209

| 3209 | 286.47 | 47 | 20.3.11.2.3 |  |  | What are "these matrices"? | change "These matrices are compressed in the form of angles…." to "These matrices V(k) are compressed in the form of angles…." |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Counter

Replace “These matrices are compressed in the form of angles, which are sent to the transmitter. The transmitter can use these angles ….” with "The beamforming matrices V(k) found by the beamformee are compressed in the form of angles which are sent to the beamformer. The beamformer can use these angles …”

Replace “receiving STA” with “beamformee” in line 45, page 286

Replace “transmitter” with “beamformer” in line 47, page 286

CID 3210

| 3210 | 286.53 | 53 | 20.3.11.2.3 |  |  | The sentence is not clear | Replace "The matrix compression is defined as follows" with "The matrix V per tone shall be compressed as follows." |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

CID 3211

| 3211 | 287.62 | 62 | 20.3.11.2.3 |  |  | The sentence "Beamforming matrix is column-wise phase invariant" is true. No need to say with "may be". | Change "This means V may be equivalent to…" to "This means V is equivalent to …" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept