IEEE P802.11
Wireless LANs

| Draft Text Changes for ‘Express Forwarding’ in a Mesh |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mathilde Benveniste | Avaya Labs-Research | 233 Mt Airy Road Basking Ridge, NJ 07920, USA | 973-761-6105 | benveniste@ieee.org |

Abstract

Changes to TGs Draft 1.03 are presented in this document to enable inclusion of the optional feature ‘express forwarding’ for multi-hop transmissions.  A description of ‘express forwarding’ appears in Doc. IEEE 802.11-07-0415r2.

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Inserted text appears in red underlined characters; deleted text appears in blue characters with a strike through line.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Modify subclause 7.1.3.5a.2 as follows:

- Mesh Flags field

The Mesh Flags field, shown in s5, is 8 bits in length and the flags therein are used to control mesh-specific header processing, e.g., for mesh address extension.

| B0 | B1 7 | B2 7 |
| --- | --- | --- |
| Address Extension (AE) | Time-sensitive QoS (TSQ) | Reserved |
| Bits: 1 | 1 | 7 6 |

The “Address Extension (AE)” flag is used to indicate the existence of — and thereby the use of — the mesh address extension field for mesh address extension based on 6-address format: If AE = 1, the mesh address extension field follows the Mesh Sequence Number field and provides two additional address fields for mesh address extension (see 11A.3.4 for the usage of address fields in 6-address format); if AE is 0, the mesh address extension field is not present. The “AE” flag can be set to 1 only when both “To DS” and “From DS” fields are set to 1.

The “Time-Sensitive QoS (TSQ)” flag is used to indicate that the Duration field value has been extended for Express Forwarding purposes.  If TSQ = 1 for a transmitted frame, an MP that supports Express Forwarding shall process the Duration field value of the frame when updating its NAV according to the rules given in subclause 9.22.  If TSQ = 0 for a transmitted frame, an MP shall process the Duration field value of the frame to update its NAV setting according to the rules of the coordination function.  An MP that does not support Express Forwarding shall always process the Duration field value of the frame to update its NAV setting according to the rules of the coordination function, regardless of the TSQ flag setting.  Only frames of type Extended with subtypes Mesh Data or Mesh Management and user priority 6 or greater may have the TSQ flag set.

The reserved field is set to zero.

Inset text below after subclause 9.21

9.22   Express Forwarding

Express Forwarding (EF) is an optional access method that allows MPs that support EF to access the channel with lower contention than would otherwise be possible when forwarding selected frames that traverse multi-hop paths.  A frame of type Extended with subtypes Mesh Data or Mesh Management may be express-forwarded if its TSQ flag in the Mesh_Flags field of the Mesh Header field is set.  Such a frame is referred to as a ‘TSQ frame’. Express Forwarding is an optional feature.  If an MP that does not support EF receives a TSQ frame for forwarding, it shall forward such frame using conventional access mechanisms.  An MP that supports EF shall extend the Duration field value of a TSQ frame to enable the next MP forwarding the frame to access the channel faster.

All MPs not supporting EF inherently obey the NAV rules of the coordination function.  They shall thus process the Duration field value to update their NAV settings.  If an MP that supports EF must forward a received TSQ frame, it shall process the Duration value of a TSQ frame to enable faster channel access.  Only Time-Critical frames may be transmitted sooner than TSQ frames.  Time-Critical frames are frames of type Extended with subtype Mesh Management with user priority of 6 or greater and …??.   If an MP that supports EF has a Time-Critical frame ready to transmit (i.e. the backoff timer for the corresponding EDCAF is expired), it shall process the Duration field value of a transmitted TSQ frame when updating its NAV to enable it to transmit the Time-Critical frame immediately following the completion of a TSQ frame transmission.  If the frame pending transmission at an MP is not Time Critical, the MP shall process the Duration field value of a transmitted frame to update its NAV setting according to the rules of the coordination function.

    - Behavior of an MP transmitting a TSQ frame

An MP that supports EF shall extend the Duration field value by the value of Dot11MeshDTC when transmitting a TSQ frame.

9.22.2   Behavior of an MP receiving a TSQ frame

When an MP that supports EF receives a TSQ frame, it shall process the Duration field value of the received frame when updating its NAV as follows:

- the MP that is the intended receiver of the transmitted  TSQ frame shall update its NAV setting using the time interval (Duration field value – value of Dot11MeshDEF) if it must forward the received frame;

- an MP that has a time-critical frame ready to transmit (that is, with expired backoff timer) shall update its NAV setting using the time nterval (Duration field value – value of Dot11MeshDTC);

- all MPs other than the MPs in (a) or (b) shall process the Duration field value to update their NAV settings according to the rules of the coordination function.

References:

- Draft_P802.11s_D1.03.pdf , April 8, 2007

- M. Benveniste,  “‘Express’ Forwarding in a Multi-hop Wireless Network,”

- Doc. IEEE 802.11-07-0415r2, May 2007