IEEE P802.11
Wireless LANs

| LB97 PHY Comment Resolutions: EVM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission suggests resolutions of LB97 PHY comments related to the sub-topic EVM. The following CIDs are addressed: 183, 1612, 1613, 1614, 1918, 2740, 2741, 2742, 2998, and 3395.

The changes marked in this document are based on TGn Draft 2.0

Revision History:

Rev 00: Initial document.

Rev 01: Incorporated comments by Tomoya Yamaura regarding CIDs 1612, 1613 and 1614.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CID        Sec.            Pg.     Ln.             Comment                     Proposal                          Proposed Resolution

| 183 | 20.3.20.7.4 | 296 |  | The transmit EVM does not measure the isolation between the transmit chains. Isolation is a critical parameter that can cause a key hole effect thereby reducing the effective channel rank and affect performance. | Use the same EVM procedure upto step c) then compute the cross correlation between the signals choosing two of them at a time and divide by the product of the square root of the energies in the two chosen signals. Take the absolute value of this number and it shall be less than -25dB without antennas. | Reject. Reason for rejection: EVM test will capture this sufficiently. Poor isolation would increase EVM and device would not pass existing EVM requirements. |
| --- | --- | --- | --- | --- | --- | --- |

| 1612 | 20.3.20.7.3 | 296 | 44 | There is a description of "In this table, the number of spatial streams is equal to the number of transmit antennas." But this is not necessary. For example, some devices may have 4 transmit chains (antennas) but support only 3 spatial streams. Even if we remove this sentence, there is no problem because equation (20-89) shown EVM value shall be measured using average of spatial streams. | Remove "In this table, the number of spatial streams is equal to the number of transmit antennas." | Counter. Modify the sentence and include “utilized” transmit/receive ports to address the case in the comment with 4 Tx antennas and 3 spatial streams. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter.

TGn Editor: on page 296, line 44, modify the text as follows:

“In this table, the number of spatial streams is equal to the number of transmit antennas. The number of spatial streams under test shall be equal to the number of utilized transmitting STA antenna (output) ports and also equal to the number of utilized testing instrumentation input ports. In the test NSS = NSTS with equal modulation MCSs shall be used. Each output port of the

transmitting STA shall be connected through a cable to one input port of the testing instrumentation.”

| 1613 | 20.3.20.7.4 | 297 | 44 | According to the equation (20-89), we cannot measure EVM for unequal modulation (MCS>32). I believe that MCSs using unequal modulation are not necessary to measure EVM, because it could be as good as MCSs using equal modulation. | Add statement that this EVM test shall be done only for MCSs using equal modulation. | Accept in principle. See CID 1612 proposed resolution. |
| --- | --- | --- | --- | --- | --- | --- |

| 1614 | 20.3.20.7.4 | 297 | 44 | In the equation (20-89), N_SS is used. However, if STBC is used, how to measure this ? | Replace "N_SS" with "N_STS" in this equation. | Reject. Reason for the rejection: the test is defined for Nss = Nsts to simplify the testing procedure. For example, in the case of Ntx = 4 and Nss = 3, receiver may have Nrx = 3, in which case special channel coupling matrix has to be defined for the EVM test, wich complicates the testing procedure and spec. This is clarified in the CID 1612 proposed resolution. |
| --- | --- | --- | --- | --- | --- | --- |

| 1918 | 20.3.20.7.4 | 296 |  | The transmit EVM does not measure the isolation between the transmit chains. Isolation is a critical parameter that can cause a key hole effect thereby reducing the effective channel rank and affect system performance. | Use the same EVM procedure up to step c), then compute the cross correlation between the signals, choosing two of them at a time, and divide by the product of the square root of the energies in the two chosen signals. Take the absolute value of this number and it shall be less than -25dB without antennas. | Duplicate of CID 183. Reject. |
| --- | --- | --- | --- | --- | --- | --- |

| 2740 | 20.3.20.7.3 | 296 | 41 | "The relative constellation RMS error, averaged over subcarriers, OFDM frames, and spatial streams shall not exceed a data-rate dependent value according to Table n74 (Allowed relative constellation error versus constellation size and code rate)." How is the averaging done? | Indicate how the constellation error is averaged. Or reference 20.3.20.7.4 if that is where the averaging is defined. | Reject. Reason for rejection: Eq. 20-89 shows averaging mathematically. |
| --- | --- | --- | --- | --- | --- | --- |

| 2741 | 20.3.20.7.3 | 296 | 44 | "In this table, the number of spatial streams is equal to the number of transmit antennas." But the table doesn't mention spatial streams. | Please clarify. or remove "In this table, ". | Counter. See CID 1612. Sentence deleted. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter.

| 2742 | 20.3.20.7.4 | 296 | 52 | "The transmit modulation accuracy test shall be performed by instrumentation capable of converting the transmittedsignals into a streams of complex samples at 40 Msample/s or more, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, analog to digital quantization noise, etc." You can't say "shall ... etc". It's an open-ended normative requirement. | List everything that's relevent to the test and remove the etc. | Counter. Remove "etc" in the sentence. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter

TGn Editor: on page 296, line 52, modify the following text:.

“The transmit modulation accuracy test shall be performed by instrumentation capable of converting the transmitted signals into a streams of complex samples at 40 Msample/s or more, with sufficient accuracy in term of I/Q arm amplitude and phase balance, dc offsets, phase noise, and analog to digital quantization noise, etc.”

| 2998 | 20.3.20.7.4 | 296 |  | The transmit EVM does not measure the isolation between the transmit chains. Isolation is a critical parameter that can cause a key hole effect thereby reducing the effective channel rank and affect performance. | Use the same EVM procedure upto step c) then compute the cross correlation between the signals choosing two of them at a time and divide by the product of the square root of the energies in the two chosen signals. Take the absolute value of this number and it shall be less than -25dB without antennas. | Duplicate of CID 183. Reject. |
| --- | --- | --- | --- | --- | --- | --- |

| 3395 |  | 297 | 45 | In eq. 20-89, EVM is measured for all the spatial streams instead of space time streams. | Modify the equation to measure over all space time streams. Change i_{SS} = 1 to N{SS} changed to i_{STS} = 1 to N_{STS}. Also require modification the defintion of the variables i_{SS} in the following sentences | Reject. Reason for the rejection: the test is defined for Nss = Nsts to simplify the testing procedure. For example, in the case of Ntx = 4 and Nss = 3, receiver may have Nrx = 3, in which case special channel coupling matrix has to be defined for the EVM test, wich complicates the testing procedure and spec. This is clarified in the CID 1612 proposed resolution. |
| --- | --- | --- | --- | --- | --- | --- |