IEEE P802.11
Wireless LANs

| LB97 submission to resolve PHY Vectors comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa Israel 31015 | +972-4-8651547 | Asssaf.kasher@intel.com |

Abstract

This document provides suggested resolutions for comments with the following CIDs: 2901, 2902, 2630, 2907, 270, 2647, 2656, 2657, 2681, 2926, 3170.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| 2901 | The purpose of "Table n55—PPDU format as a function of CH_BANDWIDTH and CH_OFFSET parameters" is not completely clear - it does not include the already defined HT-PPDU and non-HT PPDU. | make this table consistent with the defined terminology |
| --- | --- | --- |

Suggestion: reject

Reason: throughout the PHY section format is used both for the time domain structure of the PPDU and for the frequency domain structure of the PPDU.

| 2902 | Current definition of the TXVECTOR does not allow specifying transmission of 20MHz mask over CH_OFF_20U and CH_OFF_20L | Change the definition to allow specifying the 20MHz Tx mask for any of two adjacent 20 MHz channels in the 40MHz channel. |
| --- | --- | --- |

Suggestion: reject

Reason: Transmissions conforming to the mask can be specified through the interface as usuall 20MHz transmission with a different central frequency

| 2630 | Seeing as we've defined a brand new shiny interface to allow configuration of the PHY. I wonder if we should also tell it about some other relevant things like its operating channel and operating channel width. | Add any other parameters to this table that are necessary to control the PHY receive operation. This should include channel and channel width. |
| --- | --- | --- |

Suggestion: Counter

See Below

| 2907 | Table n55 and n56 are not consistent. Table n56 declares for CH_BANDWIDTH with FORMAT is NON_HT: Enumerated type: NON_HT_CBW40 for non-HT duplicate, NON_HT_CBW20 for all other non-HT, whereas Table n55 lists the options CH_OFF_20U and CH_OFF_20L under NON_HT_CBW40. | In Table n55, make the NON_HT table entries consistent with the HT table entries, by moving the CH_OFF_20L entry and CH_OFF_20U entry of NON_HT_CBW40 to NON_HT_CBW20. |
| --- | --- | --- |

Suggestion: Accept

TGn Editor: In table n55 (n55—PPDU format as a function of CH_BANDWIDTH and CH_OFFSET parameters) in page 215 of D2.0 move the CH_OFF_20L entry and CH_OFF_20U entry of the NON_HT_CBW40 line to the NON_HT_CBW20 line

| 270 | Is Table n57 supposed to contain a complete list of HT PHY parameters? If so, a few are missing. | Add ANTENNA_SET_ON, N_TX, EXPANSION_MAT_TYPE, CHAN_MAT_TYPE, RCPI |
| --- | --- | --- |

Suggestion: counter

TGn Editor: add the following lines to table n57 (lines 28-31 of page 231 draft D2.0)

| HT PHY Parameter | 2.4 GHz Operation defined by Clause 19 | 5 GHz Operation defined by clasue 17 |
| --- | --- | --- |
| N_TX | - | - |
| EXPANSION_MAT_TYPE | - | - |
| CHAN_MAT_TYPE | - | - |
| RCPI | - | - |

| 2647 | "see note 1" The note says that these values do not apply for HT devices. But this is an interface for an HT device. | If an HT device does not support these channel widths, remove 10MHz and 5MHz rates (and the note). If it does support them, remove the note. |
| --- | --- | --- |

Suggestion: defer

TGn Editor: change note 1 in page 230 line 43 as follows:

Note 1 -- Channel spacings of 5 or 10MHz indicated for L_DATARATE do not apply to an HT deviceSTA using NON-HT-DUP, UPPER-20-OFDM, LOWER-20-OFDM modulations”

| 2656 | "NO_EXPANSION_MAT indicates that EXPANSION_MAT is not valid" This is unnecessary. Parameters can be present or absent. | Remove this entry. Change the condition for this row to: "EXPANSION_MAT is present" Line 45 of page 227, replace the complex condition with "Otherwise". |
| --- | --- | --- |

Suggestion: Accept

| 2657 | CHAN_MAT is missing its "otherwise" case | Add a row: condition=otherwise, Value=not present. |
| --- | --- | --- |

Suggestion: Accept

| 2681 | "Value in non-HT 20 MHz channel" and the other headings. Bearing in mind the confusion about what is and what is not a 40 MHz channel, these headings should relate to explicit combinations of TXVECTOR parameters. | Reword headings to relate to either combinations of the TXVECTOR parameters, or the format names introduced in table n55. |
| --- | --- | --- |

Suggestion: defer

TGn Editor: change the header of  Table n58—Timing related constants in page 238 lines 35-49 as follows:

| paramter | Value in non-HT 20 MHz channel Value in non-HT frame | Value in 20 MHz HT channel Value in HT-frame with TXVECTOR primitive CH_BANDWIDTH set to HT_CBW_20 | Value in 40 MHz channel Value in a frame with TXVECTOR primitive CH_BANDWIDTH set to HT_CBW40 or NON_HT_CBW40 |
| --- | --- | --- | --- |

| 2926 | ERP-OFDM preamble missing in "condition" column | Add ERP-OFDM in condition column along with its type in value column. |
| --- | --- | --- |
| 3170 | ERP-OFDM preamble missing in "condition" column | Add ERP-OFDM in condition column along with its type in value column. |

Suggestion: Reject

Reason: ERP-OFDM has only one preamble type (clause 17 preamble) and therefore does not need to have the  preamble_type primitive

The following Text adds operating channel and channel offset to the PHY-SAP interface to enable setting static parameters of the PHY and not rely on the MIB.

TGn Editor: in clause 12 replace all the occurrences of PHY-RXCONFIG with PHY-CONFIG

TGn Editor: Add the following lines to (the additions to) table 83 in page 208 line 25

| Parameter | Associate vector | Value |
| --- | --- | --- |
| OPERATING_CHANNEL | PHYCONFIG_VECTOR | The operating channel the PHY is set to use. |
| CHANNEL_OFFSET | PHYCONFIG_VECTOR | Enumerated type: CH_OFFSET_NONE – indicates operation in 20MHz HT stations CH_OFFSET_ABOVE – indicates operation in 40MHz with the secondary channel above the primary CH_OFFSET_BELOW – indicates operation in 40MHz with the secondary channel below the primary |

TGn Editor: Change line 65 in page 209 as follows

This primitive is a request by the MAC sublayer to the local PHY entity to configure the PHY receiver.