IEEE P802.11
Wireless LANs

| Normative Text for TIM Broadcast |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-16-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Menzo Wentink | Conexant | Oudegracht 3, Utrecht, the Netherlands | +31 65 183 6231 | menzo.wentink@conexant.com |
| Jari Jokela | Nokia | Visiokatu 1, 33720 Tampere, Finland | +358504860445 | jari.jokela@nokia.com |
| Marc de Courville | Motorola | Parc les Algorithmes, 91193 Gif-sur-Yvette, France | +33169352518 | marc.de.courville@motorola.com |

Abstract

This document contains normative text for a TIM broadcast proposal to meet the TGv Power Saving objective, REQ2010, improving mobile client power efficiency and battery life. The text is aligned with P802.11-REVma/D9.0 and P802.11v D0.07.

The TIM broadcast is a mechanism by which the Access Point periodically transmits a TIM frame, which is a mangement action frame.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## 7. Frame formats

- 7.2.3.1 Beacon frame format

Insert a new element in Table 8 as indicated below:

- Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 30 | TIM Broadcast Information | TIM Broadcast Information is present only if dot11WirelessManagementImplemented is true and the AP is capable of generating TIM frames. |

- 7.3.2.4 Association Request frame format

Insert a new element in Table 10 as indicated below:

Table 10—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 14 | TIM Broadcast Information | TIM Broadcast Information is present only if dot11WirelessManagementImplemented is true and the client intends to receive TIM frames. |

- 7.2.3.5 Association Response frame format

Insert a new row into table 11 as shown below:

Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | TIM Broadcast Information | TIM Broadcast Information is present only if dot11WirelessManagementImplemented is true and the AP is capable of generating TIM frames. |

- 7.2.3.6 Reassociation Request frame format

Insert a new element inTtable 12 as indicated below:

Table 12—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 14 | TIM Broadcast Information | TIM Broadcast Request is present only if dot11WirelessManagementImplemented is true and the client intends to receive TIM frames. |

- 7.2.3.7 Reassociation Response frame format

Insert a new row into table 13 as shown below:

Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | TIM Broadcast Information | TIM Broadcast Information is present only if dot11WirelessManagementImplemented is true and the AP is capable of generating TIM frames. |

      - Probe Response frame format

Insert a new element in Table 15 as indicated below:

- Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 26 | TIM Broadcast Information | TIM Broadcast Information is present only if dot11WirelessManagementImplemented is true and the AP is capable of generating TIM frames. |

## 7.3 Management frame body components

## 7.3.2 Information elements

Insert Element ID x+14 into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| TIM Broadcast Information | x+14 |
| --- | --- |

Add a new clause after 7.3.2.48:

- 7.3.2.49 TIM Broadcast Information element

The TIM Broadcast Information element contains information about the periodic broadcast of TIM frames. It shall be present in Beacon, Probe Response, Association Response and Reassociation Response frames if the AP is capable of transmitting TIM frames and if dot11WirelessManagementImplemented is true. It shall be present in Association Request and Reassociation Request frames if the associating station intends to receive TIM frames. The format of the TIM Broadcast Information element is shown in Figure TB1.

|  | Element ID | Length | TIM broadcast offset 1 | TIM rate 1 | … | TIM broadcast offset n | TIM rate n |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 1 |  | 2 | 1 |

- Figure TB1 — TIM Broadcast Information element format

The Length field is set to n * 3 if transmitted by an AP, and set to 0 if transmitted by a non-AP station, in which case no other fields shall be present.

The TIM broadcast offset field set to the number of time units (TU) offset relative to the DTIM TBTT a TIM frame is scheduled for transmission .

The TIM rate provides an indication of the rate in units of 500 kbps which is used to transmit the TIM frame.

## 7.4 Action frame format details

## 7.4.6 Wireless Network Managemen action details

Insert the following entry in Table v47 and change the reserved values into 16-255:

Table v47—Wireless Network Management Action field values

| 15 | TIM frame |
| --- | --- |

Add a new clause after 7.4.6.15:

## 7.4.6.16 TIM frame details

The format of the TIM frame is shown in Figure vT2.

|  | Category | Action | Check Beacon | TIM |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6-257 |

- Figure vT2 — TIM frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating TIM frame, as specified in Table v47 in 7.4.6.

The Check Beacon field contains the number of the latest significant Beacon update. The Check Beacon field shall be incremented when the AP includes an information element inside the Beacon which is related to a channel switch or when another significant change occurred to the beacon (which is at the AP´s prerogative to decide).

The TIM field contains a TIM starting with the Element ID. AID 0 shall not be included in the TIM.

## 11. MAC sublayer management entity

## 11.2 Power management

## 11.2.1 Power management in an infrastructure network

Add the following section:

## 11.2.1.12 TIM broadcast

If TIM broadcast is supported, the AP shall include the TIM Broadcast Information Element in Beacon, Probe Response, and (Re)Association Response frames.

TIM broadcast shall be enabled when at least one client is associated which included in its (Re)Association request a TIM Broadcast Information element. TIM broadcast shall be disabled otherwise.

If TIM broadcast is supported and enabled, the AP shall attempt to transmit at least one TIM frame per DTIM period, at the indicated offset from the DTIM TBTT. The AP is advised to schedule a TIM at the same rate as the Beacon, and a TIM at an intermediate rate which is not DSSS. If TIM broadcast is not supported or not enabled, the AP shall not transmit TIM frames.

When the client receives a TIM frame with a Check Beacon field which is higher (modulo 255) than the previously received Check Beacon field, then the client shall attempt to receive the next Beacon.

To obtain the TIM, a client may receive a TIM frame instead of a Beacon.

TIM frames may be transmitted when scheduled, and do not need to be buffered for transmission after a DTIM Beacon.

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

### A.4.3 IUT configuration

### A.4.14 Wireless Network Management extensions

Insert the following entries into A.4.14 in Annex A as follows:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME12 | TIM broadcast | 7.2.1.10, 7.2.1.11, 11.2.1.12 | CFv:O | Yes, No. N/A |