IEEE P802.11
Wireless LANs

| TGy LB104 Submission for non-Editorial comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., San Jose, CA 95134 | 408-527-0815 | petere@cisco.com |

Abstract

This document is aligned with P802.11-2007, P802.11k/D7.0, P802.11r/D5.0 for baseline, P802.11y/D2.0, and  submissions 07/0603r1 (CID 1093) and yyy 3-5-11.14 comments  and 07/0674r2 Editorial comments and addresses the following LB104 comments:

CIDs: 1002, 1007, 1008, 1030, 1096, 1097, 1098, 1099, 1107, 1146, 1147, 1148, 1201, 1206, 1207, 1213, 1214, 1229, 1230, 1231, 1232, 1233, 1254

CIDs: 1210, 1234

CIDs: 1009, 1010, 1012, 1013, 1014, 1015, 1016, 1020, 1108, 1260, 1261, 1262, 1263, 1664, 1265

CIDs: 1006, 1104, 1105, 1106, 1133, 1187, 1236, 1237, 1253

CIDs: 1011, 1024, 1050, 1053, 1078, 1079, 1080, 1081, 1082, 1083, 1086, 1087, 1088, 1089, 1090, 1091, 1092, 1094, 1137, 1139, 1147, 1148, 1149, 1151, 1152, 1153, 1235

CIDs: 1115, 1116, 1117, 1154, 1156, 1157, 1158, 1238, 1250, 1251, 1252

CIDs: 1009, 1010, 1012, 1013, 1014, 1015, 1016, 1020, 1108, 1260, 1261, 1262, 1263, 1664, 1265

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http://standards.ieee.org/board/pat/pat-slideset.ppt>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGy Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGy Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGy amendment with the baseline documents).

TGy Editor:  Editing instructions preceded by “TGy Editor” are instructions to the TGy editor to modify existing material in the TGy draft.   As a result of adopting the changes, the TGy editor will execute the instructions rather than copy them to the TGy Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# General

Comment 1021 says “The channel switch and spectrum management needs be executed in a secured and protected connection. Without proper protection, the network operation could be easily attacked and cause network chaos”. Proposed Comment Resolution: “Commenter is encouraged to provide a proposed resolution in sufficient detail so that the specific wording of the changes that will cause the negative voter to change his or her vote to "approve" can readily be determined. One of Tgy's approved Principles (06/668r1) is: The amendment should not duplicate functionality that is being standardized in other Task Groups that are likely to complete before 802.11y. (Examples include parts of 802.11k and 802.11w). As the US 3650 MHz band is lightly licensed, we do not see an FCC requirement for "protected connection" in this band, and ask for a pointer to such a requirement.” Propose Reject Comment 1254.

Proposed Resolution:

Reject based on discussion in 07/0673: 1021

# Clauses 7 and 9

T CIDs: 1002, 1007, 1008, 1030, 1096, 1097, 1098, 1099, 1107, 1146, 1147, 1148, 1201, 1206, 1207, 1213, 1214, 1229, 1230, 1231, 1232, 1233, 1254

Discussion:

In 11y/D2.0:

Comments 1002, 1030 and 1231, 7.3.2.50, note the Channel Switch Count field is not defined, and CID 1002 recommends bring back the D1.0 text that was removed by accepting CID 359. That text has ‘shalls’ in it. Proposed Comment Resolution: ‘text from 7.3.2.20 will be used, replacing’shall be set to’ with indicates, replacing ‘shall be set to zero’ with ‘or zero’ and  ‘shall occur’ with ‘occurs’. Propose Accept Comments 1030, 1231, and Accept in Principle 1002.

Comment 1107 asks that presentation 07/0401r3 RTS/CTS long slottime option be incorporated in the next draft. At a  May 15th evening meeting with more than 20 people attending, including the TGn chair, editor, Coexistence leads and several of the PHY people. A lively discussion was followed by a strawpoll, asking “How many people would like to see the LongSlotTime Option in the next version of IEEE 802.11y draft?” with a result of Yes 3, No 10, Abstain 5. Proposed Comment Resolution: “a  May 15th evening meeting with more than 20 people attending, including the TGn chair, editor, Coexistence leads and several of the PHY people. A lively discussion was followed by a strawpoll, asking “How many people would like to see the LongSlotTime Option in the next version of IEEE 802.11y draft?” with a result of Yes 3, No 10, Abstain 5.” Propose Reject Comment 1107.

Comments 1201 and 1230, 7.3.2.50, note the Channel Switch Mode values are not described, and both ask to describe the meaning of these values. Proposed Comment Resolution: “transmission, as specified in 11.9.7.1 and 11.9.7.2.” Propose Accept in Principle Comments 1201 and 1230.

Comment 1096 7.3.2.49, says to resolve the differences between this draft and the 11k draft for LCI, create a common over-the-air LCI between TGk and TGy, then describe it, and the Information Elements that use it. Similarly, Comment 1206 says Why not just be consistent with the RFC, and use an 8-bit field for DATUM? Proposed Comment Resolution: “It costs an extra octet of transmission in every beacon frame to use an 8-bit field for datum, and only three values are necessary. The extra transmission energy creates interference for other stations. Suggested remedy insufficiently described, it appears that making the MIB LCI elements the same accomplishes the purpose”. Propose Reject Comment 1096, 1206.

Comment 1147 7.3.2.49, says the Altitude Type 3 sentence is poorly formatted and suggests changing Figure 112z Altitude Type to Height Above Ground. The remedy does not allow regulation to require Altitude Type 1 or 2. Proposed Comment Resolution “Accept reformatting sentence, but reject changing Figure 112z, because regulation may require Altitude Type 1 or 2.” Propose Accept in Principle Comment 1147.

Comment 1214 9.8.3, asks “To which mib variable does "regulatory classes capability" refer? Proposed Comment Resolution “the two mib variables are dot11RegulatoryClassesImplemented and dot11RegulatoryClassesRequired. There are legacy STAs with neither mib variable, so the text is a compromise to indicate the following cases - Implemented is true, but the STA has not been configured, Implemented is false, and the legacy case where the mib variables are not present.” Propose Reject Comment 1214.

Comment 1254 asks why new Altitude Type 3 is needed, and asks for clarification. AT1 can be defined by DATUM, and can be height above a model earth (WGS-84), or above the high tide line (NAD-27), neither of which is visible or easily found by a licensed radio operator. Proposed Comment Resolution: “AT 3 height above ground is independent of DATUM, and is easy for an operator to find or measure'” Propose Reject Comment 1254.

Propose Accept Comments 1007, 1008, 1030, 1097, 1098, 1099, 1146, 1148, 1207, 1213, 1231, 1232 and 1233 without discussion.

Proposed Resolution:

Accept: 1007, 1008, 1030, 1097, 1098, 1099, 1146, 1148, 1207, 1213, 1231, 1232, 1233

Accept in Principle based on discussion and editorial instructions in 07/0673: 1002, 1147, 1201, 1230

Reject based on discussion in 07/0673: 1096, 1107, 1206, 1214, 1254

Editorial Instructions:

Tgy Editor: insert text in 7.2.3, pg 2, D2.0 as follows

Table 8: Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 33 | Supported Regulatory Classes | The Supported Regulatory Classes information element shall be present if dot11ExtendedChannelSwitchImplemented is true. |

Tgy Editor: change text in 7.3.2 Table 26, pg 5, D2.0 as follows

Supported Regulatory Classes (see 7.3.2.51)	<ANA>	3 to 34255

Tgy Editor: change text in 7.3.2.49, pg 6, D2.0 as follows

Altitude Type (AT) 3: Height Above Ground, meters - is defined to be in meters and is formatted in 2’s complement fixed-point 22-bit integer part with 8-bit fraction.

Tgy Editor: change text in 7.3.2.50, pg 7, D2.0 as follows

The Channel Switch Mode field indicates any restrictions on transmission until a channel switch. An AP in a BSS or a STA in an IBSS sets the Channel Switch Mode field to either 0 or 1 on transmission, as specified in 11.9.7.1 and 11.9.7.2.

The New Regulatory Class field is set to the number of the regulatory class after the channel switchto which the STA is moving, as defined in Annex J.

The New Channel Number field is set to the number of the channel after the channel switchto which the STA is moving. The channel number is a channel from the STA’s new Regulatory Class as defined in Annex J.

The Channel Switch Count field either indicates the number of TBTTs until the STA sending the Channel Switch Announcement element switches to the new channel or 0. A value of 1 indicates that the switch occurs immediately before the next TBTT. A value of 0 indicates that the switch occurs at any time after the frame containing the element is transmitted.

Tgy Editor: change text in 7.3.2.51, pg 7, D2.0 as follows

The length of the Supported Regulatory Classes element is between 1 and 25332 octets.

Tgy Editor: change text in 7.4.1.6, pg 8, D2.0 as follows

The Action Value field is set to indicate an Extended Channel Switch Announcement frame, as defined in Table 44.

Tgy Editor: change text in 9.8.3 and 9.8.4, pg 9, D2.0 as follows

### 9.8.3 Operation with Regulatory Classes

When dot11RegulatoryClassesImplemented is true and dot11LCIDSERequired is true, the following statements apply:

- When dot11RegulatoryClassesRequired is false, or where Regulatory Classes domain information is not present in a STA, that STA is not required to change its operation in response to an information element or element-specific information field that contains a Regulatory Class;.

- When dot11RegulatoryClassesRequired is true, or where Regulatory Classes domain information is present in a STA, the STA shall indicate Regulatory Class information in the Country Information element and Supported Regulatory Classes information element;.

- When dot11RegulatoryClassesRequired is true and a STA is capable of operating as specified in more than one Regulatory Class, the STA shall include the Country Information and SupportedRegulatoryClasses elements in Association frames and Reassociation frames;.

- When dot11RegulatoryClassesRequired is true, or where Regulatory Classes domain information is present, and the STA parsing a Country Information element finds a First Channel Number or Regulatory Class with a Reserved value, the STA shall ignore the remainder of the Country Iinformation element and shall parse any remaining management frame body for additional information elements.

9.8.4 Operation with Coverage Classes

The default PHY parameters are based on aAirPropagationTime having a value of 1 µs or less, and SlotTime and other MAC timing are based on the PHY timing parameters. When dot11RegulatoryClassesImplemented is trueregulatory classes capability is implemented, it is possible to manage the MAC timing of stations that can receive Beacon frames or Probe Responses frames that contain the Country Information element (7.3.2.9), to increase fairness in contending for the medium. Radio waves propagate at ~300 m/µs in free space, and, for example, 3 µs would be the ceiling for BSS maximum one way distance of ~450 m (~900 m round trip). When dot11RegulatoryClassesImplemented is true and dot11LCIDSERequired is true, tThe Coverage Class field of the Country Information element shall be processed when received by the associated STA or dependent STA, replacing the current aAirPropagationTime used in IFS (9.2.3) with one indicated by the Coverage Class field, and the STA  shall recalculate the different IFSs that are a function of aAirPropagationTime.

Using the Country Information element, an AP can change Coverage Class and Max Transmit Power Level to enhance outdoor operation. When dot11RegulatoryClassesImplemented is true and dot11LCIDSERequired is true, and the Max Transmit Power Level is different from the Transmit Power limit indicated by the Regulatory Class, the associated STA or dependent STA shall operate at a transmit power at or below that indicated by the lesser of the two limits.

Clause 10

T CIDs: 1210, 1234

Discussion:

In 11y/D2.0:

Comment 1210 notes that 11r/D5.0 introduced association parameter changes, and requests that D2.0 be updated to reflect such changes. Propose Accept Comment 1210, 1234

Proposed Resolution:

Accept 1210, 1234

Editorial Instructions:

Tgy Editor: insert Content of FT Authentication Information Elements in MLME-ASSOCIATE. {request, confirm, indication, response} text in 10.3.6, pg 11-13

Change the primitive parameter lists as shown:

MLME-ASSOCIATE.request	(
PeerSTAAddress,
AssociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,

RCPI,

RSNI,

Content of FT Authetication Information Elements,
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

MLME-ASSOCIATE.confirm	(
ResultCode,
CapabilityInformation,
AssociationID,
EDCAParameterSet,

Content of FT Authetication Information Elements,
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

MLME-ASSOCIATE.indication	(
PeerSTAAddress,
CapabilityInformation,
ListenInterval,

SSID,
Supported Rates,
RSN,
QoSCapability,

RCPI,

RSNI,

Content of FT Authetication Information Elements,
SupportedRegulatoryClasses,

DSEregisteredlocation, 
VendorSpecificInfo
)

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
EDCAParameterSet,

RCPI,

RSNI,

Content of FT Authetication Information Elements,
SupportedRegulatoryClasses, 
DSEregisteredlocation, 
VendorSpecificInfo
)

# Clause 11.9.7 CSA procedures

CIDs: 1009, 1010, 1012, 1013, 1014, 1015, 1016, 1020, 1108, 1260, 1261, 1262, 1263, 1664, 1265

Discussion:

Similar text for Extended Channel Switch Announcement element is in 11n/D2.0, 11v/D0.11 and 11y/D2.0, with 11y/D2.0 being the newest version to receive LB comments (after 11n/D2.0) A strawpoll in TGn in London showed very strong preference in having one version of text that can be used in all amendments, and they voted unanimously to approve the text from 11-06/1775r2 to resolve all comments about ECSA. TGn brought their D2.0 comments to TGy for common resolution, which took place May 15th evening session with more than 20 people attending, including the TGn chair, editor, Coexistence leads and several whose TGn comments were brought by Bjern Bjerke.

Comments 1012 and 1013 ask when switching regulatory classes on the same channel, does a beaconing STA need announce a channel switch to legacy devices, without saying which kind of legacy devices are to be considered. There are legacy devices that do not know about channel switch in the 2.4 GHz band, and there are legacy devices that do not know about regulatory class in the 5 GHz band, and there are legacy devices that do not know about ECSA. In general, channel switch is best effort, and announcements are unacknowledged, and in particular, none of the legacy devices know how to switch regulatory classes. There is no benefit in sending CSA when changing regulatory classes on the same channel.

Proposed Comment Resolutions:

1012 “comment asks when switching regulatory classes on the same channel, does an AP need announce a channel switch to legacy devices, without saying which kind of legacy devices are to be considered. There are legacy devices that do not know about channel switch in the 2.4 GHz band, and there are legacy devices that do not know about regulatory class in the 5 GHz band, and there are legacy devices that do not know about ECSA. In general, channel switch is best effort, and announcements are unacknowledged, and in particular, none of the legacy devices know how to switch regulatory classes. There is no benefit in sending CSA when changing regulatory classes on the same channel.”

1013 “comment asks when switching regulatory classes on the same channel, does a DFS owner need announce a channel switch to legacy devices, without saying which kind of legacy devices are to be considered. There are legacy devices that do not know about channel switch in the 2.4 GHz band, and there are legacy devices that do not know about regulatory class in the 5 GHz band, and there are legacy devices that do not know about ECSA. In general, channel switch is best effort, and announcements are unacknowledged, and in particular, none of the legacy devices know how to switch regulatory classes. There is no benefit in sending CSA when changing regulatory classes on the same channel.”

Propose Reject Comments 1012 and 1013.

Comments 1015  11.9.7.1 and 1016 11.9.7.2 on changing channels within the same regulatory class, reword a compound sentence into two sentences, and make it possible to send both CSA and ECSA, as is requested in Comments 1009, 1010, 1261, 1262, 1264 and 1265. Proposed Comment Resolutions:

1015 “change second sentence to permit sending both: "is false, the AP shall send the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames." CID 1217 corrects grammar of element and frame.”

1016 “change second sentence to permit sending both: "is false, the DFS owner shall send the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames." CID 1217 corrects grammar of element and frame.”

1261 “May 15th discussion (07/0673) chose CID 1015 resolution to modify (2). Remedy unclear what sentence should be deleted.”

1262 “May 15th discussion (07/0673) chose CID 1015 resolution to modify (2).”

1264 “May 15th discussion (07/0673) chose CID 1016 resolution to modify (2). Remedy unclear what sentence should be deleted.”

1265 “May 15th discussion (07/0673) chose CID 1016 resolution to modify (2).”

Propose Accept in Principle Comments 1015 and 1016. Propose Reject Comments 1261, 1262, 1264 and 1265.

Comment 1263 11.9.7.2, allows using both ECSA and CSA when changing regulatory classes. Proposed Comment Resolution “with correct spelling of 'Channel.'” Propose Accept in Principle Comment 1263. Comment 1014 suggests similar text to CID 1263, but fails to address changing regulatory class on the same channel. Proposed Comment Resolution ."CID 1263 suggests change that allows switching regulatory class on same channel, as well as to a different channel.” Propose Reject Comment 1014.

Comment 1020 11.9.7.3, asks what is “this” country? Proposed Comment Resolution “Country (7.3.2.9)” Propose Accept Comment 1020.

Comment 1108 11.9.7.1, asks about making consideration of STAs in power save mode, and suggests a text change that was discussed May 15th and found wanting. Proposed Comment Resolution “strawpoll May 15th (07/0673) with commenter present voted 14-0 to change the remedy to delete 'If possible, '” Propose Accept in Principle Comment 1108.

Comment 1260 11.9.7.1, asks about the change in baseline text, that after May 15th discussion appears not needed.

“baseline text will not be changed, leaving original meaning.” Propose Accept in Principle Comment 1260.

Propose Accept Comments 1009, 1010, 1011 without discussion

Proposed Resolution:

Accept: 1009, 1010, 1011, 1020

Accept in Principle: based on discussion and editorial instructions in 07/0673: 1014, 1015, 1016, 1108, 1260, 1263

Reject: based on discussion and editorial instructions in 07/0673; 1014, 1261, 1262, 1264 and 1265

Editorial Instructions:

Tgy Editor: replace text in 11.9.7, pg 21-22, D2.0 as follows

      - Selecting and advertising a new channel in an infrastructure BSS

Change second sentence of 11.9.7.1 as follows:

An AP may make use of the information in the Supported Channel, and if present, Supported Regulatory Classes  elements, and the results of measurements undertaken by the AP and other STAs in the BSS to assist the selection of the new channel.

Insert new second paragraph and change third paragraph of 11.9.7.1 as follows:

In the following text, wherever Channel Switch Announcement is referred to, the following rules apply:

- If an AP is switching to a new channel in a different regulatory class, then the AP shall use either the Extended Channel Switch Announcement element and frame instead of the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.

- If an AP is switching to a channel within the same regulatory class, and dot11ExtendedChannelSwitchImplemented is true, then the AP shall send the Extended Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.  If  dot11ExtendedChannelSwitchImplemented is false, the AP shall send the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.

An AP shall inform associated STAs that the AP is moving to a new channel and maintain the association by advertising the switch using Channel Switch Announcement elements in Beacon frames, Probe Response frames, and Channel Switch Announcement frames until the intended channel switch time. The AP may force STAs in the BSS to stop transmissions until the channel switch takes place by settingusing the Channel Switch Mode field in the Channel Switch Announcement element to 1. If possible, tThe channel switch should be scheduled so that all STAs in the BSS, including STAs in power save mode, have the opportunity to receive at least one Channel Switch Announcement element before the switch. The AP may send the Channel Switch Announcement frame in a BSS without performing a backoff, after determining that the WM is idle for one PIFS period.

Insert new first paragraph in 11.9.7.2 as follows:

- 11.9.7.2 Selecting and advertising a new channel in an IBSS

In the following text wherever Channel Switch Announcement is referred to, the following rules apply:

- If a DFS owner is switching to a new channel in a different regulatory class, then the DFS owner shall use the Extended Channel Switch Announcement element and frame instead of the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.

- If a DFS owner is switching to a channel within the same regulatory class, and dot11ExtendedChannelSwitchImplemented is true, then the DFS owner shall send the Extended Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.  If dot11ExtendedChannelSwitchImplemented is false, the DFS owner shall send the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.

- The DFS owner shall inform associated STAs that it is moving to a new channel and maintain the association by advertising the switch using Extended Channel Switch Announcement and Channel Switch Announcement elements in accordance with rules 1) and 2), in Beacon frames, Probe Response frames, and Action frames until the intended channel switch time.

Insert new text after 11.9.7.2:

      - Advertising supported regulatory classes

When dot11ExtendedChannelSwitchImplemented is true, the Current Regulatory Class octet in the Supported Regulatory Classes element shall indicate the Regulatory Class in use for transmission and reception. The List of Regulatory Class(es) field shall list in ascending order all Regulatory Classes that the STA is capable of operating with, for this Country (7.3.2.9).

When dot11ExtendedChannelSwitchImplemented is true, the Supported Regulatory Classes element shall be included in Association Request frames, as described in 7.2.3.4, Association Response frames, as described in 7.2.3.5, in Reassociation Request frames, as described in 7.2.3.6, Reassociation Response frames, as described in 7.2.3.7, Probe Request frames, as described in 7.2.3.8 and Probe Response frames, as described in 7.2.3.9.

Clause 17

CIDs: 1006, 1104, 1105, 1106, 1133, 1187, 1236, 1237, 1253

Discussion:

In 11y/D2.0:

Comment 1105 suggests changing the CS signalling, removing PMD_CS. Proposed Comment Resolution: “May 15th discussion (07/0673) resolved CID 1105 by removing all amendment changes to 17.5” Propose Accept Comment 1105 by removing all changes to 17.5 and references to PMD_CS and PMD_ED. Consequently Comments 1106, 1134 and 1136 are on 17.5 text that is removed by CID 1105. Proposed Comment Resolution: “May 15th discussion (07/0673) resolved CID 1105 by removing all amendment changes to 17.5” Propose Reject Comment 1106.

Comment 1187 says “as defined  in 802.11 rev-ma9.0, RSSI is a parameter that takes on only 16 values and they a only to be used as a "relative measure of the received signal power". RSSI is defined in each of the PHY clauses, and the Clause 17 PHY 17.5.5.7.2 defines it with 255 levels. Proposed Comment Resolution: “17.5.5.7.2 defines RSSI as having up to 255 levels, enough for ED purposes”. Propose Reject Comment 1187

Propose Accept Comments 1006, 1104, 1106, 1133, 1236, 1237 and 1253 without discussion.

Proposed Resolution:

Accept: 1006, 1104, 1105, 1106, 1133, 1236, 1237 and 1253

Reject based on discussion in 07/0673: 1106, 1187

Editorial Instructions:

Tgy Editor: delete first change to 17.1, the last two sentences in first paragraph of 17.1 pg 25, D2.0

Tgy Editor: change lowest signal from -45 dBm to -40 dBm in Figure 254a  pg 27, D2.0

Tgy Editor: insert sentence after the editing instruction in 17.3.9.2  pg 26 line 56, D2.0

A transmit spectrum mask is more stringent than another if any of its defined points requires less emissions than the mask it is compared with.

Tgy Editor: insert text before first sentence of second paragraph in 17.3.10.5  pg 28, D2.0

A transmit spectrum mask is more stringent than another if any of its defined points requires less emissions than the mask it is compared with.

Tgy Editor: delete Figure 264 and EDITORIAL NOTE on  pg 29, D2.0

Tgy Editor: delete first two sentences in last inserted text on  pg 29, D2.0

Tgy Editor: delete editing instruction and third paragraph on  pg 30, D2.0

Tgy Editor: delete all text about 17.5 on  pages 30-33, D2.0

Annexs A and D

CIDs: 1011, 1024, 1050, 1053, 1078, 1079, 1080, 1081, 1082, 1083, 1086, 1087, 1088, 1089, 1090, 1091, 1092, 1094, 1137, 1139, 1147, 1148, 1149, 1151, 1152, 1153, 1235, 1247, 1248, 1249

Comment 1024 says dot11LCIDSETable asserts it is dot11smt 14, yet “it is not introduced as part of dot11smt.” It appears to be introduced in dot11smt on the previous page p37, line 60. Proposed Comment Resolution: “D2.0 p37 line 60 attempts to insert dot11LCIDSETable into dot11smt. If there is more required, let the TGy editor know and it will be done”. Propose Accept Comment 1024.

Comments 1050 and 1137 are reassertions of LB94 comments that the editor failed to do when creating D2.0.

Comments 1053, 1082, 1091, 1092 will be withdrawn if submission 07/0603 LB104 DSE reporting CID 1093.doc is approved. Propose Accept in Principle Comments 1053, 1082, 1091, 1092.

Comment 1139 says that if 11k fixes their mistake in D7.0, then dot11SMTbase9 should be used by 11y. Propose Accept in Principle Comment 1139.

Comment 1235 says to put the dot11DSE time limits in the MIB and rely on default values for normal operation. Propose Accept in Principle Comment 1235.

Propose Accept Comments 1078, 1079, 1080, 1081, 1083, 1086, 1087, 1088, 1089, 1090, 1094, 1139, 1147, 1148, 1149, 1151, 1152, 1153, 1247, 1248 and 1249 without discussion.

Proposed Resolution:

Accept 1011, 1024, 1050, 1078, 1079, 1080, 1081, 1083, 1086, 1087, 1088, 1089, 1090, 1094, 1147, 1147, 1149, 1247, 1248 and 1249

Accept in Principle based on discussion and editorial instructions in 07/0673: 1053, 1082, 1091, 1092, 1139 and 1235

Editorial Instructions:

Tgy Editor: change text in Annex A, Table A.4.17 second through fourth entries, pg 36, line 47, D2.

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *DSE1 | Fixed station operation with RegLoc | 11.14.1 | CFy:O.1 | Yes □ No □ N/A □ |
| *DSE2 | Enabling APstation operation with RegLoc | 11.14.1 | CFy:O.1 | Yes □ No □ N/A □ |
| DSE2.1 | Enabling APstation creation of BSA | 11.14.2 | DSE2:M | Yes □ No □ N/A □ |
| DSE2.2 | Enabling APstation operation with DSE | 11.14.1 | DSE2:M | Yes □ No □ N/A □ |

Tgy Editor: insert text in Annex D, after dot11LCIDSERequired TruthValue, pg 38, line 10, D2.0

,

dot11ExtendedChannelSwitchImplemented TruthValue

Tgy Editor: insert text in Annex D, after dot11LCIDSERequired, pg 38, line 41, D2.0

dot11ExtendedChannelSwitchImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

Implementation is capable of supporting Extended Channel

Switch Announcement. The capability is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry 60 }

Tgy Editor: insert text in Annex D, Dot11LCIDSEEntry after dot11DependentEnablementIdentifier, pg 39, line 29, D2.0

,

dot11DSEAssociateTimeLimit Unsigned32,

dot11DSEFailHoldTime Unsigned32,

dot11DSERenewalTime Unsigned32,

dot11DSETransmitDivisor Unsigned32

Tgy Editor: replace text in Annex D, in dot11LCIDSELatitudeInteger, pg 40, line 12, D2.0, also dot11LCIDSELongitudeInteger

SYNTAX Integer32 (-359..359)

Tgy Editor: replace text in Annex D, in dot11LCIDSELatitudeFraction, pg 40, line 25, D2.0, also dot11LCIDSELongitudeFraction

SYNTAX Integer32 (-16777215..16777215)

Tgy Editor: change text in Annex D, in dot11LCIDSEAltitudeType, pg 41, line 27, D2.0

hagm : Height Above Ground in meters, in 2s-complement

fixed-point 22-bit integer part with 8-bit fraction"

DEFVAL{ 3 }

Tgy Editor: replace text in Annex D, in dot11LCIDSEAltitudeInteger, pg 40, line 45, D2.0

SYNTAX Integer32 (-2097151..2097151)

Tgy Editor: replace text in Annex D, in dot11LCIDSEAltitudeFraction, pg 41, line 61, D2.0

SYNTAX Integer32 (-127..127)

Tgy Editor: change text in Annex D, in dot11LCIDSEDatum, pg 42, line 12, D2.0

"Datum is a three-bit value encoding the horizontal and vertical

references used for the coordinates given in this LCI. IETF

RFC-3825 defines the values of Datum. Type 1 is WGS-84, the

Coordinate system used by GPS. Type 2 is NAD83 with NADV88

Vertical reference. Type 3 is NAD83 with Mean Lower Low Water

Vertical datum. All other types are reserved."

Tgy Editor: replace text in Annex D, in dot11DependentenablementIdentifier, pg 43, line 18, D2.0

SYNTAX Integer32 (0..65535)

Tgy Editor: insert text in Annex D, Dot11LCIDSETable after dot11DependentEnablementIdentifier, pg 43, line 31, D2.0

dot11DSEAssociateTimeLimit OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11DSEAssociateTimeLimit indicates the maximum number of

Seconds that a dependent STA may transmit in a DSE frequency

Band while becoming associated with a control STA. Unless another

Value is mandated by regulatory authorities, the value

Shall be 32 seconds."

DEFVAL { 32 }

::= { dot11LCIDSEEntry 19 }

dot11DSEAssociateFailHoldTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11DSEAssociateFailHoldTime indicates the number of seconds

That a dependent STA must not transmit in a DSE frequency band

When it fails to become associated with a control STA within

Dot11DSEAssociateTimeLimit seconds. Unless another value is

Mandated by regulatory authorities, the value shall be 512 seconds.”

DEFVAL { 512 }

::= { dot11LCIDSEEntry 20 }

dot11DSERenewalTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11DSERenewalTime indicates the maximum number of seconds

that a dependent STA may operate in a DSE frequency band without

receiving and decoding and enabling signal from it’s control STA.

Unless another value is mandated by regulatory authorities, the

Value shall be 60 seconds.”

DEFVAL { 60 }

::= { dot11LCIDSEEntry 21 }

dot11DSETransmitDivisor OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11DSETransmitDivisor indicated the value used by a dependent

STA when operating in a DSE frequency band and transmitting. The

Dependent STA sends an Action frame when the sum of

Dot11TransmittedFragmentCount, dot11MulticastTransmittedFrameCount

And dot11ReceivedFragmentCount modulo dot11DSETransmitDivisor

Equals zero. Unless another value is mandated by regulatory

Authorities, the value shall be 256.”

DEFVAL { 256 }

::= { dot11LCIDSEEntry 22 }

Tgy Editor: delete dot11TIThreshold,4 text in Annex D, dot11PhyOFDMComplianceGroup3, pg 51, line 64, D2.0

Tgy Editor: insert text in Annex D, after dot11LCIDSERequired TruthValue, pg 53, line 1, D2.0

,

dot11ExtendedChannelSwitchImplemented TruthValue

Annexs I and J

CIDs: 1115, 1116, 1117, 1154, 1156, 1157, 1158, 1238, 1250, 1251, 1252

All comments apply to specific requirements in Annex J.2 that only apply to 3650-3700 MHz Operation in USA.

Proposed Comment Resolution “Will qualify these requirements by changing leadin to ‘Further requirements when dot11LCIDSERequired is true:’ Propose Accept Comments 1154, 1156, 1157, 1158, 1238, 1250, 1251, 1252.

Comment 1115 suggests rewording the 5 MHz channel bandwidth requirement so that it is not required of all stations, without any rational for such change. The 5 MHz requirement is for all stations because it is most robust, and will likely be used for enablement. Proposed Comment Resolution “The 5 MHz requirement is for all stations because it is most robust, and will likely be used for enablement.”  Propose Reject Comment 1115.

Comment 1116 suggests requiring 10- and 20 MHz channel bandwidth operation of all stations, without any rational for such change. The requirement has not changed from D1.0. 10- and 20 MHz channel bandwidth operations are optional for fixed stations, as they can operate at higher power, and it is more expensive to provide multiple channel bandwidths. Proposed Comment Resolution “The 10 and 20 MHz channel bandwidth requirements have not changed since D1.0, and are optional for fixed stations, as they can operate at higher power, and it is more expensive to provide multiple channel bandwidths.”  Propose Reject Comment 1116.

Comment 1117 suggests “Change all occurances where use of frequency is implied to restrict the operation to 3.65GHz only” Proposed Comment Resolution “Commenter is encouraged to provide a proposed resolution in sufficient detail so that the specific wording of the changes that will cause the negative voter to change his or her vote to "approve" can readily be determined.”  Propose Reject Comment 1117.

Propose Accept Comments 1154, 1156, 1157, 1158, 1238, 1250, 1251, 1252.

Proposed Resolution:

Accept: 1154, 1156, 1157, 1158, 1238, 1250, 1251, 1252

Reject based on discussion in 07/0673: 1115, 1116, 1117

Editorial Instructions:

Tgy Editor: change beginning sentence of last paragraph in J.2  pg 57, line 29, D2.0

Further requirements when dot11LCIDSERequired is true: of this type of operation include the following:

Tgy Editor: Insert text after last listed element of last paragraph in J.2  pg 57, line 47, D2.0

—The values of dot11DSEAssociateTimeLimit, dot11DSEAssociateFailHoldTime, dot11DSERenewalTime, and dot11DSETransmitDivisor shall be 32 seconds, 512 seconds, 60 seconds and 256 respectively.