IEEE P802.11
Wireless LANs

| BSS Maximum Idle Time Advertisement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Thomas Kuehnel | Microsoft Corp | One Microsoft Way, Redmond, WA 98075 | +1-425-706-4351 | tkuehnel@microsoft.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave, Sunnyvale, CA 94089 | +1-408-754-1213 | subbu@arubanetworks.com |
| Henry Ptasinski | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1-408-543-3316 | henryp@broadcom.com |
| Emily Qi | Intel Corporation | 2111 NE 25th Hillsboro, OR 97124 | +1-503-264-7799 | Emily.h.qi@intel.com |
| Jiandong Ruan | Microsoft Corp | One Microsoft Way, Redmond, WA 98075 | +1-425-705-5863 | jruan@microsoft.com |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA 95134 | +1-408-853-5570 | allant@cisco.com |
| James Tsai | Intel Corporation | 2111 NE 25th Hillsboro, OR 97124 | +1-503-712-5509 | James.tsai@intel.com |
| Qi Wang | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1-408-922-8798 | qi.wang@broadcom.com |

Abstract

This submission contains the normative text for an AP to advertise how long a non-AP STA may remain inactive before the AP disassociates the STA. This submission is related to TGv Objective: Power Saving (Req. #2010).

The text is aligned with P802.11v-D0.11

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[Insert the following changes to 3]

- Definitions

3.v.5  BSS Max Idle Period: the time period during which a STA can refrain from transmitting frames to its associated AP before being disassociated. This time period is specified in units of 1000 TUs.

[Make the following changes to 7.2.3.5]

##### 7.2.3.5 Association Response frame format

[Insert the following rows in Table 11]

| Order | Information | Notes |
| --- | --- | --- |
| 15 | BSS Max Idle Period | The BSS Max Idle Period element may be present if dot11WirelessManagementImplemented is true. |

[Make the following changes to 7.2.3.7]

##### 7.2.3.7 Reassociation Response frame format

[Insert the following rows in Table 13]

| Order | Information | Notes |
| --- | --- | --- |
| 16 | BSS Max Idle Period | The BSS Max Idle Period element may be present if dot11WirelessManagementImplemented is true. |

[Make the following changes to 7.3.2]

### 7.3.2 Information elements

[Insert the following rows into Table 26]

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| BSS Max Idle Period (see 7.3.2.65) | x+16 | 3 |
| Reserved | x+1617, 220 |  |

[Insert the following after 7.3.2.64]

##### 7.3.2.65 BSS Max Idle Period element

The BSS Max Idle Period element contains the time period a non-AP STA can refrain from transmitting frames to the AP before the AP disassociates the STA.

|  | Element ID | Length | Max Idle Period | Idle Options |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 1 |

Figure vn. BSS Max Idle Period element format

The minimum value of the Length field is 3.

The Max Idle Period field indicates the number of 1000 TUs that will pass before an AP disassociates an inactive non-AP STA. A non-AP STA is considered inactive if the AP has not received a frame of a frame exchange sequence initiated by the STA for a time period equal to or greater than the time specified by the Max Idle Period field value.

The Idle Options field is a bit-field indicating the options associated with the BSS Max Idle capability.

The Idle Options field is shown in Figure vXX.

|  | B0 | B1 | B7 |
| --- | --- | --- | --- |
|  | Protected Keep-Alive Required | Reserved |  |
| Bits: | 1 | 7 |  |

- Figure vXX—Idle Options field

- The Protected Keep-Alive Required bit set to 1 indicates that the STA must send a protected frame to the AP to reset the Idle Timer, defined in 11.2.1.1, on the AP for the STA. If the Protected Keep-Alive Required bit is set to 0, then the STA sends either an unprotected or protected frame to the AP to reset the Idle Timer on the AP.

- Layer management

  - MLME SAP Interface

Change 10.3.6 as follows:

    - Associate

      - MLME-ASSOCIATE.confirm

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.confirm                              (
ResultCode,
CapabilityInformation,
AssociationID,
SupportedRates,
EDCAParameterSet,
WirelessManagementCapabilities,
FBMSResponse,
SupportedRegulatoryClasses,

BSSMaxIdlePeriod
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BSSMaxIdlePeriod | As defined in BSS Max Idle Period element | As defined in BSS Max Idle Period element | Indicates the BSS Max Idle Period parameters of the AP. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.6.4 MLME-ASSOCIATE.response

- 10.3.6.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
WirelessManagementCapabilities,
FBMSResponse,
SupportedRegulatoryClasses,

BSSMaxIdlePeriod
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BSSMaxIdlePeriod | As defined in BSS Max Idle Period element | As defined in BSS Max Idle Period element | Indicates the BSS Max Idle Period parameters of the AP. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

Change 10.3.7 as follows:

### 10.3.7 Reassociate

- 10.3.7.12 MLME-REASSOCIATE.confirm

- 10.3.7.12.2 Semantics of the service primitive

Change the primitive parameters as shown:

MLME-REASSOCIATE.confirm	(
Resultcode,
CapabilityInformation,
AssoicationID,
SupportedRates,
EDCAParameterSet,
Content of FT Authentication Information Elements,
WirelessManagementCapabilities,
FBMSResponse, 
SupportedRegulatoryClasses,

BSSMaxIdlePeriod)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BSSMaxIdlePeriod | As defined in BSS Max Idle Period element | As defined in BSS Max Idle Period element | Indicates the BSS Max Idle Period parameters of the AP. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.7.14 MLME-REASSOCIATE.response

- 10.3.7.14.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
Content of FT Authentication Information Elements,
WirelessManagementCapabilities,
FBMSResponse, 
SupportedRegulatoryClasses,

BSSMaxIdlePeriod)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BSSMaxIdlePeriod | As defined in BSS Max Idle Period element | As defined in BSS Max Idle Period element | Indicates the BSS Max Idle Period parameters of the AP. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

[Insert the following section in 11.2.1 after the last paragraph]

##### 11.2.1.1 BSS Max Idle Period Management

- An AP may advertise the BSS Max Idle Period information element, defined in 7.3.2.65, in every Association or Reassociation response if dot11WirelessManagementImplemented is set to true. The Idle Options field may require STA’s to send protected or unprotected keep-alive frames.

- The AP shall maintain an Idle Timer for each STA associated to the BSS. After the STA associates to the AP, the Idle Timer shall be initialized to the Max Idle Period value. If the Idle Options field requires protected keep-alive frames, then the Idle Timer shall be reset to the Max Idle Period upon receipt of a protected frame from the STA. If the Idle Options field allows unprotected or protected keep-alive frames, then the Idle Timer shall be reset to Max Idle Period upon receipt of any frame from the STA.

- While no keep-alive frames are received from the STA, the Idle Timer value shall be decremented by 1 every 1000 TUs until it reaches a value of zero.

- Once the Idle Timer reaches zero, the AP may disassociate the STA.

[Insert the following rows to A.4.16]

### A.4.16 Wireless Network Management extensions

| RME12 | BSS Max Idle Period | 7.3.2.65 | CFv:O | Yes, No, N/A |
| --- | --- | --- | --- | --- |