IEEE P802.11
Wireless LANs

| TGn LB97 Submission |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comments assigned to the author:

- 61,62, 129 – pending approval

It also contains a number of straw polls intended to discover the will of the membership,  where no single obvious choice is available.

The changes marked in this document are based on TGn Draft version D2.06.

R1: Changes made during MAC Ad-hoc Telecon 2007-06-20 noted like this.

R2: updated resolutions to match result of straw polls

R3:  As discussed and approved (in part) during the MAC telecom.

R4:  Move the approved discussed comments into a section (for reference) and working on those comments that were deferred. approved comments removed.  Pending comment resolutions provided.  This version approved at the MAC ad-hoc on 13 Sept 2007.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# CID 129

| 129 | 469.10 | Annex S | Is the intention of nav-set sequence sent in non-HT PPDU? Then "(RTS[+HTC] CTS[+HTC]+ampdu ma-no-ack-htc+ampdu+ampdu-end)" in l.11, p.469 won't meet that. If the nav-set sequence can be also sent in HT PPDU (under the condition of the Operational Mode), then NAV setting by HT PPDU using A-MPDU needs to be added. | Correct as in comment. |
| --- | --- | --- | --- | --- |

### Discussion:

The straw poll taken earlier disallows the cited sequence.

Note, CID 129 is shown as approved by motion 183, but it has been returned by the Editor with an ER status: “I believe this was not approved in the MAC ad-hoc,  as the submission 11-07/0706r3 shows this as "defer for now".   In any case the resolution doesn't provide enough detail to implement.”

### Proposed resolution:  Counter

Accept in principal the changes proposed to “nav-set” and make the same addition to “L-sig-protection-set”.

Change Annex S nav-set term as follows:

(* These are the series of frames that establish NAV protection for an HT sequence *)

nav-set = (RTS[+HTC] CTS[+HTC]) |

CTS+self |

(Data[+HTC]+individual[+null][+QoS+normal-ack] Ack) |

Data[+HTC]+individual [+QoS+(block-ack)] | (#922,60)

Data+group[+null][+QoS] |

( 1{ Data[+HTC]+individual+QoS+implicit-bar+a-mpdu}+a-mpdu-end  BlockAck[+HTC] ) |

(BlockAckReq[+HTC] (BlockAck[+HTC]|Ack[+HTC](#922))) |

(BlockAck[+HTC] Ack);

Change Annex S l-sig-protection-set as follows:

L-sig-protection-set = (RTS+L-sig[+HTC] CTS+L-sig[+HTC]) | (#922)

(Data+individual+L-sig [+HTC][+null][+QoS+normal-ack] Ack

[+HTC](#922) +L-sig) | (#922)

( 1{ Data+L-sig [+HTC]+individual+QoS+implicit-bar+a-mpdu}+a-mpdu-end  BlockAck+L-sig[+HTC] ) |

(BlockAckReq+L-sig[+HTC] (BlockAck[+HTC]|Ack[+HTC](#922))+L-sig) |

(BlockAck+L-sig[+HTC] Ack[+HTC])+L-sig(#922));

# CID 61

| 61 | 469.34 | Annex S | "burst-BAR BlockAck ¦" As the "burst-BAR" can contain +HTC, BlockAck can also have +HTC. Also if the "burst-BAR" is BlockAckReq, then the response will be Ack for delayed BlockAck case. | Change the cited part to "burst-BAR (BlockAck¦Ack) [+HTC] ¦" |
| --- | --- | --- | --- | --- |

### Proposed Resolution: counter

The comment and proposal are correct.  The same issue needs to be addressed in the initiator-sequence-BA and HT-ack-sequence terms.

Change “initiator-sequence” as follows (D2.06):

initiator-sequence = (* No BA expected, no RD granted *)

burst

(* BlockAckReq (BAR) delivered, BA expected. No RD *)

(burst-bar (BlockAck|Ack) [+HTC]) |

(* No BAR delivered, RD granted *)

(burst-rd (

burst |

burst-bar initiator-sequence-ba

)

) |

(burst-rd-bar (BlockAck|Ack) [+HTC]) |

(burst-rd-bar (

burst-ba |

burst-ba-bar initiator-sequence-ba

)

) |

ht-ack-sequence |

psmp-burst (#123) |

link-adaptation-exchange ;

Change “initiator-sequence-ba” with the same change as shown above for “initiator-sequence”.

Change “ht-ack-sequence” as follows:

(* These are sequences that occur within an ht-txop-sequence that have an ack response *)

ht-ack-sequence = (BlockAck+delayed[+HTC] Ack[+HTC] ) |

(BlockAckReq+delayed[+HTC] Ack[+HTC]) |

(Data[+HTC]+individual[+null][+QoS+normal-ack] Ack[+HTC]);

# CID 62

| 62 | 471.04 | Annex S | "Data[+null][+HTC]+QoS+(no-ack¦block-ack) ¦" Is QoS Null frame with BlockAck policy allowed? Don't think so. | Change the cited part to "Data[+null][+HTC]+QoS+no-ack ¦". |
| --- | --- | --- | --- | --- |

Comment:  the comment is correct, the resolution is not.

QoS Null is explicitly required to have “normal ack”.

In 7.1.3.5.3: “For QoS  Null (no data) frames, this is the only permissible value for the Ack Policy subfield. “

However, that is specific to a non-AMPDU frame.   This term is used both in non-AMPDU and AMPDU contexts.

Question:  do we allow a QoS Null in an A-MPDU,  and, if so,  what is its Ack Policy setting?  I would argue that 7.4a.3 does not allow QoS Null in A-MPDU, and so its Ack Policy setting is moot.  Certainly [+null] should be excluded from this term.

The next issue relates to “no-ack”,  which is incompatible with a-mpdu.   To exclude the no-ack case requires “frame-not-requiring-response” to be split into A-MPDU and non-A-MPDU cases.

### Proposed resolution:  Counter

(* A PPDU not requiring a response is either a single frame not requiring response, or an A-MPDU of such

frames.*)

ppdu-not-requiring-response =

frame-not-requiring-response-non-ampdu |

1{frame-not-requiring-response-ampdu+a-mpdu}+a-mpdu-end;

(* A frame-not-requiring-response-non-ampdu  is a frame that does not require a response and that may be sent outside A-MPDU.  It includes those frames that do not require a response that are not allowed within an A-MPDU. *)

frame-not-requiring-response-non-ampdu =

Data[+HTC]+QoS+no-ack  |

frame-not-requiring-response-ampdu

(* A frame-not-requiring-response-ampdu  is a frame that does not require a response and can be sent within an A-MPDU. It is one of the delayed BA policy frames sent under ‘no ack’ Ack Policy, or

Data that doesn’t require an immediate ack, or a Management Action No Ack frame. A frame-not-requiring-response-ampdu may be included with any of the following sequences in any position, except the initial position

when this contains a BlockAck or Multi-TID BlockAck: ppdu-bar, ppdu-ba-bar, ppdu-ba, ppdu-rd, ppdu-rdbar,

ppdu-ba-rd-bar, psmp-ppdu *)

frame-not-requiring-response-ampdu =

BlockAck[+HTC]+delayed-no-ack |

BlockAckReq[+HTC]+delayed-no-ack |

Data[+HTC]+QoS+block-ack |
	ma-no-ack-htc;

# CID 60

| 60 | 469.14 | Annex S | "Data[+HTC]+individual[+null][+QoS+(normal-ack¦block-ack)] ¦" QoS+normal-ack is already covered in the previous case and if it is QoS+normal-ack, then Ack response is required. | Change the cited part to "Data[+HTC]+individual[+null][+QoS+block-ack] ¦". |
| --- | --- | --- | --- | --- |

Comment: The comment is correct.  The resolution is not correct and identifies an error in the nav-set term,  which is corrected in the edits shown for CID 129.  The [+null] is removed because QoS Null must have a “normal ack” ack policy.

### Proposed Resolution:  Counter