IEEE P802.11
Wireless LANs

| Resolution proposal to some PSMP CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Naveen Kakani | Nokia | 6021 Connection Drive, Irving, TX, USA | +1 214 763-1041 | naveen.kakani@nokia.com |
| Adrian P. Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This document describes corrections to be inserted to the TGn draft 2.0 that will address comments from the LB97 regarding PSMP feature at the MAC layer.

The text here is aligned with P802.11-REVma/D9.0 and P802.11n-D2.0

R2 contains changes discussed in the PSMP ad-hoc meeting on 2007-07-11.

R

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# CID’s addressed :

| CID | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 960 | 3 | circular definition. PSMP is being defined in terms of PSMP participants | write a new non-circular definition | Counter : Text as shown in doc 1-07/0730r2 PSMP: 2007-07-11 unanimous |
| 1957 | 9.15 | PSMP has not been shown to greatly improve either power consumption or capacity with respect to scheduled APSD. | Remove all forms of PSMP or provide evidence that PSMP is vastly superior to S-APSD in order to justify its incredible complexity. | Reject : S-PSMP is expected to provide benefit of statistical multiplexing (i.e., the resource allocated to provide retries is shared between the STAs that are part of the PSMP sequence, rather than being allocated to each STA, even if it is unused most of the time.). PSMP 2007-07-11: unanimous |
| 1958 | 9.15 | In practice, not all QoS-loving power-saving WLAN devices will run CODECs which operate with the same service interval (or coding rate). PSMP sets the service granularity based on the first device to set up a PSMP session. If this device is in the minority w.r.t. service intervals of the population of potential users, the majority do not get to reap the supposed benefits of using PSMP. | Remove all forms of PSMP or determine a way to gracefully solve the incompatible service interval issue. | Counter : The service interval granularity signals the current minimum service interval. But it doesn’t imply that the service interval cannot be changed to a lower value when there is a request for any S-PSMP session. Please see attached note in document 11-07/0730r2 PSMP: unanimous |
| 1959 | 9.15 | PSMP is an immensly complicated feature and is, therefore, prone to implementation error/differences. Testing of PSMP to ensure interoperabilty of different implementations will be nearly impossible. The 802.11 specification is there to ensure that if the spec is followed that different devices are guaranteed to be interoperable. By adding this feature to the 802.11n ammendment we are reducing the quality of the spec as I see no way that what is specified for PSMP will guarantee interoperable implementations. | Remove PSMP in all its forms from the 802.11n ammendment. Especially from the MAC perspective, the complexity of the ammendment will be greatly reduced by not having to account for so many corner cases. | Counter : There is explicit signaling provided to indicate if there is support for PSMP and S-PSMP. Text as shown in document 11-07/0730r2 |
| 1868 | 11.2.1.12 | Similar to the text "The STA shall be awake to receive at the start of the PSMP-DTT determined from a STA_INFO that has the STA_INFO Type field set to 2 and the AID field matching the STA’s AID where the PSMP-DTT Duration is not set to 0." we also need to add required text associated with Broadcast and Multicast PPDUs sent to the STA. | Please add. | Counter : Text as shown in document 11-07/0730r2 This adds a note that indicates that multicast is transmitted only when the existing power-save rules allow it. Support for broadcast within has been removed. PSMP: Unanimous |
| 2859 | 11.2.1.1.2 | It is not clear what is the Power Management state of the PSMP supporting station. Is it in PS mode or not? Does such a station to be awake to get DTIM or not? Is this station able to understand TIM? Answer to these question is important in relation to PSMP station's ability to get management messages and groupcast frames. There is no need for special carry of groupcasting in PSMP if the PSMP station can get it in regular way. | Define the Power management state of the PSMP station in relation to Power management in an infrastructure network. | Counter : PSMP is an aggregation mechanism. It can be used at the DTIM period to transmit Multicast data. If a STA is PSMP capable and if a PSMP frame is transmitted at the DTIM period which can signal all the Multicast data that would be transmitted then PSMP capable STAs can choose to be awake to receve Multicast data that they are interested in. Text as shown in doc 11-07/0730r2 inserts a note to clarify this. Also remove the use of broadcast within PSMP, as this adds no benefit. PSMP 2007-07-11: unanimous |

| 823 | 7.3.1.28 | The STA_INFO parsing is not as compliated as it is hard to read. Additionally figure n20 has STA_INFO set to 1 when the text says 0. Figure N19 is not addressed at all in the text | Change figures to STA Info with container bytes. Then have the container bytes described later. Use 7.3.1.11 Action field as an example where the category is the only thing described. IN this case it would be STA_INFO. Rewrite the text such that it makes sense. | Counter: changes made by 11-07/0730r2 simplify the mechanism by removing the broadcast STA-INFO type. The group believes that the description is unambiguous and correct in D2.04. The cross-reference has been corrected by resolution to editorial comments in D2.02. PSMP: Unanimous |
| --- | --- | --- | --- | --- |
| 1516 | 7.3.2.50 | There is a field of "Service Interval Granularity", and definition says "Used for scheduled PSMP only". So, how to encode this field if scheduled PSMP is NOT used, for example, 'PSMP STAs Only' field is 0, and PSMP incapable devices is associated ? | Add encoding rule for this field when scheduled PSMP is not used. I guess it should be Reserved, when 'PSMP STAs Only' field is 0. | Counter : Change Table n30, page 73, rows 21 – 29 When PSMP Support field is 1 and S-PSMP support field is 1 the service interval granularity field is encoded as follows 0 = 5 1 = 10 2 = 15 3 = 20 4 = 25 5 = 30 6 = 35 7 = 40 When either of PSMP Support field or S-PSMP support field are 0 the Service Interval Granularity field is Reserved Use B4 of HT-Information Sub-element to signal support for S-PSMP. Text as shown in doc 11-07/0730r2 PSMP: Unanimous |
| 1934 | 7.3.2.50 | Enabling the PSMP STAs Only bit gives an unfair advantage to HT STAs over legacy STAs in that the legacy guys aren't aware that their association attempt will be rejected. | Either remove this bit (set to reserved = 0) and replace it with a unique PSMP-only status code, or at least require that an AP setting this bit be required to also disallow association from all non-PSMP STAs, both HT and legacy. | Counter : The following changes delete the bit and any associated text. Editor : Delete Subclause 9.15.1, Remove PSMP STAs Only field. This is effected by 11-07/0730r2 (which reuses this bit position for another purpose). PSMP: Unanimous |
| 1095 | 7.4.9.5 | normative statements don't belong in clause 7 | move this paragraph to an appropriate place in the standard | Counter : Moved the quoted text to 9.15.2.1. Text as shown in document 11-07/0730r2 PSMP: Unanimous |
| 845 | 9.15.4 | There is very low probability that substantial number of clients will be awake in the same time to be ready to get the PSMP frame. So the unscheduled PSMP does not bring any improvement in relation to UAPSD | Exclude the unscheduled PSMP mechanism and remove this entire subclause | Reject: Unscheduled PSMP is an aggregation mechanism that can be used to aggregate to/from PSMP capable STA when the AP knows that they are awake. AP knows that these STAs are awake either by a trigger (U-APSD), scheduled SP (S-APSD), PS-Poll, DTIM period. PSMP: Unanimous |

TGn Editor: Modify the definition of PSMP  in section Definitions  (page 3, line 47) as shown below

(CID 960) 3.n41 power save multi-poll (PSMP): A mechanism that provides a time schedule that is  used by an AP

and its STAs to access the medium. The mechanism is controlled using the PSMP management action frame.

TGn Editor: Make the following changes to text in section 7.3.1.28 in TGn Draft 2.02 as shown below  (page 41, line 56). ).  The changes include deletion of Figure 53k.

7.3.1.28 PSMP STA Info field

The PSMP STA Info field is used by the PSMP frame (see 7.4.9.5 (PSMP frame format). The PSMP STA

Info field defines the allocation of time to the downlink (PSMP-DTT) and/or uplink (PSMP-UTT) associated

with a single receiver address. There are (CID 2859)two variants of the structure for the individually addressed,

(CID 2859)and multicast cases. The length of the PSMP STA Info field is 8 octets.

The structure of the STA Info field is defined in Figure n18 (PSMP STA Info fixed field (individually ad-

dressed)) (CID 2859)and Figure n20 (PSMP STA Info fixed field (multicast)).

(CID 2859)When STA_INFO Type is set to 0, the PSMP STA Info field is structured as defined in Figure n20 (PSMP

STA Info fixed field (multicast)), and relates to the transmission of broadcast data by the AP. The STA_INFO

Type field indicates the format of the remainder of the structure. When STA_INFO Type is set to 1, the PSMP

STA Info field is structured as defined in Figure n20 (PSMP STA Info fixed field (multicast)), and relates to

the transmission of multicast data by the AP. When STA_INFO Type is set to 2, the PSMP STA Info field is

structured as defined in Figure n18 (PSMP STA Info fixed field (individually addressed)) related to the exchange

of data with a single STA.

The PSMP-DTT Start Offset field indicates the start of the PPDU that has the DL data of the STA. The offset

is specified relative to the end of the PSMP frame. It is given as an integer number of 4 μs.

The PSMP-DTT Duration field indicates the end of DL data of a STA relative to the start of the PPDU that

contains the first frame destined to the STA. It contains a duration in units of 16 μs.

NOTE—The PSMP-DTT Duration field granularity (16 μs) affects power saving, while the PSMP-UTT granularity (4

μs) affects the utilization of the wireless medium, so it is appropriate to represent the latter more precisely.

If no PSMP-DTT is scheduled for a STA, but a PSMP-UTT is scheduled for that STA, then the PSMP-DTT

Duration is set to 0 and the PSMP-DTT Start Offset is reserved.

The STA_ID field contains the AID of the STA to which the PSMP STA Info field is directed.

The PSMP Multicast ID (B21 to B63) field contains the least significant 43 bits of a 48 bit MAC address. Use

of this field is described in 9.15.2.8 (PSMP (CID 2859)broadcast and multicast rules). B63 contains the least significant

bit of the multicast address.The PSMP-UTT Start Offset field indicates the start of the PSMP-UTT. The first

PSMP-UTT is scheduled to begin after a SIFS interval from the end of the last PSMP-DTT described in the

PSMP. The offset is specified relative to the end of the PSMP frame. It is specified in units of 4 μs.

The PSMP-UTT Duration field indicates the maximum length of a PSMP-UTT for a STA. PSMP-UTT Du-

TGn Editor: Make the following changes to Figure n36 in subclause 7.3.2.53 in TGn Draft 2.02 as shown below  (page 72, line 14-19)

7.3.2.53 HT Information element

|  |  |  |  | B0 B1 | B2 | B3 | B4 | B5 B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Primary Channel | Secondary Channel Offset | STA Channel Width | RIFS Mode | PSMP STAs Only S-PSMP Support | Service Interval Granularity |

--

---

Figure 112ai—HT Information element format

TGn Editor: Make the following changes to Table43I  in subclause 7.3.2.53 in TGn Draft 2.02 as shown below  (page 73, line 24-29)

Table 43l—HT Information element

| (CID 1516)PSMP STAs Only (CID 1934, 1959)S-PSMP Support | Indicates whether an AP accepts Association requests only from PSMP capable STAs Signal support for Scheduled PSMP | Set to 0 if Association requests are accepted regardless of PSMP capability Set to 1 if Association requests are accepted from only PSMP capable STA When PSMP Support = 0, S-PSMP support is set to 0 When PSMP support = 1, the S-PSMP Support field is: - Set to 0 if AP does not support S-PSMP - Set to 1 is AP supports S-PSMP |
| --- | --- | --- |
| Service Interval Granularity | (CID 1516)Duration of the shortest Service Interval. This field is defined when S-PSMP Support is set to 1, otherwise it is reserved. | Set to 0 for 5 ms Set to 1 for 10 ms Set to 2 for 15 ms Set to 3 for 20 ms Set to 4 for 25 ms Set to 5 for 30 ms Set to 6 for 35 ms Set to 7 for 40 ms |

TGn Editor: Make the following changes to text in section 7.4.8.4 in TGn Draft 2.02 as shown below  (page 79, line 81)

7.4.9.5 PSMP frame format

PSMP (Power Save Multi-Poll) is a management action frame of category HT.

The DA field of this frame is the broadcast address or a multicast address. (See 9.15.2.8 (PSMP broadcast and(CID 2859) multicast rules)).

The PSMP Parameter Set is used to describe the PSMP-DTT and PSMP-UTT that immediately follows the

PSMP frame.

------

The Action field is set to 2 (representing PSMP).

(CID 1095)In a PSMP frame, the STA_ID fields of all its STA Info fields with STA_INFO Type set to 2 shall be unique, i.e., each STA identified in the PSMP frame is identified exactly once.The PSMP STA Info fields within a

PSMP frame are ordered by STA_INFO Type as follows: broadcast (STA_INFO Type=0), (CID 2859)multicast

(STA_INFO Type=1) and then individually addressed (STA_INFO Type=2).

---

TGn Editor: Make the following changes to text in section .9.15.1  in TGn Draft 2.02 as shown below  (page 134, line 42)

(CID 1934) 9.15.1 Signaling (#1283) support for PSMP STAs Only

The AP may support a BSS containing only STAs that are PSMP capable. This approach may allow separation

of multimedia and data-oriented BSSs. An AP that intends to allow the association of only PSMP capable

STAs (#1857) shall set the PSMP STAs Only field in the HT Information element to 1. A PSMP aware

STA may decide to associate or not with a BSS based on the value of the PSMP STAs Only field, thereby

possibly shortening its scanning time (#2304).

TGn Editor: Make  the following changes to text in section 9.15.2.1 in TGn Draft 2.02  (page 134, line 2)

9.15.2.1 PSMP frame transmission (PSMP-DTT and PSMP-UTT)

A PSMP sequence (#2308) may be used to transmit Broadcast/Multicast frames along with unicast frames.

Unicast frames shall be scheduled after Broadcast/Multicast frames.

(CID 1095) In a PSMP frame, the STA_ID fields of all its STA Info fields with STA_INFO Type set to 2 (unicast) shall be unique, i.e., each STA identified in the PSMP frame is identified exactly once.

Unicast entries in the PSMP frame shoul….

TGn Editor: Make the following changes in TGn Draft 2.02  (page 140, line 44)

9.15.2.8 PSMP (CID 2859)broadcast and multicast rules

Insert the following new subclause:

9.15.2.8.1 Rules at the AP

This subclause defines rules that shall be followed by a PSMP Capable AP for the transmission of multicast

(CID 2859)and broadcast data frames (data and management) during a PSMP sequence.

(CID 2859)A STA_INFO record with STA_INFO Type set to 0 (Broadcast) may contain Data frames with the Address1

field set to any group address. A PSMP frame shall not contain more than one of these records.

Each separate multicast group address for which (CID 2859)frames are transmitted during a PSMP sequence shall (CID 2859)either:

— not have its own STA_INFO record present in the PSMP frame and transmit Data frames with the

matching Address1 field only during the PSMP-DTT associated with the STA_INFO record with

STA_INFO Type set to 0 (Broadcast), or

— have a single STA_INFO record with STA_INFO Type set to 1 (Multicast) present in the PSMP

frame and transmit (CID 2859)Data frames with the matching Address1 field only during the PSMP-DTT indicated

in this record or during the PSMP-DTT corresponding to a STA_INFO record with

STA_INFO Type set to 0, when present.

The DA of the PSMP shall be set to the broadcast address; except if the PSMP contains only a single non-null

PSMP-DTT and this PSMP-DTT contains frames for a multicast address, in which case the DA of the PSMP

frame may be set to this multicast address.

(CID 1868) NOTE – the transmission of a multicast frame within a PSMP sequence does not change the rules regarding when that frame can be transmitted.   This means that,  if there is a power-saving STA in the BSS,  the multicast frame is transmitted following a DTIM beacon according to the rules in 11.2.1.

Insert the following new subclause:

9.15.2.8.2 Rules at the STA

This subclause defines rules that shall be followed by a PSMP Capable STA for the reception of multicast

(CID 2859)frames during a PSMP sequence.

The STA shall be awake to receive during (CID 2859)any PSMP-DTT identified by broadcast STA_INFO record and all PSMP-DTTs identified by a multicast STA_INFO record where the PSMP Multicast ID field matches the

least significant bits of any address within its dot11GroupAddressesTable.

TGn Editor: Add the following Note in TGn Draft 2.02 Subclause 9.15.3  (line 35, page 141) 9.15.3 as indicated along with instructions to the 802.11 base standard editor.

9.15.3 Schedule PSMP

A PSMP capable STA may request the creation of a PSMP Session by transmitting a TSPEC in which the

APSD field is set to 0, the Schedule field is set to 1 and the Aggregate field is set to 1, indicating Scheduled

PSMP (as described in 11.4.4b (PSMP Management)). (CID 1934)A STA shall transmit this request to an AP only if the AP has “S-PSMP Support” field in the HT-Information element set to 1.

An AP shall not respond back to a TSPEC request in which the APSD and Schedule subfields indicate Scheduled

PSMP if the TSPEC request did not indicate Scheduled PSMP. An AP can respond back to a TSPEC

request that indicates Scheduled PSMP with a response indicating either Scheduled PSMP or S-APSD.

A response to a TSPEC request containing the Schedule and APSD subfields set to the value Scheduled

PSMP creates a PSMP session, which exists for the lifetime of the TS.

While one or more PSMP sessions exist with the same SP, the AP shall periodically initiate a PSMP sequence by transmitting a PSMP frame using the service period indicated to the STA in response to the received

TSPEC. The AP shall not schedule a PSMP-DTT or a PSMP-UTT for a STA outside its service period.

(CID 1957, 1958,) Note: An AP can simultaneously maintain multiple PSMP sessions with distinct Service Intervals. The Service Intervals of an AP’s PSMP sessions are multiples of the Service Interval granularity. It is possible that an AP can combine the schedule of multiple PSMP sessions into a single PSMP frame if the start times of the PSMP sessions coincide. For example, the schedule carried by a PSMP frame related to a PSMP session at 20 msec and 30 msec service intervals can be combined into a single PSMP frame once every 3 service intervals of PSMP session at 20 msec or once every 2 service intervals of the PSMP session at 30 msec.

TGn Editor: Make the following changes in TGn Draft 2.02. Subclause 11.2.1.12 (page 183) as indicated

11.2.1.12 PSMP Power management

(CID 1868) An AP transmits a PSMP frame containing a  schedule only for  STAs that are awake. A STA in Power Save mode is awake as defined in section 11.2.1.4 (U-APSD, S-APSD), 11.2.1.5 (PS-poll) or at DTIM period(CID 2859, 845)

The AP shall signal the end of the Service Period for all STA by setting the More PSMP field to 0, or by sending

CF-End instead of the next PSMP frame.

NOTE1—The AP can also signal the end of a service period on a per-STA basis using the EOSP field set to 1 in the QoS

Control field, as defined in 7.1.3.5.2 and 11.2.1.5. This field remains set to 1 for any retransmissions of the same frame

and no more new frames are sent to this particular STA in the current SP.

NOTE2—(CID 1868)If the STA is awake at the start of S-PSMP session, the operation of the More Data field in the Frame Control field and the TIM element are defined by the SAPSD rules in 11.2.1.4, 11.2.1.5 and 11.2.1.9.

A STA shall wake up at start of the next PSMP frame if the More PSMP field is set to 1 in the current PSMP

frame, unless the STA has been permitted to return to sleep through the reception of a frame addressed to it

with the EOSP field set to 1 or the maximum SP interval has elapsed.

A PPDU containing MPDUs addressed to a STA shall not start after expiry of the STA's PSMP-DTT. A STA

completes the reception of any PPDU that starts before the end of the PSMP-DTT. If no frames addressed to

a STA begin within a PSMP-DTT, it can assume that no frame addressed to it will arrive during this PSMP

sequence.

The STA shall be awake to receive at the start of the PSMP-DTT determined from a STA_INFO that has the

STA_INFO Type field set to 2 and the AID field matching the STA’s AID where the PSMP-DTT Duration

is not set to 0.