IEEE P802.11
Wireless LANs

| Proposed update text for QoS Map distribution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hong Cheng | Panasonic Singapore Labs | BLK1022 TaiSeng Ave #06-3530 Singapore 534415 | +65-65505477 | Hong.cheng@sg.panasonic.com |
| Dave Stephenson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA 95134 | +1-408-527-7991 | daves@cisco.com |

Abstract

This is the proposed text for clause 11.10.3 of IEEE802.11u Draft D0.04 to address the action point from the April Ad Hoc meeting.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Introduction:

Following are the proposed update text for clause 11.10.3 regarding the QoS Map Set distribution procedures. It is based on the IEEE802.11u D0.04 draft.

    - Interworking Procedures: User Plane Support for QoS Mapping from External Networks

Maintaining proper end-to-end QoS is an important factor when providing interworking services. This is because the interworked networks may employ different network-layer (layer 3) QoS practices.  For example, the use of a particular differentiated services code point (DSCP) for a given service may be different between different networks.  To ensure the proper QoS over-the-air in the IEEE802.11 AN=, the mapping from DSCP to UP for the corresponding network needs to be identified and made known to the non-AP STA. If an inconsistent mapping is used then:

- Admission control at the AP may incorrectly reject a service request, because the non-AP STA used the incorrect UP.

- The user may be given a different QoS over the IEEE 802.11 AN than expected, e.g. a lower QoS may be provided by the IEEE 802.11 AN than the STA expected.

Therefore, a non-AP STA should request the QoS Map from an AP prior to transmitting frames at other than best-effort user priority (UP = 0).  The non-AP STA requests the QoS Map by transmitting a QoS Map Request action frame to the AP to which it’s associated.  In response, the AP transmits the QoS Map Set element in a QoS Map Configure action frame. Upon receiving the QoS Map Set element, the non-AP STA is then capable of deciding the proper user priority and in turn the AC to use over-the-air for a particular service. The AP shall ensure the correctness of the QoS mapping information at the non-AP STA associated to itself. Therefore, when the AP detects a change in the QoS mapping information, it shall update the non-AP STA using a new QoS Map Set element.

      - QoS Map Distribution Operation

The QoS Map Distribution operation described below provides a method for synchronize the QoS mapping at the STA and AP.

After the non-AP STA finished the RSNA security procedures, its SME shall generate a MLME-QoSMap.request primitive to the MLME. The MLME of the non-AP STA shall send a QoS Map Request action frame to the AP.

When the MAC entity at the AP receives a QoS Map Request action frame from a non-AP QSTA, the MLME shall generate a MLME-QoSMap.indication primitive to its SME.

When SME of the AP receives a MLME-QoSMap.indication, it shall generate a MLEM-QoSMap.response primitive. The SME of the AP can also decide when to generate an unsolicited MLME-QoSMap.response primitive to the MLME based on its configuration status.

When the MLME of the AP receives a MLME-QoSMap.response, it shall send a QoS Map Configure action frame to the corresponding non-AP STA MAC entity.

      - Transport of QoS Map Set

Typically, the QoS Map Set is configured on an AP for each external network to which the BSS is interworked.  Most of the time, the QoS Map Set remains static. In this case, the QoS Map Set shall be unicated to the requesting non-AP STA using the QoS Map Configure action frame.

However, it is possible that the QoS Map Set may need to be changed during operation of the BSS.  If the QoS map set corresponding to a particular SSID is changed, the AP shall unicast the new QoS map set using an unsolicited QoS Map Configure action frame to each non-AP STA associated to that SSID. Alternatively, the AP may broadcast an unsolicited QoS Map Configure action frame to the non-AP STAs in the BSS.  The AP shall not transmit a QoS map configure action frame using any multicast address.

References: