IEEE P802.11
Wireless LANs

| Network Type Selection Editorial Fixes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trpz.com |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503-264-8014 | necati.canpolat@intel.com |
| Vivek Gupta | Intel Corp | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 712 1754 | vivek.g.gupta@intel.com |
| Dave Stephenson | Cisco Systems | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Hong Cheng | Panasonic Singapore Laboratories | BLK1022 TaiSeng Ave #06-3530 TaiSeng Ind. Est. Singapore 534415 | +65 65505477 | hong.cheng@sg.panasonic.com |
| Amy Zhang | Huawei Technologies | Bantian, Longgang District Shenzhen 518129,P.R.China | +86 755 896 50361 | amyzhang@huawei.com |
| Colin Blanchard | BT Group | MLB1 PP8, BT Adastral Park Ipswich, Suffolk UK IP5 3RE | +44 1473 605353 | Colin.Blanchard@bt.com |
| Lars Falk | TeliaSonera | SE-20521 Malmö, Sweden | +46-70-5945555 | Lars.p.falk@teliasonera.com |
| Christian Kuhtz | EarthLink | 1375 Peachtree St, Level A, Atlanta, GA 30309 | 404 840 7684 | kuhtzch@corp.earthlink.net |
| Srinivas Sreemanthula | Nokia | 6000 Connection Dr, Irving, TX | +1 972 894 5000 | srinivas.sreemanthula@nokia.com |
| Angelo Centonza | Roke Manor Research | Old Salisbury Lane Romsey, Hampshire SO51 0ZN, UK | +44 1794 833185 | angelo.centonza@roke.co.uk |
| George Bumiller | Research in Motion | 295 Phillip Street Waterloo, Ontario Canada N2L 3W8 | +1 201 327 1136 | g.bumiller@ieee.org |
| Manoj Deshpande | Qualcomm | 5775 Morehouse Drive San Diego, CA 92121 | +1 858 651-3717 | manojd@qualcomm.com |
| Yongho Seok | LG Electronics |  |  | yhseok@lge.com |

Abstract

This document addresses editorial concerns in 11-07/0573r2.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

After approval, several editoral notes were submitted against the network type selection proposal from 11-07/0573r2.  This document addresses the outstanding comments.

# Editing Instructions

- Normative references

Insert the following references alphabetically:

Wi-Fi Alliance, "Best Current Practices for Wireless Internet Service Provider (WISP) Roaming", version 1.0.  B. Anton, B. Bullock, and J. Short, February 2003.

- Definitions

Insert the following new definition alphabetically, renumbering as necessary:

UAM	Universal Access Method

      - Probe Response frame format

Change row 25 so that the HESSID is present if the MIB variable is true:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 25 | HESSID | HESSID is be present if dot11InterworkingEnabled is true |

Make the indicated changes to the following clause, renumbering figures where appropriate:

      - HESSID Information element

The Homogenous Extended Service Set ID (ESSID) Information element contains description of HESSID. The format of the HESSID element is shown in Figure u1.

|  | Element ID | Length | HESSID | Network Type |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |

Figure u1—HESSID Information element format

The Element ID field is equal to the HESSID value in Table 26.

The value of the Length field is 7.

The HESSID field specifies the definition of HESSID.

In the infrastructure mode, the HESSID definition includes the BSSID value of one of the group of APs. The HESSID and the SSID together to provide is a unique value that is advertised in beacons and probe responses so that the non-AP STA is aware of continued applicability of previously discovered interworking and advertising services when moving from one AP to another within the scope of the HESSID.

The Network Type field is defined in Figure u13a.  This field is used to advertise the type of network for every SSID included in the HESSID set.

|  | b0 | b2 | b3 | b4 | b5 | b6 | b7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Network Type Code |  | NASR | Intranet/ Internet | Advertisement Policy |  | Reserved |
| Bits: | 3 |  | 1 | 1 | 2 |  | 1 |

Figure u13a—HESSID Network Type Field

Bits 0-2 are the Network Type Code bits.  These bits are set with the value from Table u1a.

Table u1a—Network Type Code bit assignments

| Network Type Code Bits (b0-b1-b2) | Meaning | Description |
| --- | --- | --- |
| 000 | Private network | Network requires user accounts for access |
| 001 | Private network with guest access | Network requires user accounts for access, but guest accounts area available |
| 010 | Chargeable | Access to the network requires payment. Further information on types of charges may be available through other methods (802.21, UAM, etc). |
| 011 | Free | No charges apply for network use. Bits 4-6 are also used in this case. |
| 100 | Emergency network | The network supports emergency services. Further information is available from a GAS native query (see 7.3.2.53) |
| 101-111 | Reserved | Not used. |

Bit 3 is the Next Authentication Step Required (NASR) bit.  This bit is set to one to indicate that the network requires a further authentication step, as defined in 7.3.2.54.

Bit 4 is the Internet/Intranet bit.  This bit is only used when the Network Type Code in Table u1a is set to 011.  It is set to 1 to indicate that the network provides Internet access, and 0 to indicate the network provides access only to local networks.  If the Network Type Code is not 011, these bits is set to 0.

Bits 5 and 6 are the Advertisement Policy field, which has values set according to Table u1b.  These bits are only used when the Network Type Code in Table u1a is set to 011  If the Network Type Code is not 011, these bits is set to 0.

Table u1b—Advertisement Policy bit assignments

| Advertisement Policy bits (b5 – b6) | Meaning | Description |
| --- | --- | --- |
| 00 | No Advertisements | The network does not require the end user to view commercial advertisements |
| 01 | Advertisements present | The network requires end users to view advertisements to support the service |
| 10 | Mixed advertisement policy | The network may require end users to view advertisements, but only for certain services. |
| 11 | Reserved | Not used. |

Bit 7 is reserved and is set to zero.

References:

IEEE P802.11u™/D0.04 -  Draft P802.11u-D0.04-adhoc review-2007-04-25.doc

IEEE 802.11 document # 11-07/0573r2 – Network Type Selection, Matthew Gast, et. al.