IEEE P802.11
Wireless LANs

| LB97 Submission to resolve maximum L_LENGTH comment CID 332 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Douglas Chan | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1-408-527-9344 | dougchan@cisco.com |
| Brian Hart | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1-408-525-3346 | brianh@cisco.com |
| Brett Douglas | Cisco Systems, Inc. | 301 S. Prospect Rd, Suite 4, Bloomington, IL 61704 | +1-309-661-7656 | brettld@cisco.com |
| Luke Qian | Cisco Systems, Inc. | 4125 Highlander Parkway, Richfield, OH 44286 | +1-330-523-2051 | lchia@cisco.com |

Abstract

This document suggests resolution for LB97 comment, CID 332.  We suggest a discretionary note (some informative text) for implementers on the value of L_LENGTH.  We also found duplicate text in the draft that specifies the same condition for L_LENGTH and so we provide editorial instructions to address this accordingly.  The changes are based upon the TGn D2.02 draft.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This document describes a countered resolution for LB97 comment 332 on the maximum L_LENGTH value.  We also found duplicate text in the draft that specifies the same condition for L_LENGTH and so we provide editorial instructions to address this accordingly.

| CID | Comment | Proposed Change |
| --- | --- | --- |
| 332 | A maximum L_LENGTH of 4095 octets can led to the medium being consumed for too long a time and affect performances of applications, especially those that are time-critical, like voice-related services. | Change the maximum of L_LENGTH to 2400 octets or a reasonable value in this range. |

Discussion:

The maximum L_LENGTH is determined by the number of bits in the L-SIG allocated for it and should not be changed.  Moreover, L_LENGTH is limited by the TXOP Limit, which has a maximum of 8160 µs that should not cause significant performance degradations.

However, as stated in Fig. 18 of Clause 7.1.2 General frame format of 802.11ma, the maximum length of an unencrypted 802.11 frame is only 2340 octets.  Thus there may be devices with mechanisms in place that would refuse transmissions with L_LENGTH larger than 2340.  Therefore, we should provide a discretionary note to implementers of this.

Suggestion:

Counter comment with changes described in document 07/0776r1.

TGn Editor: Add to the last paragraph of sub clause 9.13.4 “Setting the L-LENGTH and L-RATE parameters for HT_MF PPDUs” at about line 14 on page 126 in TGn draft D2.02 with the underlined text:

The maximum value of L_LENGTH shall be 4095.

Note – The transmission of frames with L_LENGTH above 2340 octets should be accompanied by a fall back mechanism using a reduced L_LENGTH if it is determined that the mechanism fails to effectively suppress non-HT transmissions.  How this is determined is outside the scope of this standard.

TGn Editor: Delete the last paragraph of sub clause 9.13.5.1 “General rules” at about line 28 on page 128 in TGn draft D2.02:

The maximum value of L_LENGTH shall be 4095.