IEEE P802.11
Wireless LANs

| Mesh Frame Formats |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd, Lake Mary, FL 32746 | +1-407-562-4050 | Guenael.Strutt@motorola.com |
| Jan Kruys | Cisco Systems | Cisco Way Bld 14 San Jose, CA | + 31 348 453719 | jkruys@cisco.com |
| W. Steven Conner | Intel | 2111 NE 25th Ave, Hillsboro, OR | 503-264-8036 | w.steven.conner@intel.com |

Abstract

Resolution of LB93 comments related to frame formats in Draft D1.03

This contribution  is based on the frame formats presented earlier in this session (11-07/550r2)

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Note: All instructions to the TGs editor are shown in purple and are based on IEEE 802.11s Draft D1.03.  Changes to text in D1.03 are shown with insertions in blue and deletions in red.

#1 - Change the last sentence of the first paragraph of 7.1.2 in D1.03 as shown:

7.1.2 General frame format

The format of mesh management frames of subtype Mesh Multihop Action is defined in Error: Reference source not found.

#2 - Change the contents of Table 1 in D1.03 as shown:

- Type and subtype fields

| - Valid type and subtype combinations |  |  |  |
| --- | --- | --- | --- |
| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| 00 | Management | 1111 | Multihop Action Reserved |
| 11 | Extended | 0000 | Mesh Data |
| 11 | Extended | 0001 | Mesh Management |
| 11 | Extended | 0010-1111 | Reserved |

#3 - Change the contents of To/From DS combinations in data frames in D1.03 as shown:

- To DS and From DS fields

| - To/From DS combinations in data frames |  |
| --- | --- |
| To DS and From DS values | Meaning |
| To DS = 1 From DS = 1 | A data frame using the four-address format. The presence of the Mesh Header field is indicated by setting To DS = 1 and From DS = 1 in data frames transmitted between peer MPs with an established peer link. This standard does not define procedures for using this combination of field values. |

#4 - Change the second paragraph of Clause 7.1.3.5a.1 in D1.03 as shown:

7.1.3.5a.1 General

The Mesh Header field, shown in Error: Reference source not found, is present in all data frames of type Extended with subtypes Mesh Data and Mesh Managementtransmitted between peer MPs with an established peer link.  Data frames including the Mesh Header field are referred to as Mesh Data frames.

#5 - Replace the text in 7.2.3 in  D1.03 with the following instructions and text describing changes in the baseline as shown:

- Management frames

Change the text of Clause 7.2.3 as shown:

The frame format for a management frame is independent of frame subtype and is defined in Figure 35.

| Octets: 2 | 2 | 6 | 6 | 6 | 2 | 0-2312 |  | 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Duration/ ID | Address 1 (DA) | Address 2 SA | Address 3 BSSID | Sequence Control | Frame Body |  | FCS |  |
| MAC Header |  |  |  |  |  |  |  |  |  |
| Figure 35---Management frame format |  |  |  |  |  |  |  |  |  |

A STA uses the contents of the Address 1 (DARA) field to perform the address matching for receive decisions.  In the case where the Address 1 (DARA) field contains a group address and the frame type is other than Beacon, if the STA is a member of a BSS or IBSS the Address 3 (BSSID) field also is validated to ensure that the broadcast or multicast originated from a STA in the BSS of which the receiving STA is a member.  If the frame type is Beacon, other address matching rules apply, as specified in 11.1.2.3. Frames of type Probe Request with a group address in the Address 1 field are processed as described in 11.1.3.2.1.

The address fields for all management frames do not vary by frame subtype subtypes except Multihop Action are as follows.

The Address 3 (BSSID) field of the management frame is determined as follows:

- If the STA is an AP or is associated with an AP, the BSSID is the address currently in use by the STA contained in the AP.

- If the STA is a member of an IBSS, the BSSID is the BSSID of the IBSS.

- In management frames of subtype Probe Request, the BSSID is either a specific BSSID, or the wildcard BSSID as defined in the procedures specified in 11.1.3.

- For management frames transmitted by MPs, the BSSID field is not used and should be set to 0.

The Address 1 (RA=DA) field is the destination of the frame.

The Address 2 (TA=SA) field is the address of the STA transmitting the frame.

The address fields for the management frame subtype Multihop Action are as follows.

The Address 1 (RA) field is the the receiver of the frame.

The Address 2 (TA) field is the transmitter of the frame.

The Address 3 (DA) field is the destination of the frame.

(the rest of the subclause is unchanged)

#6 - Insert the following new clause after 7.2.3.12 in D1.03:

7.2.3.13 Multihop Action Frame format

The frame body of a management frame of subtype Multihop Action contains the information shown in Table sA.

Table sA—Multihop Action frame body

| Order | Information |
| --- | --- |
| 1 | Multihop Info |
| 2 | Action |
| Last | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. |

#7 - Remove 7.2.4 Extended Frames from D1.03

#8 - Insert the following new clause after 7.3.1.17 in D1.03:

7.3.1.18 Multihop Info field

The Multihop Info field is 1 octet in length and contains capability information bits.

The format of the Multihop Info field is defined in Figure sB.

| Octets: 6 | 1 |  |
| --- | --- | --- |
| Source Address (SA) | Mesh Time To Live (TTL) |  |
| Figure sB – Multihop Info field |  |  |

The SA subfield is the address of the MAC entity that initiated the MMPDU in the frame body field.

The TTL subfield is the 8 bits in length and is used to mitigate the possibility of transient loops in a mesh network by ensuring frames that are caught in a loop are eventually discarded.

#9 – Remove 7.3.1.34 Mesh Action field from D1.03

#10 – Remove 7.4.9.11 Action Encapsulation frame format from D1.03

#11 – Make the following edits to 7.4a Mesh Action (including its subclauses) in D1.03

- Replace all instances of the term “Mesh Action” with “Multihop Action”

- Replace the first paragraph of 7.4a with: “This subclause describes the Multihop Action frame formats, including the Action Details field, allowed in action categories defined in Table 24 in 7.3.1.11.”

- Insert a new row at the beginning of the frame body format table for each subclause of 7.4a with the following values –  Order:1, Information: Multihop Info

#12 – Make the following edits to 11A.3.4 Frame addressing and forwarding in a mesh network

- Remove all instance of “and Mesh Management” from this clause

#13 – Change 11A.3.4.5 and 11A.3.4.6 in D1.03 as shown:

11A.3.4.5 Note on 7.2.3 Management Frames

Management frames which utilize the normal 3-address management frame headers specified in 7.2.3, such as Mesh Management Action frames (3-address action frames) described in 7.4.6,  All management frames except for Multihop Action frames are transmitted only one hop to peer MPs.

Note that in several cases, the reception and processing of an 3-address action frame leads to the transmission of a new action frame of the same type that may include an identical or a modified version of the contents from the IEs of the received action frame.

11A.3.4.6.5.1 Forwarding of Multihop Action Encapsulation Frames

Multihop Action Encapsulation frames (see 7.2.3.13 7.4.9.11) are forwarded according to the procedures in Error: Reference source not found and Error: Reference source not found by using the Source MP address field value as the SA and the Destination MP address field value as the DA with respect to mesh forwarding.

Upon reception of an Action Encapsulation frame, the destination MP performs the requested action as defined in the encapsulated frame.  Similarly, if the action requires a response message, the destination MP generates such response and send it back with a non-mesh action encapsulation frame.

References:

- doc.: IEEE 802.11-07/0023r27 Resolution of comments received during IEEE 802.11 Letter Ballot 93