IEEE P802.11
Wireless LANs

| Reorganization of Action Frame Formats |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd, Lake Mary, FL 32746 | +1-407-562-4050 | Guenael.Strutt@motorola.com |
| Jan Kruys | Cisco Systems | Cisco Way Bld 14 San Jose, CA | + 31 348 453719 | jkruys@cisco.com |
| W. Steven Conner | Intel | 2111 NE 25th Ave, Hillsboro, OR | 503-264-8036 | w.steven.conner@intel.com |

Abstract

This document is related to reorganization of action frame formats and categories for the TGs draft. It establishes the basic structure of the text on action frame formats through editorial reorganization of frame formats already included in the draft.  More work is needed to address technical comments related to frame formats and to incorporate frame formats that are currently missing in the draft.  This contribution is related to the earlier presented frame format discussion (11-07/550r2).

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Note: All instructions to the TGs editor are shown in purple and are based on IEEE 802.11s Draft D1.03.  Changes to text in D1.03 are shown with insertions in blue and deletions in red.

7.3.1.11 Action field

Change the contents of Table 24 in D1.03 shown.

| - Category values |  |  |
| --- | --- | --- |
| Value | Meaning | See subclause |
| 8 | Mesh Peer Link Management | 7.4.9 |
| 9 | Mesh Link Metric | 7.4.10 |
| 10 | Mesh HWMP | 7.4.11 |
| 11 | Mesh RA-OLSR | 7.4.12 |
| 12 | Mesh Interworking | 7.4.13 |
| 13 | Mesh Resource Coordination | 7.4.14 |
| 14 | Mesh Security Architecture (MSA) | 7.4a.1 |
| 1597-126 | Reserved | --- |

Insert the following new clauses after 7.4.8:

- Mesh Peer Link Management action frame details

Action frame formats for Mesh Peer Link Management are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Table sA1.

| Table sA1 - Mesh Peer Link Management Action field values |  |  |
| --- | --- | --- |
| Action field value | Description |  |
|  | Peer Link Open |  |
|  | Peer Link Confirm |  |
| 2 | Peer Link Close |  |
| 3-255 | Reserved |  |

7.4.9.1 Peer Link Open action frame details

The Peer Link Open frame is used to open a peer link using the procedures defined in 11A.1.5.

Move the contents of 7.2.3.4 Association Request frame format shown in D1.03 to this clause, replacing the term “Association Request” with “Peer Link Open” and renumbering the order of the information elements from 17-21 to 1-5.

Delete 7.2.3.6 Reassociation Request frame format

7.4.9.2 Peer Link Confirm action frame details

The Peer Link Confirm frame is used to open a peer link using the procedures defined in 11A.1.5.

Move the contents of 7.2.3.5 Association Response frame format shown in D1.03 to this clause, replacing the term “Association Response” with “Peer Link Confirm” and renumbering the order of the information elements from 17-22 to 1-6.

Delete 7.2.3.7 Reassociation Response frame format

7.4.9.1 Peer Link Close action frame details

The Peer Link Close frame is used to open a peer link using the procedures defined in 11A.1.5.

Move the contents of 7.2.3.3 Disassociation frame format shown in D1.03 to this clause, replacing the term “Disassociation” with “Peer Link Close” and renumbering the order of the information element from 3 to 1.

7.4.10 Mesh Link Metric action frame details

Action frame formats for Mesh Peer Link Management are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Table sA2.

| Table sA2 - Mesh Link Metric action field values |  |  |
| --- | --- | --- |
| Action field value | Description |  |
|  | Link Metric Request |  |
| 1 | Link Metric Report |  |
| 2-255 | Reserved |  |

7.4.10.1 Link Metric Request frame format

Move the contents of clause 7.4.9.1 to this clause, and change “category Mesh” to “category Mesh Link Metric”.

7.4.10.2 Link Metric Report frame format

Move the contents of clause 7.4.9.2 to this clause, and change “category Mesh” to “category Mesh Link Metric”.

7.4.11 Mesh HWMP action frame details

Action frame formats for Mesh HWMP are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Table sA3.

| Table sA3 - Mesh HWMP action field values |  |  |
| --- | --- | --- |
| Action field value | Description |  |
|  | Path Request |  |
| 1 | Path Reply |  |
| 2 | Path Error |  |
| 3 | Root Announcement |  |
| 4-255 | Reserved |  |

7.4.11.1 Path Request frame format

Move the contents of clause 7.4.9.2 to this clause, and change “category Mesh” to “category Mesh HWMP”.

7.4.11.2 Path Reply frame format

Move the contents of clause 7.4.9.3 to this clause, and change “category Mesh” to “category Mesh HWMP”.

7.4.11.3 Path Error frame format

Move the contents of clause 7.4.9.4 to this clause, and change “category Mesh” to “category Mesh HWMP”.

7.4.12 Mesh RA-OLSR action frame details

Action frame formats for Mesh RA-OLSR are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Table sA4.

| Table sA4 - Mesh RA-OLSR action field values |  |  |
| --- | --- | --- |
| Action field value | Description |  |
|  | HELLO |  |
| 1 | Topology Control (TC) |  |
| 2 | Multiple Interface Declaration (MID) |  |
| 3 | Local Association Base Advertisement (LABA) |  |
| 4 | Local Association Base Checksum Advertisement |  |
| 5 | Association Base Block Request (ABBR) |  |
| 6-255 | Reserved |  |

Move the contents of clause 7.4.9.12 to this clause, and change “category Mesh” to “category Mesh RA-OLSR”.

7.4.13 Mesh Interworking action frame details

Action frame formats for Mesh Interworking are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Table sA5.

| Table sA5 - Mesh Interworking action field values |  |  |
| --- | --- | --- |
| Action field value | Description |  |
|  | Portal Announcement |  |
| 1-255 | Reserved |  |

7.4.13.1 Portal Announcement frame format

Move the contents of clause 7.4.9.13 to this clause, and change “category Mesh” to “category Mesh Interworking”.

7.4.14 Mesh Resource Coordination action frame details

Action frame formats for Mesh Resource Coordination are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Table sA6.

| Table sA6 - Mesh Resource Coordination action field values |  |  |
| --- | --- | --- |
| Action field value | Description |  |
|  | Congestion Control Request |  |
| 1 | Congestion Control Response |  |
| 2 | Neighborhood Congestion Announcement |  |
| 3 | Mesh Deterministic Access |  |
| 4 | Beacon Timing Request |  |
| 5 | Beacon Timing Response |  |
| 6 | Mesh Channel Switch Announcement |  |
| 7 | Connectivity Report |  |
| 8-255 | Reserved |  |

7.4.14.1 Congestion Control Request frame format

Move the contents of clause 7.4.9.5 to this clause, and change “category Mesh” to “category Mesh Resource Coordination”.

7.4.14.2 Congestion Control Response frame format

Move the contents of clause 7.4.9.6 to this clause, and change “category Mesh” to “category Mesh Resource Coordination”.

7.4.14.3 Neighborhood Congestion Announcement frame format

Move the contents of clause 7.4.9.7 to this clause, and change “category Mesh” to “category Mesh Resource Coordination”.

7.4.14.4 Mesh Deterministic Access frame format

Move the contents of clause 7.4.9.8 to this clause, and change “category Mesh” to “category Mesh Resource Coordination”.

7.4.14.5 Beacon Timing Request frame format

Move the contents of clause 7.4.9.9 to this clause, and change “category Mesh” to “category Mesh Resource Coordination”.

7.4.14.6 Beacon Timing Response frame format

Move the contents of clause 7.4.9.10 to this clause, and change “category Mesh” to “category Mesh Resource Coordination”.

7.4.14.7 Mesh Channel Switch Announcement frame format

Move the contents of clause 7.4.9.15 to this clause, and change “category Mesh” to “category Mesh Resource Coordination”.

7.4.14.9 Connectivity Report frame format

Move the contents of clause 7.4.9.14 to this clause, and change “category Mesh” to “category Mesh Resource Coordination”.

References:

- doc.: IEEE 802.11-07/0023r27 Resolution of comments received during IEEE 802.11 Letter Ballot 93