IEEE P802.11
Wireless LANs

| Comment Resolutions for STA Statistics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR 97229 | 503-264-7799 | Emily.H.Qi@intel.com |

Abstract

This document is a normative text proposal to provide comment resolution for TGv Internal Review comment 9, 10 and others. Changes are shown with respect to TGv draft D0.06 and TGw Draft 1.0.

The intent of the document is to use Microsoft Word’s “track changes” from the tools menu to convey the changes to be made.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGv Editor: Change 7.3.2.21.8 as marked with Microsoft Word’s “track changes”:

### 7.3.2.21.8 STA Statistics Request

Change Figure k11 in 7.3.2.21.8 as follows:

|  | Randomization Interval | Measurement Duration | Group Identity | Triggered Reporting (optional) |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | variable |

Figure k11—Measurement Request field format for a STA Statistics Request

Insert a new row into Table k4 in 7.3.2.21.8 as shown below:

Table k4—Group Identity for a STA Statistics Request

| Statistics Group Name | Group Identity |
| --- | --- |
| RSNA Counters | 4 |
| Reserved | 45 – 255 |

Insert the following at end of 7.3.2.21.8:

The Triggered Reporting field is used to specify trigger conditions and thresholds for triggered STA Statistics measurements. It is present only when setting up triggered reporting for Statistics Groups STA Counters from dot11CountersTable or RSNA Counters, see 11.11.9.5.

The format of the Triggered Reporting field for STA Counters is shown in Figure v1. The fields marked as optional are the fields that are only present if the appropriate bit in the STA Counter Trigger Condition is set to 1.

|  | STA Counter Trigger Condition | dot11Failed Threshold (optional) | dot11Retry Threshold (optional) | dot11Multiple Retry Threshold (optional) | dot11Frame Duplicate Threshold (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 4 | 4 | 4 | 4 |
|  | dot11RTS Failure Threshold (optional) | dot11ACK Failure Threshold (optional) | dot11FCS Error Threshold (optional) | Measurement Count | Trigger Timeout |
|  | 4 | 4 | 4 | 4 | 2 |

- — Triggered Reporting Field for STA Counters

The STA Counter Trigger Condition field is a bit-field that specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the STA Counter Trigger Condition field is shown in Figure v2.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7-B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dot11Failed | dot11Retry | dot11Multiple Retry | dot11Frame Duplicate | dot11RTS Failure | dot11ACK Failure | dot11FCS Error | Reserved |
| Bits | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 9 |

- — STA Counter Trigger Condition field

- Each bit in the STA Counter Trigger Condition field is set to 1 to request that a STA Statistics Report be generated when the corresponding STA counter defined in Figure k25 (in ) increases by a value greater than or equal to the value of the corresponding STA counter threshold in Figure v1, within the total number of MSDUs given in Measurement Count. One or more trigger conditions may be set with specified thresholds.

The Measurement Count field contains a number of MSDUs used in determining trigger thresholds as described above.

The dot11Failed Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the dot11Failed condition. This field is present when the dot11Failed condition bit in the STA Counter Trigger Condition field is set to 1.

The dot11Retry Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the dot11Retry condition. This field is present when the dot11Retry condition bit in the STA Counter Trigger Condition field is set to 1.

The dot11MultipleRetry Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the dot11MultipleRetry condition. This field is present when the dot11MultipleRetry condition bit in the STA Counter Trigger Condition field is set to 1.

The dot11FrameDuplicate Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the dot11FrameDuplicate condition. This field is present when the dot11FrameDuplicate condition bit in the STA Counter Trigger Condition field is set to 1.

The dot11RTSFailure Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the dot11RTSFailure condition. This field is present when the dot11RTSFailure condition bit in the STA Counter Trigger Condition field is set to 1.

The dot11ACKFailure Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the dot11ACKFailure condition. This field is present when the dot11ACKFailure condition bit in the STA Counter Trigger Condition field is set to 1.

The dot11FCSError Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the dot11FCSError condition. This field is present when the dot11FCSError condition bit in the STA Counter Trigger Condition field is set to 1.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA shall not generate further triggered STA Statistics Reports after a trigger condition has been met.

The format of the Triggered Reporting field for RSNA Counters is shown in Figure v3. The fields marked as optional are the fields that are only present if the appropriate bit in the RSNA Trigger Condition is set to 1.

|  | RSNA Trigger Condition | dot11RSNAStats CMACICVErrors Threshold (optional) | dot11RSNA StatsCMACReplays Threshold (optional) | dot11RSNA StatsTKIPHdrErrors Threshold (optional) | dot11RSNA StatsBIPReplays Threshold (optional) | dot11RSNA StatsBroadcastDHVMismatches Threshold (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 4 | 4 | 4 | 4 | 4 |
|  | Measurement Count | Trigger Timeout |  |  |  |  |
| Octets: | 4 | 2 |  |  |  |  |

- —Triggered Reporting Field for RSNA Counters

The RSNA Trigger Condition field is a bit-field that specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the RSNA Trigger Condition field is shown in Figure v4.

|  | B0 | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- | --- |
|  | dot11RSNAStats CMACICVErrors | dot11RSNA StatsCMACReplays | dot11RSNA StatsTKIPHdrErrors | dot11RSNAStats BIPReplays | dot11RSNA StatsBroadcastDHVMismatches |
| Bits: | 1 | 1 | 1 | 1 | 1 |
|  | B5-B15 |  |  |  |  |
|  | Reserved |  |  |  |  |
| Bits: | 11 |  |  |  |  |

- —RSNA Trigger Condition field

- Each bit in the Trigger Condition field is set to 1 to request that a STA Statistics Report be generated when the corresponding RSNA Error or Failure counter defined in Figure v5 (see 7.3.2.22.8) increases by a value grater than or equal to the value of the corresponding RSNA Failure Threshold in Figure v3 within the total number of MPDUs given in Measurement Count. One or more trigger conditions may be set with specified thresholds.

The dot11RSNAStatsCMACICVErrors Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCMACICVErrors condition. This field is present when the dot11RSNAStatsCMACICVErrors condition bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsCMACReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCMACReplays condition. This field is present when the dot11RSNAStatsCMACReplays condition bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsTKIPHdrErrors Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsTKIPHdrErrors condition. This field is present when the dot11RSNAStatsTKIPHdrErrors condition bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsBIPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsBIPReplays condition. This field is present when the dot11RSNAStatsBIPReplays condition bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsBroadcastDHVMismatches Threshold field contains a value representing the number of discarded disassociate or deauthenticate frames to be used as the threshold value for the dot11RSNAStatsBroadcastDHVMismatches condition. This field is present when the dot11RSNAStatsBroadcastDHVMismatches condition bit in the RSNA Trigger Condition field is set to 1.

The Measurement Count field contains a number of MSDUs to be used in determining trigger thresholds as described above.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA shall not generate further triggered STA Statistics Reports after a trigger condition has been met.

TGv Editor: Change 7.3.2.22.8 as marked with Microsoft Word’s “track changes”:

### 7.3.2.22.8 STA Statistics Report

Change Figure k24 in 7.3.2.22.8 as follows:

|  | Measurement Duration | Statistics Group Data | Reporting Reason |
| --- | --- | --- | --- |
| Octets: | 2 | variable | variable |

Figure k24—Measurement Report field format for a STA Statistics Report

Insert a new row into table k8 as shown below:

Table k8—Group Identity for a STA Statistics Report

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 11 | dot11RSNAStatsCMACICVErrors (Counter32) dot11RSNAStatsCMACReplays (Counter32) dot11RSNAStatsTKIPHdrErrors (Counter32) dot11RSNAStatsBIPReplays (Counter32) dot11RSNAStatsBroadcastDHVMismatches (Counter32) |
| 12– 255 | None |

Insert the following text at the end of 7.3.2.22.8:

The format of the Measurement Report field for RSNA Counters Group is shown in Figure v5.

|  | dot11RSNAStatsCMACICVErrors | dot11RSNAStatsCMACReplays | dot11RSNAStatsTKIPHdrErrors |
| --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 |
|  | dot11RSNAStatsBIPReplays | dot11RSNAStatsBroadcastDHVMismatches |  |
| Octets: | 4 | 4 |  |

- —Measurement Report field format for RSNA Counters Group

The Reporting Reason field is a bit-field indicating the reason that the measuring STA sent the STA Statistics report. It is only present if Statistics Group Name is STA Counters from dot11CountersTable or RSNA Counters.

The Reporting Reason field for STA Counters is shown in Figure v6.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7-B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dot11Failed | dot11Retry | dot11Multiple Retry | dot11Frame Duplicate | dot11RTS Failure | dot11ACK Failure | dot11FCS Error | Reserved |
| Bits | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 9 |

- —Reporting Reason field for STA Counters

The Reporting Reason field for RSNA Counters is shown in Figure v7.

|  | B0 | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- | --- |
|  | dot11RSNAStats CMACICVErrors | dot11RSNA StatsCMACReplays | dot11RSNA StatsTKIPHdrErrors | dot11RSNAStats BIPReplays | dot11RSNA StatsBroadcastDHVMismatches |
| Bits: | 1 | 1 | 1 | 1 | 1 |
|  | B5-B15 |  |  |  |  |
|  | Reserved |  |  |  |  |
| Bits: | 9 |  |  |  |  |

- —Reporting Reason field for RSNA Counters

In a requested STA Statistics Report, all bit fields in the Reporting Reason field are set to 0.