IEEE P802.11
Wireless LANs

| 40 MHz operation in 2.4 GHz Forty_MHZ_Intolerant bit |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |
| Vinko Erceg | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 |  | verceg@broadcom.com |
| Jason Trachewsky | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 |  | jat@broadcom.com |
| Jim Petranovich | Conexant |  |  | Jim,petranovich@conexant.com |
| Johnny Zweig | Apple | 1 Infinite Loop Cupertino, CA 95014 |  | justjohnny@apple.com |
| John Dorsey | Apple | 1 Infinite Loop Cupertino, CA 95014 | +1 408 974-3722 | jdorsey@apple.com |
| James Woodyatt | Apple | 1 Infinite Loop Cupertino, CA 95014 |  | jhw@apple.com |
| Dave Andrus | Apple | 1 Infinite Loop Cupertino, CA 95014 |  | dandrus@apple.com |
| Daniel Borges | Apple | 1 Infinite Loop Cupertino, CA 95014 |  | drborges@apple.com |
| Douglas Chan | Cisco | 170 W. Tasman Dr. San Jose, CA 95134 | 1 408.527.9344 | dougchan@cisco.com |
| Luke Qian | Cisco | 170 W. Tasman Dr. San Jose, CA 95134 |  | lchia@cisco.com |
| Bill McFarland | Atheros |  |  | billm@atheros.com |

Abstract

This document describes new mechanisms to be added to the TGn draft 1.07 that will address issues raised in various CIDs of LB84 regarding coexistence between 20 MHz BSS and 20/40 MHz BSS. The mechanisms provided in the proposal include discovery and propagation of information regarding associated and neighboring 40 MHz intolerant devices and rules for forbidding 40 MHz operation when such information suggests an overlapping condition between 40 MHz capable and 40 MHz intolerant devices. Adoption of the submission would address a number of CIDs which are listed in the body of the document, being too numerous to list here.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# REVISION NOTES :

r04 – 06/01/11 –

- exclude primary from detection of legacy, but not for detection of forty_mhz_intolerant indication

- clearly make the distinction between the Secondary Channel Offset and STA Channel Width as the BSS width indication vs the AP’s desired RX channel width – secondary channel offset is correct field for indicating BSS width indication

- for HT-AP-19 that is not capable of switching between 20 and 20/40, change shall to may for setting the 40mhz intolerant bit

- for an HT-AP-19 that has at least one non-HT STA associated, changed shall to may for setting the 40mhz intolerant bit

- modified 40mhz-affected-channel definition by adding the words “closest to and” in several places

- modified AP advertisements of changes to BSS width by adding the phrase “beginning at the next DTIM or next TBTT if no DTIMs are transmitted” in several places

r03 – 06/01/11 –

- Added requirement for the detection of one unicast, non-mgmt, non-null data frame from the same BSS as the detected beacon.

- changed OBSS Scanning for AP from shall to should

r02 – 06/01/03 –

- Removed paragraph requiring HT-AP-19 to delay move to 40 mhz at startup – now require only OBSS Scan.

- changed HT-AP-19 requirement to send 11k Beacon Request MA frame to associated HT-STA to a narrower requirement – HT-AP-19 only needs to send this to HT-STA that advertise 40 mhz capability

Summission Note: The CID list is not correct – it will be updated in a later version.

# CID :

# A

426, 7193, 8281, 4791, 295, 704, 1750, 6813, 7893, 689, 705

C

10902, 7315, 104, 258, 286, 288, 430, 431, 689, 705, 706, 1493, 1560, 1558, 1728, 2848, 3006, 3010, 3471, 3501, 3502, 3602, 4570, 4571, 7010, 7195, 7312, 7313, 7314, 7376, 7871, 7925, 8138, 8282, 8186, 8194, 10380, 14, 49, 69, 106, 174, 175, 291, 295, 449, 704, 712, 1069, 1495, 1503, 1517, 1559, 1561, 1625, 1635, 1657, 1657, 1661, 1718, 1750, 2730, 2737, 2738, 3117, 3453, 3513, 3514, 3515, 3882, 4007, 4048, 4084, 4288, 4574, 4640, 4791, 4792, 4804, 6115, 6122, 6123, 6769, 6813, 6938, 7006, 7012, 7024, 7178, 7282, 7319, 7320, 7331, 7372, 7385, 7424, 7448, 7473, 7767, 7839, 7840, 7873, 7878, 7879, 7893, 7894, 7913, 7922, 7923, 7926, 8014, 8021, 8045, 8127, 8200, 8255, 8263, 8281, 8284, 9241, 9248, 9249, 10016, 10017, 10018, 10020, 10208, 10209, 10210, 10212, 10247, 10261, 10271, 10293, 10379, 10388, 11730, 11731, 11732, 11733, 11975, 12111, 12200, 12201, 12202, 12245, 6799, 12265

R

Proposed resolution for ACCEPTED comments:

Accept – changes effected through the adoption of 11-07-1940-xx-000n-40mhz-operation-in-24ghz-40mhz-intolerant-bit.doc

Proposed resolution for COUNTERED comments:

Counter – see the changes effected through the adoption of 11-07-1940-xx-000n-40mhz-operation-in-24ghz-40mhz-intolerant-bit.doc

# Not all listed comments are found in one of the following categories:

Add duplicate legacy modes:

689, 705

25mhz spacing:

104, 258, 286, 288, 430, 431, 689, 705, 706, 1493, 1560, 1558, 1728, 2848, 3006, 3010, 3471, 3501, 3502, 3602, 4570, 4571, 7010, 7195, 7312, 7313, 7314, 7376, 7871, 7925, 8138, 8282, 8186, 8194, 10380

CCA on secondary:

14, 49, 69, 106, 174, 175, 291, 295, 449, 704, 712, 1069, 1495, 1503, 1517, 1559, 1625, 1635, 1657, 1657, 1661, 1718, 1750, 2730, 2737, 2738, 3117, 3453, 3513, 3514, 3515, 3882, 4007, 4048, 4084, 4288, 4574, 4640, 4791, 4792, 4804, 6115, 6122, 6123, 6769, 6813, 6938, 7006, 7012, 7024, 7178, 7282, 7319, 7320, 7331, 7372, 7385, 7424, 7448, 7473, 7767, 7839, 7840, 7873, 7878, 7879, 7893, 7894, 7913, 7922, 7923, 7926, 8014, 8021, 8045, 8127, 8200, 8255, 8263, 8281, 8284, 9241, 9248, 9249, 10016, 10017, 10018, 10020, 10208, 10209, 10210, 10212, 10247, 10261, 10271, 10293, 10379, 10388, 11730, 11731, 11732, 11733, 11975, 12111, 12200, 12201, 12202, 12245

Complete list:

14, 49, 69, 104, 106, 174, 175, 258, 286, 288, 291, 295, 295, 426, 430, 431, 449, 689, 704, 704, 705, 706, 712, 1069, 1493, 1495, 1503, 1517, 1558, 1559, 1560, 1625, 1635, 1657, 1657, 1661, 1718, 1728, 1750, 1750, 2730, 2737, 2738, 2848, 3006, 3010, 3117, 3453, 3471, 3501, 3502, 3513, 3514, 3515, 3602, 3882, 4007, 4048, 4084, 4288, 4570, 4571, 4574, 4640, 4791, 4791, 4792, 4804, 6115, 6122, 6123, 6769, 6813, 6813, 6938, 7006, 7010, 7012, 7024, 7178, 7193, 7195, 7282, 7312, 7313, 7314, 7315, 7319, 7320, 7331, 7372, 7376, 7385, 7424, 7448, 7473, 7767, 7839, 7840, 7871, 7873, 7878, 7879, 7893, 7893, 7894, 7913, 7922, 7923, 7925, 7926, 8014, 8021, 8045, 8127, 8138, 8186, 8194, 8200, 8255, 8263, 8281, 8281, 8282, 8284, 9241, 9248, 9249, 10016, 10017, 10018, 10020, 10208, 10209, 10210, 10212, 10247, 10261, 10271, 10293, 10379, 10380, 10388, 10902, 11730, 11731, 11732, 11733, 11975, 12111, 12200, 12201, 12202, 12245,

TGn Editor: Insert the following text as new definitions in the correct location alphabetically within clause “3. Definitions” at about line 54 on page 54 in TGn draft D1.07:

HT-AP-19: An HT AP that is operating on a 20 MHz channel or set of 20 MHz channels that belong to the channel set that is defined in clause 19.

40MHZ-affected-channel excluding the primary channel: any 20 MHz channel of the nine 20 MHz channels comprising the five 20 MHz channels that are closest to and centered on the primary channel and the five 20 MHz channels that are closest to and centered on the secondary channel, or any 10 MHz channel in the seventeen 10 MHz channels comprising the nine 10 MHz channel that are closest to and centered on the primary channel and the nine 10 MHz channels that are closest to and centered on the secondary channel, or any 5 MHz channel in the thirty five 5 MHz channels comprising the eighteen 5 MHz channel that are closest to and centered on the primary channel and the eighteen 5 MHz channels that are closest to and centered on the secondary channel

TGn Editor: Change figure n30 “HT Capabilities Info Field” at about line 54 on page 54 in TGn draft D1.07 by changing the name of the field “reserved” at bit location 14 to “Forty_MHZ_Intolerant”

TGn Editor: Insert a new row into table n23 “Subfields of the HT Capabilities Info field” on about page 56 line 29 of TGn draft D1.07, as shown:

| Forty_MHZ_Intolerant | When sent by an AP, indicates whether other BSSs receiving this information are required to prohibit 40 MHz transmissions. When sent by a STA, indicates whether the AP associated with this STA is required to prohibit 40 MHz transmissions by all members of the BSS. (See 9.20.7) | Set to 0 by an AP if the AP allows use of 40 MHz transmissions in neighboring BSSs. Set to 1 by an AP if the AP does not allow use of 40 MHz transmissions in neighboring BSSs. Set to 0 by a STA to indicate to its associated AP that the AP is not required to restrict the use of 40 MHz transmissions within the AP’s BSS. Set to 1 by a STA to indicate to its associated AP that the AP is required to restrict the use of 40 MHz transmissions within the AP’s BSS. |
| --- | --- | --- |

TGn Editor: Change the last referenced HT Action frame in the first paragraph of subclase “7.4.8.1 HT Action field” on page 68 at about line 50 of TGn draft D1.07 from “7.4.8.9 (Antenna Selection Indices Feedback frame format)” to “7.4.8.10 (HT Information Exchange frame format):

TGn Editor: Insert the following new row into table n29 of TGn draft D1.07 on page 69 at about line 25 to appear in the correctly ordered location of the table:

| 8 | HT Information Exchange |
| --- | --- |

TGn Editor: Change the entry found in the “Action field value” column of the last row of table n29 of TGn draft D1.07 on page 69 at about line 25 from “8-255” to “9-255”.

TGn Editor: Insert the following text as a new editor instruction and a new subclause 7.4.8.10 to appear between the end of the existing subclause “7.4.8.9 Antenna Selection Indices Feedback frame format” and the editor instruction to insert the new subclause “7.4a Aggregate MPDu (A-MPDU)” as found on page 73 at about line 53 of TGn draft D1.07:

Insert the following new sublcause:

7.4.8.10 HT Information Exchange frame format

The HT Information Exchange management action frame is of category HT. The format of its frame body is defined in Table n37a (HT Information Exchange).

Table n37a – HT Information Exchange

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | Action |
| 3 | HT Information |

The category field is set to the value 7 (representing HT).

The action field is set to 8 (representing HT Information Exchange).

The HT Information field contains at least one octet that is formatted as shown in figure n29a:

Figure n29a– HT Information field

| Bits | 0 | 1 | 2 | 3-7 |
| --- | --- | --- | --- | --- |
| Field name | Information Request | Forty_MHz_Intolerant | STA Channel Width | Reserved |

The Information Request bit is used to indicate that a transmitting STA is requesting the recipient to transmit an HT Information Exchange management action frame with the transmitting STA as the recipient. When the Information Request bit is set to 1, the recipient of the HT Information field shall transmit an HT Information Exchange management action frame with the transmitting STA as the recipient. When the Information Request bit is set to 0, then the recipient of the HT Information field may transmit an HT Information Exchange management action frame with the transmitting STA as the recipient, but should not.

The Forty_MHz_Intolerant bit indicates whether the transmitting STA is tolerant of 40 MHz operation. When the Forty_MHZ_Intolerant bit is set to 1, it indicates that the STA that has transmitted management action frame containing the HT Information field is intolerant of 40 MHz operation.

The STA Channel Width bit indicates the current operating width of the transmitting STA. The setting of the STA Channel Width bit in the last successfully acknowledged transmission of the HT Information field indicates the current operating channel width of the transmitting STA. If the value of the STA Channel Width was 1 in the last successfully acknowledged transmission of the HT Information field, then the STA may transmit 40 MHz PPDUs. If the value of the STA Channel Width was 0 in the last successfully acknowledged transmission of the HT Information field, then the STA’s transmissions shall be 20 MHz PPDUs.

TGn Editor: Insert the following text as two new paragraphs to appear between the second  and third paragraphs of subclause 9.20.7 “STA switching from 40 MHz to 20 MHz in 20/40 MHz BSS” at about line 55 on page 155 in TGn draft D1.07 as follows:

An HT-AP-19 that indicates that it is not capable of operating its BSS in a 20/40 MHz mode may set the Forty_MHZ_Intolerant field to 1 in transmitted HT Capabilities elements.

An HT-AP-19 that indicates that it is capable of switching between a 20 MHz mode and a 20/40 MHz mode may set the Forty_MHZ_Intolerant field to 1 in transmitted HT Capabilities elements.

The following events are defined to be BSS_Width_Trigger events:

- receipt on any 40MHZ-affected channel excluding the primary channel, a beacon frame that contains no HT Capabilities element, and receipt of at least one unicast, non-Null-DATA, non-Management, non-HT DATA frame from a STA of the same BSSID as the received beacon frame

- receipt on any 40MHZ-affected channel a beacon frame that contains an HT Capabilities element with the value of 1 in the Forty_MHZ_Intolerant field

- receipt of an HT Information Exchange management action frame with the Forty_MHZ_Intolerant field set to 1 from a STA with which the receiver has an active association

An HT-AP-19 that encounters any of BSS_Width_Trigger events 1, 2 or 3 shall set the STA Channel Width field to 0 in transmitted HT Information elements beginning at the next DTIM or next TBTT if no DTIMs are transmitted and shall set the Secondary Channel Offset field to 0 in transmitted HT Information elements beginning at the next DTIM or next TBTT if no DTIMs are transmitted to indicate that no secondary channel is present (i.e. that the BSS operating width is 20 mhz) and shall subsequently operate in accordance with the following restrictions:

The HT-AP-19 shall not set the STA Channel Width field to 1 in transmitted HT Information elements until thirty minutes have elapsed during which there are no BSS_Width_Trgger events.

The HT-AP-19 shall not set the Secondary Channel Offset field to a non-zero value in transmitted HT Information elements thirty minutes have elapsed during which there are no BSS_Width_Trgger events.

An HT-AP-19 that has an active association with at least one non-HT STA may set the Forty_MHZ_Intolerant bit in transmitted HT Capability elements to 1, and may set the Forty_MHZ_Intolerant bit to 1 at other times.

An HT-AP-19 that has an active association with at least one HT-STA whose last successfully indicated value of the Forty_MHz_Intolerance bit is 1 in any frame transmitted to the HT-AP-19 shall set the STA Channel Width field to 0 in transmitted HT Information elements beginning at the next DTIM or next TBTT if no DTIMs are transmitted and shall set the Secondary Channel Offset field to 0 in transmitted HT Information elements beginning at the next DTIM or next TBTT if no DTIMs are transmitted to indicate that no secondary channel is present.

An HT-AP-19 may transmit an HT Information Exchange management frame with a value of 1 in the Request Information bit to a STA including a Information element in order to request an update of the status of the Forty_MHZ_Intolerant field of the HT Capabilities element.

An HT-AP-19 that has set the Secondary Channel Offset field to 0 in its most recently transmitted HT Information element shall not transmit any 40 MHz PPDUs. An HT-AP-19 that has set the Secondary Channel Offset field to to a non-zero value in its most recently transmitted HT Information element may transmit 40 MHz PPDUs.

Before transmitting a frame containing an HT Information element with the Secondary Channel Offset field set to non-zero value, an HT-AP-19 may complete an OBSS Scan at a time no earlier than one beacon interval preceeding the transmission.

An OBSS Scan operation is defined as a passive or active scan of all of the 40-MHZ-affected-channel excluding the primary channels contained in the AP Channel Report set wherein, the per-channel scan duration is a minimum of 10 TU and where each channel in the set is scanned at least thirty times per thirty minutes.

TGn Editor: Change the last paragraph of subclause 9.20.7 “STA switching from 40 MHz to 20 MHz in 20/40 MHz BSS” at about line 4 on page 156 in TGn draft D1.07 as follows:

Recommending a transmission width of 20MHz has no effect on the BSS requirements regarding protection of 40MHz transmissions or on the STA Channel Width advertised by the associated AP. A STA should re-associate with its Supported Channel Width Set field set to 0 (indicating only 20 MHz operation) to obtain this protection. (Ed: CID 1467)

An HT-STA that is associated with an HT-AP-19 and that encounters either of the BSS_Width_Trigger events 1 or 2 shall perform either of the following operations:

- disassociate from the HT-AP-19 and re-associate with the HT-AP-19 with its Supported Channel Width Set field set to 0, indicating support only for 20 MHz operation, and the Forty_MHZ_Intolerant field set to 1 in the HT Capability element

- successfully transmit to its associated HT-AP-19 an HT Information Exchange management action frame with the Forty_MHZ_Intolerant field set to 1 if the last successfully transmitted indication to the HT-AP-19 of the Forty_MHZ_Intolerant field was 0.

A STA that has met the above conditions and performed at least one of the above operations shall subsequently operate in accordance with the following three restrictions:

The STA shall not transmit any 40 MHz PPDUs.

The STA shall not transmit to its associated AP an HT Information Exchange management action frame with the Forty_MHZ_Intolerant field set to 0 before a Recovery Period has elapsed, where the Recovery Period conditions are described below.

The STA shall not re-associate with the HT-AP-19 with a Capability element containing a value of 0 for the Forty_MHZ_Intolerant field for a period of thirty minutes has elapsed during which there are no BSS_Width_Trigger events 1 or 2.

A STA may set the Forty_MHZ_Intolerant bit to 1 in a (re)association frame or in an HT Information Exchange management action frame sent to the AP, for reasons other than the detection of an intolerant BSS.

Before transmitting a frame containing an HT Capability element with the Forty_MHZ_Intolerant bit set to 0, an HT-STA that is associated with an HT-AP-19 shall complete an OBSS Scan at a time no earlier than one beacon interval preceeding the transmission.

An HT-STA that indicates in the Supported Channel Width Set field of its most recently transmitted HT Capability element a value of 1 shall accept all Beacon Measurement Requests from its associated HT-AP-19 that have a value of 254 for the reporting condition.

An HT-AP-19 shall assume a value of 1 for the STA Channel Width of a STA that has associated with a value of 1 in the Supported Channel Width Set field of its HT Capability element until it receives an HT Information Exchange management action frame from that STA.

An HT-STA shall assume that its associated HT-AP-19 has assumed a value of 1 for the STA Channel Width value for corresponding to its association until the HT-STA successfully transmits an HT Information Exchange management action frame to the HT-AP-19.

An HT-AP-19 that indicates in the Secondary Channel Offset field of its most recently transmitted HT Information element a non-zero value shall transmit a Beacon Request Measurement request to an HT-STA that in the set the Supported Channel Width Set field of its most recently transmitted HT Capability element to a value of 1 within 2 minutes of the initial association by that HT-STA. The request shall include the following specifics:

- The mode shall be either passive mode or active mode

- The measurement duration shall be a minimum of 10 TU, representing the time required to scan per channel

- The channel number shall be 255, indicating that iterative measurements shall be performed on all channels in the AP Channel Report

- The BSSID shall be the broadcast BSSID

- The reporting condition shall be 254 (report not necessary

- The threshold value may be non-zero, in which case, it indicates the minimum RCPI for a beacon to be examined as per the procedures in “9.20.7 STA switching from 40 MHz to 20 MHz in 20/40 MHz BSS. Received beacons that do not meet this minimum RCPI threshold do not need to be processed.

During each successive thirty minute period of continued association by an HT-STA, the associated HT-AP-19 shall transmit additional Beacon Request Measurement requests to that HT-STA such that all of the channels in the AP Channel Report shall have been scanned at least once by the HT-STA for a minimum duration of 10 TU per scanned channel during the thirty minutes.

An HT-AP-19 that indicates that it is capable of switching between a 20 MHz mode and a 20/40 MHz mode shall transmit an AP Channel Report containing at least all of the 40MHZ-affected-channels excluding the primary channel.

An HT-STA whose most recently successfully acknowledged transmission of the STA Channel Width field in any frame was set to 0 shall not transmit any 40 MHz PPDUs. An HT-STA whose most recently successfully acknowledged transmission of the STA Channel Width field in any frame was set to 1 may transmit 40 MHz PPDUs.

TGn Editor: Insert the following text and editor’s instruction immediately preceeding the heading “12. PHY service specification” at about line 15 on page 187 in TGn draft D1.07:

11.17 STA-STA HT Information Exchange

A STA that supports the HT Information Exchange management action frame type shall set the HT Information Exchange Support bit to 1 in transmitted Extended Capabilities information elements.

A STA that supports the transmission of and reception of the HT Information Exchange management action frame may transmit any information that is supported by the HT Information Exchange management action frame format to any other STA that also supports the transmission of and reception of the HT Information Exchange management action frame.

A STA shall not transmit an HT Information Exchange management action frame to a STA that does not support the transmission of and reception of the HT Information Exchange management action frame.

Any changes to information that are indicated through an HT Information Exchange management action frame take effect when the HT Information Exchange management action frame is acknowledged.

The most recently received information from within an HT Information Exchange management action frame shall supercede any earlier received information, regardless of the frame type and subtype that was used to communicate that information. For example, information may be provided within HT Information Exchange management action frames, within Management Action frames, and within other management frames.

The most recently successfully transmitted information from within an HT Information Exchange management action frame shall supercede any earlier transmitted information, regardless of the frame type and subtype that was used to communicate that information. For example, information may be provided within HT Information Exchange management action frames, within Management Action frames, and within other management frames.

A STA may transmit an HT Information Exchange management action frame that contains a value of 1 for the Request Information bit to another STA that supports the transmission of and reception of HT Information Exchange management action frame. A STA that receives an HT Information Exchange management action frame that contains a value of 1 for the Request Information bit shall transmit an HT Information Exchange management action frame that has the value of its RA corresponding to the address of the STA from which it received the HT Information Exchange management action frame that contained a value of 1 for the Request Information bit.

TGn Editor: Insert the following text and figure and editor’s instruction immediately preceeding the heading “7.3.2.30 TSPEC element” at about line 65 on page 53 in TGn draft D1.07:

7.3.2.27 Extended Capabilities information element

Change the last paragraph of the subclause as follows:

The Capabilities field is a bit field indicating the capabilities being advertised by the STA transmitting the information element. There are no capabilities defined for this field in this revision of the standard. One octet of extended information has been defined. The format of that octet appears in figure n28a “Capabilities field first octet”.

| Bits | 0 | 1-7 |
| --- | --- | --- |
| Field name | HT Information Exchange Support | Reserved |

Figure n28a – Capabilities field first octet

The HT Information Exchange Support field indicates support for the HT Information Exchange management frame type and its use. The HT Information Exchange Support field shall be set to 1 to indicate support for the communication of STA information through the transmission and reception of the HT Information Exchange management frame. The HT Information Exchange Support field shall be set to 0 to indicate a lack of support for the communication of STA information through the transmission and reception of the HT Information Exchange management frame. If a STA does not support the use of the HT Information Exchange management frame subtype, then the STA is not required to transmit the Extended Capabilities Information element.

TGn Editor: Insert the following text and editor’s instruction immediately preceeding the heading “7.3.2.25 RSN Information element” at about line 8 on page 53 in TGn draft D1.07:

7.3.2.21 Measurement Request element

7.3.2.21.6 Beacon Request

Change the last row of table 29b as follows:

| Reserved | 11-2553 |
| --- | --- |

Insert two new rows to become the last rows of table 29b as follows:

| Report not required to be issued | 254 |
| --- | --- |
| Reserved | 255 |

TGn Editor: Insert the following text and editor’s instruction immediately preceeding the heading “7.3.2.25 RSN Information element” at about line 8 on page 53 in TGn draft D1.07:

11.10.8.1 Beacon Report

Change the second sentence of the second from last paragraph as follows:

When the requested Reporting Condition value is non zero and not 254, the STA shall create and transmit a Beacon Report element for that measured frame only if the condition indicated in Table 29b is true.

Insert a new paragraph to become the last paragraph of this subclause as follows:

When the requested Reporting Condition value is 254, then the STA is not required to create and transmit a Beacon Report element in response to the received Beacon Report measurement request.

References: