IEEE P802.11
Wireless LANs

| Suggested PICS Proforma Table Additions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 2-17-1 Higashi-gotanda, Shinagawa-ku, Tokyo, 141-0022 Japan | +81-3-6409-3201 | sako@wcs.sony.co.jp |
| Donald Eastlake 3rd | Motorola | 111 Locke Drive, Marlborough, MA 01757 USA | +1-508-786-7554 | Donald.Eastlake@ motorola.com |

Abstract

This document provides the suggested updates to the current draft specificaiton which potentially resolve many PICS related CIDs.

## Summary of some PICS issues CIDs

- Accept: General G07 CIDs 51, 657, 789, 996, 1217, 1218, 1876, 1896, 1905, 2020, 2108, 3448, 3498, 4686, and 5606 – There should be PICS.

- Accept: MAC MDA CID 3493 – MDA should be reflected in the PICS. Remove “optional” from the main text.

- Accept: MAC Editorial CID 2182 – MDA should be reflected in the PICS.

- Accept: MAC Editorial CID 4133 – No need to indicate optionality at MDA specification, this should be clear from the PICS.

- Counter: MAC Synchronization CIDs 3412, 4251 – Resolution note: Recommend accept, but request to remove from motion on 3/13 due to insufficient detail to implement in draft.  Remove "(Optional)" from page 190 and line 10. Add a PICS table.  Need to define PICS text.

## Suggested resolution

The following will resolve 20 CIDs or about 9% of the Open CIDs in 11-07/23r50:

Delete “optional” from main text in a few cases and complete the PICS proforma table at the end of Annex A in Draft D1.07.

Suggested updates to the draft spec

Delete “(Optional)” in the heading for Section 9.21, Section 11A.11.2.1, and Section 11A.12.

# Annex A

### A.2.2 General abbreviations for Item and Supported columns

Add the following to end of A.2.2:

MP        Mesh Protocol Capability

HWM    HWMP Routing Protocol Capability

### A.4.4.2 MAC frames

Add the following to end of A.4.4.2:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| FT26 | Mesh Data Frames | 7.1.3.1, 7.1.3.5a | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT27 | Mesh Management Frames | 7.1.3.1, 7.2.4.3 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT28 | Portnal Announcement Frame | 7.4.12.1, 11A.6 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT29 | Mesh key holder security establishment frame | 7.4b.1.1 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT30 | PMK-MA delivery push frame | 7.4b.1.2 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT31 | PMK-MA confirm frame | 7.4b.1.3 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT32 | PMK-MA request frame | 7.4b.1.4 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT33 | PMK-MA delifery pull frame | 7.4b.1.5 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT34 | PMK-MA delete frame | 7.4b.1.6 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT35 | Mesh EAP emcapsulation frame | 7.4b.1.7 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT36 | HWMP Path Request Frame | 7.4.11.1 | MWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT37 | HWMP Path Reply Frame | 7.4.11.2 | MWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT38 | HWMP Path Error Frame | 7.4.11.3 | MWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FT39 | HWMP Root Announcment Frame | 7.4.11.4 | MWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR26 | Mesh Data Frames | 7.1.3.1, 7.1.3.5a | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR27 | Mesh Management Frames | 7.1.3.1, 7.2.4.3 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR28 | Portnal Announcement Frame | 7.4.12.1, 11A.6 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR29 | Mesh key holder security establishment frame | 7.4b.1.1 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR30 | PMK-MA delivery push frame | 7.4b.1.2 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR31 | PMK-MA confirm frame | 7.4b.1.3 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR32 | PMK-MA request frame | 7.4b.1.4 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR33 | PMK-MA delifery pull frame | 7.4b.1.5 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR34 | PMK-MA delete frame | 7.4b.1.6 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR35 | Mesh EAP emcapsulation frame | 7.4b.1.7 | PC35:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR36 | HWMP Path Request Frame | 7.4.11.1 | MWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR37 | HWMP Path Reply Frame | 7.4.11.2 | MWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR38 | HWMP Path Error Frame | 7.4.11.3 | MWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| FR39 | HWMP Root Announcment Frame | 7.4.11.4 | MWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |

### A.4.4.3 Frame exchange sequences

Add the following to end of A.4.4.3:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| FS3 | Peer Link Establisment | 11A.2 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |

### A.4.4.4 MAC addressing functions

Add the following to end of A.4.4.4:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| AD4 | 4 Address Format | 7.1.2 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| AD5 | 6 Address Format | 7.1.2, 7.1.3.5a | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |

Add the following to end of A.4.4:

### A.4.4.16a Mesh Protocol Capabilities

### A.4.4.16a.1 General Mesh Support

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| MP1 | Support of Mesh Capability | 5.2.9.1, 11A.1 | O | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP2 | Decrement of Mesh Frame TTL | 7.1.3.5a.1 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP3 | MP Beaconing | 11A.11.3 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP4 | MP Synchronization | 11A.11 | MP1:O | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP5 | MDA | 9.21 | MP1:O | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP5.1 | MDAOP Advertisement | 9.21.9 | MP5:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP5.2 | Neighbor MDAOP Recognition | 9.21.4-9.21.6 | MP5:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP5.3 | MDAOP Setup | 9.21.6 | MP5:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP5.4 | Access during MDAOPs | 9.21.9 | MP5:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP6 | Intra Mesh Congestion Control | 11A.10 | MP1:O | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP7 | Mesh Beacon Collision Avoidance | 11A.11.4 | MP1:O | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP8 | Mesh Power Management | 11A.12 | MP1:O | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP8.1 | MP Power Management modes | 11A.12.2-11A.12.4 | MP8:O | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP8.2 | Mesh Power Management support | 11A.12.5 | MP8:O | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP9 | Mesh channel selection | 11A.3 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| MP10 | Airlink Time metric computation | 11A.7 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |

### A.4.4.16a.2 HWMP Routing Protocol Capabilities

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| HWM1 | Hybrid Wireless Mesh Protocol | 11A.8 | MP1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| HWM2 | On demand path selection | 11A.8.1.2 | HWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |
| HWM3 | Proactive tree building | 11A.8.1.3 | HWM1:M | Yes 􀂆 No 􀂆 N/A 􀂆 |

References:

[1] IEEE Std. 802.11-2007.

[2] Draft Amendment: ESS Mesh Networking.  doc.: IEEE P802.11s/D1.07, September 2007.

[3] 11-07-0023-50-000s-lb93-tgs-draft-1-0-comment-spreadsheet.xls