IEEE P802.11
Wireless LANs

| LB97 CID 2582 BSS Width Switching |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place Sunnyvale, CA 94040 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document contains a proposed resolution for CID 2582 of LB97 from the COEX group of comments. CID 2582 raises some interesting questions about the transition between a 20/40 MHz BSS and a 20 MHz BSS and vice versa. This submission recognizes a much larger issue that results from considering this question and as a result, attempts to create a set of rules for signalling and behaviour that involves the HT Capabilities and HT Information elements. The following additional CID are potentially addressed: 2583, 2585, 3104, 2483, 3079.

General discussion:

CID 3104

TGn Editor: Insert the following heading and text as a new subclause to appear as the first subclause after “9.20 20/40 Functional Description” on page 159 at about line 47 of TGn Draft D2.04, and to be numbered approximately as 9.20.1, making appropriate adjustments to the heading numbering of other subclauses:

9.20.1 40 MHz PPDU transmission restrictions

Several fields from various frames are used to convey information between STAs regarding the support for 40 MHz PPDU transmission and reception and regarding any current prohibition against the transmission and reception of 40 MHz PPDUs.

The following rules describe the behaviour that accompanies those fields.

The fields that are used to determine the status of the transmission and reception of 40 MHz PPDUs are as follows:

- The Supported Channel Width Set field of the HT Capabilities element

- The Secondary Channel Offset field of the HT Information element

- The STA Channel Width field of the HT Information element

- The STA Channel Width field of the Notify Channel Width action frame

- The Extended Channel Switch Announcement element

The Supported Channel Width Set field is used to indicate whether or not the transmitting STA is capable of transmitting and receiving 40 MHz PPDUs.

A STA that is associated with an AP shall not transmit a value for the Supported Channel Width Set field that differs from a previously transmitted value unless the STA first dissociates or is dissociated from its BSS.

The Secondary Channel Offset field is used to indicate whether or not the BSS is occupying a 40 MHz wide pair of channels, and when a secondary channel exists, whether it is above or below the primary channel in frequency.

The STA Channel Width field is used to indicate the channel width of PPDU (40MHz and 20MHz PPDU or 20MHz PPDU only) that the STA is able to receive on. A transmitted value of zero in the STA Channel Width field indicates that the transmtting STA is not  able of receiving   40 MHz  PPDUs, beginning at the end of the transmission of the frame that contained the STA Channel Width field.

A STA shall not transmit a frame containing a STA Channel Width field with a non-zero value if the value of its most recently transmitted Supported Channel Width Set field is zero.

The Extended Channel Switch Announcement element can be used to indicate a change in the operating width of the BSS.

A STA that is associated with an infrastructure BSS (STA1) shall not transmit a 40 MHz PPDU to a STA (STA2) unless the following three conditions are true:

- The Supported Channel Width Set field of the HT Capabilities element of both STAs is set to a non-zero value

- The Secondary Channel Offset field of the most recently received HT Information element sent by the AP of the BSS has a value of 1 or 3

- The most recently received Extended Channel Switch Announcement element transmitted by the AP did not contain a value of 0 in the Channel Switch Count field while also containing a value in the New Regulatory Class field that indicates a regulatory class that does not correspond to a Channel Spacing value of 40 MHz.

If the above three conditions are met, STA1 should not transmit a 40 MHz PPDU to STA2 unless the following two conditions are true:

- Either, STA1 has not received a Notify Channel Width action frame that was transmitted by STA2, or the STA Channel Width field of the most recently received Notify Channel Width action frame at STA1 that was transmitted by STA2 is non-zero

- Either, the STA Channel Width field of the most recently received HT Information element that was transmitted by STA2 is non-zero

An AP shall not transmit a 40 MHz PPDU to another STA unless the following three conditions are true:

- The Supported Channel Width Set field of the HT Capabilities element of the AP and the STA are set to a non-zero value

- The Secondary Channel Offset field of the AP’s most recently transmitted HT Information element sent has a value of 1 or 3

- The most recently transmitted Extended Channel Switch Announcement element transmitted by the AP did not contain a value of 0 in the Channel Switch Count field while also containing a value in the New Regulatory Class field that indicates a regulatory class that does not correspond to a Channel Spacing value of 40 MHz.

If the above three conditions are met, the AP should not transmit a 40 MHz PPDU to another STA unless the following condition is true:

- Either, the AP has not received a Notify Channel Width action frame that was transmitted by the STA, or the STA Channel Width field of the most recently received Notify Channel Width action frame at the AP that was transmitted by the STA (if any), is non-zero

An HT-STA-19 that is not a member of an infrastructure BSS shall not transmit a 40 MHz Mask PPDU.

An HT-STA-17 that is not associated with an infrastructure BSS (STA1) shall not transmit a 40 MHz PPDU to a STA (STA2) unless the following three conditions are true:

- The Supported Channel Width Set field of the HT Capabilities element of both STAs is set to a non-zero value

- The Secondary Channel Offset field of the most recently received HT Information element sent by STA2 has a value of 1 or 3

- The Secondary Channel Offset field of the most recently transmitted HT Information element sent by STA1 has a value of 1 or 3

If the above three conditions are met, STA1 should not transmit a 40 MHz PPDU to a STA2 unless the following   condition is true

- STA1 has not received a STA Channel Width field that was transmitted by STA2, or the value of the most recently received STA Channel Width field at STA1 that was transmitted by STA2, is non-zero

Independently of the values of the five fields, a non-AP STA that is a member of a 20/40 MHz BSS shall not transmit 20 MHz frames in the secondary channel.

TGn Editor: Search and destroy the following text, wherever you find it in the TGn Draft D2.04:

An AP may communicate its STA Channel Width either by transmitting a 20/40 BSS Coexistence Element (in beacons and probe response frames) or by transmitting the Notify Channel Width management action frame.

An HT STA whose most recently successfully acknowledged transmission of the STA Channel Width field in any frame was set to 0 shall not transmit a 40 MHz mask PPDU.

A STA that receives from another STA, a Notify Channel Width frame with a value of 0 in the STA Channel Width field should not transmit a 40 MHz mask PPDU to that STA. The reaction of an FC HT STA or FC HT AP to change the channel width of PPDUs addressed to the sender of the Notify Channel Width frame might not be immediate after receiving the frame. A STA that has transmitted a Notify Channel Width frame knows that the requested channel width has been adopted after receiving an HT frame of the requested channel width from the STA to which it addressed the Notify Channel Width frame.

CID 2582

TGn Editor: Change the second paragraph of subclause “11.9.8.4 Channel management at the AP” on page 190 at about line 49 of TGn Draft D2.04 as follows (note that one of the changes is to split the pargraph into two paragraphs).

An AP switches the BSS channel width between 20/40 MHz and 20 MHz by changing the value of the Supported Channel Width Set field of the HT Capabilities element and/or the Secondary Channel Offset field of the HT Information element in the Beacon and/or by changing the value of the Secondary Channel Offset field of the Secondary Channel Offset element.

In order to maintain existing associations while performing a channel pair relocation, an AP may inform associated HT STAs that it is making the change by including an Extended Channel Switch Announcement element in Beacon, Probe Response, and Extended Channel Switch Announcement frame transmissions until the intended channel switch time. The New Channel Number of the Extended Channel Switch Announcement element represents the new channel in the case that the BSS after relocation/width change will be a 20 MHz BSS, or the primary channel of the new pair of channels in the case that the BSS after relocation/width change will be a 20/40 MHz BSS. When changing to a new pair of channels, the New Regulatory Class field specifies the position of the secondary channel relative to the new primary channel, i.e., either above or below.

CID 2583

TGn Editor: Within subclause “11.9.8.4 Channel management at the AP”, change the first paragraph of the subclause at page 190 at about line 41 of TGn Draft D2.04 as follows:

While operating a 20/40 MHz BSS, a STA that is the DFS owner in an IBSS or an AP may decide to move the BSS and/or adjust the BSS channel width if, for example, another BSS starts to operate in either or both of the primary or secondary channels, or radar is detected in either or both of the primary or secondary channels or for other reasons that are beyond the scope of this standard. Specifically, the AP or STA may move its BSS to a different pair of channels, or change from a 20/40 MHz BSS to a 20 MHz BSS using either the primary channel of the previous channel pair or any other available 20 MHz channel. The AP or STA may also change from a 20 MHz BSS to a 20/40 MHz BSS, or move the channel of the 20 MHz BSS.

TGn Editor: Within subclause “11.9.8.4 Channel management at the AP”, delete table “79.0.a-Channel switching and channel width adjustment combinations” on page 191 at about line 6 of TGn Draft D2.04 and delete the paragraph that precedes that table which begins “When an AP uses the Extended Channel Switch Announcement” and insert the following text in its place:

The AP or STA moving the BSS or adjusting its channel width selects a combination of operating parameters from any single row of any one of the tables in Annex J that is appropriate for the current operating domain of the AP or STA. The AP or STA selects one channel number from the Channel Set column of the selected row. The AP or STA includes the selected information in subsequently transmitted frames that contain any combination of the following three elements:

  - HT Information element

  - Channel Switch Announcement element

  - Extended Channel Switch Announcement element

The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements to 1 if the Behavior Limit parameter of the selected row contains the value 13. The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements to 3 if the Behavior Limit parameter of the selected row contains the value 14. The AP or STA shall set the Secondary Channel Offset field of transmitted HT Information elements to 0 if the Behavior Limit parameter of the selected row contains neither the value 13 nor the value 14.

The AP or STA shall set the New Channel Number field of transmitted Channel Switch Announcement elements and Extended Channel Switch Announcement elements to the value of the selected channel from the selected row.

The AP or STA shall set the New Regulatory Class field of transmitted Extended Channel Switch Announcement elements to the value of the Regulatory Class column of the selected row.

Basic functionality in BSS 20/40 MHz mode

CID 3331

TGn Editor: Within subclause “11.15.1 Basic functionality in BSS 20/40 MHz mode” delete the last sentence of the second paragraphat about line 45 of page 192 of TGn Draft D2.04, as shown:

An HT AP declares its channel width capability (20 MHz only or 20/40 MHz) in the Supported Channel Width Set field of the HT Capabilities element.

| CID | Commentor | Page | Clause | Proposed Resn Status | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 2582 | Stephens, Adrian | 199.59 | 11.9.8.4 | A | "The STAs may disassociate and associate or reassociate to accommodate the new capability." What is this saying? Re-associate with the same AP (which is unnecessary) or go elsewhere? What stops a STA from doing is already? - nothing | Remove the sentence or rewrite as a note saying they can go to another AP if they want to. | Accept – text is removed per editor instructions found in 11-07-2020r7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2583 | Stephens, Adrian | 200.01 | 11.9.8.4 | A | What I don't understand is the role of regulatory classes when an AP switches between 20/40 and 20. When it swiches between these, must it also change regulatory class to a 40 MHz or 20MHz class? Or can it stay in the 40MHz class with the Secondary Channel Offset indicating 20MHz operation? | Please add text that describes how secondary channel offset and regulatory class are modified when switching between 20/40 and 40. | Accept – text added per 11-07-2020r7 |
| 2585 | Stephens, Adrian | 200.23 | 11.9.8.4 | C | Table n52 mistakenly believes that the Supported Channel Width set determines the 20/40 vs 20 mode of operation of the BSS. It does not. Secondary channel offset does. | Replace "Supported Channel Width Set: 0" with "Secondary Channel Offset set to 0" "Supported Channel Width Set: 1" with "Secondary Channel Offset set to a non-zero value" | Counter – table deleted and text created to replace the information and correct it as suggested as per 11-07-2020r7. |
| 3104 | Zaks, Artur | 80.00 |  | C | Not sufficient definition of AP's intention to change channel width. | Change sentence :"If an AP wishes to receive 20 MHz packets, it broadcasts this management action frame to all STAs." to: "When AP decides to change the Channel width it will send broadcast Notify Channel Width Action frame." | Counter – see new text from 11-07-2020r7, which gives all conditions that determine when a 40 mhz frame may be transmitted, based on the named fields and other fields. |
| 2483 | Stephens, Adrian | 164.24 | 9.20.4 | A | How does 20/40 switching work with ISSS? IBSS is an HT feature. We have not properly addressed how an IBSS operates channel width switching. | I propose that 40MHz in 2.4 GHz not be allowed in an IBSS (this is because the 20/40 coexistence rules require the presence of the AP to manage response to BSS width tirgger events). Review the language attached to the channel switching and use of extended channel switch announcement to ensure that it speaks of STAs, not APs. This will allow its use by the DFS owner of a 5GHz 40MHz IBSS. | Accept – 20/40 MHz BSS operation in IBSS in 2.4 GHz is disallowed per 11-07-0614r10. IBSS channel switching for 5 GHz is accommodated in changes as per 11-07-2020r7. |
| 3079 | Zaks, Artur | 164.00 | 9.20.4 | A | Operation in the IBSS is not defined | Define the operation in the IBSS | Accept – IBSS 20/40 MHz BSS is not allowed in 2.4 GHz, and changes which allow the DFS owner mechanism to be used in 5 GHz allow the operation in 5 GHz – see 11-07-0614r10 and 11-07-2020r7. |

| 556 |  | 80.13 | 7.4.9.2 |  | The AP capabilities after broadcasting this frame are unclear. The AP "wishes" to receive 20 MHz transmissions, but is it required to receive or reject 40 MHz transmissions? | Add a sentence stating how an AP is expected to treat 40 MHz transmissions after sending a Notify Channel Width frame signaling 20 MHz width. | Counter – clarified rules for when it is ok to TX 40 mhz as per 11-07-2020r7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1745 |  | 164.24 | 9.20.4 |  | What is difference between STA Channel Width and Supported Channel Width? And for that Channel Width (in the Notify Channel Width field) as well? | please clarify | Counter – clarified meaning of these fields as per new text from 11-07-2020r7 |
| 2484 |  | 164.24 | 9.20.4 |  | This is a coex or mac issue. We've got an over-abundance of "width" fields. This results in over-complex and confusing specification in 9.20.4. To summarise, we have a: Supported Channel Width set - i.e. what the STA can support. Static. Secondary Channel offset - indicates whether a BSS is 20 or 20/40. STA Channel width - indicates whether the STA wants to receive only 20MHz frames. I question whether the last one should exist in the HT Information element. Under what conditions would an AP want to permit 20/40 operation in its BSS, but still only be able to receive 20 MHz itself? Surely if the interference is so bad in the secondary channel that it wanted 20MHz frames sent to it, the interference is also likely to be bad for the whole BSS. In that case the AP should either relocate or switch the whole BSS to 20MHz operation. | Remove STA Channel width from the HT information element. Remove all associated text. | Counter – clarified meaning of these fields as per new text from 11-07-2020r7 |
| 2492 |  | 164.30 | 9.20.4 |  | "When a STA is associated with an AP that declares a Supported Channel Width of 20/40 MHz, the STA may transmit frames in the 20 MHz wide primary channel as well as the 40 MHz wide channel." This is way too broad. There are times when it can't - for example resulting from the CCA rules, or when it's a responder to a 20MHz STA. This needs to be turned into a statement that is always true. | "Reword: ""If a STA is associated with an AP that declares a Supported Channel Width of 20 MHz, the STA shall not transmit any 40 MHz mask PPDUs."" Add the following: ""A STA shall only transmit a 40 MHz mask PPDU when the secondary channel offset field of its BSS is set to a non-zero value""" | Counter – clarified rules for when it is ok to TX 40 mhz as per 11-07-2020r7 |
| 2504 |  | 165.35 | 9.20.4 |  | "The AP may communicate its STA Channel Width either by transmitting the HT Information Element (in beacons and probe response frames) or by transmitting the Notify Channel Width management action frame." This statement is unnecessary as the definition of the frames already permits this communication. | Delete the sentence or reword as a note thus: "NOTE-The AP can communicate its STA Channel Width either by transmitting the HT Information Element (in beacons and probe response frames) or by transmitting the Notify Channel Width management action frame." | Counter – clarified rules for when it is ok to TX 40 mhz as per 11-07-2020r7 |
| 2506 |  | 165.39 | 9.20.4 |  | "may request that frames sent to it be of 20 or 40 MHz width." "May" is a normative verb granting permission. It needs to be related to actions. This could be reworded "may transmit a Notify Channel Width action frame". However there's nothing to stop it doing this. | Reword: "A 20/40 MHz capable non-AP STA in a 20/40 MHz capable BSS may transmit a Notify Channel Width frame in order to control whether STAs transmitting to it are limited to 20 MHz or may use 40 MHz.." | Counter – clarified rules for when it is ok to TX 40 mhz as per 11-07-2020r7 |
| 2507 |  | 165.42 | 9.20.4 |  | "The reaction of both STA and AP to switch the channel width may be not immediate after receiving these frames." Unclear. Also incorrect use of the normative verb "may". Also an AP is a STA, so "STA and AP" is incorrect. | Reword thus: "The reaction of a non-AP STA or an AP that receives a Notify Channel Width frame is not necessarily immediate." | Counter – text deleted as per 11-07-2020r7 |
| 3049 |  | 165.42 | 9.20.4 |  | The definition is ambigues. | re-define the paragraph | Counter – text deleted as per 11-07-2020r7 |
| 3050 |  | 165.42 | 9.20.4 |  | "these frames" is not clear | explicitely define what frames | Counter – text deleted as per 11-07-2020r7 |
| 2508 |  | 165.44 | 9.20.4 |  | "The STA that has transmitted one of these frames shall decide that the related request has been adopted after having received an HT frame of requested specified channel width." 1. Wrong article. 2. These frames needs to be expanded. 3. "Related" is awkward 4. "shall decide" is poor normative specification 5. Another issue is that the peer may transmit a 20MHz frame for other reasons (e.g. due to 20/40 CCA rules), so receving a 20MHz frame is not, by itself, enough to know that the request has been "adopted". The question is what normative action must the transmitting STA perform based on the response from the peer. If the transmitting STA is still capable of receiving 20/40, it really doesn't matter when the response arrives because it has no effect - i.e. the STA still continues to be capable of receivng 20/40. If, however, there is a change of capability, the STA must delay that change until the response arrises, and can be reworded. If this is the case, the next paragraph is wrong because the STA is now effectively a 20MHz STA. | Reword: "A STA (STA A) should not transmit a 40MHz frame to a STA (STA B) if the last Notify Channel Width frame received from STA B contained a Channel Width field set to 0." | Counter – clarified rules for when it is ok to TX 40 mhz as per 11-07-2020r7 |
| 3051 |  | 165.44 | 9.20.4 |  | "these frames" is not clear | explicitely define what frames | Counter – text deleted as per 11-07-2020r7 |
| 3078 |  | 167.12 | 9.20.4 |  | The definition is ambigues. | re-define the paragraph | Counter – clarified rules for when it is ok to TX 40 mhz as per 11-07-2020r7 |

| 3331 |  | 201.57 | 11.15.1 |  | There is no BSS Channel Width field in any element. | Change "BSS Channel Width" to "Secondary Channel Offset" | Counter – deleted the sentence as per 11-07-2020r7. |
| --- | --- | --- | --- | --- | --- | --- | --- |

References: