IEEE P802.11
Wireless LANs

| 802.11 TGn LB97 Submission, PSMP ad-hoc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-06-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comments assigned to the author: 2331, 2332.

The changes marked in this document are based on TGn Draft version D2.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Comments

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 2331 | 143.27 | 9.15.2.7 | "Data transmitted within a PSMP sequence (PSMP-DTT and PSMP-UTT) shall be acknowledged using MTBA." This statement is incorrect becuase later down we also allow "no ack" and "block ack". | Replace with: "A BlockAck or BlockAckReq frame transmitted within a PSMP sequence (PSMP-DTT or PSMP-UTT) shall be the Multi-TID variant (MTBA or MTBAR respectively)." |

### Discussion:

Consideration of these comments was delayed while comment 843 (“remove HT-delayed BA”) was decided in the MAC ad-hoc.   The MAC ad-hoc decided to remove HT-delayed BA.  The resolution of these comments must be consistent with that decision.

Note, the resolution below indicates that non-HT delayed BA is not permitted in PSMP either.

### Proposed Resolution:

Accept.

| 2332 | 143.37 | 9.15.2.7 | "If a Block Ack agreement exists for a data frame, then such frames transmitted during either a PSMP-DTT or a PSMP-UTT with an individual receiver address shall be a QoS data subtype and shall have one of the following Ack Policy values: Scheduled Ack Under MTBA/PSMP or Block Ack" Line 64 mentions delayed block ack. This begs the quesion if the quoted sentence needs to be specific to one or other type of block ack. I think that it is specific to HT immediate. If that is the case, we need to allow only Block Ack policy for HT-delayed BA agreements. | Insert: "an HT-immediate" before "block ack agreement" Insert the following new sentence after the quoted one: "If an HT-delayed Block Ack agreement exists for a data frame, then such frames transmitted during either a PSMP-DTT or a PSMP-UTT with an individual receiver address shall be a QoS data subtype and shall have the Ack Policy field set to Block Ack" |
| --- | --- | --- | --- | --- |

### Discussion:

This change is clearly unnecessary, now that HT-delayed BA has been removed.

### Proposed Resolution:

Reject.  See resolution to CID 843.   This removes HT-delayed BA from the draft,  so the change proposed by the commenter is no longer necessary.