IEEE P802.11
Wireless LANs

| TGn LB97 Submission containing proposed comment resolutions for ad-hoc “Frame Formats” comment-group “BlockAck” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +972547885738 | solomon.trainin@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comments assigned to the author:

- CIDs   2010, 2026, 147, 669, 155, 2142, 2045 counter

- CIDs   3,  2025, 142, 2030, 2033, 2034, 2036, 2037, 2041, 2043, 676, 2047, 2048, 144, 145, 2049, 2052, 2056,  accept

- CIDs   2106, 679, 1943, 2143, reject

- CIDs   673, 603, 675, Transfer to MAC

- CIDs   604, 2039, 1880, 148,  2053, 2054,   Transfer to PSMP

- CIDs   2834  Straw poll

- CID     2031 reconsider (approved FF motion 151)

(40 CIDs)

The changes marked in this document are based on TGn Draft version D2.02.

The r1 resolves the CIDs including the 679

,

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 2010 | 16.25 | 7.1.3.5.3 |  | The references quoted for Implict Block Ack "9.2.8, 9.3.3 and 9.9.2.3." are irrelevant. | Replace with relevant references. | Counter As per commentar recommendation Implement as in 20070610 11-07-xxxx-00-000n-tgn-lb97-frame-formats-BA |
| 3 | 22.13 | 7.1.4 |  | "associated Acks" BlockAcks should be also included. | Change it to "associated response frames". | Accept As per commentar recommendation |
| 673 | 23.00 | 7.2.17 |  | Figure 31 : Why do you need additional bit for Compressed Bitmap when the only allowed Block ACK between HT-STAs is compressed Block ACK | Change B2 in Figure 31 to reserved | The same issue raised in CID 3091of MAC BA comments Transfer to MAC |
| 603 | 24.00 | 7.2.1.7 |  | Table n9: since the BAR and BA Ack policy subfield are defined only for HT-delayed agreement, and the HT-delayed block ack only uses compressed BA and BAR variants, why the values are also mentioned to be used for simple BlkAck and simple BlkAckReq in this table? | Only mention for the compressed BlkAckReq/BlkAck and Multi-TIB BlkAckReq/BlkAck. | The same issue raised in CID 3091of MAC BA comments Transfer to MAC |
| 675 | 24.00 | 7.2.1.7 |  | Table n10 : The only allowed Block ACK between HT STAs is compressed Block ACK. | Delete the column for Compressed Bitmap from Table n10 | The same issue raised in CID 3091of MAC BA comments Transfer to MAC |
| 2025 | 24.06 | 7.2.1.7 |  | "The BAR Ack Policy field is set to this value in all individually addressed BlockAckReq,..." There are currently no provisions for BlockAckReq... to be group addressed. Therefore the "individually addressed" is unnecessary and inconsistent with usage elsewhere which does not call this out. | Remove "individually addressed" from Table n9 | Accept As per commentar recommendation |
| 140 | 24.13 | 7.2.1.7 |  | "In the case of HT-delayed Block Ack, the addressee returns an ACK." Why do we need to mention this again, the table is anyway for HT-Delayed BA. | Remove this redundant statement. | Counter Already implemented in D2.02 |
| 2026 | 24.13 | 7.2.1.7 |  | "In the case of HT-delayed Block Ack, the addressee returns an ACK." This whole table is specific to this case, so this can be simplified. | Replace quoted text with: "The addressee returns an ACK". | Counter Already implemented in D2.02 |
| 142 | 24.21 | 7.2.1.7 |  | "in which the sender does not require immediate acknowledgement to the containing BlockAck/BlockAckReq frame (as appropriate)" | Whats a "containing" BA/BAReq frame? Remove the text "to the containing BlockAck/BlockAckReq frame (as appropriate)" | Accept As per commentar recommendation |
| 2030 | 24.55 | 7.2.1.7.1 |  | "The Compressed Bitmap subfield of the BAR control field of the Basic BlockAckReq frame has the value 0." This duplicates the definition in table n10 | Either remove this line or reference table n10 instead of defining the value locally. | Accept Remove the text as per commentar recommendation |
| 2033 | 25.20 | 7.2.1.7.2 |  | "The BAR Ack Policy subfield of the BAR control field of the Compressed BlockAckReq is defined in 7.2.1.7 (Block Ack Request (BlockAckReq) frame format)." Seeing as 7.2.1.7 defines the values of these fields, it is not necessary to do so here. | Remove the quoted text. | Accept As per commentar recommendation |
| 2034 | 25.21 | 7.2.1.7.2 |  | "The Multi-TID subfield of the BAR control field of the Compressed BlockAckReq frame has the value 0. This is already defined in table n10, so there is no need to repeat it here. | Remove the quoted text | Accept As per commentar recommendation |
| 2036 | 25.26 | 7.2.1.7.2 |  | "The Compressed Bitmap subfield of the BAR control field of the Compressed BlockAckReq frame has the value 1 and has the same meaning as defined in 7.2.1.7 (Block Ack Request (BlockAckReq) frame format). The compressed bitmap subfield is defined in table n10. There is no need to duplicate it here. | Remove the quoted text. | Accept As per commentar recommendation |
| 2037 | 25.42 | 7.2.1.7.3 |  | "The Starting Sequence Number subfield is the sequence number of the first MSDU for which this BlockAck is sent, and is set to the same value as in the immediately previously received BlockAckReq frame." This implies that there is always a previously received BlockAckReq frame. This is not so under implicit block ack rules, which state: "The Starting Sequence Number of the Block Ack Starting Sequence Control field of the BlockAck frame shall be set to any circular modulo 2^12 value in the range from (WinEnd_R - 63) to WinStart_R inclusive." | Replace quoted text with: "The Starting Sequence Number subfield is the sequence number of the first MSDU for which this BlockAck is sent. The value to go in this field is defined in 9.10.7.5." | Accept As per commentar recommendation The quoted sentence is in the 7.2.1.8.3 Compressed BlockAck variant on page 30 in D2.02 |
| 604 | 25.48 | 7.2.1.7.3 |  | "The BAR Ack Policy subfield of the Multi-TID BlockAckReq frame is reserved" , is not clear. Since MTBAR can be used with HT-delayed Block Ack Agreement, this subfield is used. |  | Transfer to PSMP |
| 2039 | 25.50 | 7.2.1.7.3 |  | The note: "NOTE—The BAR Ack policy field in BlockAckReq and BlockAck is only defined for HT-delayed BlockAck. It is reserved under HT-immediate BlockAck." is potentially misleading, because Multi-TID is only used under PSMP scheduled ack, when neither HT-delayed not HT-immediate would seem to apply. This also begs the question of what goes in the "Block Ack Policy" field of the related ADDBA. Is Multi-TID under PSMP a variant of immediate BA, or Delayed BA? It must be one or the other. Depending on the answer to this question, we may need to modify the text quoted above. | Recommend somewhere we tie Multi-TID BA operation to a BA agreement established using immediate BA. We can then clarify the note further thus: "NOTE-The BAR Ack policy field in BlockAckReq and BlockAck is only defined for HT-delayed BlockAck. It is reserved under HT-immediate BlockAck (including scheduled acknowledgement under a PSMP session)." Alternatively, delete HT delayed BA. | Transfer to PSMP |
| 2041 | 25.55 | 7.2.1.7.3 |  | "The Compressed Bitmap subfield of the BAR control field of the Multi-TID BlockAckReq frame has the value 1 and has the same meaning as defined in 7.2.1.8 (Block Ack (BlockAck) frame format).". This duplicates specification of table n10. | Remove the quoted sentence | Accept As per commentar recommendation |
| 2043 | 25.61 | 7.2.1.7.3 |  | "that there are 3 TID values present in the Multi-TID BlockAckReq frame’s BAR Information field, as shown in Figure n6 (BAR Information field (MTBAR))." This is misleading because figure n6 doesn't show 3 TID balues. | Replace with: "that there are 3 TID values present in the Multi-TID BlockAckReq frame’s BAR Information field." | Accept As per commentar recommendation |
| 2045 | 28.61 | 7.2.1.8.1 |  | "Bit position n of the Block Ack bitmap, if set to 1, acknowledges receipt of an MPDU with an MPDU sequence control value equal to (Block Ack Starting Sequence Control + n)." The problem with this is that the Block Ack Starting Sequence Control is a structure consisting of two integer fields. It is itself not an integer. | Replace "(Block Ack Starting Sequence Control + n)" with "((Starting Sequence Number * 16) + n)" (2 occurances this paragraph). | Counter “Treat both the MPDU sequence control and Block Ack Starting Sequence Control as an 16 bit integer” Add this text to this paragraph CID 2031 (wrongly mentioned as 1875 in D2.03) should be reconsidered |
| 676 | 29.00 | 7.2.1.8.2 |  | The only allowed bit map between HT STAs is compressed bit map | Delete lines 32 and 33 | Accept As per commentar recommendation Already removed in the D2.04 |
| 2047 | 29.10 | 7.2.1.8.2 |  | "The BlockAck of compressed format is mandatory for all HT STAs. BlockAck negotiated between HT STAs shall use the compressed format." The first statement is ambiguous, but unnecessary in the light of the second sentence. The second statement disallows an HT STA from using Multi-TID BlockAck, which is clearly not its intent. | Delete the quoted text, and insert as a continuation of the paragraph on page 27, line 54: "The values of the Multi-TID and Compressed Bitmap fields determine which of three possible BlockAck frame variants is represented, as indicated in the Table n11 (BlockAck frame variant encoding).", the following text: "BlockAck negotiated between HT STAs shall use the compressed variant when transmitted outside a PSMP sequence, and shall use the Multi-TID variant when transmitted inside a PSMP sequence". Consider making a matching statement in 7.2.1.7 at line 63 of page 23 after: "The meaning of the BAR Information field depends on the BlockAckReq frame variant type. The meaning of this field is explained within the subclause for each of the BlockAckReq frame variants." | Accept As per commentar recommendation and consideration |
| 2048 | 29.14 | 7.2.1.8.2 |  | "A block ack agreement established between two HT STAs using Delayed BlockAck Policy is referred to as an HT-delayed BlockAck. A block ack agreement established between two HT STAs using Immediate BlockAck Policy is referred to as an HT-immediate BlockAck." These statements have nothing to do with the Compress BlockAck frame format. They are also duplicated in clause 3 (defintions) and 11.5.1.1 and 11.5.1.2. | Delete the quoted text. | Accept As per commentar recommendation and consideration |
| 144 | 29.15 | 7.2.1.8.2 |  | This definition has nothing to do with compressed BA per-se and does not belong here. | Delete or move to Section 3, or at the starting of Block ACK section. | Accept the deletion as recommended and as in the CID 2048 |
| 145 | 29.18 | 7.2.1.8.2 |  | This definition has nothing to do with compressed BA per-se and does not belong here. | Delete or move to Section 3, or at the starting of Block ACK section. | Accept the deletion as recommended and as in the CID 2048 |
| 147 | 29.42 | 7.2.1.8.2 |  | "and is set to the same value as in the immediately previously received BlockAckReq frame" "Immediately previous" may not always be correct, what happens if this is HT-delayed BA and the immediately previous BAReq was for a different TID? | Change sentence to "and is set to the same value as in the immediately previously received BlockAckReq frame of the same TID that is being acknowledged." | Counter Solved in CID 2037 by adding text that refers to 9.10.7.5 |
| 669 | 29.42 | 7.2.1.8.2 |  | According to 9.10.7.5, the Starting Sequence Number is not aleays necessarily set to the same value in the immediately previously received BlockAckReq frame | Delete the entire sentence and provide a reference to clause 9.10 for the rules on setting the SSN in the BA. | Counter Solved in CID 2037 by adding text that refers to 9.10.7.5 |
| 2049 | 29.60 | 7.2.1.8.2 |  | "This is indicated by the value 1 in the Compressed Bitmap field of the BA Control Field." The meaning of "this" is ambiguous. However, it can at most be a functional duplication of table n11. | Remove the quoted text. | Accept As per commentar recommendation |
| 1880 | 30.00 | 7.2.1.8.3 |  | It is not clear if a station is allowed to include in a MTBA frame the BA Information fields related to all the BA agreements established under PSMP policy, regardless of the TIDs of the frames correctly received during the previous PSMP transmit opportunity (either UTT or DTT). Note: obviously such previous PSMP trasmit opportunity shall include at least either an explicit or implicit MTBA request. | Allow such behaviour if not currently allowed. | Transfer to PSMP |
| 2052 | 30.22 | 7.2.1.8.3 |  | "The TID_INFO subfield of the BA Control field of the Multi-TID BlockAck frame contains the number of instances Per TID Info, of the Block Ack Starting Sequence Control field and Block Ack bitmap present in the MTBA as given by TID_INFO + 1, i.e., a value of 2 in the TID_INFO field means that there are 3 copies of the Block Ack Starting Sequence Control field and Block Ack bitmap values present in the Multiple TID Block Acknowledgement frame’s BA Information field." This is misleading, ungrammatical and way too long. | Replace with: "The TID_INFO subfield of the BA Control field of the Multi-TID BlockAck frame contains the number of TIDs, less one, for which information is reported in the BA Information field. For example, a value of 2 in the TID_INFO field means that information for 3 TIDs is present." | Accept As per commentar recommendation |
| 148 | 30.28 | 7.2.1.8.3 |  | "The TID_INFO value in the Multiple TID Block Acknowledgement frame’s BA Information field may contain a value that is different from the TID_INFO value of the corresponding MTID BlockAckRequest frame." 1. What does "different mean", more or less? Can the transmitter request for 3 TIDs and the responder respond with only 2 TIDs? Is that the meaning? If so what happens to the outstanding TID BA? Whose responsibility is it to retrieve the BA for any outstanding TIDs, the receiver's or the transmitter's? 2. What happens if the receiver sends back BAs for more TIDs than requested? There is no restriction on the receiver to respond with the BAs of the asked for TIDs, it could possibly respond with BAs of unasked for TIDS. In these two cases, should the transmitter ignore the BAs of the unasked for TIDs? What happens if the receiver clears its partial state assuming that the transmitter has "accepted" the BA for an unasked for TID? | Define the behavior clearly and unambiguously. | Transfer to PSMP |
| 2053 | 30.32 | 7.2.1.8.3 |  | "The BA Information field within the Multi-TID BlockAck frame comprises 1 or more copies of the Per TID Info, Block Ack Starting Sequence Control field and the Block Ack bitmap, as shown in Figure n10 (BA Information field (MTBA)). The number of copies of the Per TID Info, Block Ack Starting Sequence Control field and the Block Ack bitmap that appear in the Multi-TID Block Ack frame’s BA Information field is indicated by the TID_INFO field of the BA Control field" There are a number of things wrong with this. Firstly, the Per TID info etc.. are not copies, but may contain distinct values. Secondly, the TID_INFO field has already been defined, so there's no need to do it all over again. | Replace with the following: "The BA Information field within the Multi-TID BlockAck frame contains 1 or more instances of the Per TID Info field, Block Ack Starting Sequence Control field and the Block Ack bitmap, as shown in Figure n10 (BA Information field (MTBA))." | Transfer to PSMP |
| 2054 | 30.41 | 7.2.1.8.3 |  | "The Starting Sequence Number subfield is the sequence number of the first MSDU for which this BlockAck is sent, and is set to the same value as in the immediately previously received BlockAckReq frame" This is not necessarily so. MTBA can sent implicitly, without any previous BAR. | Replace quoted text with: "The Starting Sequence Number subfield is the sequence number of the first MSDU for which this BlockAck is sent. The value to go in this field is defined in 9.10.7.5." | Transfer to PSMP |
| 2056 | 30.60 | 7.2.1.8.3 |  | "The Block Ack bitmap within the Multi-TID BlockAck frame contains an 8-octet compressed Block Ack Bitmap," The problem here is that the terminology "compressed Block Ack bitmap" doesn't agree with the name of the field "Block Ack Bitmap". There's no need to keep repeating compressed. | Remove all instances of "compressed" in this paragraph. | Accept As per commentar recommendation |
| 2106 | 63.42 | 7.3.2.49.2 |  | Why have we got a field for HT-delayed BA support? There is already a capability bit for delayed BA support. Does it actually mean support for the "no ack" BA Ack Policy? | Either rename it and describe it as support for the "no ack" BA Ack Policy setting under delayed BA, or remove the field. | Reject This capability is to support the HT-delayed BA as it is defined in 9.10.8.3 Operation of HT-delayed BlockAck This section includes relation between delayed BA and RD and the dynamical behavior of no-ACK as well. It is not part of the Delayed Block Ack capability So no need to rename the capability |
| 679 | 88.00 | 7.4a.4 |  | Disallow Block ACK to be aggregated with data frames in A-MPDU - Block ACK is sent at rates to provide reliable transmissions. In A-MPDU all the MPDU's are transmitted at the same rate - In some instances you might want to get an ACK for the Block Ack. It is not clear if you can do that all the time when you aggregate with other MPDU's - Block ACK is a control frame that can be used for improved protection from non-HT STAs but when aggregated with other A-MPDUs you loose the benefit | Update Table n42 to disallow aggregation of Block ACK in A-MPDU | This issue has been already solved by resolution to CID 672 (reject) in Doc: 11-07-0542-02-000n-lb97-frame-format-comments-xls-mpdu |
| 2142 | 88.01 | 7.4a.4 |  | "The A-MPDU shall only contain MPDUs as described in Table n42 (A-MPDU contents using HT-immediate BlockAck), Table n43 (A-MPDU contents using HT-delayed BlockAck), and Table n44 (A-MPDU contents MPDUs using MTBA/PSMP)." It is not clear if these are mutually exclusive or additive - i.e. can you mix delayed Block Ack and immediate Block Ack. If it is a "pick and choose" from the tables, does the constraint "One TID per aggregation" apply to the whole A-MPDU, or just bits chosen from that table? Also it's unclear how the "using explicit table" applies. Is this a new special case, or does it apply to any table (e.g. PSMP)? If it does apply to PSMP, how can CTS be sent as the NAV will be set by the preceding PSMP frame. | I think the logic goes as follows: .Identify whether it's PSMP or not. If it is, the contents are defined by the PSMP table (copy the relevant explicit feedback entries there) Otherwise, the aggregation is limited to data and/or an immediate BA of a single TID. The Ack Policy and then BA policy set up for that TID identifies which table applies. I think this can be described through an initial table that calls out these conditions in columns, and has rows that point to the table that defines the permitted contents. | Counter Remove the Delayed BA from the spec (CID 2834) Remove CTS from the Table 45n and fix the related places in the explicit beamforming. Implemented in Doc: 11-07-xxxx-00-000n-tgn-lb97-frame-formats-BA |
| 1943 | 88.12 | 7.4a.4 |  | Comments for BlockAck MPDUs should specify that this case is only possible in RDG scenarios. | Update comments accordingly. | Reject BlockAck aggregation is not limited to RDG only. The BlockAck can be aggregated with Action no Ack as well. The proper aggregation behavior has been already specified |
| 2143 | 88.17 | 7.4a.4 |  | It is not clear if "One TID per aggregation" applies only to the QoS Data, or it also applies to the BlockAck. |  | Reject This particular row applies to the QoS data only as it is stated in header of the most left column No such a limitations is defined for Block Ack - see the row of Block Ack |
| 2834 | 88.26 | 7.4.a.4 |  | The HT-delayed BA does not bring any added value. As it is mentioned in the basic spec the delayed Block Ack mechanism is primarily intended to allow existing implementations to use this feature with minimal hardware changes and also to allow inexpensive implementations that would use the processing power on the host. As long the HT-immediate BlockAck is mandatory in .11n and any resources limitation related to this feature are already solved by the partial state approach there is no need for the delayed mechanism at all. | Remove the HT-delayed BlockAck from the spec | Straw poll: Remove the HT-delayed BA from the spec Yes - No - Abstain - |
| 155 | 89.23 | 7.4a.4 |  | This problem is not unique to 11n: In EDCA, there is no way to differentiate between two flows, and the same seqn number space is being used for both flows (one with NoACK and and one with BlockACK). If the receiver misses a NoACK frame, it will have a hole in the received seq space, and cannot release the non-no-ack frames from the rx reorder buffer, not knowing that a NoACK frame is missing. | Allow only one ACK Policy at a time for the same RA/TID to prevent this problem from occurring. If a BA setup has been done for an RA/TID, disallow MSDUs with QoSNOACK at the MAC-SAP. | Counter Already solved by CID 154 Doc: 11-07-0542-02-000n-lb97-frame-format-comments-xls-mpdu |

CID 2010

TGn Editor:  Change the text in 7.1.3.5.3 (Ack Policy subfield) on page 17 at line 27 in D2.02 as follows:

… according to the procedures defined in 9.2.8a, 9.3.3 and 9.9.2.3. 9.10.7.5, 9.10.8.3, 9.14.3 and 9.17.3

CID 2142

TGn Editor:  Remove the row that contains CTS in Table 57v (A-MPDU contents MPDUs using explicit feedback) on page 85 at line 44 in D2.02

TGn Editor:  Change the text in 9.17.3 (Explicit feedback beamforming) on page 154 that starts at line 45 in D2.02 as follows:

- If a control response frame (#2418) ACK or BA is required (CTS, ACK, BA), (#2418) both the feedback response frame (#2405) and the control response frame may be aggregated in an A-MPDU. If a CTS response is required the feedback response shall not be aggregate with CTS and may be sent in A-MPDU aggregate with an ACK or BA in the same TXOP. Otherwise, the feedback response frame (#2405) shall be sent a SIFS after the reception of the sounding PPDU. If NDP sounding is used, the transmission of the feedback response frame (#2405) may follow the NDP, but the control response frame is transmitted a SIFS after reception of the PPDU that elicited the control response.