IEEE P802.11
Wireless LANs

| TGn LB97 Submission containing proposed comment resolutions for ad-hoc “COEX” comment-group “DFS Channel Selection” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +972547885738 | solomon.trainin@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comments assigned to the author:

- CIDs  1770 Accept

- CIDs  2572, 2893, 2575, 2894 Defer to sync with TGy draft 3.0

The changes marked in this document are based on TGn Draft version D2.02.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2572 | 198.40 | 11.9.7.1 |  |  | "If an AP is switching to a channel within the same regulatory class, and dot11ExtendedChannelSwitchImplemented is true, then the AP shall send the Extended Channel Switch Announcement element and Extended Channel Switch Announcement frame, else the AP shall send the Channel Switch Announcement element and Channel Switch Announcement frame. Optionally the Extended Channel Switch Announcement element and Extended Channel Switch Announcement element frame may be used." This option is always true for HT APs. So it says when we switch within the same regulatory class, we must use the extended element/frame. This would ensure that legacy .11h STA's get left behind. | Should allow the use of the normal channel switch signalling as well as the extended signalling. | Defer to sync with TGy D3.0 The 11.9.7 (Selecting and advertising a new channel) is not a part of the TGn draft |
| 2893 | 198.41 | 11.9.7.1 |  |  | The dot11ExtendedChannelSwitchImplemented is subject of AP and STA as well. How knows the DFS owner that the dot11ExtendedChannelSwitchImplemented is true in all recipients of the Extended Channel Switch Announcement? | Add Extended Channel Switch Implemented subfield in the Extended Capabilities element. Add a following rules to this subclause "A STA shall set the Extended Channel Switch Implemented subfield in the Extended Capabilities element to 1 if the STA’s dot11ExtendedChannelSwitchImplemented dot11MultiDomainCapabilityEnabled are true; otherwise, it shall be set to 0. A STA shall set dot11ExtendedChannelSwitchImplemented to TRUE before associating with a BSS or IBSS in which the Extended Channel Switch Implemented subfield is set to 1 in the Extended Capabilities element in Beacon frames and Probe Response frames received from the BSS or IBSS." | Defer to sync with TGy D3.0 The 11.9.7 (Selecting and advertising a new channel) is not a part of the TGn draft |
| 2575 | 198.61 | 11.9.7.2 |  |  | "If a DFS owner is switching to a channel within the same regulatory class, and dot11ExtendedChannelSwitchImplemented is true, then the DFS owner shall send the Extended Channel Switch Announcement element and Extended Channel Switch Announcement frame," This option is always true for HT APs. So it says when we switch within the same regulatory class, we must use the extended element/frame. This would ensure that legacy .11h STA's get left behind. | Should allow the use of the normal channel switch signalling as well as the extended signalling. | Defer to sync with TGy D3.0 The 11.9.7 (Selecting and advertising a new channel) is not a part of the TGn draft |
| 2894 | 199.14 | 11.9.8.1 |  |  | The definition mandates that HT STA sets to true all the following attributes: dot11RegulatoryClassesImplemented, dot11RegulatoryClassesRequired, dot11ExtendedChannelSwitchImplemented. It means that all HT STA implements the Extended Channel Switching. Not in all environments there is need for it. For example this feature is not so important for the 20MHz capable Home BSS that sits in 2.4GHz band. So this feature should be configurable | Add Extended Channel Switch Implemented subfield in the Extended Capabilities element. Add a following rules to this subclause "A STA shall set the Extended Channel Switch Implemented subfield in the Extended Capabilities element to 1 if the STA’s dot11ExtendedChannelSwitchImplemented and dot11MultiDomainCapabilityEnabled are true; otherwise, it shall be set to 0. A STA shall set dot11ExtendedChannelSwitchImplemented to TRUE before associating with a BSS or IBSS in which the Extended Channel Switch Implemented subfield is set to 1 in the Extended Capabilities element in Beacon frames and Probe Response frames received from the BSS or IBSS." | Similar to 2893 Defer to sync with TGy D3.0 |
| 1770 | 201.01 | 11.9.8.4 |  |  | 11.9.8.1 states "For an HT STA, the following MIB attributes shall be set to true: dot11RegulatoryClassesImplemented, dot11RegulatoryClassesRequired, and dot11ExtendedChannelSwitchImplemented." so why is this if statement necessary? | remove paragraph | Accept As the commenter recommends |