IEEE P802.11
Wireless LANs

| TGn LB97 Submission containing proposed comment resolutions for ad-hoc “COEX” comment-group “Channel Switching” |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +972547885738 | solomon.trainin@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comments assigned to the author:

- CIDs 1935, 1936, 3103, 557, 2121, 2592, 1089, 2120, 2121, 2122, 3332, 3331, 263, 264, 3333 Accept or Counter

- CIDs 546, 3102, 2187 Reject

- CIDs  419, 2123, Transfer to editorial

- CIDs  2487 Transfer to different document of the same Ad Hoc group

The changes marked in this document are based on TGn Draft version D2.02.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| CID | Page | Clause | Duplicate of CID | Resn Status | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 546 | 73.06 | 7.3.2.50 |  |  | The form of the directions in the Encoding field does not match most of the other entries in the table (i.e., "Set to x if…"). | Change the first line to: "Set to 0 if only 20 MHz transmissions may be sent to the STA". Change the second line to: "Set to 1 if any channel width in the Supported Channel Width set may be used when sending to the STA". | Reject There are different styles of encoding description in the table. This style fits the definition. |
| 419 | 73.07 | 7.3.2.50 |  |  | "(subject also its Supported Channel Width Set)." Y'know, I can't figure out what this is trying to say. "subject to its Supported Channel Width Set"? "subject is also its Supported Channel Width Set"? | Please clarify | Transfer to editorial |
| 1935 | 73.30 | 7.3.2.50 |  |  | The text is unclear on whether Operating mode is determined based on capabilities or current conditions. For example, if a 40 MHz capable STA changes its channel width from 40 to 20 MHz, does this require that the AP which was formerly in Operating Mode "0" now indicate its Operating Mode as "2"? | Clarify the text | Counter Implement as in document 11-07-2055-00-000n-tgn-lb97-COEX-ch-switching |
| 1936 | 73.30 | 7.3.2.50 |  |  | The text is unclear on how Operating mode is (or is not) affected when an HT AP changes its channel width. | Clarify the text | Counter See CID 1935 |
| 3102 | 77.00 | 7.4.1.5 |  |  | Secondary Channel Offset value 0 is redundant | Change the sentence "It may be present when switching to a 20 MHz channel (in which case the secondary channel offset is set to 0)." to: "in 20 MHx channel Secondary Channel Offset element is not present." | Reject The current definition allows eliminating the Secondary channel Offset element when switching to a 20 MHz channel. |
| 3103 | 80.00 | 7.4.9.2 |  |  | No definition when STA should use this Action Frame. | Add following definition: " STA sends Notify Channel Width Action Frame to AP in the BSS or to an all non-AP STAs in the IBSS if it wants to change the existing channel width operation mode. AP complies with STA request by changing the Operating mode of BSS. All STAs in the IBSS. STAs in the IBSS comply with STA request by changing the Operating mode of IBSS" | Counter Implement as in document 11-07-2055-00-000n-tgn-lb97-COEX-ch-switching |
| 557 | 80.13 | 7.4.9.2 |  |  | This clause should contain an informative note indicating that protection requirements in the BSS do not change as a result of this frame. | Add: "Note: Notify Channel Width has no effect on BSS requirements for protection of 40 MHz transmissions. A 20/40 MHz capable STA which has signaled use of the 20 MHz primary channel using Notify Channel Width must still receive 40 MHz transmissions (e.g., for updating the NAV). A STA which will only receive 20 MHz transmissions must reassociate, indicating support for 20 MHz only. See 9.20.4 (Switching between 40 MHz and 20 MHz)." | Counter, There is no protection mode associated with transmission of 40MHz channel width frames so no need to address this part of the recommendation. Insertion of reference to the behavioral subclause should be implemented as in CID 3103 |
| 1089 | 80.13 | 7.4.9.2 |  |  | normative statements don't belong in clause 7 | move this paragraph to an appropriate place in the standard | Counter As in CID 3103 |
| 2120 | 80.13 | 7.4.9.2 |  |  | "If an AP wishes to receive 20 MHz packets, it broadcasts this management action frame to all STAs." We probably need to move this to 9.20.4, or delete it as a duplicate. | Move normative text to clause 9 or delete if it is a duplicate. | Counter As in CID 3103 |
| 2121 | 80.13 | 7.4.9.2 |  |  | "This frame can be sent by both non-AP STA and AP." This is inaequate, we need a reference. | Replace with: "The Notify Channel Width frame is used by both non-AP STAs and APs to determine what channel width frames they currently are able to receive, following the rules defined in 9.20.4." | Counter As in CID 3103 |
| 2122 | 80.14 | 7.4.9.2 |  |  | "In addition, the AP indicates its current STA Channel Width in the HT Information Element in the beacon." This has no relevance to the Notify Channel Width frame format. | Delete it - it's a duplicate of what's in 9.20.4. | Counter As in CID 3103 |
| 2123 | 80.18 | 7.4.9.2 |  |  | "The Category field is set to 7 (representing HT).". "The Action field is set to 0 (representing Notify Channel Width)." This is duplication of normative definition, which is bad. | Replace with: "The Category field is set to the value for HT, specified in Table 24." "The Action field is set to the value for Notify Channel Width, specified in Table n31" Make similar changes for all the HT Action frames. | Transfer to editorial |
| 2187 | 109.61 | 9.6.8 |  |  | "An HT STA which transmits a control response frame shall send the frame using the same channel width as that on which it received the frame that elicited the response transmission." This is a rather complex statement. Needs simplification. | "An HT STA that receives a frame that elicits a control frame transmission shall send the frame using the same channel width as the received frame". | Reject The change does not reflect the difference of the channel width the frame is sent and the channel width it is actually received by the responding STA. It may differ in case of non-HT dup. |
| 2487 | 164.24 | 9.20.4 |  |  | This subclause mixes a number of topics: 1. Scanning and detection of intolerance/OBSS 2. Signalling a change in BSS channel width 3. Signalling a change in STA operating channel width These should be described separately | Separate this subclause according to the themes identified above. | Transfer this CID to be solved in the document 11-07-2020-01-000n-lb97-cid-2582-bss-width-switching |
| 3331 | 201.57 | 11.15.1 |  |  | There is no BSS Channel Width field in any element. | Change "BSS Channel Width" to "Secondary Channel Offset" | Counter The same resolution as CID 263 |
| 3332 | 201.57 | 11.15.1 |  |  | It would be handy to describe what the other field is for. There are three fields in the two elements and this text only discusses two of the three. What is the meaning of the STA Channel Width field of the HT Information element? | Provide an explanation of "the third field" | Counter The document 11-07-2020-07-000n-lb97-cid-2582-bss-width-switching explains in details relation between three mentioned fields |
| 263 | 201.58 | 11.15.1 |  |  | "BSS Channel Width" is not a subfield in HT Information element | Change "BSS Channel Width" to "STA Channel Width" | Counter Implement as in document 11-07-2055-01-000n-tgn-lb97-COEX-ch-switching |
| 264 | 202.01 | 11.15.1 |  |  | "BSS Channel Width" is not a subfield in HT Information element | Change "BSS Channel Width" to "STA Channel Width" | Counter The same resolution as 263 |
| 3333 | 202.07 | 11.15.1 |  |  | Allow STA to send 20 mhz control frames on the secondary channel. | Make the change indicated in the comment. | Counter The same resolution as in CID 3272 in 11-07-0614-10-000n-lb97-coex-20-40-coex-comments-resolution |
| 2592 | 202.14 | 11.15.1 |  |  | "The PPDU Formats for Beacon transmission are defined in Table n53 (PPDU Formats for Beacon transmission)." Isn't this already covered in detail in 9.6.2.1? | Delete the quoted sentence and table n53. | Accept As per commenter recommendation |

CID 1935

TGn Editor:  Add following note on page 73 at line 18 in the column “Encoding” in the Table 43l (HT Information element (continued)) in D2.02

NOTE – Any change of STA Channel Width value does not impact the value of HT protection

CID 3103

TGn Editor:  Insert following sentence as a first one on page 77 at line 63 in the subclause 7.4.8.2 (Notify Channel Width frame format) in D2.02

An STA sends Notify Channel Width Action Frame to another STA if it wants to change the Channel Width of frames that the STA sends to it. See definition in 9.20.4

CID 263

TGn Editor: In D2.05 on page 191 at line 38 change the text as follows:

An AP switches (#43) the BSS channel width (#43) between 20/40 MHz BSS and 20 MHz BSS by changing the value …

TGn Editor: In D2.05 on page 193 at line 42 replace “BSS” by “STA” in two places

TGn Editor: In D2.05 on page 193 at line 50 change the text as follows:

A 20/40 MHz capable HT STA, 20 MHz capable HT STA or non-HT STA may be associated in a BSS with BSS Channel Width equal to 20/40 MHz BSS.

TGn Editor: In D2.05 on page 194 at line 4 replace “BSS Channel Width” by “AP Supported Channel Width set”

TGn Editor: In D2.05 on page 194 at line 24 change the text as follows:

A PCO capable AP advertises its BSS as 20/40 MHz BSS Channel Width and PCO Capable