IEEE P802.11
Wireless LANs

| TGy LB106 Submission for non-Editorial comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., San Jose, CA 95134 | 408-527-0815 | petere@cisco.com |

Abstract

This document is aligned with P802.11-2007, P802.11k/D7.0, P802.11r/D6.1 for baseline, P802.11y/D3.0, and  submissions 07/2058r3 (misc PHY comments), 07/2080r2 LB106 Submission for DSE comments and 07/2072r2 LB106 Editorial comments and addresses the following LB106 comments:

CIDs: 2006, 2013, 2014, 2015, 2021, 2039, 2045, 2053, 2062, 2063, 2066, 2067, 2068, 2070, 2072, 2073, 2074

CIDs: 2002, 2075, 2076, 2077, 2078

CIDs: 2003, 2004, 2008, 2016, 2023, 2044

## Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGy Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGy Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGy amendment with the baseline documents).

TGy Editor:  Editing instructions preceded by “TGy Editor” are instructions to the TGy editor to modify existing material in the TGy draft.   As a result of adopting the changes, the TGy editor will execute the instructions rather than copy them to the TGy Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Clauses 7, 9 and 10

CIDs: 2006, 2013, 2014, 2015, 2021, 2039, 2045, 2053, 2062, 2063, 2066, 2067, 2068, 2070, 2072, 2073, 2074

Discussion:

In 11y/D3.0:

| 2072 | 7.2.3.1 | 3 | 24 | T | Y | As it is stated in the subclause 11.9 of the basic spec "STAs shall use the DFS procedures defined in this subclause if dot11SpectrumManagementRequired is true." The Extended Channel switch functionality is part of the 11.9 definition, so both attributes dot11SpectrumManagementRequiredshoud and dot11ExtendedChannelSwitchImplemented should be mentioned as requirement for the Extended Channel Switch Announcement information element presence. The same comment applies to any appearance of the Extended Channel Switch Announcement in 7.2.3.4 - 7.2.3.9 | The attribute dot11SpectrumManagementRequired enables wide range of features. In the current spec there is no way to separately declare support of them. Clear specification should be provided to allow or disallow separate use of the extended channel switching |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2073 | 7.2.3.1 | 3 | 27 | T | Y | As it is stated in the subclause 11.9 of the basic spec "STAs shall use the DFS procedures defined in this subclause if dot11SpectrumManagementRequired is true." The Supported Regulatory Classes functionality is part of the 11.9 definition, so both attributes dot11SpectrumManagementRequiredshoud and dot11ExtendedChannelSwitchImplemented should be mentioned as requirement for the Supported Regulatory Classes information element presence. The same comment applies to any appearance of the Supported Regulatory Classes in 7.2.3.4 - 7.2.3.9 | The attribute dot11SpectrumManagementRequired enables wide range of features. In the current spec there is no way to separately declare support of them. Clear specification should be provided to allow or disallow separate use of the Supported Regulatory Classes information element. |

Comments 2072 and 2073 note that DFS procedures, (in particular 11.9.7 Selecting and advertising a new channel), are available only when dot11SpectrumManagementRequired is true, yet P802.11y uses dot11ExtendedChannelSwitchImplemented in Clause 7 text. Suggested Remedy is to have individual MIB variables for Extended Channel Switch Announcement and Supported Regulatory Classes.

Comment Resolution:Commenter writes "I would see that the text at the line 24 on page 3 should be changed this way:

The Extended Channel Switch Announcement information element may be present only if dot11ExtendedChannelSwitchImplemented, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired are true." Will add a normative statement in 11.9.7 “When dot11ExtendedChannelSwitchImplemented is true, dot11MultiDomainCapabilityEnabled, dot11SpectrumManagementReqired and dot11RegulatoryClassesRequired shall be true.”

Propose Accept in Principle Comments 2072 and 2073

| 2021 | 7.2.3.1 | 3 | 20 | T | Y | The Notes for DSE registered location here and elsewhere in Clauses 7 and 10 refer to 'if dot11DSERequired is true or dot11RegLocRequired is true', when dot11LCIDSERequired is true when either of these is true. The quoted text should be replaced by 'if dot11LCIDSERequired is true' everywhere it occurs. | Per comment |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “also replace 'if dot11RegLocRequired is true or dot11DSERequired is true'”

Proposed Accept in Principle Comment 2021.

Tgy Editor: Replace 'if dot11DSERequired is true or dot11RegLocRequired is true' and ‘if dot11RegLocRequired is true or dot11DSERequired is true’ with 'if dot11LCIDSERequired is true' in D3.0

| 2045 | 7.2.3.9 | 5 | 17 | T | Y | The Supported Regulatory Classes element in Probe Response "is present if … is true" However, a Supported Regulatory Classes element in a Beacon (see 7.2.3.1) "may be present if … is true" | Claify why is there a difference, and correct as appropriate. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Commenter notes inconsistency between Beacon frame and Probe Response frame for the same IE. Proposed Comment Resolution: “Many persistent Beacon frame elements (11, 14, 17, 18, 21) are Noted as "shall be present". Few Probe Response frame elements (13, 16, 17) use "shall be present", most (6, 7, 8, 9, 18, 19, 20, 21, 22) use "is present.” Will change Supported Regulatory Classes element Notes in Beacon to “shall be present” and delete “only.”

Propose Accept Comment 2045.

Tgy Editor: Change Notes in Supported Regulatory Classes in 7.2.3.1 pg 3 line 28 in D3.0

"shallmay be present only if”

| 2053 | 7.3.2.22.11 | 7 | 24 | T | Y | The text states that it is mandatory for a STA to support the generation of a DSE report if dot11LCIDSERequired is true. However, the next sentence says it is always optional | Remove contradiction |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution : Accept

Tgy Editor: Delete third sentence in 7.3.2.22.11, pg 7, D3.0:

It is optional for a STA to support the generation of this report.

| 2039 | 7.3.2.50 | 10 | 63-64 | T | Y | A zero value for the Channel Switch Count field to indicate "… that the switch occurs anytime after the frame containing the element is transmitted." seems to be useless. If the transition occurs arbitarirly, there is no use in telling other STAs. | Delete sentence and reword previous sentence to become "A value of 0 indicates that the switch occurs immediately before the next TBTT." |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment 2039 on 7.3.2.50 ECSA. Proposed Comment Resolution: ‘Text is identical to 7.3.2.20 CSA, and works identically. The AP says "I am leaving this channel", and the receiving STAs know not to wait TBTT before tuning to another frequency. Propose Reject Comment 2039.

| 2013 | 9.8.3 | 13 | 12 | T | Y | The 802.11-2007 standard is silent about parcing the Country Information element information, and the statements should apply to any station when dot11RegulatoryClassesImplemented is true. | Remove "and dot11LCIDSERequired is true" from the first sentence, and add a PICS capability tests about Clause 9.8.3 to A.4.12. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2014 | 9.8.4 | 13 | 48 | T | Y | The 802.11-2007 standard is silent about resolving situations where the received Max Transmit Power Level differs from the Transmit Power limit indicated by the Regulatory Class, and the statement should apply to any station when dot11RegulatoryClassesImplemented is true. | Remove "and dot11LCIDSERequired is true" from the sentence, and change PICS A.4.13 RC6 and RC7 accordingly. |
| 2015 | 9.8.4 | 13 | 42 | T | Y | The 802.11-2007 standard is silent about resolving situations where the Country Information element is received by an unassociated station, and the statement should apply to any station when dot11RegulatoryClassesImplemented is true. | Remove "and dot11LCIDSERequired is true" from the sentence, and change PICS A.4.13 RC5 accordingly. |
| 2062 | 9.8.3 | 13 | 12 | T | N | Please delete condition that dot11LCIDSERequired is true. | Delete altogether or make optional so condition is not resitcted to dependent station operations only. |
| 2063 | 9.8.4 | 13 | 42 & 49 | T | N | Please delete condition that dot11LCIDSERequired is true. | Delete altogether or make optional so condition is not resitcted to dependent station operations only. |
| 2074 | 9.8.3 | 13 | 12 | T | Y | As it is stated in "When dot11RegulatoryClassesImplemented is true and dot11LCIDSERequired is true, the following statements apply:" the defined rules applies to the STA that enables the Dependent Station Enablement procedures only. It seems that the rules may be useful for any station that operates with regulatory classes | Extend the rules for any station that operates with regulatory classes |

Comments 2013, 2062 and 2074, 9.8.3, note the base standard is silent about the creation or parcing of Regulatory Class information in the Country Information element. Proposed Comment Resolution: “Will Remove "and dot11LCIDSERequired is true" from the first sentence, and add a PICS capability tests about Clause 9.8.3  to A.4.10.” Propose Accept Comment 2074. Propose Accept in Principle Comments 2013 and 2062.

Comments 2014, 2015 and 2063, 9.8.4, note the base standard is silent about receiving the Country Information element by an unassociated station and about when Max Transmit Power Limit differs from the Transmit Power limit indicated by the Regulatory Class. Proposed Comment Resolution: “Will Remove "and dot11LCIDSERequired" from the fourth sentence, "and dot11LCIDSERequired is true" from the sixth sentence, and change PICS capability tests about Clause 9.8.4 in A.4.13.” Propose Accept in Principle Comments 2014, 2015, 2063.

Propose Accept Comments 2006, 2066, 2067, 2068 and 2070 without discussion.

Proposed Resolution:

Accept: 2006, 2013, 2045, 2053, 2066, 2067, 2068, 2070 and 2074

Accept in Principle based on discussion and editorial instructions in 07/2073r1: 2014, 2015, 2021, 2062, 2063 and 2072 and 2073

Reject based on discussion in 07/2073r1: 2039

Editorial Instructions:

Tgy Editor: change text in Figures 117a and 117b, pg 12, D3.0 from ‘Action’ to ‘Action Value’

Tgy Editor: change text in 7.4.1.6, pg 12 line 17, D3.0 as follows

"element is set as field contains the Extended Channel Switch Announcement element, excluding the Element ID and Length fields, described in 7.3.2.50."

Tgy Editor: change length of ExtendedChannelSwitchAnnouncement element from 6 to 4 in Figure 117a, pg 12 line 6, D3.0

Tgy Editor: change text in 7.4.1.7, pg 12 line 42, D3.0 as follows

"element is set as field contains the DSE Registered Location Announcement element, excluding the Element ID and Length fields, described in 7.3.2.49."

Tgy Editor: change text in 9.8.3 and 9.8.4, pg 13, D3.0 as follows

### 9.8.3 Operation with Regulatory Classes

When dot11RegulatoryClassesImplemented is true and dot11LCIDSERequired is true, the following statements apply:

- When dot11RegulatoryClassesRequired is false, or where Regulatory Classes domain information is not present in a STA, that STA is not required to change its operation in response to an information element or element-specific information field that contains a Regulatory Class

- When dot11RegulatoryClassesRequired is true, or where Regulatory Classes domain information is present in a STA, the STA shall indicate Regulatory Class information in the Country Information element and Supported Regulatory Classes information element;

- When dot11RegulatoryClassesRequired is true and a STA is capable of operating as specified in more than one Regulatory Class, the STA shall include the Country Information and SupportedRegulatoryClasses elements in Association frames and Reassociation frames;

- When dot11RegulatoryClassesRequired is true, or where Regulatory Classes domain information is present, and the STA parsing a Country Information element finds a First Channel Number or Regulatory Class with a Reserved value, the STA shall ignore the remainder of the Country Information element and shall parse any remaining management frame body for additional information elements.

9.8.4 Operation with Coverage Classes

The default PHY parameters are based on aAirPropagationTime having a value of 1 µs or less, and SlotTime and other MAC timing are based on the PHY timing parameters. When dot11RegulatoryClassesImplemented is true, it is possible to manage the MAC timing of stations that can receive Beacon frames or Probe Responses frames that contain the Country Information element (7.3.2.9), to increase fairness in contending for the medium. Radio waves propagate at ~300 m/µs in free space, and, for example, 3 µs would be the ceiling for BSS maximum one way distance of ~450 m (~900 m round trip). When dot11RegulatoryClassesImplemented is true and dot11LCIDSERequired, the Coverage Class field of the Country Information element shall be processed when received by the associated STA or dependent STA, replacing the current aAirPropagationTime used in IFS (9.2.3) with one indicated by the Coverage Class field.

Using the Country Information element, an AP can change Coverage Class and Max Transmit Power Level to enhance outdoor operation. When dot11RegulatoryClassesImplemented is true and dot11LCIDSERequired is true, and the Max Transmit Power Level is different from the Transmit Power limit indicated by the Regulatory Class, the associated STA or dependent STA shall operate at a transmit power at or below that indicated by the lesser of the two limits.

Tgy Editor: change text in 10.3.36.1.4, pg 20 lines 64-65, D3.0 as follows:

On receipt of this primitive, the MLME constructs an Extended Channel Switch Announcement frame and sends it to the PLME for transmission.

On receipt of this primitive, the MLME constructs and transmits an Extended Channel Switch Announcement frame.

# Clause 11.9.7 CSA procedures

CIDs: 2002, 2075, 2076, 2077, 2078

| 2075 | 11.9.7.1 | 24 | 23 | T | Y | Using of an Extended Channel Switch Announcement element and frame and a Channel Switch Announcement element and frame actually will present the same information so it is not clear why the use of the Extended Channel Switch Announcement element and frame is mandated. The same comment applies to 11.9.7.2 | Explain clearly when each of the infromation elements and frames should be used and why |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The referenced text is when switching “to a new channel in a different regulatory class”, and if sending both ECSA and CSA the information that is the same are the channel switch mode, count and New Channel Number, and the information that differs is the New Regulatory Class, whose requirements are unique per regulatory class.

Proposed Comment Resolution: “The change in regulatory class is the information that differs between ECSA and CSA. The only cases where regulatory class is changed and both ECSA and CSA are sent, are when the requirements signified by the new regulatory class are met by all STAs that act on the Channel Switch Announcement.”

Propose Accept in Principle Comment 2075.

| 2076 | 11.9.7.1 | 24 | 38 | T | Y | Paragraph that starts at line 38 does not define behavior of the Extended Channel Switch Announcement element | Define behavior for the Extended Channel Switch Announcement element |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution:

“Will change initial text of second paragraph to “In the following text:” and make corresponding change to 11.9.7.2. Commenter writes “add the following text before paragraph the starts with “An AP shall inform associated STAs”:

In the following text, wherever Channel Switch Announcement is referred to both the Extended Channel Switch Announcement and Channel Switch Announcement should be used as defined in 1) and 2)”.”

Propose Accept in Principle Comment 2076

| 2077 | 11.9.7.1 | 24 | 32 | T | Y | "If dot11ExtendedChannelSwitchImplemented is false, the AP shall send the Channel Switch Announcement and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames." | If doc11ExtendedChannelSwitchImplemented is false, how can an AP send both Extended Channel Switch Announcement and the Channel Switch Announcements and frames since the AP does not have the Extended Channel Switch capability? Section 11.9.7.1 is under clause 11 for DFS in 5Ghz band, do the rules specified here also apply to the operation in other bands (e.g. 2.4Ghz and 3.65Ghz)? Clarify. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2078 | 11.9.7.2 | 24 | 64 | T | Y | "If dot11ExtendedChannelSwitchImplemented is false, the DFS shall send the Channel Switch Announcement and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames." | If doc11ExtendedChannelSwitchImplemented is false, how can a DFS owner send both Extended Channel Switch Announcement and the Channel Switch Announcements and frames since the DFS owner does not have the Extended Channel Switch capability? Section 11.9.7.2 is under clause 11 for DFS in 5Ghz band, do the rules specified here also apply to the operation in other bands (e.g. 2.4Ghz and 3.65Ghz)? Clarify. |

There are legacy devices that do not know about channel switch in the 2.4 GHz and 5 GHz band, and there are legacy devices that do not know about regulatory class in the 5 GHz band, and there are legacy devices that do not know about ECSA. In general, channel switch is best effort, and announcements are unacknowledged, and in particular, none of the legacy devices know how to switch regulatory classes.

Proposed Comment Resolutions: “No further clarification needed. The fourth sentence of 11.9 allows DFS procedures to be used in other bands. Other text makes their use mandatory for operation in other bands.

* The STA may choose to implement ECSA without setting dot11ExtendedChannelSwitchImplemented to true

* 802.11n specifies in 11.9.8 that dot11ExtendedChannelSwitchImplemented shall be true for HT STAs operating in either or both of the 2.4 and 5 GHz bands. The 3.65 band is covered by TGy. Comment lacks specific objections with proposed resolution in sufficient detail so that the specific wording of the changes that will cause the negative voter to change his or her vote to "approve" can readily be determined.”

Propose Reject Comments 2077 and 2078.

| 2002 | 11.9.7.3 | 26 | 16 | T | Y | "The List of Regulatory Class(es) field shall list in ascending order all Regulatory Classes that the STA is capable of operating with, for this Country (7.3.2.9)." "Country" is still not defined to my satisfaction. | "The List of Regulatory Class(es) field shall list in ascending order all Regulatory Classes that the STA is capable of operating with, for the Country that is specified in the Country information element (7.3.2.9)." |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comment 2002 without discussion

Proposed Resolution:

Accept: 2002

Accept in Principle based on discussion in 07/2073r1: 2075 and 2076

Reject: based on discussion in 07/2073r1; 2077 and 2078

Editorial Instructions:

Tgy Editor: insert text in 11.9.7, pg 24, D3.0 as follows

Insert new sentence at end of 11.9.7 as follows:

When dot11ExtendedChannelSwitchImplemented is true, dot11MultiDomainCapabilityEnabled, dot11SpectrumManagementRequired and dot11RegulatoryClassesRequired shall be true.

Tgy Editor: change text in second paragraph of 11.9.7.1, pg 24, D3.0 as follows

      - Selecting and advertising a new channel in an infrastructure BSS

Insert new second paragraph and change third paragraph of 11.9.7.1 as follows:

In the following text, wherever Channel Switch Announcement is referred to, the following rules apply:

- If an AP is switching to a new channel in a different regulatory class, then the AP shall use either the Extended Channel Switch Announcement element and frame instead of the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames when the requirements signified by the new regulatory class are met by all STAs that act on the Channel Switch Announcement.

Tgy Editor: change text in 11.9.7.1, pg 24-25, D3.0 as follows

Insert new sentence and change third paragraph of 11.9.7.1 as follows:

In the following text, wherever Channel Switch Announcement is referred to both the Extended Channel Switch Announcement and Channel Switch Announcement should be used as defined in 1) and 2).

Tgy Editor: change text in 11.9.7.2, pg 24-25, D3.0 as follows

Insert new text at start of  first paragraph in 11.9.7.2 as follows:

- 11.9.7.2 Selecting and advertising a new channel in an IBSS

In the following text wherever Channel Switch Announcement is referred to, the following rules apply:

- If a DFS owner is switching to a new channel in a different regulatory class, then the DFS owner shall use the Extended Channel Switch Announcement element and frame instead of the Channel Switch Announcement element and frame, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames when the requirements signified by the new regulatory class are met by all STAs that act on the Channel Switch Announcement.

Tgy Editor: insert sentence in 11.9.7.2, pg 25 line 8, D3.0 as follows

In the following text, wherever Channel Switch Announcement is referred to both the Extended Channel Switch Announcement and Channel Switch Announcement should be used as defined in 1), 2) and 3).

Insert new text after 11.9.7.2:

      - Advertising supported regulatory classes

When dot11ExtendedChannelSwitchImplemented is true, the Current Regulatory Class octet in the Supported Regulatory Classes element shall indicate the Regulatory Class in use for transmission and reception. The List of Regulatory Class(es) field shall list in ascending order all Regulatory Classes that the STA is capable of operating with, for the country that is specified in the Country Information elementthis Country (7.3.2.9).

Annexs A and D

CIDs: 2003, 2004, 2008, 2016, 2023, 2044

| 2003 | A.4.10 | 35 | 54 | T | Y | There is an editor's instruction here to add entries into a table. However, the text here does not contain any entries to be added. | If there are entries to be added, put those entries into the draft. If there are no entries to be added, delete the editor's instruction. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment 2003 says the editing instruction to Insert these entries after OF4.13 Power level appears to be without text. Proposed Comment Resolution: “Frontmatter Page v, Editorial Note 3 says that tables may "float" (this one floated to page 36, line 2), and "Please do not report it as a defect in the draft." Propose Reject Comment 2003.

| 2023 | Annex A | 37 | 57 | T | Y | PICS A.4.17 DSE4, Extended Channel Switch procedure, should be part of A.4.12, after SM20, and depending on CF10. | Per comment |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “CF15 requires CF10, so CF10 does not need to be in the Status field when DSE4 is moved to A.4.12.” Propose Accept in Principle Comment 2023.

| 2004 | D | 44 | 29 | T | Y | The Datum field in the DSE LCI from figure 85q, the DSE registered location from Figure 112z, and the text of 7.3.2.49 all indicate that this is a 3-bit field rather than the 8-bit field defined in RFC 3825. A previous comment 1206 from LB #104 was rejected when it was proposed to expand it to 8-bits, so that 802.11y would use without modification to RFC 3825. From 11-07-673r3 (page 12), the approved change for this item did not include changing the three-bit to 8-bit nor did it change the values from (0..7) to (0..255). | Change 8-bit to three-bit and change (0..255) to (0…7) Thus reflecting the fact that the datum is using only 3-bits (thus 8 possible values (0..7)). |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Commenter's email says "the simple fix is to change (0..255) to (1..3) after the INTEGER for the Datum MIB element". Propose Accept in Principle Comment 2004.

| 2008 | Annex D | 42 | 34 | T | Y | dot11LCIDSETable elements that are defined in RFC-3825 should be stored in the MIB big-endian, and the DESCRIPTIONs changed to say they are big-endian. | Alter DESCRIPTIONs of dot11LCIDSELatitudeResolution, dot11LCIDSELatitudeInteger, dot11LCIDSELatitudeFraction, dot11LCIDSELongitudeResolution, dot11LCIDSELongitudeInteger, dot11LCIDSELongitudeFraction, dot11LCIDSEAltitudeType, dot11LCIDSEAltitudeResolution, dot11LCIDSEAltitudeInteger, dot11LCIDSEAlitiudeFraction, and dot11LCIDSEDatum per comment. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comment 2008.

| 2016 | Annex D | 40 | 19 | T | Y | All the dot11StationConfigTable elements and dot11LCIDSE Table entries should be read-only, as they can only be changed from their default values by a licensed operator. | Change dot11LCIDSERequired, dot11DSERequired and all accessible parts of dot11LCIDSETable from read-write to read-only. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comment 2016.

| 2044 | Annex D |  |  | T | Y | The definitions of various parameters uses the clause "The capability is disabled otherwise" However, the definition provides semantics rather than describing a capability and so the "The capability is disabled otherwise" makes no sense | In each case, properly define the semantics in the "otherwise case" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Of the four occurrences of the phrase in Annex D text, two indicate capabilities and two are indications of requirements. The description text of dot11RegLocRequired and dot11DSERequired will be changed, and commas will be added after "disabled" in all occurrences.”

Propose Accept in Principle Comment 2044.

Proposed Resolution:

Accept 2008, 2016

Accept in Principle based on discussion and editorial instructions in 07/2073r1: 2004, 2023 and 2044.

Reject based on discussion in 07/2073r1: 2003.

Editorial Instructions:

Tgy Editor: Insert text in Annex A, Table A.4.10 after MD14 entry, pg 36, line 24, D3.

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| MD15 | Operation with Regulatory Classes Multiple classes in Country Information element Multiple Classes in Association and Reassociation frames | 9.8.3 9.8.3 | CF15:M CF15:M | Yes □ No □ N/A □ Yes □ No □ N/A □ |

Tgy Editor: change text in Annex A, Table A.4.12 after SM6 entry, pg 36, line 52, D3.

Insert new entry after SM20 as shown:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| SM21 | Channel switch procedure Transmission of extended channel switch announcement and channel switch procedure by an AP Transmission of extended channel switch announcement and channel switch procedure by a STA Reception of extended channel switch announcement and channel switch procedure by a STA | 11.9.7 11.9.7 11.9.7 | (CF15&CF1):M (CF15&CF2):M CF15:M | Yes □ No □ N/A □ Yes □ No □ N/A □ Yes □ No □ N/A □ |

Tgy Editor: Change text in Annex A, Table A.4.13 after RC4 entry, pg 37, line 2, D3.

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RC5 | CoverageClass 0-31 CoverageClass operation when not associated | 9.8.4 9.8.4 | CF15:M CF15:M | Yes □ No □ N/A □ Yes □ No □ N/A □ |
| RC6 | Power level, equivalent Max Transmit Power Level and Regulatory Class Power level operation when not associated | 9.8.4 9.8.4 | CF15:M CF15:M | Yes □ No □ N/A □ Yes □ No □ N/A □ |
| RC7 | Power level, different Max Transmit Power Level and Regulatory Class Power level operation when not associated | 9.8.4 9.8.4 | CF15:M CF15:M | Yes □ No □ N/A □ Yes □ No □ N/A □ |

Tgy Editor: delete DSE4 text in Annex A, Table A.4.17, pg 37, line 46-55, D3.

Tgy Editor: change text in Annex D, in and after dot11LCIDSEImplemented (four places), pg 40, line 12, D3.0

is disabled, otherwise the default value of this attribute is false.

Tgy Editor: change text in Annex D, in dot11LCIDSERequired and dot11DSERequired, pg 40, line 19 and 42, D3.0

MAX-ACCESS read-onlywrite

Tgy Editor: change text in Annex D, in dot11LCIDSETable wherever MAX-ACCESS is read-write, pg 41-46, D3.0

MAX-ACCESS read-onlywrite

Tgy Editor: change text in Annex D, Dot11LCIDSEDatum, pg 44, line 24, D3.0

,

SYNTAX INTEGER (0..255)(1..3)

Tgy Editor: insert text in Annex D, in dot11LCIDSELatitudeResolution, dot11LCIDSELatitudeInteger, dot11LCIDSELatitudeFraction, dot11LCIDSELongitudeResolution, dot11LCIDSELongitudeInteger, dot11LCIDSELongitudeFraction, dot11LCIDSEAltitudeType, dot11LCIDSEAltitudeResolution, dot11LCIDSEAltitudeInteger, dot11LCIDSEAlitiudeFraction, and dot11LCIDSEDatum Descriptions, pg 42-44, D3.0

This field is derived from IETF RFC-3825, and is accessed big-endian.

Tgy Editor: change seventh paragraph in Annex J.2A, , pg 55, line 19-22, D3.

- All stations shall use Multi-Domain capability (dot11MultiDomainCapabilityEnabled true), Spectrum Management capability (dot11SpectrumManagementRequired true), Regulatory Classes (dot11RegulatoryClassesRequired true), Supported Regulatory Classes and Extended Channel Switch Announcement (dot11ExtendedChannelSwitchImplemented true), and shall not use Channel Switch Announcement.