IEEE P802.11
Wireless LANs

| Access Point Collaboration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Alex Ashley | NDS Ltd | NDS Ltd, One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley@nds.com |
| Robert Miller | AT&T | AT&T Labs - Research 180 Park Avenue, Bldg. 103, Room B251 Florham Park, NJ 07932 | +1 973 236 6920 | rrm@research.att.com |
| Yongho Seok | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |

Abstract

This document describes a mechanism to allow multiple APs on the same channel to cooperatively share time on the wireless medium. This enables enhanced QoS by allowing collaborating APs to avoid allocating overlapping contention free periods and reducing back off periods due to collisions.

Background:

The spectrum assignments in which IEEE 802.11 can be used are a finite resource that is becoming more congested as 802.11 becomes more popular. The use of this spectrum can be made more efficient by techniques such as intelligent channel selection, transmit power control and time division. This document describes a method to allow access points to collaborate by sharing time on the wireless medium.

The proposal is designed to fulfill requirement 2050 “Access Point Coordination” in the TGV objectives document [1].

This document is based upon TGv draft v1.03 and the IEEE P802.11-2007 baseline.

11.9 DFS procedures

Insert the following new clause after 11.9.7:

11.9.8 AP collaboration

11.9.8.1 Suppressing channel access for AP collaboration

An AP in a BSS may schedule suppressed intervals, which are time  intervals during which channel accesses by the member STAs of the BSS are suppressed.  During a suppressed interval scheduled by the AP of a BSS, all member STAs of the BSS shall yield their channel access attempts to channel access attempts by member STAs of BSSes that are not suppressed.     These suppressed intervals can be used to provide time-period based collaboration between APs operating on the same channel.

The AP that is suppressing its BSS is referred to as a “suppressed AP” and the AP that is able to use this period of suppressed channel access in other BSSes is referred to as the “recipient AP”. Correspondingly the BSS of a suppressed AP is referred to as a “suppressed BSS” and the BSS of a recipient AP is referred to as a “recipient BSS”.  There can be multiple suppressed APs and multiple recipient APs as there may be more than two BSS operating on the same channel within radio range.

11.9.8.2 AP collaboration using suppressed intervals

Time-period based collaboration between APs operating on the same channel can be controlled by appropriate configuration of the dot11MgmtOptionBeaconOffset, dot11MgmtOptionGrantPeriodOffset, dot11MgmtOptionGrantPeriodLength, dot11MgmtOptionSuppressedPeriodOffset and dot11MgmtOptionSuppressedPeriodLength MIB variables.

A BSS that has dot11MgmtOptionAPCollaborationImplemented set TRUE and has a non-zero value of dot11MgmtOptionSuppressedPeriodLength shall schedule suppressed intervals.  The dot11MgmtOptionBeaconOffset establishes a reference time for scheduling suppressed intervals by specifying the offset (in TUs) from the TBTT of the BSS.  The dot11MgmtOptionGrantPeriodOffset and dot11MgmtOptionGrantPeriodLength specifies the offset and duration, both in TUs, of a grant time window relative to the reference time.  The dot11MgmtOptionSuppressedPeriodOffset and dot11MgmtSuppressedPeriodLength specifies the offset and duration, both in TUs, of a suppressed time window relative to the reference time.

The period of suppressed medium access starts at TBTT + dot11MgmtOptionBeaconOffset + dot11MgmtOptionSuppressedPeriodOffset and ends at

TBTT + dot11MgmtOptionBeaconOffset + dot11MgmtOptionSuppressedPeriodOffset + dot11MgmtSuppressedPeriodLength. During the period starting at TBTT + dot11MgmtOptionGrantPeriodOffset and ending at TBTT + dot11MgmtOptionGrantPeriodOffset + dot11MgmtOptionGrantPeriodLength medium access shall not be suppressed, regardless of the values of dot11MgmtOptionSuppressedPeriodOffset and dot11MgmtSuppressedPeriodLength.

The method used by an AP to suppress its BSS is not specified in this version of the specification. The dot11MgmtOptionSuppressionAlgorithm is used to specify the suppression algorithm to be used by the AP.

| Code | Meaning | See subclause |
| --- | --- | --- |
| 0 – 126 | Reserved |  |
| 127-255 | Vendor-specific |  |

Co-channel BSSes can collaborate by scheduling concurrent suppressed intervals in certain BSSes to provide channel access precedence to other BSSes.  Figure v98 illustrates how this can be done.  In this example, BSS#1 has a suppressed interval scheduled within its BI.  During this suppressed interval, channel accesses by members STAs of BSS#1 are suppressed, thus effectively member STAs of BSS#2 receive precedence in their rights to access the channel.

Figure v98—Example timing of silence and grant periods

Insert the following new rows into A.4.18:

### A.4.18 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME13 | AP Collaboration | 11.9.8 | (CFv&SM7):O | Yes, No, N/A |

Annex D

Insert the following after dot11MgmtOptionMultiBSSIDEnabled:

dot11MgmtOptionAPCollaborationImplemented		TruthValue,

dot11MgmtOptionSuppressionAlgorithm		INTEGER,

dot11MgmtOptionBeaconOffset				INTEGER,

dot11MgmtOptionGrantPeriodOffset			INTEGER,

dot11MgmtOptionGrantPeriodLength			INTEGER,

dot11MgmtOptionSuppressedPeriodOffset			INTEGER,

dot11MgmtOptionSuppressedPeriodLength			INTEGER }

Insert the following after dot11MgmtOptionMultiBSSIDEnabled OBJECT-TYPE:

dot11MgmtOptionAPCollaborationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates the capability of the AP to support collaboration using the Time Collaboration capability bit  of the Extended Capabilities information element, the CF-Offer and CF-Response frames. The default value of this attribute is FALSE.”

::= { dot11WirelessMgmtOptionsEntry 8 }

dot11MgmtOptionSuppressionAlgorithm OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall be a set to the current BSS suppression method"

::= { dot11WirelessMgmtOptionsEntry 14 }

dot11MgmtOptionBeaconOffset OBJECT-TYPE

SYNTAX INTEGER (-1..32767)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute contains the offset, in TUs, from the common time reference until the start of the Beacon transmission. A value of -1 indicates no common time reference has been established for this AP.”

::= { dot11WirelessMgmtOptionsEntry 9 }

dot11MgmtOptionGrantPeriodOffset OBJECT-TYPE

SYNTAX INTEGER (0..131071)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute contains the offset, in TUs, from the scheduled start of the Beacon transmission until the start of the time period granted exclusively for use by this AP.”

::= { dot11WirelessMgmtOptionsEntry 10 }

dot11MgmtOptionGrantPeriodLength OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute contains the duration, in TUs, of the period granted exclusively for use by this AP. When zero, no period has been granted for exclusive use by this AP.”

::= { dot11WirelessMgmtOptionsEntry 11 }

dot11MgmtOptionSuppressedPeriodOffset OBJECT-TYPE

SYNTAX INTEGER (0..131071)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute contains the offset, in TUs, from the scheduled start of the Beacon transmission until the start of the time period during which channel access attempts by member STAs of this BSS is to remain suppressed.”

::= { dot11WirelessMgmtOptionsEntry 12 }

dot11MgmtOptionSuppressedPeriodLength OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute contains the duration, in TUs, of the suppressed period for this AP. When zero, no suppressed period has been set for this AP.”

::= { dot11WirelessMgmtOptionsEntry 13 }

References:

[1]	11-05-0827-13-000v-tgv-objectives.doc