IEEE P802.11
Wireless LANs

| TGy LB106 Submission for DSE comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., San Jose, CA 95134 | 408-527-0815 | petere@cisco.com |

Abstract

This document is aligned with P802.11-2007, P802.11k/D7.0, P802.11r/D6.1 for baseline, P802.11y/D3.0, and submissions 07/2058r1 (misc PHY comments), 07/2073r1 non-Editorial comments and 07/2072r1 Editorial comments, and addresses the following LB106 comments:

CIDs: 2034, 2035, 2049, 2050, 2051

CIDs: 2020, 2022, 2028, 2054

CIDs: 2030, 2031, 2032, 2033, 2038, 2040, 2042, 2052, 2055, 2056, 2057, 2058, 2059, 2060, 2069

CIDs: 2018, 2047, 2048

CID: 2061

## Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGy Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGy Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGy amendment with the baseline documents).

TGy Editor:  Editing instructions preceded by “TGy Editor” are instructions to the TGy editor to modify existing material in the TGy draft.   As a result of adopting the changes, the TGy editor will execute the instructions rather than copy them to the TGy Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Clause 3

CIDs: 2034, 2035, 2049, 2050, 2051

Discussion:

In 11y/D3.0:

| 2034 | 3.34a | 1 | 53 | E | N | All messages in digital wireless communication are coded. The sentence should read "… upon its ability to continue to receive messages from the enabling AP." | Change text as suggested |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comment 2034.

| 2051 | 3.34a | 1 | 54 | T | Y | The text speaks of an association between the dependent STA and the enabling AP. However, this is confusing because I understand that this is not intended to be an 802.11 association. | Modify nomenclature to call relationship between the dependent STA and the enabling AP a "registration" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “As the FCC uses "registration" for licensed operators and stations in required databases and regulations, it would be very confusing to replace "enablement" with "registration"”. Propose Reject Comment 2051.

| 2049 | 3.34b | 1 | 59 | T | Y | Definition uses term "restricted channel" However, this is not defined anywhere | Define "restricted channel" A similar comment applies to 3.48a, which defines "restricted bands" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Adding definition of restricted channel “, which is a channel where transmission is restricted to licensed operators and stations operating under their control”.” Propose Accept in Principle Comment 2049.

| 2035 | 3.48a | 1 | 61 | E | N | All messages in digital wireless communication are coded. "An AP that broadcasts coded signals" is a very obscure definition. It's misleading because readers may think that the successful detection of a preamble or other parts of an AP's beacon may be sufficient for the dependent STA to be allowed to transmit. As far as I understand, the dependent STA must successfully receive an enabling AP's beacon frame to interpret the IEs within. | "3.48a enabling AP: An AP that broadcasts information that are used by dependent STAs to determine which restricted bands they may use." |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 2050 | 3.54a | 1 | 65 | E | Y | The text uses "station" However, "STA" would be more consistent with the rest of clause 3 | Replace "station" with STA Similar comment applies to other clauses in draft |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comments 2035 and 2050 without discussion.

Proposed Resolution:

Accept: 2034, 2035 and 2050

Accept in Principle: based on discussion and editorial instructions in 07/2080r0; 2049

Reject: based on discussion in 07/2080r0; 2051

Editorial Instructions:

Tgy Editor: change text in 3, pg 1, D3.0 as follows

3.34a dependent STA: A station that must receive and successfully decode an enabling signal before it is allowed to transmit. Once enabled byassociated with an enabling AP, the dependent STA’s operation becomes contingent upon its ability to continue to receivedecode messages from the enabling AP.

3.34b dependent STA enablement (DSE): The process by which a dependent STA gains permission to transmit on a restricted channel, which is a channel where transmission is restricted to licensed operators and stations operating under their control.

3.48a enabling AP: An AP that broadcasts informationcoded signals that are used by dependant STAs to determine which restricted bands they may use. Restricted bands are those where transmission is restricted to licensed operators and stations operating under their control.

3.54a fixed stationSTA:  A station that is only authorized to operate at a specific location.

# Clause 5

Tgy Editor: insert text after Clause 4 DSE, pg 2 line 15, D3.0 as follows

5.1.1.1 The wireless station (STA)

Change the first paragraph of 5.1.1.1 as shown:

In wired LANs, an address is equivalent to a physical location. This is implicitly assumed in the design of

wired LANs. In the design of wired LANs it is implicitly assumed that an address is equivalent to a physical location. In IEEE 802.11, the addressable unit is a station (STA). The term implies no more than the origin or/and destination of a message. Physical and operational characteristics are defined by modifiers that are placed in front of the term STA. For example, in the case of location and mobility, the addressable units are the Fixed STA, the Portable STA and the Mobile STA. The STA is a message destination, but not (in general) a fixed location.

Conceptually, it is important to realize that a station can take on multiple distinct characteristics, each of which shape its function. For example, a single addressable unit can simultaneously be a Portable STA, a QoS STA, a Dependant STA and a Hidden STA.

Insert new text after 5.2.7.11:

5.2.8 Operation in licensed frequency bands

802.11 has been amended to make use of a number of frequencies that are licence by the national regulators for a given country. While effort is made to generalize these amendments so that they can be reused in the same bands in other countries, it should be noted that the projects that drafted said changes were focused on the rules single country. Countries may impose specific requirements for radio equipment in addition to those specified in this standard.

5.2.8.1 Types of licenses

License types are frequency band specific, with either exclusive use or shared use in a given area. The types of licenses available continue to evolve as radios and service requirements evolve.

5.2.8.2 Operational logic and regulatory privileges

In the regulatory domain, a wireless device must adhere to published operational procedures as a condition of a license or as a condition for an exemption from needing a license. In order to meet this requirement a wireless device or its license holder may have to change how the device is operated in differing circumstances. For example, a station may have to change its behaviour or even cease operation while moving.

In addition, a STA may gain permission to operate in an enhanced manner through a registration and declaration process specified by a regulatory authority. These enhancements are referred to as the registered STA’s privileges.

The base level privilege of a Registered STA is the ability to operate autonomously in a licensed band (it does not need to hear an enabling signal)

Registration may also entitle a station to additional privileges that are further shaped by conditional operational procedures.

For example, if regulations stated that:

1.       A fixed STA that is a registered STA may use higher power

2.       A control STA that is a registered STA may take responsibility for dependant stations

The following scenario could arise:

·                                 An enabling AP that is also a fixed STA could use increased power to reach potential dependant STAs.

·                                 A portable STA may be granted permission to be an enabling AP, but it may not be permitted to operate at higher power in order to reach potential dependant stations.

### 5.3 Logical service interfaces

Change text at end of table in Clause 5.3

a) Authentication

b) Association

c) Deauthentication

d) Disassociation

e) Distribution

f) Integration

g) Data confidentiality

h) Reassociation

i) MSDU delivery

j) DFS

k) TPC

l) Higher layer timer synchronization (QoS facility only)

m) QoS traffic scheduling (QoS facility only)

n) Radio measurement

o) DSE

### 5.3.1 SS

Change text at end of table in Clause 5.3.1

a) Authentication

b) Deauthentication

c) Confidentiality

d) MSDU delivery

e) DFS

f) TPC

g) Higher layer timer synchronization (QoS facility only)

h) QoS traffic scheduling (QoS facility only)

i) Radio measurement

j) DSE

### 5.3.2 DSS

Change text at end of table in Clause 5.3.2

a) Authentication

b) Disassociation

c) Distribution

d) Integration

e) Reassociation

f) QoS traffic scheduling (QoS facility only)

g) DSE

# Clause 7

CIDs: 2020, 2022, 2028, 2054

| 2022 | 7.2.3.1 | 3 | 20 | E |  | If a description of how a dependent STA comes under the control of an enabling AP is created (maybe in response to other unsatisfied comments, e.g. LB104 1034 or 1101), then the variable dot11LCIDSERequired should be renamed globally to something more descriptive of enablement. | Per comment |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “LCIDSE conveys location-based dependent STA enablement in six letters of a 19 letter name. LCI is used in IETF geopriv and related areas like emergency services, and DSE is widely used in P802.11y D3.0. Fifty-two letters is about the length of a line of DESCRIPTION in a MIB.”

Propose Reject Comment 2022.

| 2054 | 7.3.2.21.11 | 6 | 39 | T | Y | The text refers to the AP with which the STA is associated. However, it is unclear if this is the enabling AP (with which it is registered) or the local AP (with which it is associated - in 802.11 speak) | Clarify to which AP the clause applies. If it is the enabling AP, how does the STA return the report if it cannot actually communicate directly with the enabling AP |
| --- | --- | --- | --- | --- | --- | --- | --- |

Commenter notes ambiguity about STA sending DSE request. Proposed Comment Resolution: “It is mandatory to generate a report in response to a request from either the enabling AP or the AP with which it is associated.”

Propose Accept Comment 2054.

Tgy Editor: Change second sentence in 7.3.2.21.11 pg 6 line 38 in D3.0

"It is mandatory for a STA in a BSS with dot11LCIDSERequired true to generate a DSE report in response to a DSE request if the request is received from the AP with which it is associated or the enabling AP, and dot11LCIDSERequired is true.”

Tgy Editor: Replace figure 79m of 7.3.2.21.11, pg 6 line 25, D3.0 as follows

|  | RequesterSTAAddress | ResponderSTAAddress | Regulatory Class | Channel Number | Measurement Start Time | Measurement Duration |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 1 | 1 | 8 | 2 |

Tgy Editor: insert text after second paragraph of 7.3.2.21.11, pg 6 line 42, D3.0 as follows

The RequesterSTAAddress is the individual MAC address of the requesting STA that grants enablement. The length of the RequesterSTAAddress field is six octets.

The ResponderSTAAddress is the individual MAC address of the responding STA that operates based on the enablement. The length of the ResponderSTAAddress field is six octets.

Tgy Editor: Replace figure 85p of 7.3.2.22.11, pg 7 line 27, D3.0 as follows

|  | RequesterSTAAddress | ResponderSTAAddress | Regulatory Class | Channel Number | Actual Measurement Start Time | Measurement Duration | Reported DSE LCIs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 1 | 1 | 8 | 2 | variable |

Tgy Editor: insert text before second paragraph of 7.3.2.22.11, pg 6 line 37, D3.0 as follows

The RequesterSTAAddress is the individual MAC address of the requesting STA that grants enablement. The length of the RequesterSTAAddress field is six octets.

The ResponderSTAAddress is the individual MAC address of the responding STA that operates based on the enablement. The length of the ResponderSTAAddress field is six octets.

| 2028 | 7.3.2.49 | 9 | 60 | T | N | The impact of being in a National Policy Area or border is undescribed. It must have some impact else it need not exist. E.g. How should STA behavior change in this case? | Describe (perhaps in clause 11) the need/use of this bit |
| --- | --- | --- | --- | --- | --- | --- | --- |

Commenter notes signaling of presence within a National Policy area or near a national border and asks for description of some implications of such signaling.Proposed Comment Resolution: “Clause 11.14 rewrite discusses both the setting and reception of RegLoc Agreement.”

Propose Accept in Principle Comment 2028.

| 2020 | 7.4 | 11 | 23 | T |  | If a description of how a dependent STA comes under the control of an enabling AP is created (maybe in response to other unsatisfied comments, e.g. LB104 1034 or 1101), and other Action frames are defined, then DSERegisteredLocationAnnouncement should be moved from Spectrum management into a new catagory of Action frames that involve dependent stations. | Per comment |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Creating a new category of Action frames for three Actions involving dependent stations does not appear worth the effort. Keeping them together with Spectrum Management Action frames will lead to broader use in 802.11.”

Propose Reject Comment 2020.

Proposed Resolution:

Accept: 2054

Accept in Principle based on discussion and editorial instructions in 07/2080r0: 2028

Reject based on discussion in 07/2080r1: 2020, 2022

Editorial Instructions:

Tgy Editor: Change Table 26 DSE registered location Length, pg 5 line 45, D3.0 as shown below:

“2022”

Tgy Editor: Replace Figure 85q, pg 8 line 3, D3.0 as shown below:

Tgy Editor: Replace Figure 112z, pg 9 line 8, D3.0 as shown below:

Tgy Editor: Change third sentence of 7.3.2.49, pg 9 line 41, D3.0 as shown below:

“The length field is set to 1820.”

Tgy Editor: Change tenth paragraph of 7.3.2.49, pg 10 line 5, D3.0 as shown below:

Dependent Enablement Identifier is a 16-bit field with a value set by the enabling AP via the DSE enablement frameregistered location element in the (re)association response as specified in 11.14.2 and 11.14.3, or zero.

Tgy Editor: Insert new text before NOTE of 7.3.2.49, pg 10 line 7, D3.0 as shown below:

Regulatory Class indicates the channel set for which the enablement request or report or announcement applies. Regulatory Class and Channel Number together specify the channel frequency and channel bandwidth for which the report applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the enablement request or report or announcement applies. Channel Number is defined within a Regulatory Class as shown in Annex J.

Tgy Editor: Change part of Table 44 of 7.4.1, pg 11 line 54, D3.0 as shown below:

- Table 44—Spectrum management Action Value field values

| Action Value field value | Description |
| --- | --- |
| 6 | DSE Registered Location Announcement |
| 7 | DSE Enablement |
| 8 | DSE Deenablement |
| 59-255 | Reserved |

Tgy Editor: insert text after 7.4.1.7, pg 12 line 44, D3.0 as follows

##### 7.4.1.8 DSE enablement frame format

The DSE enablement frame is an Action frame. It is transmitted by a DSE STA as part of enablement. The format of the DSE enablement frame body is shown in Figure 117c.

|  | Category | Action Value | RequesterSTAAddress | ResponderSTAAddress | Reason Result Code | Enablement Identifier |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 | 1 | 2 |

Figure 117c—DSE enablement frame body format

The Category field is set to 0 (representing spectrum management).

The Action Value field is set to indicate a DSE enablement frame.

The RequesterSTAAddress is the individual MAC address of the requesting STA that initiates the enablement process. The length of the RequesterSTAAddress field is six octets.

The ResponderSTAAddress is the individual MAC address of the responding STA that grants the enablement. The length of the ResponderSTAAddress field is six octets.

The Enablement Identifier is a 16-bit number assigned by an enabling AP to a dependent STA, otherwise it is zero.

The Reason Result Code field is used to indicate the reason that an unsolicited notification management frame of

Type DSE enablement was generated. The length of the Reason Result Code field is one octet. The Reason Result Codes that have been allocated are shown in Table 7-44a.

- Table 7-44a—Reason result code field values

| Reason Result Code field value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Unspecified reason |
| 2 | Request enablement |
| 3 | Success |
| 4 | The request has been declined |
| 5 | The request has not been successful as one or more parameters have invalid values |
| 6 | Enablement denied because enabling AP is unable to handle additional dependent STAs |
| 7 | Handshake timeout |
| 8 | The Peer STA is not reachable |
| 9-255 | Reserved |

##### 7.4.1.9 DSE deenablement frame format

The DSE deenablement frame is an Action frame. It is transmitted by a DSE STA as part of deenablement. The format of the DSE deenablement frame body is shown in Figure 117d.

|  | Category | Action Value | RequesterSTAAddress | ResponderSTAAddress | Reason Result Code |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 | 1 |

Figure 117d—DSE deenablement frame body format

The Category field is set to 0 (representing spectrum management).

The Action Value field is set to indicate a DSE deenablement frame.

The RequesterSTAAddress is the individual MAC address of the requesting STA that initiates the deenablement process. The length of the RequesterSTAAddress field is six octets.

The ResponderSTAAddress is the individual MAC address of the responding STA that grants the deenablement. The length of the ResponderSTAAddress field is six octets.

The Reason Result Code field is used to indicate the reason that an unsolicited notification management frame of

Type DSE deenablement was generated. The length of the Reason Result Code field is one octet. The Reason Result Codes that have been allocated are shown in Table 7-44b.

- Table 7-44b—Reason result code field values

| Reason Result Code field value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Unspecified reason |
| 2 | Request deenablement |
| 3 | Success |
| 4 | The request has been declined |
| 5 | The request has not been successful as one or more parameters have invalid values |
| 6 | Reserved |
| 7 | Reserved |
| 8 | The Peer STA is not reachable |
| 9-255 | Reserved |

# Clause 9.8.1

Tgy Editor: insert text after 9.8, pg 13 line 6, D3.0 as follows

### 9.8.1 Operation upon entering a regulatory domain

Insert text at end of second paragraph of 9.8.1:

When a STA enters a regulatory domain, it shall passively scan to learn at least one valid channel, i.e., a channel upon which it detects IEEE 802.11 frames. The Beacon frame contains information on the country code, the maximum allowable transmit power, and the channels to be used for the regulatory domain. Optionally, the Beacon frame may also include, on a periodic basis, the regulatory information that would be returned in a Probe Response frame. When DSE dependent STA operation is required in a regulatory domain, a dependent STA may be required to receive a Beacon frame signalling dependent enablement (11.14.3), and may need to continue passive scanning to receive such a Beacon frame directly from an enabling AP. Once the STA has acquired the information so that it is able to meet the transmit requirements of the regulatory domain, it shall transmit a Probe Request to an AP to gain the additional regulatory domain information contained in the Probe Response frame, unless the information was previously received in a Beacon frame. The STA then has sufficient information available to configure its PHY for operation in the regulatory domain.

Tgy Editor: insert text after 10.3.36.4.4, pg 23 line 56, D3.0 as follows

- 10.3.37 Enablement

This mechanism supports the process of establishing an enablement relationship with a peer MAC entity.

10.3.37.1 MLME-ENABLEMENT.request

10.3.37.1.1 Function

This primitive requests enablement with a specified peer MAC entity.

10.3.37.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ENABLEMENT.request(

RequesterSTAAddress,

ResponderSTAAddress,

EnablementTimeLimit,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| RequesterSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the MAC entity which initiates the enablement process. |
| ResponderSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the MAC entity of the enabling AP. |
| EnablementTimeLimit | Integer | ≥ 1 | Specifies a time limit (in TU) after which the enablement process will be terminated. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.37.1.3 When generated

This primitive is generated by the SME for a STA to establish enablement with a specified peer MAC entity in order to permit Class 2 frames to be exchanged between the two STAs. During the enablement procedure, the SME can generate additional MLME-ENABLEMENT.request primitives.

10.3.37.1.4 Effect of receipt

This primitive initiates an enablement procedure. The MLME subsequently issues a MLME-ENABLEMENT.confirm that reflects the results.

10.3.37.2 MLME-ENABLEMENT.confirm

10.3.37.2.1 Function

This primitive reports the results of an enablement attempt with a specified peer MAC entity.

10.3.37.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ENABLEMENT.confirm(

RequesterSTAAddress,

ResponderSTAAddress,

ResultCode,

EnablementIdentifier,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| RequesterSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the MAC entity which initiated the enablement process. This value must match the RequesterSTAAddress parameter specified in the corresponding MLME-ENABLEMENT.request. |
| ResponderSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with which the enablement process was attempted. This value must match the ResponderSTAAddress parameter specified in the corresponding MLME-ENABLEMENT.request. |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TOO_MANY_ SIMULTANEOUS_ REQUESTS, REFUSED | Indicates the result of MLME-ENABLEMENT.request. |
| EnablementIdentifier | Integer | 0-65535 | Specifies the dependent enablement identifier. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.37.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-ENABLEMENT.request for enablement with a specified peer MAC entity.

10.3.37.2.4 Effect of receipt

The SME is notified of the results of the enablement procedure.

10.3.37.3 MLME-ENABLEMENT.indication

10.3.37.3.1 Function

This primitive indicates receipt of a request from a specific peer MAC entity to establish an enablement relationship with the STA processing this primitive.

10.3.37.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ENABLEMENT.indication	(

RequesterSTAAddress,

ResponderSTAAddress,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| RequesterSTAAddress | MACAddress | Any valid individual MAC Address | Specifies the address of the peer MAC entity which initiated the enablement relationship. |
| ResponderSTAAddress | MACAddress | Any valid individual MAC Address | Specifies the address of the peer MAC entity which is the enabling AP. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.37.3.3 When generated

This primitive is generated by the MLME as a result of the receipt of an enablement request from a specific peer MAC entity.

10.3.37.3.4 Effect of receipt

The SME is notified of the receipt of this enablement request.

10.3.37.4 MLME-ENABLEMENT.response

10.3.37.4.1 Function

This primitive is used to send a response to a specific peer MAC entity that requested enablement with the STA that issued this primitive.

10.3.37.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ENABLEMENT.response(

RequesterSTAAddress,

ResponderSTAAddress,

ResultCode,

EnablementIdentifier,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| RequesterSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity which initiated the enablement relationship. |
| ResponderSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity which is the enabling AP. |
| ResultCode | Integer | SUCCESS, REFUSED | Indicates the result response to the enablement request from the peer MAC entity. |
| EnablementIdentifier | Integer | 0-65535 | Specifies the dependent enablement identifier. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.37.4.3 When generated

This primitive is generated by the SME of a STA as a response to an MLME-ENABLEMENT.indication primitive.

10.3.37.4.4 Effect of receipt

This primitive initiates transmission of a response to the specific peer MAC entity that requested enablement.

- 10.3.38 Deenablement

This mechanism supports the process of invalidating an enablement relationship with a peer MAC entity.

10.3.38.1 MLME-DEENABLEMENT.request

10.3.38.1.1 Function

This primitive requests that the enablement relationship with a specified peer MAC entity be invalidated.

10.3.38.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-DEENABLEMENT.request(

RequesterSTAAddress,

ResponderSTAAddress,

ReasonCode,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| RequesterSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with initiating the deenablement process. |
| ResponderSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity which become deenabled in the process. |
| ReasonCode | As defined in frame format | As defined in frame format | Specifies the reason for initiating the deenablement procedure. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.38.1.3 When generated

This primitive is generated by the SME for a STA to invalidate enablement with a specified peer MAC entity in order to prevent the exchange of Class 2 frames between the two STAs. During the deenablement procedure, the SME can generate additional MLME-DEENABLEMENT.request primitives.

10.3.38.1.4 Effect of receipt

This primitive initiates a deenablement procedure. The MLME subsequently issues a MLME-DEENABLEMENT.confirm that reflects the results.

10.3.38.2 MLME-DEENABLEMENT.confirm

10.3.38.2.1 Function

This primitive reports the results of a deenablement attempt with a specified peer MAC entity.

10.3.38.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-DEENABLEMENT.confirm(

PeerSTAAddress,

ResponderSTAAddress,

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| RequesterSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity which initiated the deenablement process. This value must match the RequesterSTAAddress parameter specified in the corresponding MLME-DEENABLEMENT.request. |
| ResponderSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity with which the deenablement process was attempted. This value must match the ResponderSTAAddress parameter specified in the corresponding MLME-DEENABLEMENT.request. |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TOO_MANY_ SIMULTANEOUS_ REQUESTS | Indicates the result of MLME-DEENABLEMENT.request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.38.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-DEENABLEMENT.request to invalidate the enablement relationship with a specified peer MAC entity.

10.3.38.2.4 Effect of receipt

The SME is notified of the results of the deenablement procedure.

10.3.38.3 MLME-DEENABLEMENT.indication

10.3.38.3.1 Function

This primitive reports the invalidation of an enablement relationship with a specific peer MAC entity.

10.3.38.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-DEENABLEMENT.indication	(

RequesterSTAAddress,

ResponderSTAAddress,

ReasonCode,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| RequesterSTAAddress | MACAddress | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which the enablement relationship was invalidated. |
| ResponderSTAAddress | MACAddress | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which the enablement relationship was invalidated. |
| ReasonCode | As defined in frame format | As defined in frame format | Specifies the reason the deenablement procedure was initiated. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.38.3.3 When generated

This primitive is generated by the MLME as a result of the invalidation of an enablement relationship with a specific peer MAC entity.

10.3.38.3.4 Effect of receipt

The SME is notified of the invalidation of the specific enablement relationship.

## Clause 11.3

Tgy Editor: insert text after 11., pg 24 line 3, D3.0 as follows

11.3 STA authentication and association

Change first two paragraphs of 11.3 as shown:

A STA keeps two state variables for each STA with which direct communication via the WM is needed:

— Authentication state: The values are unauthenticated and authenticated.

— Association state: The values are unassociated and associated.

These two variables create three local states for each remote STA:

— State 1: Initial start state, unauthenticated, unassociated.

— State 2: Authenticated, not associated.

— State 3: Authenticated and associated.

When dot11LCIDSERequired is true, requirements of enablement (allowed frame types) apply, as specified in 11.14.

# Clause 11.14

CIDs: 2030, 2031, 2032, 2033, 2038, 2040, 2042, 2052, 2055, 2056, 2057, 2058, 2059, 2060, 2069

| 2042 | General |  |  | E | Y | dot11AssociateFailHoldTime is used three times in the document. It should be dot11DSEAssociateFailHoldTime’. | Fix |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 2055 | 11.14 | 25 | 64 | T | Y | Page 40 says the DSE procedures (defined in 11.14) are only used when dot11DSERequired is true However, line 64 covers the case when dot11DSERequired is false | Remove reference to dot11DSERequired when false |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comments 2042 and 2055.

| 2056 | 11.14 | 25 | 50 | T | Y | The text refers to "frequency band" However, "frequency band" is not defined | Define "frequency band" in this context |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Frequency bands is undefined in the base standard and appears 18 times. Will rewrite 11.14 text being commented on to remove it.” Propose Accept in Principle Comment 2056.

| 2057 | 11.14 | 25 | 50 | T | Y | The text defines various parameters indexed by frequency band However they do not seem to be indexed by frequency band in the MIB. | Fix, or explain why not |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Will rewrite to remove apparent MIB indexing.” Propose Accept Comment 2057.

| 2052 | 11.14 |  |  | T | Y | This is similar comment to one I made in the last LB that was not addressed because I acidently marked it as not required My understanding of the intent of 11y is as follows: * Dependent AP hears enabling AP * Dependent AP registers with enabling AP, either over the air or via wire (noting the dependent AP may be a low power device unable to communicate with the enabling AP) * Enabling AP accepts registration from dependent AP and allocates unique indentity to dependent AP * Dependent STA hears enabling AP, either over the air or via wire * Dependent STA registers with enabling AP, either over the air or via wire (noting the dependent AP may be a low power device unable to communicate with the enabling AP) * Enabling AP accepts registration from dependent STA and allocates unique indentity to dependent STA * Both the dependent AP and the dependent STA may operate normally while they regularly hear the enabling AP However, if this underdstanding is correct then there are lots of unanswered questions in the draft * Where is all this described in the text? * What protocol is used for a dependent STA or a dependent AP to communicate with the enabling AP, over the wire (possibly in a different subnet) or over the air? * Is the dependent STA allowed to associate with the dependent AP for the purpose of registering over the wire with the enabling AP? The text in 11.14.3 implies not. * ... | The text needs to be completely rewritten to describe intent completely |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Will rewrite to remove concurrent associations.” Propose Accept in Principle Comment 2052.

Proposed Resolution:

Accept: 2042, 2055, 2057

Accept in Principle: based on discussion and editorial instructions in 07/2080r0; 2052, 2056

Editorial Instructions:

Tgy Editor: change third through fifth paragraphs of 11.14 pg 25 line 48-65, D3.0 as follows

STAs shall use the DSE procedures defined in this subclause if dot11RegLocRequired or dot11LCIDSERequired is true.  dot11DSERequired and dot11ExtendedChannelSwitchImplemented  shall be true when regulatory authorities require DSE. Dot11DSERequiredIt may also be set to false to configure STAs to operate as registered STAs. true in other circumstances. If licensees in more than one frequency band of a country use DSE procedures, then each dot11RegLocRequired, each dot11DSERequired, and each dot11ExtendedChannelSwitchImplemented shall be specific to a frequency band using the DSE procedures of that country. For each frequency band using the DSE procedures, the corresponding dot11ExtendedChannelSwitchImplemented shall be true.

The DSE procedures provide for the following:

--—Fixed STAstation and enabling AP operation

--—Creation of a Basic Service Area for dependent station operation

--—Dependent STA operation with DSE

For the purpose of DSE, when dot11LCIDSERequired is true, Spectrum Management Action frames shall be Class 1 frames and permitted in States 1, 2 and 3 of 11.3. a STA with dot11DSERequired set to false shall not respond to the DSE registered location request unless dot11RegLocRequired is set to true.

Tgy Editor: Insert new text before existing 11.14.1  pg 26 line 1, D3.0 as follows

### 11.14.1 Enablement and deenablement

This subclause describes the procedures used for IEEE 802.11 enablement and deenablement. A STA keeps a state variable for each STA with which enablement communication is needed:

— Enablement state: The values are unenabled and enabled.

The other states used in this description are defined in 11.3.

### 11.14.1.1 Enablement-originating STA

Upon receipt of an MLME-ENABLEMENT.request primitive, the originating STA shall become enabled with the indicated STA using the following procedure:

a) In an ESS, or optionally in an IBSS, the STA shall execute the enablement mechanism described herein.

Enablement utilizes a two-message transaction sequence. The first message asserts identity and requests enablement. The second message returns the enablement result. If the result is “successful,” the requesting STA shall be declared enabled.The STA initiating the authentication exchange is referred to as the requester, and the enabling AP to which the initial frame in the exchange is directed is referred to as the responder.

First frame

— Message type: Management

— Message subtype: Action DSE enablement

— Information items:

• STA Identity Assertion (in RequesterSTAAddress)

• Enabling AP Identity Assertion (in ResponderSTAAddress)

• Reason Result Code = 2

• Enablement identifier = 0

— Direction of message: From requester to responder

Final frame

— Message type: Management

— Message subtype: Action DSE enablement

— Information items:

• STA Identity Assertion (in RequesterSTAAddress)

• Enabling AP Identity Assertion (in ResponderSTAAddress)

• Enablement identifier

• The result of the requested enablement as defined in 7.4.1.7

— Direction of message: From responder to requester

b) If the enablement was successful, the state variable for the indicated STA shall be set to State 2.

c) The STA shall issue an MLME-ENABLEMENT.confirm primitive to inform the SME of the result of the enablement.

### 11.14.1.2 Enablement-destination STA

Upon receipt of an Action frame with DSE enablement Request Result Code = 2, the

destination STA shall enable the requesting STA using the following procedure:

a) The STA shall create a response frame with the enablement status as defined in 7.4.1.7 set in the Request Result Code, and a unique dependent enablement identifier if enablement was successful.

Final frame

— Message type: Management

— Message subtype: Action DSE enablement

— Information items:

• STA Identity Assertion (in RequesterSTAAddress)

• Enabling AP Identity Assertion (in ResponderSTAAddress)

• Enablement identifier

• The result of the requested enablement as defined in 7.4.1.7

— Direction of message: From responder to requester

b) If the enablement was successful, the state variable for the indicated STA shall be set to State 2.

c) The STA shall issue an MLME-ENABLEMENT.indication primitive to inform the SME of the enablement.

### 11.14.1.3 Deenablement-originating STA

Upon receipt of an MLME-DEENABLEMENT.request primitive, the originating STA shall

deenable with the indicated STA using the following procedure:

a) If the state variable for the indicated STA is in State 2 or State 3, the STA shall send a

DSE deenablement frame to the indicated STA.

b) The state variable for the indicated STA shall be set to State 1.

c) The STA shall issue an MLME-DEENABLEMENT.confirm primitive to inform the SME of the

completion of the deenablement.

### 11.14.1.4 Deenablement-destination STA

Upon receipt of a Deenablement frame, the destination STA shall deenable with the indicated STA

using the following procedure:

a) The state variable for the indicated STA shall be set to State 1.

b) The STA shall issue an MLME-DEENABLEMENT.indication primitive to inform the SME of

the deenablement.

| 2058 | 11.14 |  |  | T | Y | The draft seems to define measurement requests and responses. However, there is no description in 11.14 on how this should occur | Provide a description in 11.14 on how the measurements are intended to be used |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Will put usage overview description in 11.14.1.” Propose Accept in Principle Comment 2058.

| 2069 | 11.14.1 | 26 | 3 | T | N | What is the difference between a "fixed station" and an enabling AP" It seems like they are the same. | Clarify. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comment 2069

Proposed Resolution:

Accept: 2069

Accept in Principle: based on discussion and editorial instructions in 07/2080r0; 2058

Editorial Instructions:

Tgy Editor: change text in 11.14.1, pg 26, D3.0 as follows

    - Fixed STAstation and enabling AP operation

A fixed station is an AP that broadcasts its registered location, and is restricted from enabling other stations. Fixed STAsstations and enabling APs shall have dot11RegLocRequired set to true, and dot11DSERequired set to false. They shall transmit the DSE registered location element in every beacon, and shall set the Dependent STA bit in the DSE registered location element to 0. If the registered location of the fixed or enabling AP is within a national policy area, such as within a Fixed Satellite Service exclusion zone, or the registered location is within an international agreement area near a national border, the RegLoc Agreement bit in the DSE registered location element shall be set to1, signifying to other stations that additional restrictions on STAs with directional antennas may apply, otherwise it shall be set to 0.

The DSE registered location element Latitude, Longitude and Altitude fields shall be reported at their best known resolutions, which may exceed the resolutions required by regulatory authorities. The Altitude Type shall be 3, Height Above Ground meters (altitude in meters above adjacent terrain), unless another altitude type is required for operation in the regulatory domain. The Datum value shall be 1 (World Geodetic System 1984), unless another datum is required for operation in the regulatory domain.

The enabling AP can issue a Measurement Request DSE request to any of its enabled STAs, in order to receive a DSE report that may have Reported DSE LCI elements of neighboring STAs. The licensed operator may be able to use these reports to identify interfering stations or map overlapping coverage in a multiple BSS situation. Because Reported DSE LCI elements are to any destination addresses, a single DSE report may contain elements received from stations that are not yet associated to a BSS.

| 2030 | 11.14.2 | 26 | 24 | E | N | Typo? "dependentstaenabled" | Fix typo |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “Not a typo, see Page 44 line 58. Per CID 2007, dot11RegLocDSE is changed to TruthValue, then this becomes "true".” Propose Accept in Principle Comment 2030.

Proposed Resolution:

Accept in Principle: based on discussion and editorial instructions in 07/2080r0; 2030

Editorial Instructions:

Tgy Editor: change text in 11.14.2, pg 26, D3.0 as follows

    - Creation of a DSE service area for dependent station operation

An enabling AP may allow the creation of a DSE service area for dependent STA operation where regulatory requirements permit. An enabling AP sets dot11RegLocDSE to truedependentstaenabled, and signifies the creation of a DSE service area by setting the RegLoc DSE bit in the DSE registered location element to 1. Dependent STA operation is conditional on receiving and decoding a DSE registered location element with RegLoc DSE bit set to 1 directly from an enabling AP. Before attempting enablement withassociation to any one enabling AP, a dependent STA may have detected several enabling APs, and may attempt enablementassociation with any one and fail, then attempt enablementassociation to another and another. An enabling AP shall assign Dependent EnablementStation Identifiers in a way that makes them unique among enabledassociated STAs to maximize the ability to identify sources of interference.

An enabling AP may request dependent STAs to perform Measurement Request DSE requests and make DSE reports. The details of sending Measurement Requests from outside a BSS are unspecified.

| 2033 | 11.14.3 | 33 | 1 | T | N | I understood one usage case is an enabling AP enabling an AP, which in turn enables its STAs. Is this mode still supported? I couldn't figure out how it would work. | Please tell the commenter his knowledge is outdated, or provide extra detail on how this indirect enablement might work. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “FCC regulation prohibits indirect enablement. Any station that wants to transmit and is not registered, must directly receive and decode an enabling signal from a registered base station.” Propose Reject Comment 2033.

| 2059 | 11.14.3 | 27 | 1 | T | Y | The text provides a picture of a "typical" state machine. Why does the draft need a "typical" state machine? | Remove diagram or provide better context |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “a picture is sometimes worth 1000 words, and 802.11-2007 Figures 15.7, 15.9, 17.15, 17.7, 18.8 and 18.10 show typical state machines. The state machine diagram clarifies the decision to change states, and its consequences.” Propose Accept in Principle Comment 2059.

| 2031 | 11.14.3 | 27 | 2 | E | N | Figure could be more helpful | Annotate figure with more detailed conditions: e.g. RegLocDSE == 1 received |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “CID 2038 changed text in Unenabled box to “Enabling message received” which many would find more helpful. The figure shows the time limits to clarify that dependent stations have time-limited grants to become enabled and remain enabled.  Emailed commenter for suggestions..” Propose Reject Comment 2031.

| 2038 | 11.14.3 | 27 | 11 | E | N | The sentence "Receive and decode an Enabling Signal?" in Figure 204a seems to imply that a message was received that allows the STA to transmit. | Change sentence to "Enabling message received?" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comment 2038.

| 2040 | 11.14.3 | 27 | 42-47 | T | Y | A station can use active or passive scanning. With passive scanning, a STA does not transmit any frames. It solely seeks for beacon frames. With active scanning, a STA transmits probe request frames after some period the WM is deteced as idle. "The current text reads "For DSE, the following statements apply: A STA with dot11DSERequired set to true shall not operate in an infrastructure BSS or IBSS unless it has received a Beacon frame or Probe Response frame from a enabling AP with the Spectrum Management bit set to 1 in the Capability Information field, and with the RegLocDSE bit set to 1 in the DSE registered location element." To receive a Probe Response the STA needs to transmit a Probe Request. Once it has transmitted a Probe Request in search of an AP it may have already violated the regulatory limitations. | Do not allow active scanning when the STA is known to operate in a frequency band that requires an enabling AP to be allowed to transmit. Therefore, change the text accordingly to not to allow a station to search for APs using Probe Request frames. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “will remove Probe Response from unenabled state, but retain Probe Request/Probe Response in enabled state, for use when dot11DSERenewalTime limit approaches.” Propose Accept in Principle Comment 2040.

| 2032 | 11.14.3 | 27 | 57 | T | N | DEI details are light. Why is 16 bits of DEI needed over & above the SA? It seems there is only 1 bit of info - associated to an enabling AP or not? | Provide more detail & justificaiton on DEI or simplify. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Comment Resolution: “SA is not as valid as an identifying number assigned by a licensed operator.” Propose Reject Comment 2032.

| 2060 | 11.14.3 | 28 | 7 | T | Y | The text includes "count the sum" This makes no sense | Recast sentence to remove "count the sum" |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose Accept Comment 2060.

Proposed Resolution:

Accept: 2038, 2060

Accept in Principle: based on discussion and editorial instructions in 07/2080r0; 2040 and 2059

Reject: based on discussion in 07/2080r0; 2031, 2032 and 2033

Editorial Instructions:

Tgy Editor: Change text in 11.14.3, pg 26 line 36, D3.0  as follows

Dependent STAs shall have dot11RegLocRequired and dot11DSERequired set to true.

Tgy Editor: replace Figure 204a in 11.14.3, pg 27 line 2, D3.0  as follows

Figure 204a—Dependent STA state machine

Tgy Editor: Change text in 11.14.3, pg 26-28 line 42-11, D3.0  as follows

For DSE, the following statements apply:

—A STA with dot11DSERequired set to true shall not operate in an infrastructure BSS or IBSS unless it has received a Beacon frame or Probe Response frame from an enabling AP with the Spectrum Management bit set to 1 in the Capability Information field, and with the DSE registered location element RegLoc DSE bit set to 1 in the DSE registered location element. A dependent STA that is not associated with an enabling AP shall not transmit, except to attain enablement fromauthenticate and associate with an enabling AP, unless such action is as mandated in the regulatory domain (e.g. Emergency Services);

—A dependent STA shall only attempt enablementassociation with an enabling AP if the AP is transmitting Beacon frames with the RegLoc DSE bit set to 1 in the DSE registered location element;

—An associated dependent STA shall operate under the control of the enabling AP from which it attained enablementto which it is associated;

—A dependent STA shall create a dependent DSE registered location element containing the enabling AP’s DSE registered location element and having the RegLoc DSE bit set to 0 and the Dependent STA bit set to 1. Before enablementassociation, the Dependent Enablement Identifier (DEI) field shall be set to zero. Upon attaining enablementassociation or reassociation, the DEI field shall be set to the DEI value received by the dependent STA from the enabling AP in the MLME-ENABLEMENTASSOCIATE.response or MLME-REASSOCIATE.response;

—A dependent STA that has not attained enablementis not eassociated with an enabling AP shall only transmit for up to dot11DSEAssociateTimeLimit seconds, measured from the time of first transmission, while attempting to attain enablementassociate, then if it is not enabledassociated with an enabling AP, it shall not transmit for dot11DSEAssociateFailHoldTime seconds, before it can again attempt to attain enablementauthenticate and associate;

—An enabledassociated dependent STA shall cease transmission within dot11DSERenewalTime seconds if it has not received either a Beacon frame or a Probe Response frame from it’s enabling AP with the RegLoc DSE bit set to 1 in the DSE registered location element, and set all fields in bits 16-1759 of it’s DSE registered location element to zero;

—An enabled dependent STA shall return a DSE report in response to a Measurement Request DSE request, if the request is received from the AP with which it is associated or the enabling AP from which it received enablement.;

—A dependent STA shall create a Spectrum management Action frame containing the dependent DSE Registered Location Announcement element. Whenever The dependent STA shall count the sum of dot11TransmittedFragmentCount, dot11MulticastTransmittedFrameCount and dot11ReceivedFragmentCount, modulo dot11DSETransmitDivisor has a remainder of zero, the dependent STA shall and send this Action frame to the broadcast address using normal frame transmission rules, whenever the sum modulo dot11DSETransmitDivider has a remainder of zero.

.

Annex A

Editorial Instructions:

Tgy Editor: change text in Annex A, Table A.4.12 SM4 entry, pg 36, line 30, D3.

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| SM4 | Action frame protocol for spectrum management actions Measurement Request frame Measurement Report frame TPC Request frame TPC Report frame Channel Switch Announcement frame Extended Channel Switch Announcement frame DSE Registered Location Announcement frame | 7.3.1.11, 7.4 7.4.1.1 7.4.1.2 7.4.1.3 7.4.1.4 7.4.1.5 7.4.1.6 7.4.1.7 | CF10:M CF10:M CF10:M CF10:M CF10:M CF10:M CF15:M CF15:M | Yes 􀂆No 􀂆 Yes 􀂆No 􀂆 Yes 􀂆No 􀂆 Yes 􀂆No 􀂆 Yes 􀂆No 􀂆 Yes 􀂆No 􀂆 Yes □ No □ N/A □ Yes □ No □ N/A □ |

Tgy Editor: change text in Annex A, Table A.4.17 entries, pg 36, line 47, D3.0 as shown

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *DSE1 | Fixed station operation with RegLoc | 11.14.21 | CF15:O.1 | Yes □ No □ N/A □ |
| *DSE2 | Enabling AP operation with RegLoc | 11.14.21 | CF15:O.1 | Yes □ No □ N/A □ |
| DSE2.1 | Enabling AP creation of BSA | 11.14.32 | DSE2:M | Yes □ No □ N/A □ |
| DSE2.2 | Enabling AP operation with DSE | 11.14.21 | DSE2:M | Yes □ No □ N/A □ |
| *DSE3 | Dependent STA operation with DSE | 11.14.43 | CF15:O.1 | Yes □ No □ N/A □ |
| DSE3.1 | Dependent STA Enablement | 11.14.43 | DSE3:M | Yes □ No □ N/A □ |
| DSE3.2 | Dependent STA DSE Time to Associate | 11.14.43 | DSE3:M | Yes □ No □ N/A □ |
| DSE3.3 | Dependent STA DSE Time to Not Transmit | 11.14.43 | DSE3:M | Yes □ No □ N/A □ |
| DSE3.4 | Dependent STA DSE Probe ResponseRegistered Location Announcement frame | 11.14.43 | DSE3:M | Yes □ No □ N/A □ |
| DSE3.5 | Dependent STA MLME-ASSOCIATE.response DSE | 10.3.6.4 | DSE3:M | Yes □ No □ N/A □ |
| DSE3.6 | Dependent STA MLME-REASSOCIATE.response DSE | 10.3.7.4 | DSE3:M | Yes □ No □ N/A □ |
| DSE4 | DSE request report procedure Transmission of Measurement Request DSE request by an AP Transmission of Measurement Report DSE report by a STA Channel switch procedure Transmission of extended channel switch announcement and channel switch procedure by an AP Transmission of extended channel switch announcement and channel switch procedure by a STA Reception of extended channel switch announcement and channel switch procedure by a STA | 11.14.4 11.14.4 11.9.7 11.9.7 11.9.7 | (CF15& CF1):M (CF15& CF2):M (CF15&CF1):M (CF15&CF2):M CF15:M | Yes □ No □ N/A □ Yes □ No □ N/A □ |
| DSE5 | Station Association procedure Transmission of Association Request with SupportedRegulatoryClasses by a STA Transmission of Association Response with SupportedRegulatoryClasses by an AP | 9.8.3, 11.3.2.1 9.8.3, 11.3.2.2 | (CF15& CF2:M (CF15& CF1):M | Yes □ No □ N/A □ Yes □ No □ N/A □ |
| DSE6 | Station Reassociation procedure Transmission of Reassociation Request with SupportedRegulatoryClasses by a STA Transmission of Reassociation Response with SupportedRegulatoryClasses by an AP | 9.8.3, 11.2.3.3 9.8.3, 11.2.3.4 | (CF15& CF2:M (CF15& CF1):M | Yes □ No □ N/A □ Yes □ No □ N/A □ |
| DSE7 | Probe Request procedure Transmission of Probe Request with Supported Regulatory Classes by a STA | 11.9.7.3 | CF15&CF2:M | Yes □ No □ N/A □ |
| DSE8 | Probe Response procedure Transmission of Probe Response with Supported Regulatory Classes by an AP | 11.9.7.3 | CF15&CF1:M | Yes □ No □ N/A □ |

Annex D

CIDs: 2018, 2047, 2048

| 2018 | Annex D | 44 | 48 | E |  | dot11RegLocAgreement and dot11RegLocDSE DESCRIPTIONs refer to RegLoc STA, but should refer to Enabling AP | Change RegLoc STA's to Enabling AP's in both descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2047 | Annex D | 40 | 28 | T | Y | The name of "dot11RgLocRequired" suggests that something is required. However the definition provides no hint as to what is required | Change the definition so that it is clear what is required |
| 2048 | Annex D | 40 | 40 | T | Y | The name of "dot11DSERequired" suggests that something is required. However the definition only hints that the station is required to be enabled by an "enabling AP" | Change the definition so that it is clearer what is required |

Propose Accept Comment 2018 without discussion.

Comments 2047 and 2048 are about the adequacy of DESCRIPTION fields in MIB elements.

Proposed Comment Resolution: “The DESCRIPTION text will be clarified” Propose Accept in Principle Comment 2048.

Proposed Comment Resolution: “The DESCRIPTION text will be clarified or deleted” Propose Accept in Principle Comment 2047.

Proposed Resolution:

Accept 2018

Accept in Principle based on discussion and editorial instructions in 07/2080: 2047 and 2048

Tgy Editor: Delete text in Annex D, dot11RegLocRequired, dot11StationConfigEntry 60, pg 40, lines 28-38, D3.0

Tgy Editor: Change text in Annex D, dot11DSERequired, pg 40, lines 40-50, D3.0

dot11DSERequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

operation is dependent on enablement from enabling APsregisgtered stations. The capability is disabled otherwise. This attribute, when false, indicates that station operation does not depend on enablement from enabling APs. The default value of this attribute is false."

::= { dot11StationConfigEntry 61 }

Tgy Editor: Change text in Annex D, dot11LCIDSETable, pg 40, lines 36, 37, D3.0

‘dot11DSEAssociateFailHoldTime’

Tgy Editor: Change text in Annex D, dot11RegLocAgreement and dot11RegLocDSE , pg 44, lines 47, 64, D3.0

Reports the Enabling AP’sRegLoc STA’s

# General

| 2061 | General |  |  | T | Y | The description of the DSE procedures need a rewrite to make them much clearer and match the intent of the TG | It is hard to know how to rewrite the procedures until the intent of the TG is more obvious |
| --- | --- | --- | --- | --- | --- | --- | --- |

Comment 2061 says “The description of the DSE procedures need a rewrite to make them much clearer and match the intent of the TG”. Proposed Comment Resolution: “Comment appears to be a generalization of Comment 2052 by same commenter, which only addresses 11.14. Accepting 2052 and doing the supporting message formats causes changes to other clauses.” Propose Accept in Principle Comment 2061.

Proposed Resolution:

Accepted in Principle based on discussion in 07/2080: 2061