IEEE P802.11
Wireless LANs

| LB97 Explicit SNR Resolutions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date: 2007-07-11 |  |  |  |  |  |
| Author(s): |  |  |  |  |  |
| Name | Company | Address | Phone | email |  |
| Joonsuk Kim | Broadcom Corporation. |  |  | joonsuk@broadcom.com |  |
| Matthew Fischer | Broadcom Corporation |  |  |  | mfischer@broadcom.com |
| Assaf Kasher | Intel |  |  |  |  |

Abstract

This document proposes a resolution of Letter Ballot 97 comments in Explicit SNR tab in Beam adhoc.

The changes marked in this document are based on TGn Draft 2.02

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# CID 810,811,812,813,814,815

| 810 | 45.58 | 7.3.1.30 |  |  | SNR values are defined in MAC frame fields. Acronym list defines SNR as signal to noise ratio. But in this clause SNR is described as "linear power over the tones represented". Clearly this is a power metric and not a signal to noise ratio metric. | Change SNR to Tone Set Power Indicator (TSPI) or some other name which correctly describes this metric. |
| --- | --- | --- | --- | --- | --- | --- |
| 811 | 45.58 | 7.3.1.31 |  |  | SNR values are defined in MAC frame fields. Acronym list defines SNR as signal to noise ratio. But in this clause SNR is described as "linear power over the tones represented". Clearly this is a power metric and not a signal to noise ratio metric. | Change SNR to Tone Set Power Indicator (TSPI) or some other name which correctly describes this metric. |
| 812 | 45.58 | 7.3.1.32 |  |  | SNR values are defined in MAC frame fields. Acronym list defines SNR as signal to noise ratio. But in this clause SNR is described as "linear power over the tones represented". Clearly this is a power metric and not a signal to noise ratio metric. | Change SNR to Tone Set Power Indicator (TSPI) or some other name which correctly describes this metric. |
| 813 | 45.58 | 7.3.1.30 |  |  | SNR values are defined in MAC frame fields. SNR is described as "linear power over the tones represented". This implies that the MAC layer obtains this PHY layer information by some means. Yet the TGn PHY does not define a means to measure power over a set or subset of tones. And the MAC-PHY primitive interface does not contain any variables that would seem to transport this information to MAC layer. | Modify clause 20 to specify how the "linear power over the tones represented" are measured. Modify clause 20.5 to define the MAC-PHY primitive and variable used to convey this new measurement from PHY to MAC. |
| 814 | 45.58 | 7.3.1.31 |  |  | SNR values are defined in MAC frame fields. SNR is described as "linear power over the tones represented". This implies that the MAC layer obtains this PHY layer information by some means. Yet the TGn PHY does not define a means to measure power over a set or subset of tones. And the MAC-PHY primitive interface does not contain any variables that would seem to transport this information to MAC layer. | Modify clause 20 to specify how the "linear power over the tones represented" are measured. Modify clause 20.5 to define the MAC-PHY primitive and variable used to convey this new measurement from PHY to MAC. |
| 815 | 45.58 | 7.3.1.32 |  |  | SNR values are defined in MAC frame fields. SNR is described as "linear power over the tones represented". This implies that the MAC layer obtains this PHY layer information by some means. Yet the TGn PHY does not define a means to measure power over a set or subset of tones. And the MAC-PHY primitive interface does not contain any variables that would seem to transport this information to MAC layer. | Modify clause 20 to specify how the "linear power over the tones represented" are measured. Modify clause 20.5 to define the MAC-PHY primitive and variable used to convey this new measurement from PHY to MAC. |

Proposed resolution:  Counter.

Insert new RXVECTOR as follows:

TGn editor: add the following row at the end of Table 178 (page 212 in D2.02)

| Parameter | Condition | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- | --- |
| SNR | CHAN_MAT_TYPE is CSI_MATRICES | This parameter is a measure of the received SNR per chain. SNR indications of 8 bits are supported. SNR shall be the average of the dB values with scale in each receive chain as described in 7.3.1.30 (CSI Report Field) | N | Y |
|  | CHAN_MAT_TYPE is COMPRESSED_SV or NON_COMPRESSED_SV | This parameter is a measure of the received SNR per stream. SNR indications of 8 bits are supported. SNR shall be the average of the dB values with scale in each stream as described in 7.3.1.31 (Non-compressed Beamforming Report Field) and 7.3.1.32 (Compressed Beamforming Report Field) | N | Y |

# CID 1645

| 1645 | 46.16 | 7.3.1.30 |  |  | When carrier grouping is used, it is not clear whether the values to be sent back are smoothed across tones, or sampled individual tones. This may effect the way the beamformer interprets the data | Either specify that the channel estimates should be smoothed or not smoothed, or provide an extra bit to indicate whether smoothing was used. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:  Reject.

It is not necessary for Beamformer to know whether the channels are measured with smoothing or not. We don’t believe an extra bit to indicate the channel measuring method does not improve any interpretation for Beamformer when grouping is used.

# CID 3241

| 3241 | 50.00 | 7.3.1.32 |  |  | "..where SNR_average is the dB value of linear power averaged over the tones represented. This encoding covers the SNR range from -10 to 53.75 in 0.25 dB steps." It may be better to average dB values. | Change to: Sum of the dB values divided by the number of values summed. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:  Counter.

We need to make clear how to calculate SNR as follows:

TGn editor: In Page 45 line 55, Replace “where SNR_average is the dB value of linear power averaged over the tones represented. ” with “where SNR_average is sum of the dB values of SNR per tone divided by the number of tones represented ”

TGn editor: In Page 47 line 47, Replace “where SNR_average is the dB value of linear power averaged over the tones represented.” with “where SNR_average is sum of the dB values of SNR per tone divided by the number of tones represented ”

TGn editor: In Page 51 line 18, Replace “where SNR_average is the dB value of linear power averaged over the tones represented.” with “where SNR_average is sum of the dB values of SNR per tone divided by the number of tones represented ”