IEEE P802.11
Wireless LANs

| TGn LB97 Submission for PHY FEC Comment CID #3245 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Road, San Jose CA, 95131 | +1.408.451.8109 (o) +1.408.893.9357 (m) | George.Vlantis@st.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comment: #3245

The changes marked in this document are based on TGn Draft version P802 11n D2.0.pdf.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor:” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolutions

Click on icon below for modified “FEC” sheet of PHY Comment Resolution spreadsheet Doc #11-07-0341r7.  Changes within the spreadsheet are in red text.

# (1) CID 3245 (20.3.4: page 235; line 54)

Commenter’s Comment:  “If NES = 2 (BCC only), then every other input bit is encoded using a rate 1/2 convolutional code and identical encoders are used.  After encoding, the data bits are re-assembled into one sequence (bit 0 from encoder 0, bit 0 from encoder 1, bit 1 from encoder 0, etc.) before step j.”

Proposed Change: “This is a nit, but the NES = 2 case should be clearly defined here or a reference made to the subclause defining all operations.”

Ad-hoc Notes: “Editor2 -- transferring this comment to PHY.  Commenter is pointing out that reassembly of the two encoded bit streams into a single bit stream when two BCC encoders are used does no appear to be specified in 20.3.4, and also not in 20.3.10.  This appears to be a significant hole in the spec that needs to be fixed, but this is a technical change, not an editorial change.”

History:  Reclassified from Editorial to Technical; Transferred to PHY ad-hoc; and Assigned to George Vlantis at May meeting in Montreal.

Proposed Resolution:  Counter (Accept In Principle).

(See Discussion on next page.)

Discussion:

The commenter correctly points out that, in the NES=2 case for BCC encoding, the equation for the demultiplexing of the data bits into two streams for the two BCC encoders is given in paragraph h) of subclause 20.3.4 “Overview of the PPDU encoding process”, i.e. page 235, line 50, preceding parapraph i).  However, the reassembly operation back into a single stream, following the BCC encoding process, is not described at the end of paragraph i) nor at the beginning of paragraph j), nor is there a description of how to combine the two streams in the body of 20.3.10 “HT PCLP sublayer”.

Paragraph h) of subclause 20.3.4 (line 50 of page 235) describes the multiplexing operation as follows:

“h) If BCC encoding is to be used, and the value of NES is 2, then the scrambled data bits are divided between two BCC encoders by sending alternating bits to the two different encoders, as described in 20.3.10.4 (Encoder parsing.operation).”

Note that a forward reference is provided to subclause 20.3.10.4 “Encoder parsing operation”. A description of the demultiplexing operation and the equation (20-33) is indeed given in subclause 20.3.10.4 within the first two sentences.

However, the reassemly operation following the BCC encoding process back into a single stream is nowhere to be found.  The recommendation would be to insert a paragraph between between paragraphs i) and j) that provides a forward reference to a new subclause between the current 20.3.10.5 and 20.3.10.6 on page 264.  The new subclause 20.3.10.6 “Encoder reassembly operation” is derived from the first two sentences of subclause 20.3.10.4 “Encoder parsing operation, equation 20-33, and the notation used in subcluase 20.3.10.5 “Binary convolutional coding and puncturing.  Hence, I move to authorize the editor to insert the following new paragraph j) in subclause 2.3.4 and the new subclause 20.3.10.6 to 20.3.10 given below:

TGn Editor: In D2.0, page 235, line 60, insert the following new paragraph j) before the existing paragraph j) and renumber the succeeding paragraphs:

j) If BCC encoding is used, and the value of NES is 2, then the encoded data bits from the two BCC encoders are reassembled back into a single stream by selecting alternating bits from the two different encoders, as described in 20.3.10.6 (Encoder reassembly operation).”

TGn Editor: In D2.0 page 264, line 38, insert the following new subclause 20.3.10.6 before the existing subclause 20.3.10.6 and renumber the succeeding subclauses:

20.3.10.6 Encoder reassembly operation

If two encoders are used, the BCC encoded (and conditionally punctured) output sequences {yi0} and {yi1} are reassembled back into a single bit stream by sending alternating bits from each of the two different enoders.  The ith bit from the the jth encoder, denoted yi(j), is:

b N_ES • i + j  =  yi(j)  ;  0 ≤ j  ≤  NES – 1