IEEE P802.11
Wireless LANs

| Suggested editorial updates on information element to D1.05 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |

Abstract

This document provides the suggested updates to the current draft specificaiton to fix the inconsistenciy regarding information element length, and a minor editorial clarification.

## Summary of the intention of this document

- Some inconsistency was found among descriptions in subclauses under 7.3.2 and the contents in table 7.26, in terms of table entry and information element length.
This document intends to resolve these editorial inconsistencies.

- Somehow, one of the power management related information element is described in between the congestion control related information element. It is easier to read the spec, if the description is sorted by the services/features provided by the information element.
This document intends to provide this editorial clean up.

## Suggested update to the draft spec

- Relocate subclause 6.3.2.57 (Mesh ATIM window element) after subclause 7.3.2.64 (Mesh TIM element), to assist the ease of read.

- Replace the Table 7.26 in clause 7.3.2 with the following suggested table.

| - Element IDs |  |  |
| --- | --- | --- |
| Information element | Element ID | Length (in octets) |
| Mesh Configuration |  | 17 |
| Mesh ID |  | 2 to 257 |
| Link Metric Report |  | 3 to 257 |
| Target Transmission Rate |  | 20 |
| Offered Traffic Load |  | 18 |
| Neighborhood Congestion |  | 5 |
| Peer Link Management |  | 5 to 9 |
| Mesh Channel Switch Announcement |  | 15 |
| Mesh Neighbor List |  | 4 to 257 |
| Mesh TIM |  | 6-256 |
| Mesh ATIM window parameter |  | 4 |
| Beacon Timing |  | 7 to 257 |
| MDAOP Setup Request |  | 7 |
| MDAOP Setup Reply |  | 4-257 |
| MDAOP Advertisements Request |  | 2 |
| MDAOP Advertisements |  | 8 to 257 |
| MDAOP Set Teardown |  | 9 |
| Connectivity Report |  | 16 to 257 |
| Portal Announcement (PANN) |  | 19 |
| Root Announcement (RANN) |  | 23 |
| Path Request (PREQ) |  | 39 to 257 |
| Path Reply (PREP) |  | 34 to 257 |
| Path Error (PERR) |  | 14 |
| Proxy Update (PU) |  | 12 to 252 |
| Proxy Update Confirmation (PUC) |  | 10 |
| RA-OLSR Common |  | 13 to 257 |
| Mesh Security Capability |  | 9 |
| MSA |  | 17 to 257 |
| Reserved |  |  |

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D1.05, June 2007.