IEEE P802.11
Wireless LANs

| Suggested editorial updates on information element to D1.05 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |
| W Steven Conner | Intel Corp. | JF3-206, 2111 NE 5th Ave, Hillsboro, OR 97124 | +1-503-264-8036 | w.steven.conner@intel.com |

Abstract

This document provides the suggested updates to TGs Draft D1.05 to fix inconsistencies regarding information element length, and a minor editorial clarification.

## Summary of the intention of this document

- Some inconsistency was found among descriptions in subclauses under 7.3.2 and the contents in table 7-26, in terms of table entry and information element length.
This document intends to resolve these editorial inconsistencies.

- Somehow, one of the power management related information element is described in between the congestion control related information element. It is easier to read the spec, if the description is sorted by the services/features provided by the information element. This document intends to provide this editorial clean up.

## Background

Table 7-26 in Clause 7.3.2 summarizes the Element IDs for all of the information elements defined in the 802.11 standard, including the IE name, the Element ID, and the Length.  The length stored in this table specifies the length of the Information field, including 1 octet Element ID and 1 octet Length, in addition to the information specific to the element.  Thus, the length shown in this table should be 2-octets larger than the length value that is encoded in an information element, which only specifies the length of the information contained in the element.  The valid range for the length values in Table 7-26 is 2-257 octets (corresponding to elements with 0-255 octet information length).

## Proposed update to the TGs Draft D1.05:

- Relocate subclause 7.3.2.57 (Mesh ATIM window element) after subclause 7.3.2.64 (Mesh TIM element), to assist ease of reading the draft.

- Change the contents of Table 7-26 in D1.05 as shown with Word change-tracking below:

| Table 7-26 – Element IDs |  |  |
| --- | --- | --- |
| Information element | Element ID | Length (in octets) |
| Mesh Configuration |  | 17 |
| Mesh ID |  | 2 to 34 |
| Link Metric Report |  | 3 to 257 |
| Congestion Notification |  | 10 |
| Peer Link Management |  | 5 to 9 |
| Mesh Channel Switch Announcement |  | 15 |
| Mesh Neighbor List |  | 4 to 257 |
| Mesh TIM |  | 6-256 |
| Mesh ATIM window parameter |  | 4 |
| Beacon Timing |  | 7 to 257 |
| MDAOP Setup Request |  | 7 |
| MDAOP Setup Reply |  | 4-257 |
| MDAOP Advertisements Request |  | 2 |
| MDAOP Advertisements |  | 3 to 257 |
| MDAOP Set Teardown |  | 9 |
| Connectivity Report |  | 16 to 257 |
| Portal Announcement (PANN) |  | 19 |
| Root Announcement (RANN) |  | 23 |
| Path Request (PREQ) |  | 39 to 257 |
| Path Reply (PREP) |  | 34 to 257 |
| Path Error (PERR) |  | 14 |
| Proxy Update (PU) |  | 12 to 252 |
| Proxy Update Confirmation (PUC) |  | 10 |
| HELLO |  | 15 to 257 |
| Topology Control (TC) |  | 15 to 257 |
| Multiple Interface Declaration (MID) |  | 19 to 257 |
| Local Association Base Advertisement (LABA) |  | 19 to 257 |
| Local Association Base Checksum Advertisement (LABCA) |  | 13 to 257 |
| Association Base Block Request (ABBR) |  | 20 to 257 |
| Mesh Security Capability |  | 9 |
| MSA |  | 17 to 257 |

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D1.05, June 2007.