IEEE P802.11
Wireless LANs

| Unified Mesh Management Frame format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Srini Duvvuri | Atheros Communications | 5480 Great America Parkway, Santa Clara CA 95054 |  | sduvvuri@atheros.com |
| Kevin Hayes | Atheros Communications | 5480 Great America Parkway, Santa Clara CA 95054 |  | Kevin@atheros.com |

Abstract

This document details the changes required in order to support a unified mesh management frame format.  Many of these changes are extensions to what is already contained in doc 11-07/799r7.

Note: All instructions to the TGs editor are shown in purple and are based on IEEE 802.11s Draft D1.04.  Changes to text in D1.04 are shown with insertions in blue and deletions in red.

#1 - Change the contents of Table 1 in D1.04 as shown:

- Type and subtype fields

| - Valid type and subtype combinations |  |  |  |
| --- | --- | --- | --- |
| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| 00 | Management | 1111 | Mesh Action Reserved |
| 11 | Extended | 0000 | Mesh Data |
| 11 | Extended | 0001 | Mesh Management |
| 11 | Extended | 0010-1111 | Reserved |

#2 - Change Clause 7.1.3.5a.1 in D1.04 as shown:

7.1.3.5a.1 General

The mesh header field is a 4 or to 16 22 octet field that includes:

- an 8-bit Mesh Flags field to control mesh header processing

- a time to live field for use in multi-hop forwarding to aid in limiting the effect of transitory path selection loops

- a mesh sequence number to suppress duplicates in flooding algorithms and for other services

- and optionally a 12 6-18-octet mesh address extension field containing two extended addresses resulting in enabling up to a total of 6 addresses in mesh data frames

The Mesh Header field, shown in Figure s4, is present in all Data frames of type Extended with subtypes Mesh Data and Mesh Management if and only if they are transmitted between peer MPs with an established peer link.  Data frames including the Mesh Header field are referred to as Mesh Data frames.  The Mesh Header is also included in Management frames of subtype Mesh Action.

| Octets: 1 | 1 | 2 | 12 6-18 |
| --- | --- | --- | --- |
| Mesh Flags | Mesh Time To Live (TTL) | Mesh Sequence Number | Mesh Address Extension (present in some configurations) |
| - Mesh Header field |  |  |  |

#3 – Change Clause 7.1.3.5a.2 “Mesh Flags field” in D1.04, including the addition of Table AA, as shown:

7.1.3.5a.2 Mesh Flags field

The Mesh Flags field, shown in Figure s5, is 8 bits in length and the flags therein are used to control mesh-specific header processing, e.g., for mesh address extension.

| B0 B1 |  | B2 | B1B3 B7 |
| --- | --- | --- | --- |
| Address Extension (AE) Mode |  | Multihop Control Mode | Reserved |
| Bits: 12 |  | 1 | 75 |
|  | - Mesh Flags field |  |  |

The “Address Extension (AE) Mode” flag field is used to indicate the contents existence of — and thereby the use of — the mMesh aAddress eExtension field. for mesh address extension based on 6-address format: If AE = 1, the mesh address extension field follows the Mesh Sequence Number field and provides two additional address fields for mesh address extension (see 11A.5.5 for the usage of address fields in 6-address format); if AE is 0, the mesh address extension field is not present. The “AE” flag can be set to 1 only when both “To DS” and “From DS” fields are set to 1.  Table AA defines valid values for the Address Extension Mode (the numeric values in Table AA are shown in binary) and describes the corresponding contents of the Mesh Address Extension field (see 11A.5.5 for the usage of the optional addresses contained in the Mesh Address Extension field).  If the Address Extension Mode is set to 0, the Mesh Address Extension field is not present.  For all other values, the Mesh Address Extension field follows the Mesh Sequence Number field.

The Multihop Control Mode field is used to indicate whether this frame is intended for multiple hops or for single hop forwarding.  When this field is set to 1, the TTL, Mesh Sequence Number, and Mesh Address Extension fields are all present in the Mesh Header.  Otherwise, the frame is intended for single hop usage and none of the other fields in the Mesh Header are present.  The Multihop Control Mode field is set to 1 for mesh data frames, is set to 0 for single hop mesh action frames, and set to 1 for Multihop mesh action frames.

The reserved bits are set to zero.

Table AA – Valid values for the Address Extension Mode

| Address Extension Mode value | Address Extension Mode description | Mesh Address Extension field length (octets) | Applicable frame types |
| --- | --- | --- | --- |
| 00 | No Mesh Address Extension field | 0 | Data |
| 01 | Mesh Address Extension field contains Addr4 | 6 | Management |
| 10 | Mesh Address Extension field contains Addr5 and Addr6 | 12 | Data |
| 11 | Mesh Address Extension field contains Addr4, Addr5, and Addr6 | 18 | Management |

#4 - Change Clause 7.1.3.5a.5 “Mesh Address extension field” in D1.04, as shown:

- Mesh Address Extension field

The Mesh Address Extension field, shown in Figure s6, is 6 to 18 12 octets in length and follows the Mesh Sequence Number field only when the “AE” flag Address Extension Mode subfield of the Mesh Flags field is set to 1 a non-zero value. The Mesh Address Extension field provides two up to three additional address fields, Address 5 and Address 6, for mesh address extension as defined in Table AA based on 6-address format.

| Octets: 6 | 6 | 6 |  |
| --- | --- | --- | --- |
| Address 4 | Address 5 | Address 6 |  |
| - Mesh Address Extension field |  |  |  |

Address 4 may be used in Management frames of subtype Mesh Action to add a fourth address (which is missing from the MAC header of Management frames).  Address 4 is not included in the Mesh Address Extension field of Data frames.

#5 - Replace the text in 7.2.3 in D1.04 with the following instructions and text describing changes in the baseline as shown:

- Management frames

Change the text of Clause 7.2.3 as shown:

[…]

The address fields for all management frames do not vary by frame subtype subtypes except Mesh Action are as follows.

[…]

The address fields for the management frame subtype Mesh Action are as follows.

The Address 1 (RA) field is the the receiver of the frame.

The Address 2 (TA) field is the transmitter of the frame.

The Address 3 (DA) field is the destination of the frame.

#6 - Insert the following new clause after 7.2.3.12 in D1.04:

7.2.3.13 Multihop Action Frame format

The frame body of a management frame of subtype Mesh Action contains the information shown in Table sA.

Table sA—Mesh Action frame body

| Order | Information |
| --- | --- |
| 1 | Mesh Header |
| 2 | Action |
| Last | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. |

#7- Make the following changes to each of the multihop action frame formats currently defined as subclauses of 7.4b “ Multihop Action frame format details”:

- Add the following sentences after the first paragraph of each subclause of 7.4b for each frame format: “The Multihop Control Mode subfield of the Mesh Flags field in the Mesh Header is set to 1, enabling transmission of this frame on a multihop path.”

#8 – Make the following edits to 11A.5.5 Frame addressing and forwarding in a mesh network

- In 11A.5.5.2.1, replace all instances of “in the header” with "in the Mesh Header".

- In 11A.5.5.2.2, add ", and the frame is a Mesh data frame," after "If Address 3 matches the MP’s own MAC address"

#9 – Change 11A.5.5.5 in D1.04 as shown:

11A.5.5.5 Note on 7.2.3 Mesh Management Frames

Management frames which utilize the normal 3-address management frame headers specified in 7.2.3, such as Mesh Management Action frames (3-address action frames) described in 7.4.6,  Mesh management frames may be either single-hop or multiple hop, depending on the setting of the Multihop Control field.

Note that in several cases, the reception and processing of an 3-address action frame leads to the transmission of a new action frame of the same type that may include an identical or a modified version of the contents from the information elements of the received action frame.

#10 - Insert the following new clause after 11A.5.5.5 in D1.04:

11A.5.5.6 Forwarding of Mesh Action Frames

Mesh Action frames (see 7.2.3.13) are forwarded according to the procedures in 11A.5.5.2 and 11A.5.5.3 by using the Source MP address field value as the SA and the Destination MP address field value as the DA with respect to mesh forwarding.  The transmitter sets the Multihop Control field to one.