IEEE P802.11
Wireless LANs

| TGk D8.0 SB Resolutions Assigned to Joe Kwak |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-JUL-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | Joekwak @sbcglobal.net |

Abstract

This document presents the proposed changed text needed to satisfy the Sponsor Ballot Resolutions assigned to Joe Kwak.  The comment IDs included here are: 1,2,3,4,57,58,61,62,64, and 66. The included text is based on D8.0.  WORD track changes is used to show draft changes needed to resolve the above listed comments.

7.3.2.39 BSS Average Access Delay element

The BSS Average Access Delay element contains the AP Average Access Delay which is a measure of load in the BSS and is available in both QoS APs and non-QoS APs. The element information field is defined in Figure 112i.

Figure 112i—BSS Average Access Delay element format

The Length field in octets is set to 1.

The AP Average Access Delay is a scalar indication of the relative level of loading at an AP. A low value indicates more available capacity than a higher value. If the AP is not currently transmitting any DCF or EDCA traffic, the AP Average Access Delay is set to 255. The values between 1 and 252 are a scaled representation of the average medium access delay for DCF or EDCA transmitted frames measured from the time the DCF or EDCA MPDU is ready for transmission (i.e. begins CSMA/CA access) until the actual frame transmission start time. Non-QoS APs average the access delays for all DCF transmitted frames. QoS APs average the access delays for all EDCA transmitted frames of all ACs. The AP Average Access Delay values are scaled as follows:

0: 		Access Delay  < 8 µs

1:		8 µs < Access Delay < 16 µs

2<

n<14:		n*8 µs < Access Delay  < (n+1)*8 µs

15:		120 µs < Access Delay  < 128 µs

16:		128 µs < Access Delay  < 144 µs

17<

n<106: 		(n*16)-128 µs < Access Delay  < ((n+1)*16)-128 µs

107:		1584 µs < Access Delay  < 1600 µs

108:		1600 µs < Access Delay  <  1632 µs

109<

n<246:		(n*32)-1856 µs <  Access Delay  < ((n+1)*32)-1856 µs

247:		6048 µs < Access Delay  <  6080 µs

248:		6080 µs < Access Delay  <  8192 µs

249:		8192 µs < Access Delay  <  12288 µs

250:		12288 µs <  Access Delay <  16384 µs

251:		16384 µs <  Access Delay <  20480 µs

252: 		20480 µs <  Access Delay <  24576 µs 253 24576 µs <  Access Delay

254:		Service unable to access channel

255:		Measurement not available

The values 0-253 indicate Average Access Delay when one or more frames were transmitted during the measurement window. The value 254 indicates that DCF or EDCA services are currently unable to access the channel due to continuous carrier sense mechanism deferral and that no frames were transmitted during the measurement window. The AP measures and averages the medium access delay for all transmit frames using the DCF or EDCA over a continuous thirty second measurement window. See 11.14 for accuracy requirements.

7.3.2.44 BSS AC Access Delay element

The BSS AC Access Delay element contains an Access Category Access Delay field, as shown in Figure 112m.

Figure 112n—BSS AC Access Delay element format

The Length field in octets is set to 4.

The AC Access Delay field is formatted as four subfields as shown in Figure 112r. The AC Access Delay is a scalar indication of the average access delay at a QoS AP for services for each of the indicated Access Categories. If the QoS AP is not currently transmitting any traffic using the indicated AC, the Average Access Delay for that AC is set to 255. The values between 1 and 252 are a scaled representation of the average medium access delay for transmitted frames in the indicated AC measured from the time the EDCA MPDU is ready for transmission (i.e. begins CSMA/CA access) until the actual frame transmission start time. The AC Access Delay values are scaled as follows:

0: 		Access Delay  < 8 µs

1:		8 µs < Access Delay < 16 µs

2<

n<14: 		n*8 µs < Access Delay  < (n+1)*8 µs
.

15:		120 µs < Access Delay  < 128 µs

16:		128 µs < Access Delay  < 144 µs

17<

n<106: 		(n*16)-128 µs < Access Delay  < ((n+1)*16)-128 µs

107:		 1584 µs < Access Delay  < 1600 µs

108:		1600 µs < Access Delay  <  1632 µs

109<

n<246: 		(n*32)-1856 µs <  Access Delay  < ((n+1)*32)-1856 µs

247:		6048 µs < Access Delay  <  6080 µs

248:		6080 µs < Access Delay  <  8192 µs

249:		8192 µs < Access Delay  <  12288 µs

250:		12288 µs <  Access Delay <  16384 µs

251:		16384 µs <  Access Delay <  20480 µs

252: 		20480 µs <  Access Delay <  24576 µs

253: 		24576 µs <  Access Delay

254:		Service unable to access channel

255:		Measurement not available

The values 0-253 indicate Average Access Delay when one or more frames were transmitted during the measurement window. The value 254 indicates that EDCA services are currently unable to access the channel due to continuous carrier sense mechanism deferral to higher priority AC transmissions and that no frames were transmitted during the measurement window. The QoS AP measures and averages the medium access delay for all transmit frames of the indicated AC using EDCA mechanism over a continuous thirty second measurement window. See 11.14 for accuracy requirements.

|  | Average Access Delay for Best Effort | Average Access Delay for Background | Average Access Delay for Video | Average Access Delay for Voice |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure 112o—Access Category Access Delay subfields

10.3.6 Associate

10.3.6.1 MLME-ASSOCIATE.request

10.3.6.2 MLME-ASSOCIATE.confirm

[EDITORS NOTE TO REVIEWERS:  10.3.6.2 is to be added to current draft and changed as shown.]

10.3.6.2.1 Function

This primitive reports the results of an association attempt with a specified peer MAC entity that is within an AP.

10.3.6.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.confirm(
ResultCode,
CapabilityInformation,
AssociationID,
SupportedRates,
EDCAParameterSet,
RCPI.request,
RSNI.request,
RCPI.response,
RSNI.response,
VendorSpecificInfo
)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED_REASON_UNSPECIFIED, REFUSED_NOT_AUTHENTICATED, REFUSED_CAPABILITIES_MISMATCH, REFUSED_EXTERNAL_REASON, REFUSED_AP_OUT_OF_MEMORY, REFUSED_BASIC_RATES_MISMATCH | Indicates the result of the MLMEASSOCIATE.request. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capabilities advertised by the AP. |
| AssociationID | Integer | 1–2007 inclusive | If the association request result was SUCCESS, then AssociationID specifies the association ID value assigned by the AP. |
| SupportedRates | Set of integers | 2–127 inclusive (for each integer in the set), bit 7 is set to 1 to indicate that a rate is a member of the BBSBasicRateSet. | The set of data rates (in units of 500 kb/s) that are supported by AP, including indication of which rates are part of the BSSBasic-RateSet (according to 7.3.2.2). |
| EDCA-ParameterSet | As defined in frame format | As defined in frame format | Specifies the EDCA parameter set that the STA should use. The parameter shall be present only if the MIB attribute dot11QosOptionImplemented is true. |
| RCPI.request | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| RSNI.request | Integer | As defined in 7.3.2.41 | RSNI at the time the corresponding Association Request frame was received. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| RCPI.response | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Response frame. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| RSNI.response | Integer | As defined in 7.3.2.41 | RSNI at the time the corresponding Association Response frame was received. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| Vendor-SpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.6.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-ASSOCIATE.request or receipt of an association response frame from the peer MAC entity to associate with a specified peer MAC entity that is within an AP.

10.3.6.2.4 Effect of receipt

The SME is notified of the results of the association procedure.

10.3.6.3 MLME-ASSOCIATE.indication

10.3.6.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.indication	(
PeerSTAAddress, 
CapabilityInformation, 
ListenInterval, 
SSID, 
SupportedRates,
RSN
QoSCapability,
RCPI,
RSNI
VendorSpecificInfo
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the association was received. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the association request. |
| ListenInterval | Integer | >=0 | Specifies the listen interval value provided by the peer MAC entity as part of the association request. |
| SSID | Octet string | 0-32 octets | Specifies the SSID provided by the peer MAC entity as part of the association request. |
| SupportedRates | Set of integers | 2-127 inclusive (for each integer in the set) | The set of data rates (in units of 500 kb/s) that are supported by the STA that is requesting association. |
| RSN | RSN information element | As defined in frame format | A description of the cipher suites and AKM suites supported in the BSS. Only one pairwise cipher suite and only one authenticated key suite are allowed in the RSN information element. |
| QoSCapability | As defined in frame format | As defined in frame format | Specifies the parameters within the QoS Capability that are supported by the peer MAC entity. The parameter may be present only if the MIB attribute dot11QosOptionImplemented is true. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. The element may be present only when te MIB attribute dot11RadioMeasurementEnabled is true. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Association Request frame was received. The element may be present only when te MIB attribute dot11RadioMeasurementEnabled is true. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.6.4 MLME-ASSOCIATE.response

10.3.6.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
RCPI,
RSNI
VendorSpecificInfo
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the association request was received. |
| ResultCode | Enumeration | SUCCESS, REFUSED_REASON_UNSPECIFIED, REFUSED_CAPABILITIES_MISMATCH, REFUSED_EXTERNAL_REASON, REFUSED_AP_OUT_OF_MEMORY, REFUSED_BASIC_RATES_MISMATCH | Indicates the result response to the association request from the peer MAC entity. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the association request. |
| AssociationID | Integer | 1-2007 inclusive | If the association request result was SUCCESS, then AssociationID specifies the association ID value assigned to the peer MAC entity by the AP. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. The element may be present only when te MIB attribute dot11RadioMeasurementEnabled is true. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Association Request frame was received. The element may be present only when te MIB attribute dot11RadioMeasurementEnabled is true. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.7 Reassociate

10.3.7.1 MLME-REASSOCIATE.request

10.3.7.2 MLME-REASSOCIATE.confirm

[EDITORS NOTE TO REVIEWERS:  10.3.7.2 is to be added to current draft and changed as shown.]

10.3.7.2.1 Function

This primitive reports the results of a reassociation attempt with a specified peer MAC entity that is within an AP.

10.3.7.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REASSOCIATE.confirm(
ResultCode,
CapabilityInformation,
AssociationID,
SupportedRates,
EDCAParameterSet,
RCPI.request,
RSNI.request,
RCPI.response,
RSNI.response,
VendorSpecificInfo
)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED_REASON_UNSPECIFIED, REFUSED_NOT_AUTHENTICATED, REFUSED_CAPABILITIES_MISMATCH, REFUSED_EXTERNAL_REASON, REFUSED_AP_OUT_OF_MEMORY, REFUSED_BASIC_RATES_MISMATCH | Indicates the result of the MLMEREASSOCIATE.request. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capabilities advertised by the AP. |
| AssociationID | Integer | 1–2007 inclusive | If the association request result was SUCCESS, then AssociationID specifies the association ID value assigned by the AP. |
| Supported-Rates | Set of integers | 2–127 inclusive (for each integer in the set), bit 7 is set to 1 to indicate that a rate is a member of the BBSBasicRateSet. | The set of data rates (in units of 500 kb/s) that are supported by AP, including indication of whichrates are part of the BSSBasic-RateSet (according to 7.3.2.2). |
| EDCAParameterSet | As defined in frame format | As defined in frame format | Specifies the EDCA parameter set that the STA should use. The parameter shall be present only if the MIB attribute dot11QosOptionImplemented is true. |
| RCPI.request | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| RSNI.request | Integer | As defined in 7.3.2.41 | RSNI at the time the corresponding Association Request frame was received. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| RCPI.response | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Response frame. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| RSNI.response | Integer | As defined in 7.3.2.41 | RSNI at the time the corresponding Association Response frame was received. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.7.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-REASSOCIATE.request to reassociate with a specified peer MAC entity that is within an AP.

10.3.7.2.4 Effect of receipt

The SME is notified of the results of the reassociation procedure.

10.3.7.3 MLME-REASSOCIATE.indication

10.3.7.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REASSOCIATE.indication	(
PeerSTAAddress, 
CurrentAPAddress,
CapabilityInformation, 
ListenInterval, 
SSID, 
SupportedRates,
RSN
QoSCapability,
RCPI,
RSNI
VendorSpecificInfo
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the reassociation was received. |
| CurrentAPAddress | MACAddress | Any valid individual MAC address | Specifies the address of the AP with which the peer station is currently associated. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the reassociation request. |
| ListenInterval | Integer | >=0 | Specifies the listen interval value provided by the peer MAC entity as part of the association request. |
| SSID | Octet string | 0-32 octets | Specifies the SSID provided by the peer MACentity as part of the reassociation request. |
| SupportedRates | Set of integers | 2-127 inclusive (for each integer in the set) | The set of data rates (in units of 500 kb/s) that are supported by the STA that is requesting reassociation. |
| RSN | RSN information element | As defined in frame format | A description of the cipher suites and AKM suites supported in the BSS. |
| QoSCapability | As defined in frame format | As defined in frame format | Specifies the parameters within the QoS Capability that are supported by the peer MAC entity. The parameter may be present only if the MIB attribute dot11QosOptionImplemented is true. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Reassociation Request frame. The element may be present only when te MIB attribute dot11RadioMeasurementEnabled is true. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Reassociation Request frame was received. The element may be present only when te MIB attribute dot11RadioMeasurementEnabled is true. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

[NOTE TO REVIEWERS:  New Vendor Specific rows, as shown in above table, are also to be added to tables in 10.3.7.4.2, 10.3.12.1.2, 10.312.3.2, 10.3.14.1.2, and in 10.3.14.3.2.]

## 11.14 Access Delay Measurement

Access delay measurements are AP MAC layer measurements of average medium access delay for transmitted frames measured from the time the MPDU is ready for transmission (i.e. begins CSMA/CA access) until the actual frame transmission start time.  Access delay measurement results are included in the BSS Average Delay element and in the BSS AC Access Delay element.

For the BSS Average Delay measurement, the AP shall measure and average the medium access delay for all transmit frames using the DCF or EDCA over a continuous thirty second measurement window. For the BSS AC Access Delay measurement, the QoS AP shall measure and average the medium access delay for all transmit frames of the indicated AC (see Figure 112o) using EDCA mechanism over a continuous thirty second measurement window. The accuracy for the average medium access delay shall be +/- 100 µs or better when averaged over at least 200 frames. Accuracy is not defined for measurements averaged over less than 200 frames.