IEEE P802.11
Wireless LANs

Traffic Filtering and Sleep Mode

Date: 2007-07-11

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive San Jose, CA 95134 | +1 408 853 5570 | allant@cisco.com |
| Bob O’Hara | Cisco Systems | 170 W. Tasman Drive San Jose, CA 95134 | +1 408 853 5513 | boohara@cisco.com |
| Henry Ptasinski | Broadcom Corporation | 190 Mathilda Place Sunnyvale, CA 94086 | +1 408 543 3316 | henryp@broadcom.com |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave. Hillsboro, OR | +1 503 264 7799 | emily.h.qi@intel.com |
| James Tsai | Intel Corp. | 2111 NE 25th Ave. Hillsboro, OR | +1 503 712 5509 | james.tsai@intel.com |
| Partha Narasimhan | Aruba Networks | 1322 Crossnam Ave Sunnyvale, CA 94089 | +1 408 754 3018 | partha@arubanetworks.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 408 754 1213 | subbu@arubanetworks.com |
| Thomas Kuehnel | Microsoft Corporation | One Microsoft Way Redmond, WA 98052 | +1 425 706 4351 | tkuehnel@microsoft.com |
| Moo Ryong Jeong | DoCoMo USA Labs | 3240 Hillview Ave. Palo Alto, CA 94304 | +1 650 496 4761 | jeong@docomolabs-usa.com |
| Fujio Watanabe | DoCoMo USA Labs | 3240 Hillview Ave. Palo Alto, CA 94304 | +1 650 496 4726 | watanabe@docomolabs-usa.com |
| Choong Khong Neng | BT Aisian Research Centre | 1B-17, Block 1B, Plaza Sentral Jalan Stesen Central 5 Kuala Lumpur Sentral 50470 Kuala Lumpur Malaysia | +60 3 2091 9494 | khongneng.choon@bt.com |
| Low Andy Lock Yen | BT Aisian Research Centre | 1B-17, Block 1B, Plaza Sentral Jalan Stesen Central 5 Kuala Lumpur Sentral 50470 Kuala Lumpur Malaysia | +60 3 2091 9393 | andy.low@bt.com |
| Qi Wang | Broadcom Corporation | 190 Mathilda Place Sunnyvale, CA 94086 | +1 408 922 8798 | qi.wang@broadcom.com |
| Kevin Hayes | Atheros Communications | 5480 Great America Parkway Santa Clara CA 95054 | +1 408 773 5275 | kevin@atheros.com |
| Matt Smith | Atheros Communications | 5480 Great America Parkway Santa Clara CA 95054 |  | msmith@atheros.com |
| Joe Epstein | Meru Networks | 1309 S. Mary Ave. Sunnyvale, CA 94086 | +1 408 215 5345 | jepstein@merunetworks.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This submission contains the normative text for a traffic filtering and notification service and a sleep mode provided by APs to non-AP STAs. This submission is related to TGv Objective: Power Saving (Req. #2010).

The text is aligned with P802.11v-D0.13.

##### 3 Definitions

Insert the following definitions

3.zzz Traffic filter: a set of patterns, specified by the use of TCLAS information elements, that are utilized by the Traffic Filtering Service to identify specific frames.

3.yyy Traffic filtering service (TFS):  A service provided by an AP to a non-AP STA that can reduce the number of frames sent to the non-AP STA by discarding, without transmitting, unicast frames addressed to the non-AP STA that do not match traffic filters specified by the non-AP STA. If negotiated with the AP, the frames that do match at least one of the set of specified traffic filters are indicated to the non-AP STA via a notification frame.

3.xxx Sleep Mode: An extended power save mode for non-AP STAs whereby a non-AP STA need not listen for every DTIM Beacon frame, and can negotiate not to perform GTK/IGTK updates while in this mode.  TFS can be used by a non-AP STA while in sleep mode.

##### 7.2.3.6 Reassociation Request frame format

Insert the following rows in Table 12

| Order | Information | Notes |
| --- | --- | --- |
| 23 | TFS Request | The TFS Request element is present if dot11WirelessManagementImplemented is true and the TFS bit in the Wireless Network Management Capability element is set to 1. |
| 24 | Sleep Mode | The Sleep Mode element is present if dot11WirelessManagementImplemented is true and the Sleep Mode bit in Wireless Network Management Capability element is set to 1. |

##### 7.2.3.7 Reassociation Response frame format

Insert the following rows in Table 13

| Order | Information | Notes |
| --- | --- | --- |
| 23 | TFS Response | The TFS Response element is present if dot11WirelessManagementImplemented is true and the TFS bit in Wireless Network Management Capability element is set to 1 and the TFS Request element was present in the corresponding Reassociation Request frame. |
| 24 | Sleep Mode | The Sleep Mode element is present if dot11WirelessManagementImplemented is true and the Sleep Mode bit in Wireless Network Management Capability element is set to 1 and the Sleep Mode element was present in the corresponding Reassociation Request frame. |

### 7.3.2 Information Elements

Insert the following rows into Table 26

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| TFS Request (see 7.3.2.77) | x+16 | 5-256 |
| TFS Response (see 7.3.2.78) | x+17 | 2-256 |
| Sleep Mode (see 7.3.2.79) | x+18 | 5 |
| Reserved | x+1619, 220 |  |

Modify 7.3.2.25.3 as shown.

7.3.2.25.3 RSN capabilities

The RSN Capabilities field indicates requested or advertised capabilities. The value of each of the RSN

Capabilities fields is 0 if the RSN Capabilities field is not available in the RSN information element.

The length of the RSN Capabilities field is 2 octets. The format of the RSN Capabilities field is as illustrated in Figure 91 and described after the figure.

|  | B0 | B1 | B2 – B3 | B4 – B5 | B6 – B8 | B9 | <ANA1> | B12-B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pre-Auth | No Pairwise | PTKSA Replay Counter | GTKSA Replay Counter | Reserved | PeerKey Enabled | GTK/IGTK Update Policy | Reserved |

Editoral Note – The positions shown as <ANA1> and <ANA2> will be replaced with the appropriate values once assigned by the 802.11 ANA.

— Bit <ANA1>: GTK/IGTK Update Policy. An AP sets the GTK/IGTK Update Policy subfield of the RSN Capabilities field to 1 to signal that GTK/IGTK Updates are deferred for Sleep Mode STAs, and sets the subfield to 0 when GTK/IGTK Updates will not be deferred.

7.3.2.31 TCLAS element

Insert the following row into Table 42

| Classifier type | Classifier parameters |
| --- | --- |
| 0 | Ethernet parameters |
| 1 | TCP/UDP IP parameters |
| 2 | IEEE 802.1D/Q parameters |
| 3 | Filter Offset parameters |
| 34 – 255 | Reserved |

Insert the following text add the end of section 7.3.2.31

For Classifier Type 3, the classifier parameters are defined by an offset and a filter value. The Frame Classifier field for Classifier Type 3 is defined in Figure vnx. It has a variable length.

|  | Classifier Type (3) | Classifier Mask | Filter Offset | Filter Value |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure vnx—Frame Classifier field of Classifier Type 3 for MAC Offset parameters

The value of the Filter Offset field is the number of octets following the MAC header at which the Filter Value is compared, after any necessary decryption or disaggregation.  A value of zero for the Filter  Offset indicates that the Filter Value is to be compared to the octet immediately following the MAC header.

The Filter Value is an octet string that is compared to the frame content, beginning at the octet indicated by the Filter Offset.

7.3.2.62 Wireless Network Management capability information element

Insert the following rows into Table v1

| Bit | Information | Notes |
| --- | --- | --- |
| 9 | TFS | The TFS bit set to 1 indicates the STA supports TFS as described in section 11.20.11. The TFS bit set to 0 indicates the STA does not support TFS. |
| 10 | Sleep Mode | - The Sleep Mode bit set to 1 indicates the STA supports sleep mode as described in section 11.20.12. The Sleep Mode bit set to 0 indicates the STA does not support sleep mode. |
| 11-n*8 | Reserved | - All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

Insert the following after 7.3.2.76

##### 7.3.2.77 TFS Request element

The TFS Request element defines information about the traffic filters that will be enabled at the AP for the requesting non-AP STA.

|  |  |  |  |  |  | One or more TFS subelements |
| --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | TFS ID | TFS Action Code | TFS Subelement Count | TFS Subelements |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |

Figure vn. TFS Request element format.

The Element ID field is equal to the TFS Request value in Table 26.

The Length field is set to 3+n, where n indicates the total length of all TFS Subelements contained in the element.

The TFS ID field indicates a unique ID for the set of traffic filters specified in the TFS subelements.

The TFS Action Code field is bit significant.  It defines the actions taken at the AP when a frame matches a traffic filter.  The functions of the bits in this field are shown in Table vnt.

| Bit | Information | Notes |
| --- | --- | --- |
| 0 | Delete | Setting this bit to 1 indicates the traffic filter is to be deleted when a frame matches the traffic filter. A value of 0 for this bit indicates no deletion of the traffic filter. |
| 2 | Notify | - Setting this bit to 1 indicates the STA is to be sent a TFS Notify frame when a frame matches the traffic filter. Setting this bit toto 0 indicates the AP does not send a TFS Notify frame to the requesting STA. |
| 3-7 | Reserved | - All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

Figure vnt – TFS Action Code field values

The TFS Subelement Count field indicates the number of the TFS Subelements present.

The TFS Subelements field contains one or more TFS subelements. The TFS subelement contains one or more TCLAS information elements and zero or one TCLAS Processing information element, as shown in Figure vn1.

|  |  |  | One or more TCLAS Elements |  |
| --- | --- | --- | --- | --- |
|  | Subelement ID | Length | TCLAS Elements | TCLAS Processing Element |
| Octets: | 1 | 1 | variable | 3 |

Figure vn1. TFS Subelement format

The Subelement ID field uniquely identifies this subelement to be the TFS subelement.  The value of this field is 1.

The Length field indicates the number of octets that follow the field.  The value of this field is the sum of the lengths of the TCLAS element(s) plus the optional TCLAS Processing element, if present.

The TCLAS Elements field contains one or more TCLAS information elements to specify the traffic filter as defined in 7.3.2.31.

The TCLAS Processing Element field is optionally present and defines how multiple TCLAS information elements are processed as defined in 7.3.2.33.

##### 7.3.2.78 TFS Response element

The TFS Response provides information about the status of the requested traffic filter.

|  |  |  | One or more TFS Status Subelements |
| --- | --- | --- | --- |
|  | Element ID | Length | TFS Status Subelements |
| Octets: | 1 | 1 | n×2 |

Figure vn2. TFS Response element format.

The Element ID field is equal to the TFS Response value in Table 26.

The Length field is set to n×2, where n indicates the total length of all TFS Subelements contained in the element.

The TFS Status Subelement field contains the following information.

|  | TSF Response Status | TFS ID |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure vn3.  TFS Status subelement format

The TSF Response Status field indicates the status returned by the AP responding to the STA’s requested traffic filter, as indicated by Table v-n.

The TFS ID field provides the unique ID for the TFS traffic filter set.

Table v-n. TFS Status Definition

| Value | Description |
| --- | --- |
| 0 | Accept |
| 1 | Denied due toa malformed request or an ambiguous classifier |
| 2 | Denied due to the lack of resources on AP |
| 3 | Denied due to requested filter(s) matching 2 or more existing enabled traffic filters |
| 4 | Denied. By policy, requested traffic filter is not permitted to participate in TFS |
| 5 | Overridden due to the policy limits on AP |
| 6 | Denied. The AP is unable to perform the requested action |
| 7 | Overridden due to an alternate or a duplicate traffic filter set on AP |
| 8-255 | Reserved |

##### 7.3.2.79 Sleep Mode element

The Sleep Mode element is used to enter and exit the sleep mode. The format of the Sleep Mode element is shown in Figure v-10.

|  | Element ID | Length | Action Type | Sleep Mode Response Status | Update Policy | Sleep Interval |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure v-10 Sleep Mode element format

The Element ID field is equal to the Sleep Mode value in Table 26.

The value of the Length field is 1 or 3, depending on the value of Action Type.

The Action Type field is set to a number that identifies the type of sleep mode request and response. The Action Types are shown in Table v-10.

Table v-10 Action Type Definitions

| Name | Event Type | Length |
| --- | --- | --- |
| Enter Sleep Mode | 0 | 3 |
| Exit Sleep Mode | 1 | 1 |
| Reserved | 2 – 255 |  |

The Sleep Mode Response Status field indicates the status returned by the AP responding to the STA’s sleep mode request as defined in Table v-n+1. This field is valid only if the Sleep Mode element is present in the Sleep Mode Response frame and Reassociation Response frame.

Table v-n+1. Sleep Mode Response Status Definition

| Value | Description |
| --- | --- |
| 0 | Enter/Exit Sleep Mode Accept |
| 1 | Denied. The AP is unable to perform the requested action |
| 2 | Denied temporarily. The AP is unable to perform the requested action at the current time. The request can be submitted again at a later time. |
| 3 | Exit Sleep Mode Accept, GTK/IGTK update required |
| 4-255 | Reserved |

The Update Policy and Sleep Interval fields are present only if the Action Type field is set to “Enter Sleep Mode”.

The Update Policy field indicates the policy for security association updates. The set of valid Update Policy is defined in Table v-11.

Table v-11 Update Policy Definitions

| Bit | Information | Notes |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | GTK/IGTK Update Policy | The GTK/IGTK Update Policy bit set to 1 indicates that the AP will suppress GTK/IGTK updates to STAs that are in Sleep Mode. The GTK/IGTK Update Policy bit set to 0 indiates that the AP will not suppress GTK/IGTK updates to STAs that are in Sleep Mode. |
| 2-7 | Reserved |  |

The Sleep Interval field indicates to the AP how often a STA in Sleep Mode wakes to receive Beacon frames, defined as the number of DTIM intervals.  The value set to 0 indicates that the requesting Non-AP STA does not wake up at any specific interval.

### 7.4.11 Wireless Network Management action details

Insert the following rows in table v48

Table v48—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 15 | TFS Request |
| 16 | TFS Response |
| 17 | TFS Notify |
| 18 | Sleep Mode Request |
| 19 | Sleep Mode Response |
| 1520-255 | Reserved |

Insert the following after 7.4.8.15

##### 7.4.11.16 TFS Request frame format

The TFS Request frame is sent by a non-AP STA to the AP to request the specified traffic filter, defined by the TFS element.

|  | Category | Action | Dialog Token | TFS Request Element | Vendor Specific Elements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable |

Figure vn. TFS Request frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating TFS Request frame, as specified in 7.4.11.

The Dialog Token field is set to a value chosen by the STA sending the TFS Request frame to identify the request/response transaction.

The TFS Request Element field specifies the traffic filters that are requested by the non-AP STA, as specified in 7.3.2.65.

The Vendor Specific Elements field contains zero or more Vendor Specific elements, as defined in 7.3.2.26.

##### 7.4.11.17 TFS Response frame format

The TFS Response frame is sent by an AP in response to a TFS Request frame.

|  | Category | Action | Dialog Token | TFS Response Element | Vendor Specific Elements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable |

Figure vn. TFS Response frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating TFS Response frame, as specified in 7.4.11.

The Dialog Token field is set to the value in the corresponding TFS Request frame.

The TFS Response Element indicates the traffic filters that the AP  is configured to support, as specified in 7.3.2.66.

The Vendor Specific Elements field contains zero or more Vendor Specific element, as defined in 7.3.2.26.

##### 7.4.11.18 TFS Notify frame format

The TFS Notify frame is sent by an AP to a STA when a frame matching a traffic filter is encountered.

|  | Category | Action | TFS ID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure vn. TFS Notify frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating TFS Notify frame, as specified in 7.4.11.

The TFS ID field indicates the traffic filter set containing the TCLAS information element that was matched.

##### 7.4.11.19 Sleep Mode Request frame format

The Sleep Mode Request frame is sent by a non-AP STA to the AP to enter the Sleep Mode.

|  | Category | Action | Dialog Token | Sleep Mode Element | TFS Request Elements | Vendor Specific Elements |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable | variable |

Figure vn. Sleep Mode Request frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating Sleep Mode Request frame, as specified in 7.4.11.

The Dialog Token field is set to a value chosen by the STA sending the Sleep Mode Request frame to identify the request/response transaction.

The Sleep Mode Element field contains a Sleep Mode element that is requested by a non-AP STA, as described in 7.3.2.67.

The TFS Request Element field contains zero or one TFS Request elements to specify the traffic filters that are requested by a non-AP STA, as defined in 7.3.2.65.

The Vendor Specific Elements field contains zero or more Vendor Specific elements, as defined in 7.3.2.26.

##### 7.4.11.20 Sleep Mode Response frame format

The Sleep Mode Response frame is sent by an AP in response to a Sleep Mode Request frame.

|  | Category | Action | Dialog Token | Sleep Mode Element | TFS Request Element | Vendor Sepecific Element |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable | variable |

Figure vn. TFS Response frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating Sleep Mode Response frame, as specified in 7.4.11.

The Dialog Token field is set to the value in the corresponding Sleep Mode Request frame.

The Sleep Mode Element field contains a Sleep Mode element, as described in 7.3.2.67.

The TFS Request Element field contains zero or one TFS Request elements to specify the traffic filters, as defined in 7.3.2.65.

The Vendor Specific Elements field contains zero or more Vendor Specific element, as defined in 7.3.2.26.

    - Reassociate

- 10.3.7.11 MLME-REASSOCIATE.request

- 10.3.7.11.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.request	(
NewAPAddress,
ReassociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,
Content of FT Authentication Information Elements,
SupportedRegulatoryClasses,
WirelessManagementCapabilities,
PresenceParameters,
FBMSRequest,
TrafficGeneration,
TFSRequest,
SleepMode)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSRequest | As defined in the TFS Request element | As defined in the TFS Request element | Specifies the proposed service parameters for the TFS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Wireless Network Management Capability is set to 1. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed service parameters for Sleep Mode. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the Sleep Mode bit in the Wireless Network Management Capability is set to 1. |

- 10.3.7.12 MLME-REASSOCIATE.confirm

- 10.3.7.12.2Semantics of the service primitive

Change the primitive parameters as shown:

MLME-REASSOCIATE.confirm	(
Resultcode,
CapabilityInformation,
AssoicationID,
SupportedRates,
EDCAParameterSet,
Content of FT Authentication Information Elements,
SupportedRegulatoryClasses,
WirelessManagementCapabilities,
FBMSResponse, 
TFSResponse,
SleepMode
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSResponse | As defined in the TFS Response element | As defined in the TFS Response element | Specifies service parameters for the TFS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Wireless Network Management Capability is set to 1 and the TFS Request element is present in corresponding Reassociation Request frame. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies service parameters for Sleep Mode. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the Sleep Mode bit in the Wireless Network Management Capability is set to 1 and the Sleep Mode element is present in the corresponding Reassociation Request frame. |

- 10.3.7.13 MLME-REASSOCIATE.indication

- 10.3.7.13.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.indication	(
PeerSTAAddress,
CurrentAPAddress,
CapabilityInformation,
ListenInterval,
SSID,
SupportedRates,
RSN,
QoSCapability,
Content of FT Authentication Information Elements,
SupportedRegulatoryClasses,
WirelessManagementCapabilities,
PresenceParameters, 
FBMSRequest,
TrafficGeneration,
TFSRequest, 
SleepMode)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSRequest | As defined in the TFS Request element | As defined in the TFS Request element | Specifies the proposed service parameters for the TFS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Wireless Network Management Capability is set to 1. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed service parameters for the Sleep Mode. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the Sleep Mode bit in the Wireless Network Management Capability is set to 1. |

- 10.3.7.14 MLME-REASSOCIATE.response

- 10.3.7.14.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
Content of FT Authentication Information Elements, SupportedRegulatoryClasses,
WirelessManagementCapabilities,
FBMSResponse, 
TFSResponse,
SleepMode)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSResponse | As defined in the TFS Response element | As defined in the TFS Response element | Specifies service parameters for the TFS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Wireless Network Management Capability is set to 1 and the TFS Request element is present in the corresponding Reassociation Request frame. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies service parameters for Sleep Mode. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the Sleep Mode bit in the Wireless Network Management Capability is set to 1 and the Sleep Mode element is present in the corresponding Reassociation Request frame. |

Insert the following clauses after 10.3.59

### 10.3.60 TFS Setup

The following MLME primitives support the signaling of TFS Setup.  The signaling is shown in Figure vt1.

- Figure vt1 – TFS request and response exchange

#### 10.3.60.1 MLME-TFS.request

##### 10.3.60.1.1 Function

This primitive requests that a TFS Request frame be sent to the AP with which the STA is associated.

##### 10.3.60.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TFS.request	(
PeerSTAAddress,

TFSRequest,

VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity for TFS . |
| TFSRequest | As defined in the TFS Request element | As defined in the TFS Request element | Specifies the proposed service parameters for the TFS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Wireless Network Management Capability is set to 1. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the TFS Request frame. |

##### 10.3.60.1.3 When Generated

This primitive is generated by the SME to request that a TFS Request frame be sent to the AP with which the STA is associated.

##### 10.3.60.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TFS Request action management frame. The STA then attempts to transmit this to the AP with which it is associated.

#### 10.3.60.2 MLME-TFS.confirm

##### 10.3.60.2.1 Function

This primitive reports the result of a TFS procedure.

##### 10.3.60.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- TFS.confirm	(
TFSResponse,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TFSResponse | As defined in the TFS Response element | As defined in the TFS Response element | Specifies service parameters for the TFS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Wireless Network Management Capability is set to 1 and the TFS Request element is present in the corresponding Reassociation Request frame. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the TFS Response frame. |

##### 10.3.60.2.3 When Generated

This primitive is generated by the MLME as a result of the receipt of a TFS Response action management frame.  It  indicates the results of the request.

This primitive is also generated when the MLME-TFS.request contains invalid parameters and when a timeout or failure occurs.

##### 10.3.60.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the Element Status and may use the reported data.

#### 10.3.60.3 MLME-TFS.indication

##### 10.3.60.3.1 Function

This primitive indicates that a TFS Request frame was received from a non-AP STA.

##### 10.3.60.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TFS.indication	(
PeerSTAAddress,
TFSRequest,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TFS Request frame was received. |
| TFSRequest | As defined in the TFS Request element | As defined in the TFS Request element | Specifies the proposed service parameters for the TFS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and the TFS bit in the Wireless Network Management Capability is set to 1. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the TFS Request frame. |

##### 10.3.60.3.3 When Generated

This primitive is generated by the MLME when a valid TFS Request action management frame is received.

##### 10.3.60.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 9.2.7.1.1.

#### 10.3.60.4 MLME-TFS.response

##### 10.3.39.4.1 Function

This primitive is generated in response to an MLME-TFS.indication requesting a TFS Response frame be sent to a non-AP STA.

##### 10.3.60.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TFS.response	(
PeerSTAAddress,
TFSResponse,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TFS Request frame was received. |
| TFSResponse | As defined in the TFS Response element | As defined in the TFS Response element | Specifies service parameters for the TFS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and TFS bit in the Wireless Network Management Capability is set to 1 and the TFS Request element is present in corresponding Reassociation Request frame. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the TFS Response frame. |

##### 10.3.60.4.3 When Generated

This primitive is generated by the SME in response to an MLME-TFS.indication requesting a TFS Response be sent to a non-AP STA.

##### 10.3.60.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TFS Response action management frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

### 10.3.61 Sleep Mode request

This set of primitives supports the exchange of sleep mode parameter information between peer SMEs. Figure vs1 depicts the sleep mode request and response process.  Figure vs1 illustrates the basic protocol and is only an example and therefore is not meant to be exhaustive of all possible protocol uses.

- Figure vs1—Sleep Mode Request and Response Exchange

#### 10.3.61.1 MLME-SLEEPMODE.request

##### 10.3.61.1.1 Function

This primitive requests that a Sleep Mode Request frame be sent to the AP with which the STA is associated.

##### 10.3.61.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.request	(
PeerSTAAddress,

DialogToken,
SleepMode,

TFSRequest,
VendorSpecific)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Sleep Mode Request frame is to be sent. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Request frame. |
| TFSRequest | As defined in the TFS Request element | As defined in the TFS Request element | Specifies the proposed TFS service parameters for the Sleep Mode Request frame. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the Sleep Mode Request frame. |

##### 10.3.61.1.3 When Generated

This primitive is generated by the SME to request that a Sleep Mode Request frame be sent to the AP with which the STA is associated.

##### 10.3.61.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Sleep Mode Request action frame. The STA then attempts to transmit this to the AP with which it is associated.

#### 10.3.61.2 MLME-SLEEPMODE.indication

##### 10.3.61.2.1 Function

This primitive indicates that a Sleep Mode Request frame was received from a non-AP STA.

##### 10.3.61.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.indication	(
PeerSTAAddress,
DialogToken,
SleepMode,
TFSRequest,
VendorSpecific
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the Sleep Mode request frame is received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Request frame. |
| TFSRequest | As defined in the TFS Request element | As defined in the TFS Request element | Specifies the proposed TFS service parameters for the Sleep Mode Request frame. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the Sleep Mode Request frame. |

##### 10.3.61.2.3 When Generated

This primitive is generated by the MLME when a valid Sleep Mode Request frame is received.

##### 10.3.61.2.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.1 (TBD).

#### 10.3.61.3 MLME-SLEEPMODE.response

##### 10.3.61.3.1 Function

This primitive requests the transmission of Sleep Mode information to a peer entity, in response to a received Sleep Mode Request frame.

##### 10.3.61.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.response	(
PeerSTAAddress,

DialogToken,
SleepMode,

TFSRequest,
VendorSpecific)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Sleep Mode Response frame is to be sent. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Response frame. |
| TFSRequest | As defined in the TFS Request element | As defined in the TFS Request element | Specifies the proposed TFS service parameters for the Sleep Mode Response frame. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the Sleep Mode Response frame. |

##### 10.3.61.3.3 When Generated

This primitive is generated by the SME to request that a Sleep Mode Response frame be sent to a peer entity to convey Sleep Mode information.

##### 10.3.61.3.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Sleep Mode Response frame containing the Sleep Mode elements specified. This frame is then scheduled for transmission.

#### 10.3.61.4 MLME-SLEEPMODE.confirm

##### 10.3.61.4.1 Function

This primitive reports the result of a request to send a Sleep Mode Response action frame.

##### 10.3.61.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.confirm	(
PeerSTAAddress,

DialogToken,
SleepMode,

TFSRequest,
VendorSpecific)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which the Sleep Mode Response frame is received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Response frame. |
| TFSRequest | As defined in the TFS Request element | As defined in the TFS Request element | Specifies the proposed TFS service parameters for the Sleep Mode Response frame. |
| VendorSpecific | As defined in the Vendor Specific element | As defined in the Vendor Specific element | Specifies the vendor specific parameters for the Sleep Mode Response frame. |

##### 10.3.61.4.3 When Generated

This primitive is generated by the MLME when a Sleep Mode Response frame is received.

##### 10.3.61.4.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the result code.

Insert new subclauses after 11.20

### 11.20.11 TFS Procedures

Traffic filtering is a service that may be provided by an AP to its associated STAs, where the AP examines incoming frames destined for a STA and determines if any of those frames match a specific set of traffic filters that may be enabled at the AP per the request of the STA.  Unicast frames that do not match any of the traffic filters in the set are discarded.  Unicast frames that do match at least one of the set of the enabled traffic filters are delivered to the STA.  The STA may also negotiate to have a notification frame sent prior to the delivery of the frame matching the traffic filter.

When  a traffic filter for broadcast or multicast frames is enabled at the AP, the broadcast and multicast frames are still delivered using the normal delivery rules for broadcast and multicast, without regard to the frames matching the traffic filter, since other associated STAs my also want to receive these frames. Because a STA using TFS can be in power save mode for an extended period of time, broadcast and multicast frames that match the traffic filter can be delivered via the normal mechanisms, before the STA is aware that the traffic filter has been matched.  It is likely (but not guaranteed) that the STA does not receive those broadcast and multicast frames matching the traffic filter at the normal broadcast and multicast delivery time.  To prevent this from happening, the STA can request a notification frame be sent when requesting the establishment of the traffic filter.

A traffic filter is established using the TFS Request information element and the patterns to be matched specified in one or more enclosed TFS subelements.  For the traffic filter to match a frame, any TFS subelement can match the frame.  Using multiple TFS subelements in a TFS Request information element is the equivalent to a logical OR operation on the match conditions of each TFS subelement.  Using multiple TCLAS information elements in a TFS subelement is the equivalent to a logical AND operation on the match condition of each TCLAS information element.

##### 11.20.11.1 TFS non-AP STA operation

A STA supporting use of TFS shall indicate such support by setting the TFS bit in the Wireless Network Management Capability information element.  To use the TFS, the non-AP STA shall issue a MLME-TFS.request to send a TFS Request frame or shall issue a MLME-REASSOCIATE.request to send a Reassociation Request frame.  To use the TFS, either MLME request shall include a valid TFSRequest parameter with all TFS information elements that the AP  uses as triggers to set the STA’s TIM bit and optionally to send a notification to the non-AP STA. The TFS elements define the traffic filters the AP should use to set the STA’s TIM bit and optionally notify the non-AP STA.

The receipt of a MLME-TSF.confirm or MLME-REASSOCIATE.confirm with a valid TFSResponse parameter indicates to the STA that the AP has processed the corresponding TFS request.  The content of the TFSReponse parameter provides the status of each of the TFS information elements processed by the AP.

A non-AP STA may indicate that it is no longer using a particular TFS Element by transmitting a TFS Request frame without that TFS Element. The AP shall send a TFS Response frame with the Response Element Element Status field value set to Accept, upon receipt of the TFS Request.

A non-AP STA may choose to terminate use of the TFS service by sending a TFS Request frame with no TFS elements in the request thereby cancelling all traffic filters at the AP.

TFS Request frames may be sent by the non-AP STA at any time.

##### 11.20.11.2 TFS AP operation

An AP supporting use of TFS shall indicate such support by setting the TFS bit in the Wireless Network Management Capability information element.  When an AP receives a MLME-TFS.indication or MLME-REASSOCIATE.indication with a valid TSFRequest parameter, it shall establish one or more traffic filters for the requesting STA and issue a MLME-TFS.response with a TFSResponse parameter indicating the status of the associated request. When an AP establishes any filters for a requesting STA, the AP shall always establish a traffic filter that matches unicast EAPOL-Key messages addressed to the requesting STA, with bits 0 and 1 of the TFS Action Code field set to 0.

When TFS is enabled for an associated STA, the AP shall discard all unicast frames destined for the non-AP STA until a frame is found that matches one or more traffic filters established by the STA.  When a frame is found that matches one or more of the traffic filters enabled at  the STA (a matching frame), the AP shall perform the following actions, in order.

- If bit 1 of the TFS Action Code field is set for any of the traffic filters that matched the matching frame, a TFS Notify frame shall be sent to the STA.

- For an incoming unicast frame, the AP shall send the matching frame to the destination STA.

- If bit 0 of the TFS Action Code field is set for a traffic filter that matched the matching frame, the AP shall delete the traffic filter.

Note: Due to the operation of normal multicast frame delivery, a multicast frame that matches a traffic filter can result in the STA receiving indication of the multicast frame either before or after the multicast frame is transmitted by the AP, if the TFS Notify frame is queued in the STA’s power save queue.  This can result in the STA receiving the multicast frame in some cases and not receiving it in other cases.

The AP shall terminate any TFS operation for a STA when no traffic filters remain for a STA or when the AP receives an MLME-TFS.indication or MLME-REASSOCIATE.indication with a null TFSRequest.

### 11.20.12 Sleep Mode Procedure

##### 11.20.12.1 Sleep Mode Capability

An AP supporting Sleep Mode shall indicate its support by using Wireless Network Management Capability information element. A STA supporting use of Sleep Mode shall indicate it by using Wireless Network Management Capability information element.

##### 11.20.12.2 Sleep Mode non-AP STA operation

A STA supporting use of Sleep Mode shall indicate such support by setting the Sleep Mode bit in the Wireless Network Management Capability information element.  To use the Sleep Mode service, the non-AP STA shall issue a MLME-SLEEPMODE.request to send a Sleep Mode Request frame or shall issue a MLME-REASSOCIATE.request to send a Reassociation Request frame.  MLME request shall include a valid SleepMode parameter with a Sleep Mode element.  The Action Type field in the Sleep Mode element shall be set to “Enter Sleep Mode” (0x00) , and the Update Policy and Sleep Interval fields shall be included. MLME request may also include a valid TFSRequest parameter with all TFS information elements that the AP will use as triggers to set the STA’s TIM bit and optionally to send a notification to the non-AP STA.

The receipt of a MLME-SLEEPMODE.confirm or MLME-REASSOCIATE.confirm with a valid SleepMode parameter indicates to the STA that the AP has processed the corresponding Sleep Mode request frame.  The content of the Sleep Mode parameter in the Sleep Mode Response frame or Reassociation Request frame provides the status of Sleep Mode information elements processed by the AP.

While in Sleep Mode, the non-AP STA may not wake up every DTIM interval for broadcast/multicast packets, but wake up Sleep Interval. The non-AP STA may not participate in GTK/IGTK updates based on the GTK/IGTK Update Policy required by the AP.

To exit Sleep Mode, the non-AP STA shall issue a MLME-SLEEPMODE.request to send a Sleep Mode Request frame with an Action Type field in the Sleep Mode element set to “Exit Sleep Mode” (0x01).

If the non-AP STA receives a Sleep Mode Response Status of “Exit Sleep Mode Accept, GTK/IGTK update required” from the AP, the non-AP STA shall delete the GTKSA and IGTKSA.

##### 11.20.12.3 Sleep Mode AP operation

An AP supporting use of Sleep Mode shall indicate such support by setting the Sleep Mode bit in the Wireless Network Management Capability information element.  When an AP receives a MLME-SLEEPMODE.indication or MLME-REASSOCIATE.indication with a valid SleepMode parameter and an Action Type in the Sleep Mode element of “Enter Sleep Mode” (0x00), it shall make the selection on the Update Policy and Sleep Interval for the requesting STA and issue a MLME-SLEEPMODE.response or MLME-REASSOCIATE.response with a SleepMode parameter indicating the status of the request. The value of the Action Type field of the Sleep Mode element in the Sleep Mode Response frame or Reassociation Response frame shall be set to “Enter Sleep Mode” (0x00).

When Sleep Mode is enabled for an associated STA, the AP shall stop sending all directed management action frames to the non-AP STA and maintain the association for the Sleep Mode non-AP STA for the BSS Maximum Idle Time. An AP shall perform the TFS AP operation, as specified in 11.20.11 for non-AP STAs for which it has received TSF Request elements.  The AP shall set the TIM bit corresponding to the AID of the associated STA for which the AP has queued either a TFS Notify frame or matching frame.  In addition, an AP shall not perform GTK/IGTK updates for a non-AP STA, if the GTK/IGTK Update Policy bit  is set to 0.

When an AP receives a MLME-SLEEPMODE.indication with a valid SleepMode parameter and an Action Type in the Sleep Mode element of “Exit Sleep Mode” (0x01), the AP shall disable Sleep Mode service for the requesting STA, and issue a MLME-SLEEPMODE.response with a SleepMode parameter indicating the status of the associated request. If the GTK or IGTK which the non-AP STA currently holds is invalid (e.g. due to a suppressed GTK/IGTK update), the AP shall respond with Sleep Mode Response Status “Exit Sleep Mode Accept, GTK/IGTK update required”, and shall initiate a GTK/IGTK update to the non-AP STA.

Insert the following rows to A.4.14

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME13 RME13.2 RME13.3 RME13.4 | TFS TFS Request frame TFS Response frame TFS Notify frame | 11.20.11 7.4.11.16 7.4.11.17 7.4.11.18 | CFv:O M M CF1:M, CF2:O | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RME14 RME14.2 RME14.3 | Sleep Mode Sleep Mode Request frame Sleep Mode Response frame | 11.20.12 7.4.11.19 7.4.11.20 | CFv:O M CF1:M, CF2:O | Yes, No, N/A Yes, No, N/A Yes, No, N/A |

# Annex D

Insert the following entries at the end of the Dot11WirelessMgmtOptionsEntry sequence list:

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionPresenceEnabledTruthValue,

dot11MgmtOptionFBMSImplementedTruthValue,

dot11MgmtOptionFBMSEnabledTruthValue,

dot11MgmtOptionEventsEnabledTruthValue,

dot11MgmtOptionDiagnosticsEnabledTruthValue,

dot11MgmtOptionMultiBSSIDImplementedTruthValue,

dot11MgmtOptionMultiBSSIDEnabledTruthValue,

dot11MgmtOptionTFSImplementedTruthValue,

dot11MgmtOptionTFSEnabledTruthValue, dot11MgmtOptionSleepModeImplementedTruthValue,

dot11MgmtOptionSleepModeEnabledTruthValue }

On page 155, as part of Dot11WirelessMgmtOptionsEntry, add the followingMIB objects:

dot11MgmtOptionTFSImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting TFS when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 8 }

dot11MgmtOptionTFSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that TFS is enabled. TFS is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 9 }

dot11MgmtOptionSleepModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Sleep Mode when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 10 }

dot11MgmtOptionSleepModeEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that Sleep Mode is enabled. Sleep Mode is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 11 }