IEEE P802.11
Wireless LANs

| [Annex-B Comment Resolution] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dennis Ward | Sirius Satellite Radio | 27200 Haggerty Rd. Suite B-3 | +1 248 699-0442 | dward@siriusradio.com |

Abstract

Proposed resolutions for comments made regarding Annex-B

Amend P802.11.2 D1.0 to resolve the CID’s identified in the following text, from Letter Ballot 101:

CID 1670

Add the following text:

B2.18 Shielding Effectiveness (SE): The ratio of the signal received (from a transmitter) without the shield, to the signal received inside the shield; the insertion loss when the shield is placed between the transmitting antenna and the receiving antenna.

CID 1692

Add the following text:

B.3.9 Coaxial Cable Parameters

B.3.9.1 Characteristic Impedance (Zo)

The Coaxial cable should have the same Zo as that used in the system design.  WLAN systems are typically 50 , but unique designs that do not have an external port may use some impedance other than 50 .

B.3.9.2 Loss Characteristics

The insertion loss per unit length of the cable should be measured for the frequency of interest, and should be as low a value as possible.  The loss should not exceed a value that would impair a parameter from being measured to within the required accuracy.

CID 1597

Change:

B.3.1.6 Third-order intermodulation distortion

To Read:

B.3.1.6 Third-order intermodulation distortion (IMD3)

Change references from IMD to IMD3.

CID 1099

Change “1dB” in Section B.3.3.2 to “0.5dB”

CID 1354 and CID 967

Change “100” in Section B.3.2.2 to “10 degrees”

CID 29

Change the text in B.3.6.1:  The device should have a highest operating frequency of at least 3 GHz when testing 2.4 GHz devices, and 6 GHz when testing 5 GHz devices.

To:  The device should have a highest operating frequency of at least 3 GHz when testing 2.4 GHz devices, and 6 GHz when testing 5 GHz devices.  The device minimum operating frequency should be 2 GHz or lower.

Change “3dB” in Section B.3.6.4 to “0.5dB”

CID 1355

Add the following text:

B.3.6.5 Coupling and Directivity

The device should have a coupling and directivity of 20dB or greater.

CID 1708

Add the following text

B.3.8.4 Accuracy

The device shall have attenuation accuracy greater than or equal to +/- 1dB for all attenuation values.

References:

IEEE Standard 299-1997 Method for Measuring the Effectiveness of Electromagnetic

Shielding Enclosures